## Exercise specification: Avionics-style Discrete I/O on IMXRT1050-EVKB (WOW + FLT/GRD + BIT)

### 1) Exercise title

“Discrete I/O Fundamentals Lab: Robust WOW input + Open-Drain FLT/GRD output with diagnostics (BIT) on EVKB-i.MX RT1050.”

### 2) Why this lab exists (tie to theory)

This exercise turns the theory topics into a single, testable implementation on the EVKB:

- GPIO as binary discretes (asserted / de-asserted) used in avionics (e.g., WOW, FLT/GRD, door closed).

- Correct pad electrical configuration: pull-ups/pull-downs, Schmitt trigger (hysteresis), slew/drive, open-drain vs push-pull.

- Deterministic debounce / glitch rejection, edge timestamping, and Built-In Test (BIT) for stuck-at faults.

### 3) Learning objectives (what you must demonstrate)

By the end, the learner can:

- Configure a GPIO input discrete with defined default state (pull) and noise immunity (hysteresis + software filtering).

- Implement a robust discrete truth decision using time debounce and majority voting (e.g., 3-of-5).

- Configure a GPIO output discrete as open-drain and demonstrate wired-OR behavior across boards.

- Add avionics-style diagnostics: edge timestamp log + stuck-at detection (BIT).

### 4) Hardware & tools required

Minimum (single-board variant)

- 1× IMXRT1050-EVKB, USB cable (DAP-Link/OpenSDA).

- MCUXpresso IDE + the provided NXP SDK.

- Terminal program (MCUXpresso Console, PuTTY, TeraTerm, etc.)

Recommended (multi-board avionics simulation)

- 2–3× EVKB boards (you have 4 → perfect for multi-LRU simulation).

- 5–10 jumper wires.

- 1× resistor for pull-up on shared line (e.g., 4.7 kΩ to 3.3 V) for wired-OR demo.

- (Optional) oscilloscope/logic analyzer for edge/noise observation.

Safety / electrical constraint

- This lab is 3.3 V LVCMOS only; do not connect aircraft 28 V discretes directly (in real avionics you’d use isolation / protection networks).

### 5) Discrete definitions in this lab (signals + polarity)

You will implement two discretes:

#### A) WOW discrete (input)

- Meaning: “Weight-On-Wheels asserted = aircraft on ground”

- Lab source: EVKB USER button (SW8) used as the physical input stand-in.

- Logical polarity: define WOW_ASSERTED = 1 in software.

  - If the board button is active-low, translate raw pin reading into logical WOW.

- Filtering requirement:

  - Debounce window: 5 ms stable before accepting a state change.

  - Glitch rejection: ignore pulses shorter than 1 ms (simulated noise).

#### B) FLT/GRD discrete (output)

- Meaning: shared “GROUND” status line

- Electrical mode: open-drain (wired-OR capable)

- Polarity: asserted = line pulled LOW (active-low line)

- Fail-safe intention: if a unit resets, it should release the line (input pull-up brings it HIGH).

These choices directly map to the theory guidance: pull + hysteresis for inputs; open-drain for shared lines.

### 6) Software architecture requirements (bare-metal)

Your application must be structured into these blocks:

- Board init
BOARD_InitBootPins(), BOARD_InitBootClocks(), BOARD_InitDebugConsole()

- Discrete I/O layer

  - wow_raw_read() → reads raw GPIO pin

  - wow_filter_update() → debounce + voting (produces WOW_TRUTH)

  - fltgrd_set_open_drain(assert) → asserts/releases open-drain output

- Diagnostics layer (BIT + log)

  - Edge timestamp on every accepted state change (use SysTick or GPT)

  - Stuck-at BIT checks:

    - If WOW raw stays constant for > N seconds while user is actively toggling → flag “possible stuck input”

    - If output is commanded assert/release but the monitored line does not change (in multi-board variant) → flag “possible stuck output / wiring fault”

- Main control logic

  - If WOW_TRUTH == 1 (ground) → assert FLT/GRD (pull low)

  - Else release FLT/GRD (line high via pull-up)

  - Periodically print a status line (raw, filtered, BIT flags)

### 7) Pad / electrical configuration requirements (what you must explicitly set or verify)

You must ensure the following pad behaviors are present (either via Pins Tool generated pin_mux.c or via IOMUXC_SetPinConfig() overrides):

WOW input pad

- Pull enabled (pull-up if active-low, pull-down if active-high)

- Hysteresis (Schmitt trigger) enabled for noise immunity

- Input is not left floating

FLT/GRD output pad

- Open-drain enabled

- Conservative drive strength / slew (use “as low/slow as practical” consistent with correct operation)

This is a graded requirement: your final submission must include a short note stating which pad options are enabled and where (Pins Tool screenshot or code comment).

### 8) Two execution variants (choose depending on available wiring time)

#### Variant 1 — Single-board (core fundamentals)

- Use SW8 to simulate WOW.

- Drive on-board LED as an indicator of WOW_TRUTH (not wired-OR, but still validates filtering + polarity + determinism).

- Console prints raw and filtered state + timestamps + BIT counters.

Pass criteria

- One press → exactly one accepted edge (no multiple toggles).

- Injected “fast tapping” does not cause spurious truth flips (debounce works).

- Console shows timestamped transitions and no random flips while idle.

#### Variant 2 — Multi-board avionics simulation (recommended with your 4 boards)

Simulate multiple LRUs sharing a discrete line:

- Board A (LRU-A): reads its WOW source (SW8), computes WOW_TRUTH, drives open-drain FLT/GRD.

- Board B (LRU-B): same behavior (independent).

- Board C (Monitor/CMC): reads the shared FLT/GRD line as an input and logs:

  - “LINE LOW because at least one LRU asserted”

  - timestamps + which boards are asserting (optional if you add per-board debug prints via USB serial)

Wiring

- Tie grounds together across boards.

- Create one shared node “FLT/GRD_LINE”.

- Pull up FLT/GRD_LINE to 3.3 V through a resistor (e.g., 4.7 kΩ).

- Connect Board A open-drain output pin to FLT/GRD_LINE.

- Connect Board B open-drain output pin to FLT/GRD_LINE.

- Connect Board C input pin to FLT/GRD_LINE.

Pass criteria (wired-OR)

- If either board asserts → line is LOW.

- Line becomes HIGH only when both release.

- Resetting one board must not prevent the other from asserting the line (fault containment concept).

This demonstrates the “wired-OR / open-drain discrete sharing” concept from the theory.

### 9) MCUXpresso IDE workflow requirements (how you must build it)

You must implement the lab by starting from an SDK example and then modifying it:

- Import an SDK GPIO example

  - In MCUXpresso: Import SDK example(s)…

  - Select board: evkbimxrt1050

  - Choose one of:

    - driver_examples/gpio/input_interrupt (best starting point)

    - driver_examples/gpio/led (if you want to merge later)

- Build and run the baseline example

  - Confirm the console prints and button interrupt triggers.

- Add your discrete layer + filtering + BIT

  - Modify only what’s needed; keep the SDK startup/board init.

- (For multi-board) Create 2–3 projects

  - Same codebase with compile-time role selection:

    - ROLE_LRU_A, ROLE_LRU_B, ROLE_MONITOR

- Demonstrate via Debug Console

  - Provide a log snippet showing:

    - raw transitions

    - filtered transitions

    - timestamps

    - BIT fault flags (even if “none”)

### 10) Exercise completion requires

- Firmware project(s) (MCUXpresso workspace or exported archive)

- Short configuration note covering:

  - pin mapping used

  - pad settings enabled (pull, hysteresis, open-drain, slew/drive choice)

  - debounce/voting parameters chosen and why

- Test evidence

  - console log (copy/paste) for:

    - normal operation

    - one induced fault case (e.g., forced stuck input by holding button or tying line low in multi-board)