## Exercise specification: Discrete I/O Software Debouncing on IMXRT1050-EVKB (PIT 1 kHz integrator)

This exercise implements deterministic software debouncing for a noisy mechanical discrete input on the EVKB-i.MXRT1050, using a fixed-rate timer tick (PIT) and a saturating up/down counter (“integrator”) debouncer. It maps directly to the “Discrete IO Fundamentals — Software Debouncing” theory (determinism, monotonicity, bounded latency, configurability, observability/BIT).

### 1) Goal (what you will build)

You will build a bare-metal firmware that:

- Samples a raw GPIO discrete at a fixed 1 ms rate using PIT.

- Converts raw samples into a stable debounced state using an integrator algorithm.

- Produces well-defined debounced edges: exactly one rising edge per real press, one falling edge per real release (monotonicity).

- Demonstrates both:

  - Generic example: a panel button toggles an LED exactly once per press.

  - Avionics example: WOW discrete qualifies ground-spoiler deploy only after ≥20 ms stable assertion.

### 2) Hardware / tools

- 1× IMXRT1050-EVKB

- MCUXpresso IDE

- Your uploaded NXP SDK

- Terminal (MCUXpresso console / PuTTY / TeraTerm)

Signals used (per the theory doc board definitions):

- SW8: GPIO5_IO00 (user button)

- USER LED: GPIO1_IO09

### 3) SDK starting point (must use an existing SDK example)

Start from the SDK PIT driver example present in your SDK:

- boards/evkbimxrt1050/driver_examples/pit/ (base project)

(You will add GPIO usage + the debouncer on top of it. This follows the theory recommendation to use PIT for fixed-rate sampling.)

### 4) Debouncing requirements (functional + timing)

#### Sampling

- Sampling period Ts = 1 ms (1 kHz PIT interrupt).

- All debouncing logic updates must execute in bounded, constant time per tick (no printing inside ISR).

#### Debouncer strategy: integrator (required)

Per input, maintain:

- count in range [0..MAX] (saturating)

- state (debounced) in {0,1}

- rose, fell edge flags (latched once per change)

Update rule every tick (required behavior):

- if raw==1 → count++ until MAX

- if raw==0 → count-- until 0

- commit state=1 only when count==MAX

- commit state=0 only when count==0

- set rose on 0→1, set fell on 1→0

#### Parameter sets (you must support both)

- Generic debounce: MAX = 5 → ~5 ms qualification (typical button bounce immunity).

- Avionics WOW qualify: MAX = 20 → ≥20 ms stable before “on-ground” becomes true (spoiler safety gate).

Latency requirement: Worst-case added edge-to-decision latency ≈ MAX * Ts. You must print this computed value at startup.

### 5) Application behavior requirements

#### A) Generic example behavior (toggle exactly once)

- A press/release sequence on SW8 must cause:

  - Exactly one DEBOUNCED RISE message (on press) and

  - Exactly one DEBOUNCED FALL message (on release).

- The USER LED must toggle exactly once per press (or be driven by debounced state—either is acceptable, but you must state which).

#### B) Avionics scenario behavior (WOW → spoiler logic)

Treat SW8 as WOW:

- Pressed → WOW raw asserted (map polarity in software if needed)

- Released → WOW raw deasserted

Spoiler deploy logic (software simulation):

- On debounced rising edge (WOW becomes 1) → set SPOILER_CMD = DEPLOY

- While WOW debounced state is 1 → keep SPOILER_CMD active

- When WOW debounced state returns to 0 → SPOILER_CMD = INHIBIT

Visual indication (required):

- During qualification window (i.e., while 0 < count < MAX for the WOW assert transition), blink USER LED at 2 Hz to show “validating.”

- When WOW debounced is solid 1, LED must be solid ON.

- When debounced is 0, LED must be OFF.

Logging (required):

- Every debounced edge prints: timestamp (ms), raw, count, debounced state, edge type.

- Periodic status line every 500 ms: timestamp, raw, count, debounced state.

### 6) Observability + BIT (Built-In Test) requirements

Implement and expose via console:

- raw_edge_count (counts raw transitions detected)

- debounced_edge_count (counts committed edges)

- max_count_reached_events (how often it actually qualified)

- stuck_input_warning: if raw never changes for 30 s, flag a warning (maintenance observability)

Rule of thumb expected in healthy behavior:

- raw_edge_count >= debounced_edge_count (often much higher due to bounce)

### 7) MCUXpresso IDE steps (what the learner must do)

- Import the SDK example:

  - File → Import → MCUXpresso IDE → Import SDK example(s)…

  - Board: evkbimxrt1050

  - Example: driver_examples → pit

- Add new module files:

  - debouncer.h (and optionally debouncer.c) implementing the integrator API.

- Modify pit.c (or the example’s main file) to:

  - Initialize board + debug console

  - Initialize GPIO for SW8 + LED

  - Configure PIT to 1 kHz and enable ISR

  - In main loop: consume “tick” flag, read raw, call debouncer update, apply application logic, print logs

- Build, flash, run; verify output in console.

### 8) Verification checklist (pass/fail)

Pass if all are true:

- Determinism

- Tick period is fixed at 1 ms (PIT); debouncer update runs once per tick.

- Monotonicity

- One physical press → at most one DEBOUNCED RISE

- One physical release → at most one DEBOUNCED FALL

- Qualification timing

- With MAX=20, WOW debounced must not assert earlier than ~20 ms after a true press (within 1–2 ticks tolerance).

- Avionics behavior

- LED blinks at 2 Hz during qualification; solid ON only when WOW is fully qualified.

- Observability/BIT

- Counters and stuck warning are visible and make sense during testing.

### 9) Deliverables

- MCUXpresso project (or exported archive)

- Console log showing:

  - MAX=5 behavior for 5 presses

  - MAX=20 behavior for 3 presses (show qualification delay + 2 Hz blink)

- Short note (½ page) stating:

  - chosen polarity mapping for SW8 raw

  - why Ts=1 ms and MAX values meet the intended bounce/qualification requirement