# Discrete I/O Fundamentals — Software Debouncing Lab (EVKB-i.MX RT1050)

This is the complete, executable solution for the exercise “Discrete IO Fundamentals — Software Debouncing” on the IMXRT1050-EVKB using the MCUXpresso SDK you uploaded.

It implements a deterministic, fixed-rate (1 ms) sampler using PIT and a saturating up/down counter (“integrator”) debouncer.

You will build two configurations from the same project: - GENERIC: MAX=5 (≈5 ms). One press → exactly one toggle. - AVIONICS (WOW qualify): MAX=20 (≈20 ms). Adds “validating” blink + spoiler command simulation.

## 1) Start from an existing SDK example (required)

### Import

- MCUXpresso IDE → File → Import… → MCUXpresso IDE → Import SDK example(s)…

- Board: evkbimxrt1050

- Example: driver_examples → pit

- Finish.

This creates a project whose main file is: - boards/evkbimxrt1050/driver_examples/pit/pit.c

### What you will change

You will: 1) Add two new files: debouncer.h and debouncer.c 2) Replace the content of pit.c with the lab version below

## 2) Board I/O used

### Input discrete: SW8

- SW8 is the EVKB user button on SNVS_WAKEUP, muxed as GPIO5_IO00.

- In this lab, we define RAW_LOGIC = 1 when pressed.

  - Default assumption: active-low button (pressed reads 0 on pad). You can flip a macro if your observation differs.

### Output discrete: USER LED

- EVKB USER LED is GPIO1_IO09 (already supported via USER_LED_* macros in the imported example).

## 3) Debouncing algorithm (integrator) — what this solution implements

Per tick (1 ms): - If raw==1 → count++ (saturate at MAX) - If raw==0 → count-- (saturate at 0) - state becomes 1 only when count==MAX - state becomes 0 only when count==0 - Generate edge flags rose / fell on state transitions

This produces monotonic, bounce-immune edges and a bounded qualification latency ≈ MAX * 1ms.

## 4) Build configurations (two required demos)

You will build two variants from the same project by changing preprocessor symbols.

### A) GENERIC (MAX=5)

- Purpose: show typical button debounce and exactly one action per press

- Behavior:

  - On debounced rising edge (press qualified), toggle USER LED once

### B) AVIONICS (MAX=20)

- Purpose: simulate an avionics discrete WOW qualification

- Behavior:

  - While qualifying assertion (debounced state=0 and 0<count<MAX): blink LED at 2 Hz

  - When debounced WOW=1: LED solid ON and SPOILER_CMD=DEPLOY

  - When debounced WOW=0: LED OFF and SPOILER_CMD=INHIBIT

### How to set symbols in MCUXpresso

Project → Properties → C/C++ Build → Settings → Tool Settings → MCU C Compiler → Preprocessor - For GENERIC: - Ensure no LAB_MODE_AVIONICS is defined (or define LAB_MODE_AVIONICS=0) - For AVIONICS: - Add: LAB_MODE_AVIONICS=1

Optional: If your SW8 polarity is opposite, add/override: - SW8_ACTIVE_LOW=0

## 5) Files to add

Create these two files under the project (e.g., in the same folder as pit.c).

### 5.1) debouncer.h

#ifndef DEBOUNCER_H_
#define DEBOUNCER_H_

#include <stdint.h>
#include <stdbool.h>

typedef struct
{
    uint8_t max;      /* qualification threshold */
    uint8_t count;    /* saturating integrator counter [0..max] */
    uint8_t state;    /* debounced state 0/1 */
    uint8_t rose;     /* latched edge flag */
    uint8_t fell;     /* latched edge flag */
} debouncer_t;

/* Initialize with MAX and an initial raw sample (0/1). */
void Debouncer_Init(debouncer_t *d, uint8_t max, uint8_t initial_raw);

/* Update once per sampling tick with raw sample (0/1). */
void Debouncer_Update(debouncer_t *d, uint8_t raw);

/* Edge flag consumers (return-and-clear). */
bool Debouncer_Rose(debouncer_t *d);
bool Debouncer_Fell(debouncer_t *d);

static inline uint8_t Debouncer_State(const debouncer_t *d) { return d->state; }

#endif /* DEBOUNCER_H_ */

### 5.2) debouncer.c

#include "debouncer.h"

void Debouncer_Init(debouncer_t *d, uint8_t max, uint8_t initial_raw)
{
    d->max = max;
    d->rose = 0u;
    d->fell = 0u;

    if (initial_raw)
    {
        d->count = max;
        d->state = 1u;
    }
    else
    {
        d->count = 0u;
        d->state = 0u;
    }
}

void Debouncer_Update(debouncer_t *d, uint8_t raw)
{
    d->rose = 0u;
    d->fell = 0u;

    /* Saturating integrator */
    if (raw)
    {
        if (d->count < d->max)
        {
            d->count++;
        }
    }
    else
    {
        if (d->count > 0u)
        {
            d->count--;
        }
    }

    /* Commit state only at the extremes */
    if ((d->count == d->max) && (d->state == 0u))
    {
        d->state = 1u;
        d->rose = 1u;
    }
    else if ((d->count == 0u) && (d->state == 1u))
    {
        d->state = 0u;
        d->fell = 1u;
    }
}

bool Debouncer_Rose(debouncer_t *d)
{
    bool v = (d->rose != 0u);
    d->rose = 0u;
    return v;
}

bool Debouncer_Fell(debouncer_t *d)
{
    bool v = (d->fell != 0u);
    d->fell = 0u;
    return v;
}

## 6) Replace pit.c with the lab implementation

Open the imported project’s pit.c and replace it completely with the code below.

### pit.c (full replacement)

/*
 * Discrete IO Fundamentals — Software Debouncing (EVKB-i.MX RT1050)
 *
 * Base example: driver_examples/pit
 *
 * Sampling: PIT @ 1kHz (1ms)
 * Debouncer: saturating up/down counter (integrator)
 *
 * Build modes:
 *   - Default (GENERIC): MAX=5ms, toggle LED once per press
 *   - Avionics (WOW): define LAB_MODE_AVIONICS=1, MAX=20ms, validating blink + spoiler simulation
 *
 * If SW8 polarity is opposite, define SW8_ACTIVE_LOW=0.
 */

#include <stdbool.h>
#include <stdint.h>

#include "fsl_debug_console.h"
#include "fsl_common.h"
#include "fsl_clock.h"
#include "fsl_gpio.h"
#include "fsl_iomuxc.h"
#include "fsl_pit.h"

#include "board.h"
#include "app.h"

#include "debouncer.h"

/* ------------------------- Build-time knobs ------------------------- */
#ifndef LAB_MODE_AVIONICS
#define LAB_MODE_AVIONICS (0)
#endif

#ifndef SW8_ACTIVE_LOW
#define SW8_ACTIVE_LOW (1) /* default on EVKB: pressed reads 0 */
#endif

/* PIT sampling */
#define SAMPLE_PERIOD_US (1000u) /* 1ms */

#if (LAB_MODE_AVIONICS != 0)

#define DEBOUNCE_MAX (200u)

#else

#define DEBOUNCE_MAX (5u)

#endif

#if (LAB_MODE_AVIONICS != 0)

#define AVIONICS_BLINK_TOGGLE_MS (50u)   /* demo-only: toggles every 50ms */

#endif

/* BIT / observability */
#define STUCK_WARN_MS (30000u)
#define STATUS_PRINT_MS (500u)

/* SW8: SNVS_WAKEUP -> GPIO5_IO00 */
#define SW8_GPIO GPIO5
#define SW8_PIN  (0u)
#define SW8_MUX  IOMUXC_SNVS_WAKEUP_GPIO5_IO00

/* ------------------------- PIT tick state ------------------------- */
static volatile uint32_t g_pitTicks = 0u;

void PIT_LED_HANDLER(void)
{
    PIT_ClearStatusFlags(DEMO_PIT_BASEADDR, DEMO_PIT_CHANNEL, kPIT_TimerFlag);
    g_pitTicks++; /* 1 tick == 1ms */
    SDK_ISR_EXIT_BARRIER;
}

static inline uint32_t ticks_ms(void)
{
    return g_pitTicks;
}

/* ------------------------- SW8 pad setup (SNVS) ------------------------- */
static void SW8_InitPadAndMux(void)
{
    /* Enable IOMUXC clocks (both domains) */
    CLOCK_EnableClock(kCLOCK_Iomuxc);
    CLOCK_EnableClock(kCLOCK_IomuxcSnvs);

    /* Mux SNVS_WAKEUP to GPIO5_IO00 */
    IOMUXC_SetPinMux(SW8_MUX, 0u);

    /* Configure SNVS pad:
     * - enable pull/keeper
     * - select pull-up
     * - enable hysteresis
     * - slow slew
     */
    uint32_t cfg = 0u;

    cfg |= IOMUXC_SNVS_SW_PAD_CTL_PAD_WAKEUP_PKE_MASK;
    cfg |= IOMUXC_SNVS_SW_PAD_CTL_PAD_WAKEUP_PUE_MASK; /* pull enable */
    cfg |= IOMUXC_SNVS_SW_PAD_CTL_PAD_WAKEUP_PUS(2u);  /* 100K pull-up typical */
    cfg |= IOMUXC_SNVS_SW_PAD_CTL_PAD_WAKEUP_HYS_MASK;
    cfg |= IOMUXC_SNVS_SW_PAD_CTL_PAD_WAKEUP_SRE(0u);  /* slow slew */

    /* Write the pad control register */
    IOMUXC_SNVS->SW_PAD_CTL_PAD_WAKEUP = cfg;
}

/* ------------------------- Raw read (logical) ------------------------- */
static uint8_t SW8_ReadLogical(void)
{
    /* Read pad status so we see the true input level */
    uint8_t pad = GPIO_PinReadPadStatus(SW8_GPIO, SW8_PIN);

#if (SW8_ACTIVE_LOW != 0)
    /* pressed -> 0 -> logical 1 */
    return (pad == 0u) ? 1u : 0u;
#else
    return (pad != 0u) ? 1u : 0u;
#endif
}

/* ------------------------- Application (avionics simulation) ------------------------- */
#if (LAB_MODE_AVIONICS != 0)
typedef enum
{
    SPOILER_INHIBIT = 0,
    SPOILER_DEPLOY  = 1
} spoiler_cmd_t;
#endif

int main(void)
{
    pit_config_t pitConfig;

    /* Board init from the imported PIT example */
    BOARD_InitHardware();

        /* Enable clocks for GPIO blocks used */
    CLOCK_EnableClock(kCLOCK_Gpio5);

/* Enable LED (GPIO1_IO09) */
    LED_INIT();



    /* Ensure SW8 is properly muxed and has pull/hysteresis */
    SW8_InitPadAndMux();

    /* Init SW8 as digital input */
    gpio_pin_config_t sw8_in_cfg = {
        .direction = kGPIO_DigitalInput,
        .outputLogic = 0u,
        .interruptMode = kGPIO_NoIntmode,
    };
    GPIO_PinInit(SW8_GPIO, SW8_PIN, &sw8_in_cfg);

    /* Init PIT @ 1ms */
    PIT_GetDefaultConfig(&pitConfig);
    PIT_Init(DEMO_PIT_BASEADDR, &pitConfig);
    PIT_SetTimerPeriod(DEMO_PIT_BASEADDR, DEMO_PIT_CHANNEL,
                       USEC_TO_COUNT(SAMPLE_PERIOD_US, PIT_SOURCE_CLOCK));
    PIT_EnableInterrupts(DEMO_PIT_BASEADDR, DEMO_PIT_CHANNEL, kPIT_TimerInterruptEnable);
    EnableIRQ(PIT_IRQ_ID);
    PIT_StartTimer(DEMO_PIT_BASEADDR, DEMO_PIT_CHANNEL);

    /* Debouncer init */
    debouncer_t db;
    uint8_t raw0 = SW8_ReadLogical();
    Debouncer_Init(&db, (uint8_t)DEBOUNCE_MAX, raw0);

    /* Observability counters */
    uint32_t raw_edge_count = 0u;
    uint32_t debounced_edge_count = 0u;
    uint32_t max_count_reached_events = 0u;
    uint32_t last_raw_change_ms = 0u;

    uint8_t last_raw = raw0;
    uint32_t last_status_ms = 0u;

#if (LAB_MODE_AVIONICS != 0)
    spoiler_cmd_t spoiler = SPOILER_INHIBIT;
#endif

    PRINTF("\r\n=== Software Debouncing Lab (PIT 1kHz) ===\r\n");
#if (LAB_MODE_AVIONICS != 0)
    PRINTF("Mode: AVIONICS (WOW qualify)\r\n");
#else
    PRINTF("Mode: GENERIC (button debounce)\r\n");
#endif
    PRINTF("Sample period: %u us (1 tick = 1 ms)\r\n", (unsigned)SAMPLE_PERIOD_US);
    PRINTF("Debounce MAX: %u ticks => worst-case latency ~%u ms\r\n\r\n",
           (unsigned)DEBOUNCE_MAX, (unsigned)DEBOUNCE_MAX);

    /* Main loop processes every PIT tick (catch up if needed) */
    uint32_t processed = ticks_ms();

    while (true)
    {
        uint32_t nowTicks = ticks_ms();

        while (processed != nowTicks)
        {
            processed++; /* advance logical time by 1ms */
            uint32_t t_ms = processed;

            /* Sample raw */
            uint8_t raw = SW8_ReadLogical();

            /* Raw edge counter */
            if (raw != last_raw)
            {
                raw_edge_count++;
                last_raw_change_ms = t_ms;
                last_raw = raw;
            }

            /* Update debouncer */
            uint8_t prev_state = Debouncer_State(&db);
            Debouncer_Update(&db, raw);

            /* Count qualification events (state becomes 1 at MAX) */
            if ((prev_state == 0u) && (Debouncer_State(&db) == 1u))
            {
                max_count_reached_events++;
            }

            /* Consume edges (debounced) */
            if (Debouncer_Rose(&db))
            {
                debounced_edge_count++;
                PRINTF("[%8lu ms] DEBOUNCED RISE  raw=%u count=%u state=%u\r\n",
                       (unsigned long)t_ms, raw, db.count, db.state);

#if (LAB_MODE_AVIONICS != 0)
                /* WOW asserted -> deploy spoilers */
                if (spoiler != SPOILER_DEPLOY)
                {
                    spoiler = SPOILER_DEPLOY;
                    PRINTF("[%u ms] SPOILER_CMD=DEPLOY\r\n", (unsigned long)t_ms);
                }
#else
                /* Generic: toggle LED exactly once per press */
                LED_TOGGLE();
#endif
            }

            if (Debouncer_Fell(&db))
            {
                debounced_edge_count++;
                PRINTF("[%u ms] DEBOUNCED FALL  raw=%u count=%u state=%u\r\n",
                       (unsigned long)t_ms, raw, db.count, db.state);

#if (LAB_MODE_AVIONICS != 0)
                /* WOW deasserted -> inhibit spoilers */
                if (spoiler != SPOILER_INHIBIT)
                {
                    spoiler = SPOILER_INHIBIT;
                    PRINTF("[%u ms] SPOILER_CMD=INHIBIT\r\n", (unsigned long)t_ms);
                }
#endif
            }

#if (LAB_MODE_AVIONICS != 0)
            /* LED behavior for avionics mode:
             * - validating assertion: state=0 and 0<count<MAX  => blink 2 Hz
             * - qualified asserted: state=1 => ON
             * - deasserted: state=0 and count==0 => OFF
             */
            if ((db.state == 0u) && (db.count > 0u) && (db.count < db.max))
            {
                /* 2 Hz blink: toggle every 250ms */
          bool on = ((t_ms / AVIONICS_BLINK_TOGGLE_MS) & 1u) ? true : false;
                if (on)
                {
                    USER_LED_ON();
                }
                else
                {
                    USER_LED_OFF();
                }
            }
            else if (db.state != 0u)
            {
                USER_LED_ON();
            }
            else
            {
                USER_LED_OFF();
            }
#endif

            /* Periodic status line (every 500ms) */
            if ((t_ms - last_status_ms) >= STATUS_PRINT_MS)
            {
                last_status_ms = t_ms;

                bool stuck_warn = ((t_ms - last_raw_change_ms) >= STUCK_WARN_MS);

                PRINTF("[%u ms] STATUS raw=%u count=%u state=%u  raw_edges=%u deb_edges=%u max_events=%u stuck_warn=%u\r\n",
                       (unsigned long)t_ms,
                       raw,
                       db.count,
                       db.state,
                       (unsigned long)raw_edge_count,
                       (unsigned long)debounced_edge_count,
                       (unsigned long)max_count_reached_events,
                       stuck_warn ? 1u : 0u);
            }
        }

       
        __NOP();
    }
}

## 7) Expected console behavior (what “correct” looks like)

### GENERIC (MAX=5)

- A single press produces exactly one line:

  - DEBOUNCED RISE ...

- A single release produces exactly one line:

  - DEBOUNCED FALL ...

- The LED toggles exactly once per press (bounce doesn’t cause extra toggles)

### AVIONICS (MAX=20)

- When you press SW8:

  - LED blinks at 2 Hz for ~20 ms while qualifying

  - Then you get DEBOUNCED RISE and SPOILER_CMD=DEPLOY

  - LED becomes solid ON

- When you release SW8:

  - You get DEBOUNCED FALL and SPOILER_CMD=INHIBIT

  - LED turns OFF

## 8) Troubleshooting

- Nothing prints / no PIT ticks

- Ensure the PIT interrupt is enabled (EnableIRQ(PIT_IRQ_ID)) and your project still links the ISR with the correct name (PIT_LED_HANDLER).

- SW8 always reads 0 or always reads 1

- Try toggling polarity: set SW8_ACTIVE_LOW=0.

- Confirm SW8_InitPadAndMux() is called before reading.

- Debounced edges never occur

- Confirm DEBOUNCE_MAX is not too large for your test.

- In AVIONICS mode (MAX=20), hold the button down at least 50–100 ms to observe qualification.

## 9) What you should observe and reason

- Two console logs:

  - GENERIC mode: 5 press/release cycles

  - AVIONICS mode: 3 press/release cycles showing ~20 ms qualification and spoiler commands

- Reason on:

  - sampling period

  - MAX values and implied latency

  - why integrator debouncing is deterministic and bounce-immune