## Exercise specification: Discrete I/O Driver — Interrupt-driven (GPIO IRQ + PIT debounce + timestamped events) on IMXRT1050-EVKB

This lab implements an avionics-style interrupt-driven discrete I/O driver with edge selection, PIT-based debounce/deglitch, safe output defaults, and an event callback + ring buffer with timestamps, then validates timing/latency and fault behavior.

## 1) Learning objectives

By the end you will be able to:

- Configure i.MX RT1050 GPIO inputs/outputs with IOMUXC + pad control and enable edge interrupts using MCUXpresso SDK (fsl_gpio.h).

- Design a reusable DIO driver supporting:

  - Rising / falling / both edge selection

  - Debounce/deglitch using PIT timing

  - Safe power-up output state

  - Timestamped events via callback + event ring buffer

- Measure and report latency and validate fault behavior (bounce, chatter, wire-off/floating).

## 2) Board I/O mapping (EVKB-i.MXRT1050)

Use the SDK board aliases exactly as the theory specifies:

- SW8 (input discrete): GPIO5 pin 0

  - SDK aliases: BOARD_USER_BUTTON_GPIO, BOARD_USER_BUTTON_GPIO_PIN, BOARD_USER_BUTTON_IRQ, BOARD_USER_BUTTON_IRQ_HANDLER

  - IRQ line is on the combined GPIO IRQ for the port (GPIO5 combined).

- User LED D9 (output discrete): GPIO1 pin 9

  - SDK aliases: BOARD_USER_LED_GPIO, BOARD_USER_LED_GPIO_PIN

  - LED is typically inverted on EVKB; rely on board macros for correct logic.

## 3) Starting point (must use SDK example)

Start from an existing SDK example in your uploaded SDK:

- boards/evkbimxrt1050/driver_examples/gpio/input_interrupt (present in your SDK zip)

You will extend it with PIT-based debounce + driver files.

## 4) Driver architecture requirements (what you must implement)

Implement the driver as reusable modules, not “all in main”.

### 4.1 dio_irq.h/.c (Discrete Input + Output API)

Per input channel configuration must include:

- GPIO_Type *base; uint32_t pin;

- bool active_high; (polarity mapping to logical level)

- edge_mode = rising/falling/both

- debounce_us (0 = no debounce)

- callback(channel, new_level, timestamp_us)

- internal runtime state:

  - irq_armed (set in GPIO ISR)

  - t_irq_us timestamp of last edge

  - last_debounced_level

Output API must include:

- dio_output_init(... safe_level ...) called at startup before enabling interrupts

- dio_write()/dio_read() with polarity handling

### 4.2 dio_timebase.h/.c (PIT timebase)

Use PIT to implement:

- uint32_t dio_timebase_get_us() for timestamps

- a periodic mechanism to support debounce windows (e.g., a free-running microsecond counter or a 1ms base).

### 4.3 eventq.h/.c (Ring buffer)

Implement a fixed-size ring buffer for debounced events:

- event record: {channel_id, logical_level, timestamp_us, reason}

- overflow policy: drop newest or drop oldest (choose one and document)

- consumer runs in main loop / low-rate task (never in ISR).

## 5) ISR hygiene rules (strict)

To match avionics best practice from the theory:

- GPIO ISR must be short:

  - read interrupt flags

  - clear the flag(s) immediately

  - record timestamp + set irq_armed

  - exit with SDK_ISR_EXIT_BARRIER

- No PRINTF in ISR

- Debounce validation + callback dispatch must happen outside the GPIO ISR (deferred handling).

## 6) Exercise tasks

### Task A — Basic interrupt capture (no debounce yet)

Goal: Prove you can configure GPIO interrupts correctly.

Implementation:

- Configure SW8 for both-edge interrupts.

- In GPIO ISR: clear flags, record t_irq_us, set a “raw edge count”.

- In main loop: when edge flag seen, read pin level and print:

  - RAW EDGE @ <us> PAD=<0/1>

Pass criteria:

- Press/release generates edges.

- You observe multiple raw edges during one press if you “tap” (bounce is visible).

### Task B — Add PIT-based debounce / deglitch (20 ms)

Goal: Convert bouncy raw edges into one clean logical transition.

Requirements:

- Set debounce_us = 20000 (20 ms) for SW8.

- On any GPIO interrupt, ISR only arms debounce: irq_armed=true, t_irq_us=now_us.

- In main loop, call dio_poll_debounce(ch):

  - if now_us - t_irq_us >= debounce_us:

    - re-read the pad level

    - map through polarity → logical_level

    - if changed from last_debounced_level:

      - update last_debounced_level

      - push event to ring buffer

      - invoke callback (or callback via event consumer)

Pass criteria:

- One physical press produces one debounced transition (RISE), one release produces one (FALL).

- Console shows RAW_EDGES >> DEBOUNCED_EDGES during testing (bounce absorbed).

### Task C — Event queue + callback dispatch (deferred ISR model)

Goal: Make the driver scalable and deterministic.

Requirements:

- ISR never calls user callbacks directly if debounce enabled; it only arms.

- Debounced events are placed in eventq.

- Main loop drains queue and:

  - prints event log lines (timestamp, channel, new level)

  - toggles LED on debounced rising edge only (“one action per press”).

Pass criteria:

- LED toggles once per press (not multiple times per bounce).

- Event ring buffer overflow is handled and reported (e.g., dropped_events).

### Task D — Avionics use-case simulation: WOW with chatter fault logic

Use SW8 as WOW and implement fault rules from the theory module.

Requirements:

- Treat power-up safe default as AIR until stable ground seen.

- WOW debounce: 50 ms (debounce_us = 50000).

- Chatter detection:

  - if two accepted transitions occur within <10 ms, latch a FAULT_CHATTER.

  - when fault latched: ignore further transitions until 100 ms stable holdoff passes.

- Indication:

  - LED ON steady when in fault

  - Console prints: WOW: FAULT due to chatter, WOW: FAULT CLEARED, WOW: GROUND/AIR with timestamps.

Pass criteria:

- Normal press/release yields clean WOW: GROUND and WOW: AIR.

- Rapid tapping can intentionally trigger chatter fault.

- Fault clears only after the stable holdoff rule.

### Task E — Latency & timing validation (measurement)

Goal: Validate design claims with measurable evidence.

Required measurements to report (in README):

- Raw IRQ latency surrogate: timestamp at ISR entry (us) vs timestamp at debounced event emission (us).

- Debounce delay: show that debounced event happens ≈ debounce_us after IRQ arming.

- ISR boundedness: prove ISR work is minimal (no prints, only flag clear + arm).

Pass criteria:

- Logs demonstrate the debounce timing clearly.

- Driver does not miss debounced events during normal human interaction.

## 7) Optional multi-board extension (use your 4 boards)

If you want a more “avionics-like” signal source than a button:

- Board A generates a pulse train on a GPIO output (e.g., 10–50 Hz).

- Board B captures it with this interrupt-driven input driver.

- Board C acts as logger/monitor.
This helps validate event rate limits and queue overflow behavior.

## 8) MCUXpresso IDE steps (what the learner must do)

- Import SDK example: gpio/input_interrupt (evkbimxrt1050).

- Add new driver files: dio_irq.[ch], dio_timebase.[ch], eventq.[ch].

- Enable PIT in the project (add fsl_pit.h usage and init).

- Enable GPIO IRQ in NVIC using the board-provided IRQ macro (and set a reasonable priority).

- Build/flash/run. Verify console output + LED behavior.

## 9) Deliverables

- MCUXpresso project (or exported archive)

- README.md including:

  - channel table (pin, polarity, edge, debounce_us, safe output state)

  - overflow policy for event queue

  - timing/latency evidence and test procedure

- Console log captures for Tasks A–D, including the fault injection run.