# Discrete IO Driver — Interrupt‑driven (EVKB‑i.MX RT1050) — Complete Solution

This is the complete, executable solution for the Interrupt‑driven Discrete IO Driver exercise on the IMXRT1050‑EVKB, implemented on top of the SDK example:

- boards/evkbimxrt1050/driver_examples/gpio/input_interrupt

It provides: - GPIO interrupt capture (both edges) - Deferred debounce using a PIT free‑running timebase (microsecond timestamps) - Timestamped debounced event queue - Avionics WOW simulation with 50 ms debounce + chatter fault latch (<10 ms between accepted transitions) + 100 ms stable holdoff - Output indication on USER LED (fault = steady ON; normal = toggles once per press)

ISR hygiene: GPIO ISR does only flag clear + timestamp + arm debounce. No PRINTF in ISR.

## 1) MCUXpresso IDE steps (import + add files)

### Step 1 — Import the base SDK example

- File → Import… → MCUXpresso IDE → Import SDK example(s)…

- Board: evkbimxrt1050

- Example: driver_examples → gpio → input_interrupt

- Finish.

### Step 2 — Add the PIT driver source to the project

The imported input_interrupt example does not include PIT sources by default.

Add this file from the SDK into your project: - devices/MIMXRT1052/drivers/fsl_pit.c

MCUXpresso: right‑click project → Add files… (or Import… → Existing files) → browse into the SDK folder and select fsl_pit.c.

Headers (fsl_pit.h) are already available via include paths; you only need to compile the .c.

### Step 3 — Add these new lab source files

Create these files in your project (same folder as gpio_input_interrupt.c is fine): - dio_timebase_pit.h, dio_timebase_pit.c - eventq.h, eventq.c - dio_irq.h, dio_irq.c

### Step 4 — Replace the example main

Replace the content of: - gpio_input_interrupt.c

with the lab main in §5.

## 2) Hardware mapping used (from the imported example)

The base example already defines: - EXAMPLE_SW_GPIO, EXAMPLE_SW_GPIO_PIN, EXAMPLE_SW_IRQ, EXAMPLE_GPIO_IRQHandler, EXAMPLE_SW_NAME

And the board header defines: - USER LED: BOARD_USER_LED_GPIO / BOARD_USER_LED_GPIO_PIN - LED polarity: LOGIC_LED_ON / LOGIC_LED_OFF (EVKB LED is active‑low)

This solution uses those macros.

## 3) Build knobs (compile‑time)

Edit the macros at the top of gpio_input_interrupt.c: - LAB_AVIONICS_MODE: - 1 = WOW + chatter fault logic (default) - 0 = simple debounce + event queue demo

- LAB_DEBOUNCE_US:

  - In avionics mode: default 50,000 us

  - In basic mode: default 20,000 us

## 4) Add these driver files

### 4.1) dio_timebase_pit.h

#ifndef DIO_TIMEBASE_PIT_H_
#define DIO_TIMEBASE_PIT_H_

#include <stdint.h>

/* Initialize PIT channel 0 as a free-running downcounter for timestamps. */
void DIO_TimebaseInit_PIT(void);

/* Monotonic timestamp (microseconds). */
uint64_t DIO_TimeNowUs_PIT(void);

/* Returns PIT clock used for conversion (Hz). */
uint32_t DIO_TimebaseClockHz_PIT(void);

#endif /* DIO_TIMEBASE_PIT_H_ */

### 4.2) dio_timebase_pit.c

#include "dio_timebase_pit.h"

#include "fsl_common.h"
#include "fsl_clock.h"
#include "fsl_pit.h"

/* Use PIT channel 0 as a free-running timer.
 * We program LDVAL=0xFFFFFFFF and let it roll over.
 */
#define TB_PIT_BASE    PIT
#define TB_PIT_CH      kPIT_Chnl_0
#define TB_LDVAL_MAX   (0xFFFFFFFFu)

static uint32_t g_tb_clk_hz = 0u;
static uint32_t g_ticks_per_us = 0u; /* integer ticks/us (works with 24MHz = 24) */

static uint32_t g_last_cval = 0u;
static uint64_t g_tick_accum = 0u; /* accumulated PIT ticks (monotonic) */

uint32_t DIO_TimebaseClockHz_PIT(void)
{
    return g_tb_clk_hz;
}

void DIO_TimebaseInit_PIT(void)
{
    pit_config_t cfg;

    PIT_GetDefaultConfig(&cfg);
    PIT_Init(TB_PIT_BASE, &cfg);

    /* Stop channel, program LDVAL, clear flag, then start. */
    PIT_StopTimer(TB_PIT_BASE, TB_PIT_CH);

    TB_PIT_BASE->CHANNEL[TB_PIT_CH].LDVAL = TB_LDVAL_MAX;
    PIT_ClearStatusFlags(TB_PIT_BASE, TB_PIT_CH, kPIT_TimerFlag);

    PIT_StartTimer(TB_PIT_BASE, TB_PIT_CH);

    /* The official PIT example for this board uses kCLOCK_OscClk as the PIT source clock. */
    g_tb_clk_hz = CLOCK_GetFreq(kCLOCK_OscClk);
    if (g_tb_clk_hz == 0u)
    {
        g_tb_clk_hz = 24000000u; /* last-resort safe guess */
    }

    g_ticks_per_us = g_tb_clk_hz / 1000000u;
    if (g_ticks_per_us == 0u)
    {
        g_ticks_per_us = 1u;
    }

    /* Initialize accumulators */
    g_last_cval = PIT_GetCurrentTimerCount(TB_PIT_BASE, TB_PIT_CH);
    g_tick_accum = 0u;
}

uint64_t DIO_TimeNowUs_PIT(void)
{
    /* Protect against concurrent calls from main + ISR.
     * (GPIO ISR calls this to timestamp the interrupt.)
     */
    uint32_t primask = DisableGlobalIRQ();

    uint32_t cur = PIT_GetCurrentTimerCount(TB_PIT_BASE, TB_PIT_CH);

    /* PIT counts down. Normal case: cur <= last.
     * If cur > last -> rollover occurred.
     */
    uint32_t delta_ticks;
    if (cur <= g_last_cval)
    {
        delta_ticks = g_last_cval - cur;
    }
    else
    {
        /* Rollover: last -> 0, reload to LDVAL, then down to cur */
        delta_ticks = (g_last_cval + 1u) + (TB_LDVAL_MAX - cur);
    }

    g_last_cval = cur;
    g_tick_accum += (uint64_t)delta_ticks;

    EnableGlobalIRQ(primask);

    /* Convert ticks to microseconds using integer ticks/us.
     * With 24MHz clock: 24 ticks = 1 us.
     */
    return (g_tick_accum / (uint64_t)g_ticks_per_us);
}

### 4.3) eventq.h

#ifndef EVENTQ_H_
#define EVENTQ_H_

#include <stdint.h>
#include <stdbool.h>

typedef enum
{
    EVT_NONE = 0,
    EVT_DEBOUNCED_RISE,
    EVT_DEBOUNCED_FALL,
    EVT_FAULT_CHATTER_LATCHED,
    EVT_FAULT_CHATTER_CLEARED,
} evt_type_t;

typedef struct
{
    uint64_t t_us;
    uint8_t type;     /* evt_type_t */
    uint8_t channel;  /* channel id */
    uint8_t level;    /* logical level 0/1 */
} evt_t;

typedef struct
{
    evt_t *buf;
    uint32_t depth;
    volatile uint32_t wr;
    volatile uint32_t rd;
    volatile uint32_t dropped; /* drop-newest policy */
} eventq_t;

void EventQ_Init(eventq_t *q, evt_t *storage, uint32_t depth);
bool EventQ_Push(eventq_t *q, const evt_t *e); /* returns false if dropped */
bool EventQ_Pop(eventq_t *q, evt_t *out);
uint32_t EventQ_Count(const eventq_t *q);

#endif /* EVENTQ_H_ */

### 4.4) eventq.c

#include "eventq.h"

static inline uint32_t idx(const eventq_t *q, uint32_t v)
{
    return (v % q->depth);
}

void EventQ_Init(eventq_t *q, evt_t *storage, uint32_t depth)
{
    q->buf = storage;
    q->depth = depth;
    q->wr = 0u;
    q->rd = 0u;
    q->dropped = 0u;
}

bool EventQ_Push(eventq_t *q, const evt_t *e)
{
    uint32_t count = q->wr - q->rd;
    if (count >= q->depth)
    {
        q->dropped++;
        return false; /* drop newest */
    }

    q->buf[idx(q, q->wr)] = *e;
    q->wr++;
    return true;
}

bool EventQ_Pop(eventq_t *q, evt_t *out)
{
    if (q->rd == q->wr)
    {
        return false;
    }

    *out = q->buf[idx(q, q->rd)];
    q->rd++;
    return true;
}

uint32_t EventQ_Count(const eventq_t *q)
{
    return (q->wr - q->rd);
}

### 4.5) dio_irq.h

#ifndef DIO_IRQ_H_
#define DIO_IRQ_H_

#include <stdint.h>
#include <stdbool.h>

#include "fsl_gpio.h"
#include "eventq.h"

typedef enum
{
    DIO_EDGE_RISE = 0,
    DIO_EDGE_FALL,
    DIO_EDGE_BOTH,
} dio_edge_mode_t;

typedef struct
{
    GPIO_Type *gpio;
    uint32_t pin;
    bool activeHigh;           /* physical->logical mapping */
    dio_edge_mode_t edgeMode;  /* informational (HW sets actual edge mode) */

    uint32_t debounce_us;      /* quiet-time debounce window */

    /* ISR-updated */
    volatile bool armed;
    volatile uint64_t t_irq_us;     /* last edge timestamp */
    volatile uint64_t last_irq_us;  /* last IRQ time (for stable-holdoff) */
    volatile uint32_t raw_edges;

    /* Debounced state */
    uint8_t debounced_level;   /* logical 0/1 */

    /* Chatter fault support (avionics) */
    bool fault_chatter;
    uint64_t last_accepted_us;
} dio_irq_in_t;

void DIO_IrqInInit(dio_irq_in_t *ch,
                   GPIO_Type *gpio,
                   uint32_t pin,
                   bool activeHigh,
                   dio_edge_mode_t edgeMode,
                   uint32_t debounce_us,
                   uint8_t safe_default_level);

/* Call from GPIO ISR when this pin generated an interrupt. */
void DIO_IrqInArmFromISR(dio_irq_in_t *ch, uint64_t now_us);

/* Service debounce in main context. Pushes debounced edge events into q.
 * If avionics chatter logic is enabled in the application, it can use:
 *   ch->fault_chatter, ch->last_irq_us, ch->last_accepted_us
 */
void DIO_IrqInService(dio_irq_in_t *ch, uint64_t now_us, uint8_t channel_id, eventq_t *q);

static inline uint32_t DIO_IrqInRawEdgeCount(const dio_irq_in_t *ch) { return ch->raw_edges; }
static inline uint8_t  DIO_IrqInDebouncedLevel(const dio_irq_in_t *ch) { return ch->debounced_level; }

#endif /* DIO_IRQ_H_ */

### 4.6) dio_irq.c

#include "dio_irq.h"
#include "fsl_common.h"

static inline uint8_t phys_to_logical(uint8_t phys, bool activeHigh)
{
    if (activeHigh)
    {
        return (phys ? 1u : 0u);
    }
    else
    {
        return (phys ? 0u : 1u);
    }
}

void DIO_IrqInInit(dio_irq_in_t *ch,
                   GPIO_Type *gpio,
                   uint32_t pin,
                   bool activeHigh,
                   dio_edge_mode_t edgeMode,
                   uint32_t debounce_us,
                   uint8_t safe_default_level)
{
    ch->gpio = gpio;
    ch->pin = pin;
    ch->activeHigh = activeHigh;
    ch->edgeMode = edgeMode;
    ch->debounce_us = debounce_us;

    ch->armed = false;
    ch->t_irq_us = 0u;
    ch->last_irq_us = 0u;
    ch->raw_edges = 0u;

    ch->debounced_level = safe_default_level ? 1u : 0u;

    ch->fault_chatter = false;
    ch->last_accepted_us = 0u;
}

void DIO_IrqInArmFromISR(dio_irq_in_t *ch, uint64_t now_us)
{
    ch->raw_edges++;
    ch->t_irq_us = now_us;
    ch->last_irq_us = now_us;
    ch->armed = true;
}

void DIO_IrqInService(dio_irq_in_t *ch, uint64_t now_us, uint8_t channel_id, eventq_t *q)
{
    /* If not armed, nothing to do */
    uint32_t primask = DisableGlobalIRQ();
    bool armed = ch->armed;
    uint64_t t_irq = ch->t_irq_us;
    EnableGlobalIRQ(primask);

    if (!armed)
    {
        return;
    }

    /* Wait until quiet-time debounce passes */
    if ((now_us - t_irq) < (uint64_t)ch->debounce_us)
    {
        return;
    }

    /* Disarm (a later IRQ will re-arm) */
    primask = DisableGlobalIRQ();
    ch->armed = false;
    EnableGlobalIRQ(primask);

    /* Sample physical pad and map to logical */
    uint8_t phys = GPIO_PinReadPadStatus(ch->gpio, ch->pin);
    uint8_t logical = phys_to_logical(phys, ch->activeHigh);

    if (logical == ch->debounced_level)
    {
        return; /* no change */
    }

    /* Commit and push event */
    ch->debounced_level = logical;

    evt_t e;
    e.t_us = now_us;
    e.channel = channel_id;
    e.level = logical;
    e.type = (uint8_t)(logical ? EVT_DEBOUNCED_RISE : EVT_DEBOUNCED_FALL);

    (void)EventQ_Push(q, &e);
}

## 5) Replace gpio_input_interrupt.c (full lab application)

Replace the example file content with this.

#include <stdbool.h>
#include <stdint.h>

#include "fsl_common.h"
#include "fsl_gpio.h"
#include "fsl_debug_console.h"

#include "app.h"
#include "board.h"
#include "pin_mux.h"

#include "dio_timebase_pit.h"
#include "eventq.h"
#include "dio_irq.h"

/* ---------------- Lab knobs ---------------- */
#define LAB_AVIONICS_MODE   (1)   /* 1: WOW + chatter fault logic; 0: basic debounce demo */

#if (LAB_AVIONICS_MODE)
#define LAB_DEBOUNCE_US     (50000u)  /* 50 ms */
#else
#define LAB_DEBOUNCE_US     (20000u)  /* 20 ms */
#endif

#define CHATTER_WINDOW_US   (10000u)   /* <10 ms between accepted transitions => fault */
#define FAULT_HOLDOFF_US    (100000u)  /* 100 ms stable (no IRQ) to clear fault */

#define EVENTQ_DEPTH        (32u)

/* Channel IDs */
enum { CH_SW8 = 0 };

/* Event queue */
static evt_t g_evt_storage[EVENTQ_DEPTH];
static eventq_t g_evtq;

/* Discrete input channel */
static dio_irq_in_t g_sw8;

/* Avionics WOW state */
static bool g_wow_ground = false; /* 1=GROUND, 0=AIR */
static bool g_fault_chatter = false;

/* Counters */
static uint32_t g_debounced_edges = 0u;

/* ---------------- GPIO ISR ----------------
 * ISR hygiene: clear flag, timestamp, arm. No prints.
 */
void EXAMPLE_GPIO_IRQHandler(void)
{
    uint32_t flags = GPIO_PortGetInterruptFlags(EXAMPLE_SW_GPIO);
    uint32_t mask = (1u << EXAMPLE_SW_GPIO_PIN);

    if ((flags & mask) != 0u)
    {
        GPIO_PortClearInterruptFlags(EXAMPLE_SW_GPIO, mask);

        uint64_t now_us = DIO_TimeNowUs_PIT();
        DIO_IrqInArmFromISR(&g_sw8, now_us);
    }

    SDK_ISR_EXIT_BARRIER;
}

static inline void LED_Init(void)
{
    USER_LED_INIT(LOGIC_LED_OFF);
}

static inline void LED_Set(bool on)
{
    if (on)
    {
        GPIO_PinWrite(BOARD_USER_LED_GPIO, BOARD_USER_LED_GPIO_PIN, LOGIC_LED_ON);
    }
    else
    {
        GPIO_PinWrite(BOARD_USER_LED_GPIO, BOARD_USER_LED_GPIO_PIN, LOGIC_LED_OFF);
    }
}

static inline void LED_Toggle(void)
{
    USER_LED_TOGGLE();
}

int main(void)
{
    /* Input config: both-edge interrupts */
    gpio_pin_config_t sw_config = {
        .direction = kGPIO_DigitalInput,
        .outputLogic = 0,
        .interruptMode = kGPIO_IntRisingOrFallingEdge,
    };

    BOARD_InitHardware();

    PRINTF("\r\n=== Interrupt-driven Discrete IO Driver Lab ===\r\n");
    PRINTF("Button: %s  GPIO base=%p pin=%u\r\n", EXAMPLE_SW_NAME, EXAMPLE_SW_GPIO, (unsigned)EXAMPLE_SW_GPIO_PIN);
    PRINTF("Mode: %s\r\n", LAB_AVIONICS_MODE ? "AVIONICS (WOW + chatter fault)" : "BASIC (debounce + events)");
    PRINTF("Debounce: %u us\r\n", (unsigned)LAB_DEBOUNCE_US);

    /* Init LED output (safe default OFF) */
    LED_Init();
    LED_Set(false);

    /* Init PIT timebase */
    DIO_TimebaseInit_PIT();
    PRINTF("Timebase: PIT clk=%lu Hz\r\n", (unsigned long)DIO_TimebaseClockHz_PIT());

    /* Init event queue */
    EventQ_Init(&g_evtq, g_evt_storage, EVENTQ_DEPTH);

    /* Init discrete input driver:
     * SW8 on EVKB is typically active-low (pressed reads 0).
     * We map pressed->logical 1 by setting activeHigh=false.
     * Power-up safe default for avionics WOW: AIR (0) until stable ground seen.
     */
    DIO_IrqInInit(&g_sw8,
                 EXAMPLE_SW_GPIO,
                 EXAMPLE_SW_GPIO_PIN,
                 false, /* activeHigh */
                 DIO_EDGE_BOTH,
                 LAB_DEBOUNCE_US,
                 0u /* safe default logical level */);

    /* Configure the GPIO pin and IRQ */
    EnableIRQ(EXAMPLE_SW_IRQ);
    GPIO_PinInit(EXAMPLE_SW_GPIO, EXAMPLE_SW_GPIO_PIN, &sw_config);
    GPIO_PortClearInterruptFlags(EXAMPLE_SW_GPIO, 1u << EXAMPLE_SW_GPIO_PIN);
    GPIO_PortEnableInterrupts(EXAMPLE_SW_GPIO, 1u << EXAMPLE_SW_GPIO_PIN);

    /* IMPORTANT: to detect a stable level that exists at boot (no edge),
     * arm one “virtual” debounce evaluation at startup.
     */
    DIO_IrqInArmFromISR(&g_sw8, DIO_TimeNowUs_PIT());

    uint64_t last_status_us = 0u;

    while (1)
    {
        uint64_t now_us = DIO_TimeNowUs_PIT();

        /* Service debounce (deferred from ISR). */
        if (!g_fault_chatter)
        {
            DIO_IrqInService(&g_sw8, now_us, CH_SW8, &g_evtq);
        }
        else
        {
            /* Fault holdoff: clear only after 100ms with no IRQ activity */
            uint64_t last_irq = g_sw8.last_irq_us;
            if ((now_us - last_irq) >= FAULT_HOLDOFF_US)
            {
                g_fault_chatter = false;
                evt_t fe = { .t_us = now_us, .type = (uint8_t)EVT_FAULT_CHATTER_CLEARED, .channel = CH_SW8, .level = g_sw8.debounced_level };
                (void)EventQ_Push(&g_evtq, &fe);

                /* Resync: force a fresh evaluation after clearing */
                DIO_IrqInArmFromISR(&g_sw8, now_us);
            }
        }

        /* Consume events */
        evt_t e;
        while (EventQ_Pop(&g_evtq, &e))
        {
            switch ((evt_type_t)e.type)
            {
                case EVT_DEBOUNCED_RISE:
                case EVT_DEBOUNCED_FALL:
                {
                    g_debounced_edges++;

#if (LAB_AVIONICS_MODE)
                    /* WOW: logical 1 = GROUND, logical 0 = AIR */
                    bool new_ground = (e.level != 0u);

                    /* Chatter detection: accepted transitions too close together */
                    if (g_sw8.last_accepted_us != 0u)
                    {
                        uint64_t dt = e.t_us - g_sw8.last_accepted_us;
                        if (dt < CHATTER_WINDOW_US)
                        {
                            g_fault_chatter = true;
                            evt_t fe = { .t_us = e.t_us, .type = (uint8_t)EVT_FAULT_CHATTER_LATCHED, .channel = CH_SW8, .level = e.level };
                            (void)EventQ_Push(&g_evtq, &fe);
                            break;
                        }
                    }
                    g_sw8.last_accepted_us = e.t_us;

                    g_wow_ground = new_ground;
                    PRINTF("[%10llu us] WOW=%s  (debounced)\r\n",
                           (unsigned long long)e.t_us,
                           g_wow_ground ? "GROUND" : "AIR");

                    /* For avionics demo: toggle LED once per GROUND transition */
                    if (e.type == EVT_DEBOUNCED_RISE)
                    {
                        LED_Toggle();
                    }
#else
                    PRINTF("[%10llu us] DEBOUNCED %s  level=%u\r\n",
                           (unsigned long long)e.t_us,
                           (e.type == EVT_DEBOUNCED_RISE) ? "RISE" : "FALL",
                           e.level);

                    /* Basic demo: toggle LED on debounced press (rise) */
                    if (e.type == EVT_DEBOUNCED_RISE)
                    {
                        LED_Toggle();
                    }
#endif
                    break;
                }

                case EVT_FAULT_CHATTER_LATCHED:
                    PRINTF("[%10llu us] FAULT_CHATTER LATCHED (accepted transitions <10ms)\r\n",
                           (unsigned long long)e.t_us);
                    LED_Set(true); /* steady ON in fault */
                    break;

                case EVT_FAULT_CHATTER_CLEARED:
                    PRINTF("[%10llu us] FAULT_CHATTER CLEARED (>=100ms stable)\r\n",
                           (unsigned long long)e.t_us);
                    LED_Set(false);
                    break;

                default:
                    break;
            }
        }

        /* Periodic status (every 500ms) */
        if ((now_us - last_status_us) >= 500000ull)
        {
            last_status_us = now_us;
            PRINTF("[%10llu us] STATUS raw_edges=%lu deb_edges=%lu dropped=%lu fault=%u deb_level=%u\r\n",
                   (unsigned long long)now_us,
                   (unsigned long)DIO_IrqInRawEdgeCount(&g_sw8),
                   (unsigned long)g_debounced_edges,
                   (unsigned long)g_evtq.dropped,
                   g_fault_chatter ? 1u : 0u,
                   (unsigned)DIO_IrqInDebouncedLevel(&g_sw8));
        }

        /* Small cooperative delay to allow debounce windows to elapse.
         * We cannot use pure __WFI() here because debounce expiry may occur
         * after the last bounce IRQ (no wake source). This 1ms delay keeps
         * CPU usage low but guarantees debounce servicing.
         */
        SDK_DelayAtLeastUs(1000u, CLOCK_GetFreq(kCLOCK_CpuClk));
    }
}

## 6) Expected behavior / test procedure

### A) Basic debounce + events

- With LAB_AVIONICS_MODE=0 and LAB_DEBOUNCE_US=20000:

  - Press → exactly one DEBOUNCED RISE

  - Release → exactly one DEBOUNCED FALL

  - LED toggles once per press

### B) Avionics WOW + chatter fault

- With LAB_AVIONICS_MODE=1 and LAB_DEBOUNCE_US=50000:

  - Press/hold SW8 long enough → WOW=GROUND

  - Release/hold → WOW=AIR

  - Rapid tapping can intentionally trigger:

    - FAULT_CHATTER LATCHED

    - LED steady ON while fault is active

  - After you stop toggling for ≥100 ms:

    - FAULT_CHATTER CLEARED

## 7) Notes / common troubleshooting

- Build fails: missing PIT symbols

- You forgot to add devices/MIMXRT1052/drivers/fsl_pit.c to the project.

- No interrupts

- Ensure EnableIRQ(EXAMPLE_SW_IRQ) is executed.

- Ensure GPIO_PortEnableInterrupts is called.

- Polarity wrong (GROUND/AIR inverted)

- Change the activeHigh argument in DIO_IrqInInit.

- Timestamp seems wrong

- Ensure PIT clock is stable; the code uses kCLOCK_OscClk as per the PIT SDK example for this board.