# Discrete IO Driver Design — Integrating IO Into A Bare‑metal Scheduler (EVKB-i.MX RT1050)

This is the complete, executable solution for the lab “Integrating Discrete IO into a Bare‑metal Scheduler” on the IMXRT1050‑EVKB, using only MCUXpresso SDK APIs.

It provides: - A 1 ms SysTick timebase - A cooperative bare‑metal scheduler (task table with fixed periods) - A discrete input driver (polarity + integrator debounce + edge events) - A ring‑buffer event queue (non‑blocking, drop‑newest policy) - A discrete output driver (request/apply split + lamp‑test override + safe default) - An avionics scenario: dual‑channel WOW consolidation + 40 ms qualify/de‑qualify + 500 ms disagree fault latch + safe inhibit - A generic scenario: SW8 press toggles an LED request; long‑press enables lamp test

✅ No GPIO interrupts are used for functional IO behavior. SysTick is the only ISR and it only increments a counter.

## 1) Base project (required)

- MCUXpresso IDE → File → Import… → MCUXpresso IDE → Import SDK example(s)…

- Board: evkbimxrt1050

- Example: driver_examples → gpio → led_output

- Finish.

This creates the project containing gpio_led_output.c and the required board/clock/pin/debug-console support.

## 2) Add the lab source files

Create these new files in the project (placing them in the same folder as gpio_led_output.c is fine): - lab_config.h - scheduler.h, scheduler.c - eventq.h, eventq.c - discrete_in.h, discrete_in.c - discrete_out.h, discrete_out.c

Important (common build issue): After creating/importing the files, confirm they are not excluded: - Right‑click each file → Resource Configurations → Exclude from Build… → ensure unchecked for your active build configuration.

## 3) Variant builds (Generic vs Avionics)

This solution supports two build variants using preprocessor symbols.

### Recommended: create two MCUXpresso build configurations

- Right‑click project → Build Configurations → Manage…

- Select Debug → New… → Copy existing

- Create:

  - Debug_Generic

  - Debug_Avionics

- For each configuration:

  - Right‑click project → Properties

  - C/C++ Build → Settings → MCU GCC Compiler → Preprocessor

  - Add symbols under “Defined symbols”:

Debug_Generic - LAB_APP_MODE=LAB_MODE_GENERIC - (Optional) LAB_ENABLE_WOW_B=0 (ties WOW_B behavior to WOW_A so you don’t need the jumper input)

Debug_Avionics - LAB_APP_MODE=LAB_MODE_AVIONICS - LAB_ENABLE_WOW_B=1 (default; only needed if you overrode it elsewhere)

- Apply and Close.

- Select configuration: Build Configurations → Set Active.

## 4) Bench wiring (WOW_B, Avionics mode)

- WOW_A is SW8 (on-board user button).

- WOW_B is mapped by default to GPIO_AD_B0_10 → GPIO1_IO10.

To drive WOW_B: - Connect GPIO_AD_B0_10 header pin to 3.3V → WOW_B = 1 - Connect to GND → WOW_B = 0

The pad config enables a weak pull-down so WOW_B defaults to 0 when floating.

## 5) Build modes

The same code supports two modes.

### Mode A — GENERIC (Task 2)

- SW8 press toggles a logical LED request

- Long press (>1000 ms) enables LAMP_TEST override (forces LED ON until release)

### Mode B — AVIONICS (Tasks 3 & 4)

- WOW_A = SW8, WOW_B = jumper input

- Consolidate WOW_TRUE:

  - Assert only if both channels TRUE for ≥40 ms

  - Deassert if either channel FALSE for ≥40 ms

- Disagree fault:

  - If WOW_A != WOW_B continuously for ≥500 ms → FAULT_WOW_DISAGREE latches

  - Clear fault by pressing c in the console (non‑blocking)

- Output behavior:

  - When WOW_TRUE=1 and no fault → LED blinks at 2 Hz (“test allowed”)

  - Otherwise → LED OFF

  - Long press on SW8 still enables lamp test override

## 6) Add these new files from src folder to the project

Create these files in the project (same folder as gpio_led_output.c is fine):

- lab_config.h

- scheduler.h, scheduler.c

- eventq.h, eventq.c

- discrete_in.h, discrete_in.c

- discrete_out.h, discrete_out.c

## 7) Replace gpio_led_output.c with the src folder provided gpio_led_output.c

## 8) MCUXpresso build + run instructions

- Build

  - Select the desired build configuration (Debug_Generic or Debug_Avionics).

  - Project → Build Project.

- Flash / Debug

  - Click Debug (bug icon). MCUXpresso will build, program the EVKB, and halt at main().

  - Press Resume (F8).

- Open the console

  - Window → Show View → Console.

  - Ensure the correct debug console is selected (the example uses the SDK debug console over the OpenSDA/DAPLink virtual COM).

### Expected behavior

Generic mode - Short press SW8 toggles LED request (you’ll see PRESS/RELEASE logs). - Hold SW8 > 1000 ms enables lamp-test (forces LED ON while held).

Avionics mode - WOW_TRUE qualifies when both WOW_A and WOW_B are true for ≥40 ms. - WOW_TRUE de-qualifies when either channel is false for ≥40 ms. - If WOW_A != WOW_B continuously for ≥500 ms, the disagree fault latches. - Press c in the console to clear the fault (non-blocking).

### If you see build errors

- Confirm the imported base example is exactly: driver_examples/gpio/led_output for evkbimxrt1050.

- Confirm all added .c files are included in the active build configuration (not excluded).

- If you see multiple definition of SysTick_Handler, ensure only scheduler.c defines it (exclude any other file that defines SysTick_Handler in your project).