## Exercise specification: Integrating Discrete I/O into a Bare-metal Scheduler on IMXRT1050-EVKB

This lab has you build a minimal cooperative bare-metal scheduler (SysTick-driven) and integrate a production-style discrete I/O driver (debounce, polarity, edge detection, timestamps, event queue, and safe outputs). It follows the structure and timing guidance from the uploaded theory module.

### 1) Learning objectives

By completing this exercise, you will be able to:

- Configure IOMUXC + GPIO for deterministic discrete inputs/outputs (pull/keeper + hysteresis for inputs, sensible drive/slew for outputs).

- Implement a 1 ms SysTick timebase and a cooperative task scheduler with fixed periods.

- Integrate a discrete input driver using an integrator debounce and generate events (RISE/FALL) with timestamps.

- Implement safe discrete outputs with known power-up state, optional interlocks, and a lamp-test override.

- Demonstrate avionics-style behavior (WOW consolidation / latching / disagree detection) inside deterministic task periods.

### 2) Target board and I/O used

Use EVKB board macros (do not hardcode if macros exist):

- Input discrete A: SW8 → BOARD_USER_BUTTON_GPIO, BOARD_USER_BUTTON_GPIO_PIN (SW8 is on GPIO5_IO00 per theory).

- Output discrete: USER LED → BOARD_USER_LED_GPIO, BOARD_USER_LED_GPIO_PIN (LED on GPIO1_IO09; typically active-low on EVKB).

- Optional input discrete B (jumper input): Choose one spare GPIO pin routed to a header (e.g., a nearby AD_B0 pin), and document it in your README + configure it in pin_mux.c using Config Tools.

Electrical notes (lab-safe):

- 3.3 V logic only; never connect aircraft 28 V discretes directly.

### 3) Required software architecture (layers)

Implement these layers exactly (file names can differ, responsibilities must match):

- HAL (Board/SDK init)

- Pin mux + pad config (IOMUXC_SetPinMux, IOMUXC_SetPinConfig)

- GPIO init (GPIO_PinInit)

- DISCRETE driver

- Polarity handling (active-low / active-high)

- Integrator debounce (saturating counter, threshold)

- Debounced level + edge detect

- Per-edge timestamp

- Optional sticky fault/latch

- Scheduler

- 1 ms tick (SysTick)

- Task table with period + next release time

- Deterministic dispatch (no blocking in tasks)

- Application

- Generic demo behavior

- Avionics scenario behavior

- Console logging (non-ISR)

### 4) Scheduler requirements (must meet)

Implement a cooperative scheduler driven by SysTick = 1 ms.

Task set (required periods):

- EventPump_Task_1ms() – drains event queue into application-visible structures

- DiscreteIn_Task_5ms() – samples raw inputs + runs debounce/integrator

- App_Task_10ms() – avionics logic and state machines

- DiscreteOut_Task_10ms() – applies output requests + lamp-test override + interlocks

- (Optional) Heartbeat_Task_100ms() – toggles a scope/debug pin for jitter measurement

Determinism constraints:

- No PRINTF in ISRs; logging must happen in 10 ms task or main loop.

- Each task must complete within its period budget (document worst-case time or measure with a GPIO “scope pin”).

### 5) Discrete input driver requirements

Implement integrator debounce as described in the theory:

Per input channel state:

- GPIO_Type *base; uint32_t pin; polarity;

- count (saturating 0..COUNT_MAX)

- threshold (assert when count >= threshold; deassert when count == 0)

- debounced boolean

- last_change_ms timestamp

- Event generation: EVENT_RISE, EVENT_FALL

Sampling:

- Inputs are sampled only in DiscreteIn_Task_5ms() (5 ms cadence).

Debounce tuning (required):

- For SW8: choose COUNT_MAX and threshold such that assert requires ~15–25 ms stable (example: 5 ms sampling, threshold 3 → ~15 ms). You must print your chosen effective debounce time.

### 6) Event queue requirements (ISR→task safe pattern)

Provide a fixed-size ring buffer queue for events.

Event payload (minimum):

- event_id (PRESS/RELEASE or RISE/FALL)

- channel_id

- timestamp_ms

Queue behavior:

- Non-blocking post (drop oldest or drop newest—choose one and document)

- Drained in EventPump_Task_1ms() into counters/log structures

You may implement events purely from the 5 ms sampling task (no GPIO interrupts) or optionally use a GPIO ISR only to timestamp edges; if you use an ISR, it must only post an event and clear flags.

### 7) Discrete output driver requirements

Outputs must be “request vs apply”: application sets desired states; driver applies them in a deterministic batch (10 ms).

Required features:

- Polarity mapping (active-low LED on EVKB)

- Failsafe default at boot (LED OFF)

- Lamp-test override: forces output ON independent of app request

- Optional interlock: output only applies if a required discrete is true (used in avionics task)

## 8) Exercise tasks (what you must implement)

### Task 1 — Minimal scheduler bring-up (timing proof)

Goal: Verify your scheduler timing before adding I/O.

- Implement SysTick 1 ms + scheduler table.

- Run Heartbeat_Task_100ms() that toggles USER LED (or a spare GPIO scope pin).

- Acceptance: average period correct (200 ms full blink cycle if toggling every 100 ms) and jitter within a few ms when idle.

### Task 2 — Generic discrete I/O demo: SW8 toggles LED + long-press lamp-test

Goal: Integrate input debounce + event queue + output request/apply.

- DiscreteIn_Task_5ms() debounces SW8 and posts PRESS/RELEASE.

- EventPump_Task_1ms() passes events to the app.

- App_Task_10ms() behavior:

  - On PRESS event: toggle a logical “LED_REQUEST”

  - Detect long press: if SW8 debounced stays asserted for >1000 ms → enable LAMP_TEST=1

  - On RELEASE: disable LAMP_TEST

- DiscreteOut_Task_10ms():

  - If LAMP_TEST active: force LED ON

  - Else: apply LED_REQUEST

- Acceptance: one press causes exactly one toggle; bounce does not create extra toggles; long press forces LED on until release.

### Task 3 — Avionics scenario: WOW consolidation + safe inhibit behavior

Goal: Implement a realistic avionics-style discrete rule in a deterministic scheduler.

Inputs:

- WOW Channel A = SW8

- WOW Channel B = optional jumper input (configured as a second discrete)

Consolidation rule (required):

- Assert WOW_TRUE only if both channels debounced TRUE for ≥40 ms

- Deassert WOW_TRUE if either channel FALSE for ≥40 ms

- Timestamp every WOW_TRUE transition and post event

Application behavior (required):

- When WOW_TRUE = 0 (in flight), inhibit a simulated “THRUST_REVERSER_TEST_CMD”.

- When WOW_TRUE = 1 (on ground), allow it.

- Represent THRUST_REVERSER_TEST_CMD on:

  - Console print + optional LED pattern (e.g., blink when allowed, off when inhibited)

Acceptance:

- WOW_TRUE never asserts unless both channels are true long enough.

- One channel drop forces deassert after the 40 ms rule.

- Console shows timestamps and deterministic task execution.

### Task 4 — Diagnostic add-on: disagree detection (500 ms)

Goal: Add sticky fault behavior consistent with avionics practice.

- If WOW A and WOW B disagree continuously for >500 ms:

  - set FAULT_WOW_DISAGREE sticky until manual clear (e.g., command via UART key or reset)

  - when fault active: force outputs to safe state (e.g., inhibit thrust reverser test)

- Acceptance: holding one channel true and the other false triggers fault after ~500 ms, and it remains latched.

### 9) MCUXpresso IDE workflow (how to build it)

Use an SDK example as your base, as recommended by the theory.

Recommended base project:

- boards/evkbimxrt1050/driver_examples/gpio/led_output

Steps:

- Import SDK example into MCUXpresso.

- Add new source files:

  - scheduler.[ch]

  - discrete_in.[ch] (debounce + timestamps)

  - eventq.[ch] (ring buffer)

  - discrete_out.[ch]

  - app.[ch]

- Configure optional second input pin (if doing Task 3/4 dual-channel) in Config Tools / pin_mux.c.

- Build/flash; verify console logs + LED behavior.

### 10) Test evidence and deliverables

Deliverables:

- Source project (MCUXpresso project or exported archive)

- A short README documenting:

  - task periods and rationale

  - debounce parameters and effective debounce time

  - polarity choices for SW8 and LED

  - event queue size and overflow policy

Evidence to capture:

- Console log showing:

  - Scheduler start banner with task periods

  - Debounced PRESS/RELEASE events (Task 2)

  - WOW_TRUE transitions with timestamps (Task 3)

  - Disagree fault latch behavior (Task 4)