## Exercise specification: Discrete I/O Driver Design Using Polling on IMXRT1050-EVKB

This exercise has you design a reusable polling-based discrete I/O driver (inputs + outputs) and then apply it to a generic demo and an avionics-style use case (WOW-gated pump command + PBIT). It is explicitly aligned to the uploaded theory content “Discrete I/O Driver Design Using Polling”.

## 1) Learning objectives

By the end, you will be able to:

- Implement deterministic polling for discretes with a fixed cadence (no interrupts).

- Build a small driver layer that abstracts:

  - pin mapping (GPIO port/pin)

  - polarity (active-high vs active-low)

  - debounce timing (Td) expressed as N samples at a chosen poll period (Tp)

- Provide an edge/event model (RISING/FALLING/NONE) from debounced state changes.

- Implement a fail-safe output default and a simple PBIT stuck-at check.

## 2) Target platform and I/O mapping (EVKB specifics)

Use these EVKB discretes as your lab I/O (as specified in the theory):

- Input discrete: SW8 = SNVS_WAKEUP → GPIO5_IO00 (typically active-low on the board).

- Output discrete: USER LED = GPIO_AD_B0_09 → GPIO1_IO09 (drive as active-high).

- Logic level: 3.3 V (no direct aircraft 28 V interfacing).

## 3) Constraints (important!)

- Do not use GPIO interrupts for functional behavior; this exercise is polling-only.

- Your polling period must be fixed and documented, and your debounce must be expressed as N = Td / Tp.

- All code must use MCUXpresso SDK APIs (fsl_gpio.h, fsl_iomuxc.h, fsl_common.h, etc.).

## 4) Required driver architecture (what you must design)

Implement these layers (as described in the theory):

### A) HAL setup layer

- Pin mux + pad config using:

  - IOMUXC_SetPinMux(...)

  - IOMUXC_SetPinConfig(...) (at least ensure SW8 is not floating; LED pad configured reasonably)

### B) Discrete channel model

Create struct models:

Input channel (io_input_t) must include:

- GPIO_Type *port; uint32_t pin;

- bool activeHigh;

- uint8_t debounceN; (samples to accept change)

- runtime: bool debounced; uint8_t cnt;

Output channel (io_output_t) must include:

- GPIO_Type *port; uint32_t pin;

- bool activeHigh;

### C) Poller API

Provide these functions (names can differ, behavior must match):

- io_input_init(...)

- io_edge_t io_input_poll(...) returning NONE / RISE / FALL

- io_output_init(...) with a fail-safe initial state

- io_output_set(out, asserted) accepting a logical command (asserted=true)

## 5) Timing model (fixed cadence)

Choose and implement:

- Poll period Tp = 5 ms (recommended by the theory).

- Two debounce targets:

  - Generic button debounce: Td = 20 ms → N = 4

  - Avionics WOW debounce: Td = 30 ms → N = 6

You must print at startup:

- Tp, Td, computed N for each channel.

## 6) Exercise tasks (what you must implement)

### Task 1 — Generic demo: “Button mirrors to LED” (debounced, polled)

Behavior:

- Poll SW8 every 5 ms.

- Debounce with N=4.

- Drive USER LED:

  - LED ON when debounced “pressed” is true

  - LED OFF when released

- Print edge messages:

  - SW8 debounced: PRESSED

  - SW8 debounced: RELEASED

Acceptance criteria:

- Repeated bounces during a press produce only one debounced press event.

- LED does not flicker during bounce.

### Task 2 — Avionics simulation: “WOW-gated Pump Command” (polled discretes)

Scenario (from the theory):

- Command rule: PUMP_A_CMD = (NOT WOW) AND MAINT_DOOR_CLOSED

- Use:

  - WOW mapped to SW8 (active-low, asserted on ground).

  - MAINT_DOOR_CLOSED can be:

    - Option A (no extra wiring): a compile-time constant true

    - Option B (recommended): another GPIO input via jumper (you define the pin)

Behavior:

- Poll every 5 ms.

- Debounce WOW with Td=30 ms → N=6.

- Fail-safe: on boot and on detected fault, deassert PUMP_A_CMD (LED OFF).

- Drive LED as the pump command (asserted = LED ON).

Expected operation:

- If SW8 pressed (WOW asserted / on ground) → pump command must be OFF

- If SW8 released (airborne) AND door closed is true → pump command ON

### Task 3 — PBIT (Power-on Built-In Test) for stuck-at detection

Implement a simple PBIT consistent with the theory:

PBIT window: first 200 ms (or up to 1 s for easier bench demonstration)

Check:

- For each input channel, record whether it toggled at least once during the window when the bench procedure asks the user to toggle (e.g., “Press/release SW8 during PBIT”).

- If not toggled → log STUCK_AT warning/fault.

- If any fault is present → force fail-safe output (pump OFF).

Acceptance criteria:

- If you never touch SW8 during PBIT, the firmware reports a stuck/no-toggle fault.

- If you do toggle SW8, no fault is reported.

## 7) MCUXpresso IDE workflow requirement (how you must build it)

You must start from an SDK example and adapt it:

Recommended starting point:

- driver_examples/gpio/led_output (already has console + LED structure)

Steps:

- Import example: Import SDK example(s)… → evkbimxrt1050 → gpio/led_output

- Add new driver files:

  - io_discrete_poll.h

  - io_discrete_poll.c

- Update pin mux (either via Pins Tool or code) to ensure:

  - LED pad bound to GPIO1_IO09

  - SW8 bound to GPIO5_IO00

- Replace main() to:

  - initialize channels

  - execute the fixed-period polling loop

  - apply Task 1 and Task 2 logic

  - run PBIT early after boot

## 8) Deliverables

You will submit:

- MCUXpresso project (or exported archive)

- io_discrete_poll.[ch] driver source with a short README explaining:

  - your Tp, Td, N calculations

  - polarity definitions (logical asserted vs physical high/low)

- Console log evidence:

  - generic mirror demo (press/release)

  - avionics mode showing pump gating

  - PBIT pass/fail behavior

## 9) Optional extensions (if you want to go further)

- Add a small ring buffer event queue for edges (timestamp + line index).

- Measure worst-case poll loop CPU time using DWT cycle counter (timing evidence).

- Add a second physical input (door closed) and demonstrate the full AND logic on real pins.