# How to use this document

You will create 4 MCUXpresso projects (plus 1 receiver/analyzer project that is reused). Each exercise section is: 1) Exercise specification (what to build + pass/fail) 2) Copy‑paste ready steps (MCUXpresso clicks + file list + exact edits) 3) Copy‑paste ready C code (each file fully listed) 4) Project skeleton (folder tree)

# Common prerequisites (do this once)

## Boards & wiring (recommended: 2 boards)

- Board A = TX / Injector

- Board B = RX / Analyzer

### Use Arduino header UART pins (recommended)

EVKB Arduino header provides: - D0 = UART_RX - D1 = UART_TX

On EVKB‑IMXRT1050 these map to LPUART3 pins: - GPIO_AD_B1_06 → LPUART3_TXD (Arduino D1) - GPIO_AD_B1_07 → LPUART3_RXD (Arduino D0)

Wire: - A D1 (TX) → B D0 (RX) - A GND → B GND

Keep the Debug Console as-is (SDK uses PRINTF/GETCHAR on its configured debug UART). We use LPUART3 only for board‑to‑board link.

## MCUXpresso IDE: import base example

- File → Import → MCUXpresso IDE → Import SDK example(s)

- Select your unzipped SDK: SDK_25_06_00_EVKB-IMXRT1050

- Board: evkbimxrt1050

- Import example: demo_apps → hello_world

You will copy this imported project to create each exercise project.

## IMPORTANT: one-time pinmux edit pattern (LPUART3 on Arduino pins)

In each exercise project (TX and RX), edit: - source/pin_mux.c - Find CLOCK_EnableClock(kCLOCK_Iomuxc); - Add these lines after it:

/* Enable LPUART3 on Arduino D1/D0 */
IOMUXC_SetPinMux(IOMUXC_GPIO_AD_B1_06_LPUART3_TXD, 0U);
IOMUXC_SetPinMux(IOMUXC_GPIO_AD_B1_07_LPUART3_RXD, 0U);
IOMUXC_SetPinConfig(IOMUXC_GPIO_AD_B1_06_LPUART3_TXD, 0x10B0U);
IOMUXC_SetPinConfig(IOMUXC_GPIO_AD_B1_07_LPUART3_RXD, 0x10B0U);

# Shared avionics framing used by all exercises

We simulate an ARINC‑429 word (32‑bit) packaged into a UART frame: - [0xA5][word LSB..MSB][CRC8(word bytes)]

Fault injections target: - label substitution - parity toggle - burst corruption

# EXERCISE CT‑1 — Compile‑time FI via preprocessor gating + call‑site UART shim

## 1) Specification

### Objective

Create two build configurations: - PROD build: FI code compiles out → no injected failures - FI build: FI code compiles in → injected conditions appear

### Compile‑time method

- Preprocessor gating: FI_POINT(feature, {...}) becomes a no‑op unless FI_ENABLE=1.

### Faults to inject

- Peripheral behavior emulation: sometimes return kStatus_LPUART_TxBusy.

- Payload corruption: sometimes flip bits so CRC/parity/label checks fail.

### Required app behavior

- If TxBusy: retry up to 3 times, then enter SAFE MODE (blink LED + log) but do not hang.

- If CRC mismatch detected before transmit: refuse to send and continue.

### Pass/Fail

- PASS if:

  - FI build: logs show occasional TxBusy retry and CRC/ARINC errors; app continues.

  - PROD build: near‑zero injected errors.

- FAIL if system hangs/resets unexpectedly or never reaches safe mode when busy persists.

## 2) Copy‑paste ready steps

### Projects

Create 2 projects: - CT1_TX_callsite_shim - RX_analyzer (reused for CT‑1/CT‑2/RT‑1/RT‑2)

How: 1. Right click imported hello_world → Copy → Paste → rename. 2. Apply the pinmux edit (Common prerequisites) in both projects.

### Build configs (TX project)

- PROD: no defines

- FI: add compiler define FI_ENABLE=1

MCUXpresso: - Project → Properties → C/C++ Build → Settings → MCU C Compiler → Preprocessor → Defined symbols - Add: FI_ENABLE=1 (only in FI config)

### Files to add (TX project)

Create folder: source/fi/ and add files: - fi_config.h - fi.h - fi.c - uart_fi_shim.h - uart_fi_shim.c - arinc429.h - arinc429.c

Replace source/hello_world.c with the TX hello_world.c provided below.

### Files to add (RX project)

Replace source/hello_world.c with the RX analyzer hello_world.c below.

## 3) Copy‑paste ready C code

### CT‑1 TX: source/hello_world.c

#include "fsl_device_registers.h"
#include "fsl_debug_console.h"
#include "board.h"
#include "app.h"

#include "fsl_common.h"
#include "fsl_lpuart.h"

#include "fi/fi.h"
#include "fi/uart_fi_shim.h"
#include "fi/arinc429.h"

#define LINK_UART LPUART3
#define MAX_BUSY_RETRY 3

static void LinkUart_Init115200(void)
{
    lpuart_config_t cfg;
    LPUART_GetDefaultConfig(&cfg);
    cfg.baudRate_Bps = 115200U;
    cfg.enableTx     = true;
    cfg.enableRx     = true;

    /* Same clock helper used by SDK LPUART examples */
    (void)LPUART_Init(LINK_UART, &cfg, BOARD_DebugConsoleSrcFreq());
}

static uint8_t crc8(const uint8_t *d, uint32_t n)
{
    uint8_t c = 0x00u;
    for (uint32_t i = 0; i < n; i++)
    {
        uint8_t b = d[i];
        for (int k = 0; k < 8; k++)
        {
            uint8_t mix = (uint8_t)((c ^ b) & 0x01u);
            c >>= 1;
            if (mix)
                c ^= 0x8Cu; /* x^8 + x^5 + x^4 + 1 */
            b >>= 1;
        }
    }
    return c;
}

static void safe_mode_blink_forever(void)
{
    PRINTF("SAFE MODE: persistent UART busy -> blinking LED\r\n");
    while (1)
    {
        LED_TOGGLE();
        SDK_DelayAtLeastUs(250000u, SystemCoreClock); /* 250 ms */
    }
}

int main(void)
{
    BOARD_InitHardware();
    LinkUart_Init115200();

    PRINTF("\r\nCT-1 TX (call-site shim) start\r\n");

    FI_Init();

#if FI_ENABLE
    FI_SetEnabled(true);
    FI_SetMask(FI_F_UART_TX_BUSY | FI_F_UART_CORRUPT | FI_F_ARINC_PARITY | FI_F_ARINC_LABEL | FI_F_MEM_BITFLIP);
    FI_SetProbability(5); /* 5% */
#else
    FI_SetEnabled(false);
#endif

    uint32_t ctr = 0;

    /* Avionics-like ARINC word stream */
    arinc429_word_t aw = { .label = 0x12u, .sdi = 1u, .data = 0x12345u, .ssm = 2u };

    while (1)
    {
        /* ===== Generic telemetry frame =====
           [0x55][ctr(4)][payload(8)][crc8(1)]
        */
        uint8_t pkt[1 + 4 + 8 + 1];
        pkt[0] = 0x55u;
        memcpy(&pkt[1], &ctr, 4);
        for (int i = 0; i < 8; i++)
            pkt[5 + i] = (uint8_t)(ctr + (uint32_t)i);
        pkt[13] = crc8(&pkt[1], 4 + 8);

        /* FI: optional SRAM bit flips */
        FI_POINT(FI_F_MEM_BITFLIP, FI_BitFlipRange(pkt, sizeof(pkt), 9));

        /* self-check: refuse transmit if CRC does not match */
        if (crc8(&pkt[1], 4 + 8) != pkt[13])
        {
            PRINTF("TX: CRC mismatch -> refuse telemetry (ctr=%u)\r\n", (unsigned)ctr);
        }
        else
        {
            int retries = 0;
            while (1)
            {
                status_t st = UART_FI_WriteBlocking(LINK_UART, pkt, sizeof(pkt));
                if (st == kStatus_Success)
                    break;
                if (st == kStatus_LPUART_TxBusy)
                {
                    PRINTF("TX: UART busy, retry %d/%d\r\n", retries + 1, MAX_BUSY_RETRY);
                    retries++;
                    if (retries >= MAX_BUSY_RETRY)
                        safe_mode_blink_forever();
                    continue;
                }
                PRINTF("TX: UART error %d\r\n", (int)st);
                break;
            }
        }

        /* ===== Avionics-like ARINC frame ===== */
        (void)ARINC429_SendWord(LINK_UART, &aw);

        ctr++;
        SDK_DelayAtLeastUs(10000u, SystemCoreClock); /* 10 ms */
    }
}

### CT‑1 shared files (TX): source/fi/fi_config.h

#ifndef FI_CONFIG_H
#define FI_CONFIG_H

#ifndef FI_ENABLE
#define FI_ENABLE 0
#endif

#ifndef FI_DEFAULT_PROB_PCT
#define FI_DEFAULT_PROB_PCT 5
#endif

#ifndef FI_SEED
#define FI_SEED 0x4D534543u
#endif

#ifndef FI_FEATURE_MASK
#define FI_FEATURE_MASK 0xFFFFFFFFu
#endif

#define FI_F_UART_TX_BUSY   (1u << 0)
#define FI_F_UART_CORRUPT   (1u << 1)
#define FI_F_ARINC_PARITY   (1u << 2)
#define FI_F_ARINC_LABEL    (1u << 3)
#define FI_F_MEM_BITFLIP    (1u << 4)
#define FI_F_EXCEPTIONS     (1u << 5)

#endif

### source/fi/fi.h

#ifndef FI_H
#define FI_H

#include <stdint.h>
#include <stdbool.h>

#include "fi_config.h"

#ifdef __cplusplus
extern "C" {
#endif

void     FI_Init(void);
void     FI_SetEnabled(bool en);
bool     FI_IsEnabled(void);

void     FI_SetMask(uint32_t mask);
uint32_t FI_GetMask(void);

void     FI_SetProbability(uint8_t pct);
uint8_t  FI_GetProbability(void);

void     FI_SetSeed(uint32_t seed);

uint8_t  FI_RandPct(void);
bool     FI_ShouldFire(uint32_t feature);

void     FI_BitFlip8(uint8_t *p, uint8_t mask);
void     FI_BitFlipRange(void *buf, uint32_t len, uint32_t everyN);

/* Runtime trigger helpers (used in RT-1) */
void     FI_NotifyEvent(uint32_t feature);
void     FI_ArmNth(uint32_t feature, uint32_t nth);
void     FI_ArmWindowMs(uint32_t feature, uint32_t start_ms, uint32_t end_ms);
uint32_t FI_NowMs(void);

/* Exception injections (used in RT-2) */
void FI_Inject_DivByZero(void);
void FI_Inject_UnalignedAccess(void);
void FI_Inject_MemFault(void);

#if FI_ENABLE
#define FI_POINT(feature, code_block) \
    do { if (FI_ShouldFire((feature))) { code_block; } } while (0)
#else
#define FI_POINT(feature, code_block) \
    do { (void)(feature); } while (0)
#endif

#ifdef __cplusplus
}
#endif

#endif

### source/fi/fi.c

#include "fi.h"

#include <string.h>
#include "fsl_device_registers.h"

static volatile uint32_t fi_mask = FI_FEATURE_MASK;
static volatile uint8_t  fi_prob = FI_DEFAULT_PROB_PCT;
static volatile bool     fi_en   = (FI_ENABLE != 0);

static uint32_t lcg_state = FI_SEED;

static uint32_t ev_count[32];
static uint32_t nth_arm[32];
static bool     nth_fired[32];

static uint32_t win_start_ms[32];
static uint32_t win_end_ms[32];
static bool     win_armed[32];

static inline uint32_t lcg_next(void)
{
    lcg_state = lcg_state * 1664525u + 1013904223u;
    return lcg_state;
}

static inline int bit_index(uint32_t feature)
{
    for (int i = 0; i < 32; i++)
        if (feature == (1u << i))
            return i;
    return -1;
}

static void dwt_init(void)
{
    CoreDebug->DEMCR |= CoreDebug_DEMCR_TRCENA_Msk;
    DWT->CYCCNT = 0;
    DWT->CTRL |= DWT_CTRL_CYCCNTENA_Msk;
}

uint32_t FI_NowMs(void)
{
    uint32_t hz = SystemCoreClock;
    if ((DWT->CTRL & DWT_CTRL_CYCCNTENA_Msk) == 0u || hz == 0u)
        return 0u;
    return (uint32_t)(DWT->CYCCNT / (hz / 1000u));
}

void FI_Init(void)
{
    SystemCoreClockUpdate();
    dwt_init();

    lcg_state = (FI_SEED ^ 0xA5A5A5A5u) + 1u;

    memset((void *)ev_count, 0, sizeof(ev_count));
    memset((void *)nth_arm, 0, sizeof(nth_arm));
    memset((void *)nth_fired, 0, sizeof(nth_fired));
    memset((void *)win_start_ms, 0, sizeof(win_start_ms));
    memset((void *)win_end_ms, 0, sizeof(win_end_ms));
    memset((void *)win_armed, 0, sizeof(win_armed));
}

void FI_SetEnabled(bool en) { fi_en = en; }
bool FI_IsEnabled(void) { return fi_en; }

void FI_SetMask(uint32_t mask) { fi_mask = mask; }
uint32_t FI_GetMask(void) { return fi_mask; }

void FI_SetProbability(uint8_t pct) { fi_prob = pct; }
uint8_t FI_GetProbability(void) { return fi_prob; }

void FI_SetSeed(uint32_t seed) { lcg_state = seed ? seed : 1u; }

uint8_t FI_RandPct(void) { return (uint8_t)(lcg_next() % 100u); }

void FI_BitFlip8(uint8_t *p, uint8_t mask) { *p ^= mask; }

void FI_BitFlipRange(void *buf, uint32_t len, uint32_t everyN)
{
    if (!fi_en || everyN == 0u)
        return;
    uint8_t *b = (uint8_t *)buf;
    for (uint32_t i = 0; i < len; i++)
    {
        if ((i % everyN) == 0u)
            b[i] ^= (uint8_t)(1u << (lcg_next() & 7u));
    }
}

void FI_NotifyEvent(uint32_t feature)
{
    int bi = bit_index(feature);
    if (bi >= 0)
        ev_count[bi]++;
}

void FI_ArmNth(uint32_t feature, uint32_t nth)
{
    int bi = bit_index(feature);
    if (bi >= 0)
    {
        nth_arm[bi] = nth;
        nth_fired[bi] = false;
    }
}

void FI_ArmWindowMs(uint32_t feature, uint32_t start_ms, uint32_t end_ms)
{
    int bi = bit_index(feature);
    if (bi >= 0)
    {
        win_start_ms[bi] = start_ms;
        win_end_ms[bi]   = end_ms;
        win_armed[bi]    = true;
    }
}

static bool window_allows(uint32_t feature)
{
    int bi = bit_index(feature);
    if (bi < 0 || !win_armed[bi])
        return true;
    uint32_t now = FI_NowMs();
    return (now >= win_start_ms[bi]) && (now <= win_end_ms[bi]);
}

static bool nth_allows(uint32_t feature)
{
    int bi = bit_index(feature);
    if (bi < 0)
        return true;

    if (nth_arm[bi] == 0u)
        return true;

    if (nth_fired[bi])
        return false;

    if (ev_count[bi] == nth_arm[bi])
    {
        nth_fired[bi] = true;
        return true;
    }

    return false;
}

bool FI_ShouldFire(uint32_t feature)
{
    if (!fi_en)
        return false;
    if ((fi_mask & feature) == 0u)
        return false;
    if (!window_allows(feature))
        return false;
    if (!nth_allows(feature))
        return false;

    return (FI_RandPct() < fi_prob);
}

/* ==== Exception injections (RT-2 uses these) ==== */
void FI_Inject_DivByZero(void)
{
    SCB->CCR |= SCB_CCR_DIV_0_TRP_Msk;
    volatile int x = 1;
    volatile int y = 0;
    volatile int z = x / y;
    (void)z;
}

void FI_Inject_UnalignedAccess(void)
{
    SCB->CCR |= SCB_CCR_UNALIGN_TRP_Msk;
    uint32_t __attribute__((aligned(4))) word = 0x12345678u;
    uint8_t *pb = (uint8_t *)&word;
    volatile uint32_t bad = *(uint32_t *)(pb + 1);
    (void)bad;
}

extern void FI_MPU_SetupDenyRegion(void);
extern uint8_t g_fi_deny_buf[32];

void FI_Inject_MemFault(void)
{
    FI_MPU_SetupDenyRegion();
    volatile uint32_t *p = (uint32_t *)g_fi_deny_buf;
    volatile uint32_t v  = *p;
    (void)v;
}

### source/fi/uart_fi_shim.h

#ifndef UART_FI_SHIM_H
#define UART_FI_SHIM_H

#include <stddef.h>
#include <stdint.h>
#include "fsl_lpuart.h"

status_t UART_FI_WriteBlocking(LPUART_Type *base, const uint8_t *data, size_t length);

#endif

### source/fi/uart_fi_shim.c

#include "uart_fi_shim.h"
#include "fi.h"

#define UART_FI_MAX_COPY 128u

status_t UART_FI_WriteBlocking(LPUART_Type *base, const uint8_t *data, size_t length)
{
    /* Inject: busy return */
    FI_POINT(FI_F_UART_TX_BUSY, return kStatus_LPUART_TxBusy;);

    /* Inject: corruption (copy to scratch to avoid writing into const buffers) */
    if (FI_ENABLE && FI_ShouldFire(FI_F_UART_CORRUPT) && (length <= UART_FI_MAX_COPY))
    {
        uint8_t scratch[UART_FI_MAX_COPY];
        for (size_t i = 0; i < length; i++)
            scratch[i] = data[i];

        FI_BitFlipRange(scratch, (uint32_t)length, 7);
        return LPUART_WriteBlocking(base, scratch, length);
    }

    return LPUART_WriteBlocking(base, data, length);
}

### source/fi/arinc429.h

#ifndef ARINC429_H
#define ARINC429_H

#include <stdint.h>
#include <stdbool.h>

#include "fsl_lpuart.h"
#include "fi.h"

typedef struct {
    uint8_t  label;
    uint8_t  sdi;
    uint32_t data;
    uint8_t  ssm;
} arinc429_word_t;

uint32_t ARINC429_Pack(const arinc429_word_t *w, bool force_bad_parity);
status_t ARINC429_SendWord(LPUART_Type *base, const arinc429_word_t *w);

#endif

### source/fi/arinc429.c

#include "arinc429.h"

#include "fi/uart_fi_shim.h"

static uint8_t crc8(const uint8_t *d, uint32_t n)
{
    uint8_t c = 0x00u;
    for (uint32_t i = 0; i < n; i++)
    {
        uint8_t b = d[i];
        for (int k = 0; k < 8; k++)
        {
            uint8_t mix = (uint8_t)((c ^ b) & 0x01u);
            c >>= 1;
            if (mix)
                c ^= 0x8Cu;
            b >>= 1;
        }
    }
    return c;
}

static uint8_t odd_parity_31(uint32_t v31)
{
    uint32_t x = v31;
    x ^= x >> 16;
    x ^= x >> 8;
    x ^= x >> 4;
    x &= 0xFu;
    uint8_t parity = (uint8_t)((0x6996u >> x) & 1u);
    return (parity == 0u) ? 1u : 0u;
}

uint32_t ARINC429_Pack(const arinc429_word_t *w, bool force_bad_parity)
{
    uint32_t v = 0u;
    v |= ((uint32_t)(w->label) & 0xFFu) << 0;
    v |= ((uint32_t)(w->sdi) & 0x03u) << 8;
    v |= ((uint32_t)(w->data) & 0x7FFFFu) << 10;
    v |= ((uint32_t)(w->ssm) & 0x03u) << 29;

    uint8_t p = odd_parity_31(v);
    if (force_bad_parity)
        p ^= 1u;
    v |= ((uint32_t)p) << 31;

    return v;
}

status_t ARINC429_SendWord(LPUART_Type *base, const arinc429_word_t *w)
{
    bool bad_parity = false;
    arinc429_word_t temp = *w;

    /* FI #1: label substitution */
    FI_POINT(FI_F_ARINC_LABEL, temp.label ^= 0x1Fu;);

    /* FI #2: parity toggle */
    FI_POINT(FI_F_ARINC_PARITY, bad_parity = true;);

    uint32_t word = ARINC429_Pack(&temp, bad_parity);

    /* Frame: [0xA5][word LSB..MSB][CRC8(word bytes)] */
    uint8_t frame[1 + 4 + 1];
    frame[0] = 0xA5u;
    frame[1] = (uint8_t)(word & 0xFFu);
    frame[2] = (uint8_t)((word >> 8) & 0xFFu);
    frame[3] = (uint8_t)((word >> 16) & 0xFFu);
    frame[4] = (uint8_t)((word >> 24) & 0xFFu);
    frame[5] = crc8(&frame[1], 4);

    /* Optional: burst noise */
    FI_POINT(FI_F_UART_CORRUPT, FI_BitFlipRange(frame, sizeof(frame), 2););

    return UART_FI_WriteBlocking(base, frame, sizeof(frame));
}

### RX Analyzer: source/hello_world.c (project RX_analyzer)

#include "fsl_device_registers.h"
#include "fsl_debug_console.h"
#include "board.h"
#include "app.h"

#include "fsl_common.h"
#include "fsl_lpuart.h"

#define LINK_UART LPUART3

static void LinkUart_Init115200(void)
{
    lpuart_config_t cfg;
    LPUART_GetDefaultConfig(&cfg);
    cfg.baudRate_Bps = 115200U;
    cfg.enableTx     = true;
    cfg.enableRx     = true;
    (void)LPUART_Init(LINK_UART, &cfg, BOARD_DebugConsoleSrcFreq());
}

static uint8_t crc8(const uint8_t *d, uint32_t n)
{
    uint8_t c = 0x00u;
    for (uint32_t i = 0; i < n; i++)
    {
        uint8_t b = d[i];
        for (int k = 0; k < 8; k++)
        {
            uint8_t mix = (uint8_t)((c ^ b) & 0x01u);
            c >>= 1;
            if (mix)
                c ^= 0x8Cu;
            b >>= 1;
        }
    }
    return c;
}

static bool arinc_odd_parity_ok(uint32_t w)
{
    /* overall odd parity check (including bit31 parity bit) */
    uint32_t x = w;
    x ^= x >> 16;
    x ^= x >> 8;
    x ^= x >> 4;
    x &= 0xFu;
    uint8_t parity32 = (uint8_t)((0x6996u >> x) & 1u);
    return (parity32 == 1u);
}

int main(void)
{
    BOARD_InitHardware();
    LinkUart_Init115200();

    PRINTF("\r\nRX analyzer start (listening on LPUART3)\r\n");

    uint32_t ok_tlm = 0, bad_tlm_crc = 0;
    uint32_t ok_arinc = 0, bad_arinc_crc = 0, bad_arinc_parity = 0, bad_arinc_label = 0;

    while (1)
    {
        uint8_t sof;
        (void)LPUART_ReadBlocking(LINK_UART, &sof, 1);

        if (sof == 0x55u)
        {
            uint8_t body[4 + 8 + 1];
            (void)LPUART_ReadBlocking(LINK_UART, body, sizeof(body));
            uint8_t c = crc8(&body[0], 4 + 8);
            if (c != body[12])
                bad_tlm_crc++;
            else
                ok_tlm++;
        }
        else if (sof == 0xA5u)
        {
            uint8_t body[4 + 1];
            (void)LPUART_ReadBlocking(LINK_UART, body, sizeof(body));

            uint8_t c = crc8(&body[0], 4);
            if (c != body[4])
            {
                bad_arinc_crc++;
            }
            else
            {
                uint32_t w = ((uint32_t)body[0]) |
                             ((uint32_t)body[1] << 8) |
                             ((uint32_t)body[2] << 16) |
                             ((uint32_t)body[3] << 24);

                uint8_t label = (uint8_t)(w & 0xFFu);

                if (label != 0x12u)
                    bad_arinc_label++;
                else if (!arinc_odd_parity_ok(w))
                    bad_arinc_parity++;
                else
                    ok_arinc++;
            }
        }

        /* Print a snapshot every ~500 frames */
        if (((ok_tlm + bad_tlm_crc + ok_arinc + bad_arinc_crc + bad_arinc_parity + bad_arinc_label) % 500u) == 0u)
        {
            PRINTF("TLM ok=%u crc=%u | ARINC ok=%u crc=%u parity=%u label=%u\r\n",
                   (unsigned)ok_tlm, (unsigned)bad_tlm_crc,
                   (unsigned)ok_arinc, (unsigned)bad_arinc_crc,
                   (unsigned)bad_arinc_parity, (unsigned)bad_arinc_label);
        }
    }
}

## 4) Project skeleton (CT‑1)

### CT1_TX_callsite_shim

source/
  hello_world.c
  pin_mux.c           (edited: enable LPUART3 on Arduino D0/D1)
  fi/
    fi_config.h
    fi.h
    fi.c
    uart_fi_shim.h
    uart_fi_shim.c
    arinc429.h
    arinc429.c

### RX_analyzer

source/
  hello_world.c
  pin_mux.c           (edited: enable LPUART3 on Arduino D0/D1)

# EXERCISE CT‑2 — Compile‑time FI via GNU linker –wrap (no call-site edits)

## 1) Specification

### Objective

Inject the same UART faults as CT‑1, but without changing application call sites.

### Compile‑time method

- GNU linker: -Wl,--wrap=LPUART_WriteBlocking

### Faults to inject

- Return kStatus_LPUART_TxBusy sometimes.

- Corrupt transmit buffer sometimes.

### Pass/Fail

- PASS if wrapper is clearly active in FI build and inactive in PROD build.

## 2) Copy‑paste ready steps

### Project

Create project: - CT2_TX_linkwrap

How: - Copy/paste hello_world to CT2_TX_linkwrap - Apply pinmux edit (enable LPUART3 TX/RX on Arduino D1/D0)

### Build configs

- PROD: no flags

- FI:

  - Define FI_ENABLE=1

  - Add linker flag -Wl,--wrap=LPUART_WriteBlocking

MCUXpresso: - Project → Properties → C/C++ Build → Settings → MCU Linker → Miscellaneous → Other linker flags - Add: -Wl,--wrap=LPUART_WriteBlocking

### Files to add

Add folder source/fi/ and copy these exact files from CT‑1: - fi_config.h, fi.h, fi.c, arinc429.h, arinc429.c

Add new wrapper file: - source/fi/wrap_lpuart_writeblocking.c

Replace source/hello_world.c with CT‑2 TX main below.

## 3) Copy‑paste ready C code

### CT‑2 TX: source/hello_world.c

#include "fsl_device_registers.h"
#include "fsl_debug_console.h"
#include "board.h"
#include "app.h"

#include "fsl_common.h"
#include "fsl_lpuart.h"

#include "fi/fi.h"
#include "fi/arinc429.h"

#define LINK_UART LPUART3

static void LinkUart_Init115200(void)
{
    lpuart_config_t cfg;
    LPUART_GetDefaultConfig(&cfg);
    cfg.baudRate_Bps = 115200U;
    cfg.enableTx     = true;
    cfg.enableRx     = true;
    (void)LPUART_Init(LINK_UART, &cfg, BOARD_DebugConsoleSrcFreq());
}

int main(void)
{
    BOARD_InitHardware();
    LinkUart_Init115200();

    PRINTF("\r\nCT-2 TX (--wrap LPUART_WriteBlocking) start\r\n");

    FI_Init();

#if FI_ENABLE
    FI_SetEnabled(true);
    FI_SetMask(FI_F_UART_TX_BUSY | FI_F_UART_CORRUPT | FI_F_ARINC_PARITY | FI_F_ARINC_LABEL);
    FI_SetProbability(5);
#else
    FI_SetEnabled(false);
#endif

    arinc429_word_t aw = { .label = 0x12u, .sdi = 1u, .data = 0x12345u, .ssm = 2u };

    while (1)
    {
        status_t st = ARINC429_SendWord(LINK_UART, &aw);
        if (st == kStatus_LPUART_TxBusy)
            PRINTF("TX: busy (from wrapper), retrying\r\n");

        SDK_DelayAtLeastUs(10000u, SystemCoreClock);
    }
}

### CT‑2 wrapper: source/fi/wrap_lpuart_writeblocking.c

#include "fsl_lpuart.h"
#include "fi.h"

extern status_t __real_LPUART_WriteBlocking(LPUART_Type *base, const uint8_t *data, size_t length);

status_t __wrap_LPUART_WriteBlocking(LPUART_Type *base, const uint8_t *data, size_t length)
{
    FI_POINT(FI_F_UART_TX_BUSY, return kStatus_LPUART_TxBusy;);

    if (FI_ENABLE && FI_ShouldFire(FI_F_UART_CORRUPT) && length <= 128u)
    {
        uint8_t scratch[128];
        for (size_t i = 0; i < length; i++)
            scratch[i] = data[i];

        FI_BitFlipRange(scratch, (uint32_t)length, 11);
        return __real_LPUART_WriteBlocking(base, scratch, length);
    }

    return __real_LPUART_WriteBlocking(base, data, length);
}

RX analyzer project from CT‑1 is reused unchanged.

## 4) Project skeleton (CT‑2)

source/
  hello_world.c
  pin_mux.c
  fi/
    fi_config.h
    fi.h
    fi.c
    arinc429.h
    arinc429.c
    wrap_lpuart_writeblocking.c

# EXERCISE RT‑1 — Runtime FI policy + debug console CLI (time & event triggers + seed)

## 1) Specification

### Objective

Runtime SWIFI controls (without reflashing): - enable/disable FI - set mask - set probability - set seed (repeatable) - inject only on Nth event and in a time window

### Required runtime triggers

- Nth event: inject on the 10th ARINC transmit

- Time window: inject only within [2000ms..2100ms] from boot

### Pass/Fail

- PASS if using the same seed reproduces the same injection pattern after reset.

## 2) Copy‑paste ready steps

### Project

Create project: - RT1_TX_cli_policy

How: - Copy/paste hello_world → rename - Apply pinmux edit (enable LPUART3 on Arduino D0/D1)

### Build config

For RT‑1 you want FI engine included: - Define FI_ENABLE=1 (single config is fine)

### Files to add

Create source/fi/ and add: - fi_config.h, fi.h, fi.c, uart_fi_shim.h, uart_fi_shim.c, arinc429.h, arinc429.c - Add: fi_cli.c (below)

Replace source/hello_world.c with RT‑1 main below.

## 3) Copy‑paste ready C code

### RT‑1 TX: source/hello_world.c

#include "fsl_device_registers.h"
#include "fsl_debug_console.h"
#include "board.h"
#include "app.h"

#include "fsl_common.h"
#include "fsl_lpuart.h"

#include "fi/fi.h"
#include "fi/uart_fi_shim.h"
#include "fi/arinc429.h"

void FI_CLI_Poll(void); /* from fi_cli.c */

#define LINK_UART LPUART3

static void LinkUart_Init115200(void)
{
    lpuart_config_t cfg;
    LPUART_GetDefaultConfig(&cfg);
    cfg.baudRate_Bps = 115200U;
    cfg.enableTx     = true;
    cfg.enableRx     = true;
    (void)LPUART_Init(LINK_UART, &cfg, BOARD_DebugConsoleSrcFreq());
}

int main(void)
{
    BOARD_InitHardware();
    LinkUart_Init115200();

    PRINTF("\r\nRT-1 TX (runtime policy + CLI) start\r\n");
    PRINTF("Type: help\r\n");

    FI_Init();

    /* Build includes FI. Start disabled; enable via CLI. */
    FI_SetEnabled(false);
    FI_SetMask(0xFFFFFFFFu);
    FI_SetProbability(5);
    FI_SetSeed(0x0000BEEFu);

    /* Required triggers (can also arm via CLI): */
    FI_ArmNth(FI_F_ARINC_PARITY, 10);
    FI_ArmWindowMs(FI_F_ARINC_PARITY, 2000u, 2100u);

    arinc429_word_t aw = { .label = 0x12u, .sdi = 1u, .data = 0x12345u, .ssm = 2u };

    uint32_t tx_ctr = 0;

    while (1)
    {
        FI_CLI_Poll();

        /* Event counting for Nth trigger */
        FI_NotifyEvent(FI_F_ARINC_PARITY);

        status_t st = ARINC429_SendWord(LINK_UART, &aw);
        if (st == kStatus_LPUART_TxBusy)
            PRINTF("TX: busy\r\n");

        if ((tx_ctr % 100u) == 0u)
        {
            PRINTF("t=%ums tx=%u FI=%d mask=0x%08x pct=%u\r\n",
                   (unsigned)FI_NowMs(), (unsigned)tx_ctr, FI_IsEnabled(),
                   (unsigned)FI_GetMask(), (unsigned)FI_GetProbability());
        }

        tx_ctr++;
        SDK_DelayAtLeastUs(10000u, SystemCoreClock);
    }
}

### RT‑1 CLI: source/fi/fi_cli.c

#include "fsl_debug_console.h"
#include <string.h>
#include <stdlib.h>

#include "fi.h"

static char line[80];
static unsigned idx = 0;

static bool Console_TryReadChar(char *out)

{

int c = DbgConsole_Getchar();   /* returns -1 if no char available */

if (c < 0)

{

return false;

}

*out = (char)c;

return true;

}

static uint32_t parse_hex(const char *s)
{
    return (uint32_t)strtoul(s ? s : "0", NULL, 16);
}

static uint32_t feature_from_name(const char *s)
{
    if (!s) return 0;
    if (!strcmp(s, "UART_TX_BUSY")) return FI_F_UART_TX_BUSY;
    if (!strcmp(s, "UART_CORRUPT")) return FI_F_UART_CORRUPT;
    if (!strcmp(s, "ARINC_PARITY")) return FI_F_ARINC_PARITY;
    if (!strcmp(s, "ARINC_LABEL"))  return FI_F_ARINC_LABEL;
    if (!strcmp(s, "MEM_BITFLIP"))  return FI_F_MEM_BITFLIP;
    if (!strcmp(s, "EXCEPTIONS"))   return FI_F_EXCEPTIONS;
    return 0;
}

static void print_help(void)
{
    PRINTF("Commands:\r\n");
    PRINTF("  help\r\n");
    PRINTF("  en 0|1\r\n");
    PRINTF("  msk HEX\r\n");
    PRINTF("  pct 0..100\r\n");
    PRINTF("  seed HEX\r\n");
    PRINTF("  nth FEATURE N\r\n");
    PRINTF("  win FEATURE START_MS END_MS\r\n");
}

void FI_CLI_Poll(void)
{
    char ch;
    if (Console_TryReadChar(&ch) != kStatus_Success)
        return;

    if (ch == '\r' || ch == '\n')
    {
        line[idx] = 0;
        idx = 0;

        char *cmd = strtok(line, " ");
        if (!cmd) return;

        if (!strcmp(cmd, "help")) { print_help(); return; }

        if (!strcmp(cmd, "en"))
        {
            char *a = strtok(NULL, " ");
            FI_SetEnabled(a && a[0] == '1');
            PRINTF("FI enabled=%d\r\n", FI_IsEnabled());
            return;
        }

        if (!strcmp(cmd, "msk"))
        {
            char *a = strtok(NULL, " ");
            FI_SetMask(parse_hex(a));
            PRINTF("mask=0x%08x\r\n", (unsigned)FI_GetMask());
            return;
        }

        if (!strcmp(cmd, "pct"))
        {
            char *a = strtok(NULL, " ");
            FI_SetProbability((uint8_t)atoi(a ? a : "0"));
            PRINTF("pct=%u\r\n", (unsigned)FI_GetProbability());
            return;
        }

        if (!strcmp(cmd, "seed"))
        {
            char *a = strtok(NULL, " ");
            FI_SetSeed(parse_hex(a));
            PRINTF("seed set\r\n");
            return;
        }

        if (!strcmp(cmd, "nth"))
        {
            char *f = strtok(NULL, " ");
            char *n = strtok(NULL, " ");
            uint32_t feat = feature_from_name(f);
            FI_ArmNth(feat, (uint32_t)atoi(n ? n : "0"));
            PRINTF("nth armed\r\n");
            return;
        }

        if (!strcmp(cmd, "win"))
        {
            char *f = strtok(NULL, " ");
            char *s = strtok(NULL, " ");
            char *e = strtok(NULL, " ");
            uint32_t feat = feature_from_name(f);
            FI_ArmWindowMs(feat,
                           (uint32_t)atoi(s ? s : "0"),
                           (uint32_t)atoi(e ? e : "0"));
            PRINTF("window armed\r\n");
            return;
        }

        PRINTF("? (type help)\r\n");
        return;
    }

    if (idx < (sizeof(line) - 1))
        line[idx++] = ch;
}

## 4) Project skeleton (RT‑1)

source/
  hello_world.c
  pin_mux.c
  fi/
    fi_config.h
    fi.h
    fi.c
    uart_fi_shim.h
    uart_fi_shim.c
    arinc429.h
    arinc429.c
    fi_cli.c

# EXERCISE RT‑2 — Runtime exception injection + MPU MemManage + deterministic reset

## 1) Specification

### Objective

Deliberately inject: - UsageFault: divide-by-zero trap - UsageFault: unaligned access trap - MemManage fault: MPU no-access region read

### Required handler behavior

For each exception: - Print a short signature (UsageFault, MemManage, etc.) - Reset deterministically (NVIC_SystemReset() is acceptable for lab)

### Pass/Fail

- PASS if each injection lands in the correct handler and the board reboots cleanly.

## 2) Copy‑paste ready steps

### Project

Create project: - RT2_TX_exceptions_mpu

How: - Copy/paste hello_world → rename - Apply pinmux edit (enable LPUART3 on Arduino D0/D1)

### Build config

- Define FI_ENABLE=1

### Files to add

Create source/fi/ and add: - fi_config.h, fi.h, fi.c, uart_fi_shim.h, uart_fi_shim.c, arinc429.h, arinc429.c, fi_cli.c (from RT‑1) - Add: fi_mpu.c - Add: fault_handlers.c

Replace source/hello_world.c with RT‑2 main below.

## 3) Copy‑paste ready C code

### RT‑2 TX: source/hello_world.c

#include "fsl_device_registers.h"
#include "fsl_debug_console.h"
#include "board.h"
#include "app.h"

#include "fsl_common.h"
#include "fsl_lpuart.h"

#include "fi/fi.h"
#include "fi/uart_fi_shim.h"
#include "fi/arinc429.h"

void FI_CLI_Poll(void); /* from fi_cli.c */

#define LINK_UART LPUART3

static void LinkUart_Init115200(void)
{
    lpuart_config_t cfg;
    LPUART_GetDefaultConfig(&cfg);
    cfg.baudRate_Bps = 115200U;
    cfg.enableTx     = true;
    cfg.enableRx     = true;
    (void)LPUART_Init(LINK_UART, &cfg, BOARD_DebugConsoleSrcFreq());
}

int main(void)
{
    BOARD_InitHardware();
    LinkUart_Init115200();

    PRINTF("\r\nRT-2 TX (exceptions + MPU) start\r\n");
    PRINTF("Use CLI from RT-1 for FI control.\r\n");

    FI_Init();

    FI_SetEnabled(false);
    FI_SetMask(0xFFFFFFFFu);
    FI_SetProbability(100); /* make exception injections obvious when enabled */

    arinc429_word_t aw = { .label = 0x12u, .sdi = 1u, .data = 0x12345u, .ssm = 2u };

    while (1)
    {
        FI_CLI_Poll();

        /* Keep sending ARINC-like traffic */
        (void)ARINC429_SendWord(LINK_UART, &aw);

        /* Exception injection points (enable FI and set mask EXCEPTIONS when ready) */
        FI_POINT(FI_F_EXCEPTIONS, {
            PRINTF("Inject divide-by-zero (UsageFault expected)\r\n");
            FI_Inject_DivByZero();
        });

        FI_POINT(FI_F_EXCEPTIONS, {
            PRINTF("Inject unaligned access (UsageFault expected)\r\n");
            FI_Inject_UnalignedAccess();
        });

        FI_POINT(FI_F_EXCEPTIONS, {
            PRINTF("Inject MPU no-access read (MemManage expected)\r\n");
            FI_Inject_MemFault();
        });

        SDK_DelayAtLeastUs(200000u, SystemCoreClock); /* slow down so you see logs */
    }
}

### MPU deny region: source/fi/fi_mpu.c

#include "fsl_device_registers.h"
#include "m-profile/armv7m_mpu.h"
#include <stdint.h>

/* Protected buffer: we will create an MPU region that forbids access to this */
uint8_t __attribute__((aligned(32))) g_fi_deny_buf[32];

void FI_MPU_SetupDenyRegion(void)
{
    ARM_MPU_Disable();

    /* Use region 7 (arbitrary choice). Base address must be aligned to region size (32B). */
    ARM_MPU_SetRegionEx(
        7u,
        ARM_MPU_RBAR(7u, (uint32_t)g_fi_deny_buf),
        ARM_MPU_RASR(
            1u,               /* XN: execute never */
            ARM_MPU_AP_NONE,  /* no access */
            0u, 1u, 0u, 0u,   /* TEX=0, S=1, C=0, B=0 */
            0u,               /* subregion disable mask */
            ARM_MPU_REGION_SIZE_32B
        )
    );

    /* Enable MPU with default privileged memory map */
    ARM_MPU_Enable(MPU_CTRL_PRIVDEFENA_Msk);
}

### Fault handlers: source/fault_handlers.c

#include "fsl_debug_console.h"
#include "fsl_device_registers.h"

void HardFault_Handler(void)
{
    PRINTF("HardFault -> reset\r\n");
    NVIC_SystemReset();
}

void MemManage_Handler(void)
{
    PRINTF("MemManage -> reset\r\n");
    NVIC_SystemReset();
}

void BusFault_Handler(void)
{
    PRINTF("BusFault -> reset\r\n");
    NVIC_SystemReset();
}

void UsageFault_Handler(void)
{
    PRINTF("UsageFault -> reset\r\n");
    NVIC_SystemReset();
}

## 4) Project skeleton (RT‑2)

source/
  hello_world.c
  pin_mux.c
  fault_handlers.c
  fi/
    fi_config.h
    fi.h
    fi.c
    uart_fi_shim.h
    uart_fi_shim.c
    arinc429.h
    arinc429.c
    fi_cli.c
    fi_mpu.c

# Quick run scripts (what to type)

## CT‑1

- Flash CT1_TX_callsite_shim (FI build) on Board A

- Flash RX_analyzer on Board B

- Observe RX counters increasing and non-zero CRC/parity/label faults

- Rebuild Board A in PROD config and reflash → faults drop to ~0

## CT‑2

- Flash CT2_TX_linkwrap (FI build) on Board A

- Flash RX_analyzer on Board B

- Confirm wrapper behavior: occasional busy logs, and RX shows faults

## RT‑1

Flash RT1_TX_cli_policy on Board A, then on debug console: - help - en 1 - seed BEEF - pct 10 - msk FFFFFFFF - nth ARINC_PARITY 10 - win ARINC_PARITY 2000 2100 Reset and re-enter the same seed → pattern repeats.

## RT‑2

Flash RT2_TX_exceptions_mpu on Board A: - en 1 - msk 00000020 (EXCEPTIONS bit) - Each injection should log the expected handler then reboot.