# Footprint-First UART App — Deliverables Bundle

This bundle matches the deliverables checklist for the Footprint-First UART App (i.MX RT1050 EVKB) solution.

Because map/size/symbol outputs depend on your local build artifacts (ELF + map generated by MCUXpresso), this document provides: - ready-to-paste file contents for README_FOOTPRINT.md and diff_summary.txt - exact commands to generate *_size.txt and largest_symbols_*.txt - where/how to obtain baseline.map and optimized.map

Folder convention: create a folder at the repo root (or project root) named footprint_artifacts/ and put all deliverables inside it.

## 1) README_FOOTPRINT.md (deliverable)

Create footprint_artifacts/README_FOOTPRINT.md with the following content:

# Footprint-First UART App (EVKB-IMXRT1050) — Footprint Evidence

## Project
- Base SDK example: `boards/evkbimxrt1050/driver_examples/lpuart/interrupt_transfer`
- Solution applied:
  - Added `footprint_config.h`
  - Replaced `lpuart_interrupt_transfer.c`
  - Patched `board.h` and `board.c` to compile-out debug console when disabled

## Build configurations
### Baseline
- Configuration name: Debug (original SDK example, unmodified flags)
- Output ELF: <PATH_TO_BASELINE_ELF>
- Output MAP: baseline.map

### Optimized
- Configuration name: Release_Footprint (size-optimized)
- Output ELF: <PATH_TO_OPTIMIZED_ELF>
- Output MAP: optimized.map

## Size results (from arm-none-eabi-size -A -d)
### Baseline
See: `baseline_size.txt`

### Optimized
See: `optimized_size.txt`

## Largest symbols (from arm-none-eabi-nm --size-sort ...)
### Baseline
See: `largest_symbols_baseline.txt`

### Optimized
See: `largest_symbols_optimized.txt`

## Summary of footprint wins
Fill these after generating the outputs:

- Flash-related sections (.text + .rodata):
  - Baseline: <BASELINE_TEXT_PLUS_RODATA_BYTES>
  - Optimized: <OPTIMIZED_TEXT_PLUS_RODATA_BYTES>
  - Delta: <DELTA_BYTES> bytes

- RAM-related sections (.data + .bss):
  - Baseline: <BASELINE_DATA_PLUS_BSS_BYTES>
  - Optimized: <OPTIMIZED_DATA_PLUS_BSS_BYTES>
  - Delta: <DELTA_BYTES> bytes

## Verification notes
- Debug console disabled in release build (prevents PRINTF/DbgConsole pull-in)
- No malloc/free used by the application code
- RX/TX staging buffers placed with `AT_NONCACHEABLE_SECTION_ALIGN(..., 32)` and verified in map file

## How to reproduce
1. Build Baseline (Debug) and collect artifacts with the commands in `collect_artifacts.sh`
2. Build Optimized (Release_Footprint) and collect artifacts with the same commands
3. Compare `baseline_size.txt` vs `optimized_size.txt`, plus map files

## 2) diff_summary.txt (deliverable)

Create footprint_artifacts/diff_summary.txt with the following content:

Footprint-First UART App — Diff Summary

A) Build flags (Optimized / Release_Footprint)
- Added compiler flags:
  - -Os
  - -ffunction-sections
  - -fdata-sections
- Added linker flags:
  - -Wl,--gc-sections
  - -Wl,--cref
- Switched to nano libc:
  - -specs=nano.specs

B) Debug console removal
- In footprint release builds:
  - SDK_DEBUGCONSOLE set to DISABLE (2U)
  - BOARD_InitDebugConsole() compiles to a no-op
  - Application does not call PRINTF/DbgConsole; uses LPUART TX directly

C) Code footprint changes in application
- Uses static storage only; no heap
- Short tip string (optional compile-time feature)
- Minimal copy routine instead of libc memcpy
- Feature gates:
  - FP_FEATURE_TIPSTRING
  - FP_FEATURE_ECHO
  - FP_FEATURE_LED_HEARTBEAT
  - FP_VERBOSE_LOG (forced 0 for release)

D) Buffer placement
- RX/TX buffers declared with AT_NONCACHEABLE_SECTION_ALIGN(..., 32)
- Verified in optimized.map: locate symbols `s_rxBuffer` and `s_txBuffer` in the expected non-cacheable region/section

## 3) baseline.map and optimized.map (deliverables)

### How to generate/locate MAP files in MCUXpresso

- Enable MAP generation in each build config:

  - Project Properties → C/C++ Build → Settings

  - Tool Settings → MCU Linker → Miscellaneous

  - Ensure a -Wl,-Map,<name>.map (or equivalent UI option) is enabled.

- Build the project.

- In the project’s build output folder (varies by workspace/config), locate the .map file.

- Copy/rename into deliverables folder:

- footprint_artifacts/baseline.map

- footprint_artifacts/optimized.map

### Map-file verification checklist

In each map file, confirm: - .text and .rodata sizes and placement (Flash/XIP) - .data initialization region and size - .bss size - Search for buffer symbols: - s_rxBuffer - s_txBuffer Confirm their section placement matches your non-cacheable intent.

## 4) baseline_size.txt and optimized_size.txt (deliverables)

Create these by running arm-none-eabi-size on the corresponding ELF.

### Commands

From a terminal where arm-none-eabi-* is on PATH:

arm-none-eabi-size -A -d <PATH_TO_BASELINE_ELF>  > footprint_artifacts/baseline_size.txt
arm-none-eabi-size -A -d <PATH_TO_OPTIMIZED_ELF> > footprint_artifacts/optimized_size.txt

### What to look for

- Flash sections: .text, .rodata

- RAM sections: .data, .bss

## 5) largest_symbols_baseline.txt and largest_symbols_optimized.txt (deliverables)

Create these by running arm-none-eabi-nm with size-sort.

### Commands

arm-none-eabi-nm --size-sort -C -r <PATH_TO_BASELINE_ELF>  | tail -n 50 > footprint_artifacts/largest_symbols_baseline.txt
arm-none-eabi-nm --size-sort -C -r <PATH_TO_OPTIMIZED_ELF> | tail -n 50 > footprint_artifacts/largest_symbols_optimized.txt

### Interpretation tips

- If you still see printf/formatting-heavy symbols in the optimized list, debug console removal or libc flags are not fully applied.

- Compare top symbols before/after to identify where most flash was saved.

## 6) Optional helper script: collect_artifacts.sh

If you want a one-command collector, create footprint_artifacts/collect_artifacts.sh:

#!/usr/bin/env bash
set -euo pipefail

# Usage:
#   ./collect_artifacts.sh <baseline.elf> <optimized.elf> [baseline.map] [optimized.map]

BASE_ELF="${1:?baseline elf required}"
OPT_ELF="${2:?optimized elf required}"
BASE_MAP="${3:-}"
OPT_MAP="${4:-}"

mkdir -p footprint_artifacts

arm-none-eabi-size -A -d "$BASE_ELF" > footprint_artifacts/baseline_size.txt
arm-none-eabi-size -A -d "$OPT_ELF"  > footprint_artifacts/optimized_size.txt

arm-none-eabi-nm --size-sort -C -r "$BASE_ELF" | tail -n 50 > footprint_artifacts/largest_symbols_baseline.txt
arm-none-eabi-nm --size-sort -C -r "$OPT_ELF"  | tail -n 50 > footprint_artifacts/largest_symbols_optimized.txt

if [[ -n "$BASE_MAP" && -f "$BASE_MAP" ]]; then
  cp -f "$BASE_MAP" footprint_artifacts/baseline.map
fi
if [[ -n "$OPT_MAP" && -f "$OPT_MAP" ]]; then
  cp -f "$OPT_MAP" footprint_artifacts/optimized.map
fi

echo "Done. Deliverables created in footprint_artifacts/"

Make it executable:

chmod +x footprint_artifacts/collect_artifacts.sh

## 7) Final deliverables checklist (must exist on disk)

Inside footprint_artifacts/: - README_FOOTPRINT.md - diff_summary.txt - baseline.map - optimized.map - baseline_size.txt - optimized_size.txt - largest_symbols_baseline.txt - largest_symbols_optimized.txt - (optional) collect_artifacts.sh