# Lab Handout

## Footprint-First UART App (i.MX RT1050 EVKB)

Theme: Build a UART echo application that is measurably smaller in Flash and RAM than a baseline SDK example, using size-focused build settings and code choices typical of production embedded systems.

Outcome: At the end, you will have: - A working UART app on EVKB-IMXRT1050 (interrupt-driven) - A repeatable baseline vs optimized footprint comparison - A complete deliverables bundle (README, map files, size outputs, largest symbols)

## 1) Learning objectives

By completing this lab, you will be able to: 1. Use MCUXpresso SDK projects as a starting point and generate comparable builds. 2. Enable compiler/linker options that allow unused code/data to be removed. 3. Avoid common sources of flash bloat (printf/DebugConsole, heavy library pulls). 4. Validate memory placement using a linker map file. 5. Produce evidence (size reports + symbol lists) that proves footprint improvements.

## 2) Hardware and software prerequisites

### Hardware

- NXP EVKB-IMXRT1050 board

- USB cable for Debug/Power

- A host PC

### Software

- MCUXpresso IDE (installed)

- MCUXpresso SDK for EVKB-IMXRT1050 imported/available

- A serial terminal program (e.g., PuTTY, Tera Term, minicom)

### Tools in PATH (recommended)

- arm-none-eabi-size

- arm-none-eabi-nm

If these aren’t in PATH, you can use the IDE’s toolchain path or run commands from the MCUXpresso toolchain terminal.

## 3) What you start from

You will start from the SDK example:

- boards/evkbimxrt1050/driver_examples/lpuart/interrupt_transfer

This project typically: - initializes LPUART - uses interrupt-driven non-blocking transfer APIs - prints status using the SDK Debug Console (PRINTF)

## 4) What you will build

You will produce two builds:

### A) Baseline build

- Original example (or near-original)

- Typical debug settings

- Usually includes Debug Console usage

### B) Optimized build (Release_Footprint)

- Size-focused compile/link flags

- Dead-code elimination enabled

- Nano libc enabled

- Debug Console fully disabled

- Application avoids PRINTF entirely (uses LPUART TX directly)

- Static memory only (no heap)

## 5) Directory structure for lab artifacts

Create this folder at project root:

- footprint_artifacts/

All deliverables you generate in this lab go inside this folder.

## 6) Step-by-step procedure

### Step 1 — Import and build the baseline project

- Open MCUXpresso IDE.

- Import the SDK example project:

  - boards/evkbimxrt1050/driver_examples/lpuart/interrupt_transfer

- Build using the existing Debug configuration.

- Confirm you get a successful build and an output ELF.

#### Step 1.1 — Run baseline on hardware

- Connect the EVKB-IMXRT1050 via USB.

- Launch a serial terminal:

  - 115200 baud

  - 8-N-1

  - No flow control

- Debug/Run and observe console output.

At this point you have functional baseline behavior.

### Step 2 — Enable map file generation (baseline)

You must produce a map file for both baseline and optimized builds.

In Project Properties for Debug configuration: - C/C++ Build → Settings - Linker settings → enable map generation (often a UI checkbox or “Other flags” entry)

Rebuild the baseline.

Then copy the produced map file into: - footprint_artifacts/baseline.map

### Step 3 — Capture baseline size and symbol data

Go to Project Properties → C/C++ Build → Settings → Build Steps → Post-build steps and paste exactly these lines (one per line):

cmd /c "if not exist ""${workspace_loc:/${ProjName}}\footprint_artifacts"" mkdir ""${workspace_loc:/${ProjName}}\footprint_artifacts"""

cmd /c "echo POSTBUILD_RAN> ""${workspace_loc:/${ProjName}}\footprint_artifacts\_postbuild_probe.txt"""

cmd /c "where arm-none-eabi-size > ""${workspace_loc:/${ProjName}}\footprint_artifacts\_where_size.txt"" 2>&1"

cmd /c "where arm-none-eabi-nm   > ""${workspace_loc:/${ProjName}}\footprint_artifacts\_where_nm.txt"" 2>&1"

cmd /c "arm-none-eabi-size -A -d ""${workspace_loc:/${ProjName}}\Debug\${BuildArtifactFileName}"" > ""${workspace_loc:/${ProjName}}\footprint_artifacts\baseline_size.txt"" 2> ""${workspace_loc:/${ProjName}}\footprint_artifacts\_size_err.txt"""

cmd /c "arm-none-eabi-nm --size-sort -C -r ""${workspace_loc:/${ProjName}}\Debug\${BuildArtifactFileName}"" > ""${workspace_loc:/${ProjName}}\footprint_artifacts\largest_symbols_baseline.txt"" 2> ""${workspace_loc:/${ProjName}}\footprint_artifacts\_nm_err.txt"""

Then do:

- Project → Clean…

- Build Project

- Right-click project → Refresh (or press F5)

What you should see

At minimum:

- footprint_artifacts\_postbuild_probe.txt

If baseline_size.txt / largest_symbols_baseline.txt are still missing, open these two files to see the reason:

- footprint_artifacts\_where_size.txt

- footprint_artifacts\_where_nm.txt

### Step 4 — Create the optimized build configuration

In MCUXpresso: 1. Open Project Properties → C/C++ Build → Manage Configurations. 2. Clone/copy Debug configuration and name it: - Release_Footprint 3. In Release_Footprint, apply size-focused flags:

#### Step 4.1 — Compiler flags (Release_Footprint)

Enable: - -Os - -ffunction-sections - -fdata-sections

#### Step 4.2 — Linker flags (Release_Footprint)

Enable: - -Wl,--gc-sections - -Wl,--cref

#### Step 4.3 — Use nano libc

Enable:  -specs=nano.specs

Goal: allow the linker to discard anything unused, and reduce runtime library size.

### Step 5 — Apply the code-level solution

Apply the solution already provided in the code canvas:

- Add footprint_config.h

- Replace the example lpuart_interrupt_transfer.c with the footprint-first version

- Patch board.h and board.c as shown:

  - Ensure debug console init is compiled out when debug console is disabled.

Key design goals in the solution: - No PRINTF - No malloc/free - Minimal tip string - Feature gates to compile out optional behavior - RX/TX buffers declared with AT_NONCACHEABLE_SECTION_ALIGN(..., 32)

### Step 6 — Disable Debug Console for the optimized build

In Release_Footprint build settings, ensure debug console is disabled.

Typical approaches: - Provide a compile define SDK_DEBUGCONSOLE=2 (disable) - Or ensure the config header sets it for release builds

Rebuild the Release_Footprint configuration.

### Step 7 — Enable map file generation (optimized)

Just like baseline, ensure Release_Footprint produces a map file.

Copy it to: - footprint_artifacts/optimized.map

### Step 8 — Capture optimized size and symbol data

- Create optimized size report:

  - arm-none-eabi-size -A -d <PATH_TO_OPTIMIZED_ELF> > footprint_artifacts/optimized_size.txt

- Create optimized largest symbol list:

  - arm-none-eabi-nm --size-sort -C -r <PATH_TO_OPTIMIZED_ELF> | tail -n 50 > footprint_artifacts/largest_symbols_optimized.txt

### Step 9 — Fill out README and diff summary

In footprint_artifacts/: 1. Create README_FOOTPRINT.md using the provided template. 2. Create diff_summary.txt using the provided template.

In README: - Record baseline vs optimized .text + .rodata and .data + .bss. - Note the most important changes that drove reductions.

## 7) How to compare baseline vs optimized

### A) Compare section sizes (size reports)

Open baseline_size.txt and optimized_size.txt and compare: - .text - .rodata - .data - .bss

Expected direction: - .text and .rodata should go down in optimized. - .data should not increase.

### B) Compare biggest symbols (nm reports)

Open largest_symbols_baseline.txt and largest_symbols_optimized.txt.

What to look for: - Baseline often has large formatting/console/serial-manager symbols. - Optimized should drop those if debug console is removed.

### C) Verify non-cacheable buffer placement (map file)

Open optimized.map and search for: - s_rxBuffer - s_txBuffer

Confirm: - alignment intent (32) - section/region placement matches your non-cacheable memory expectations

Note: Depending on linker script, the macro may enforce alignment but section mapping may require additional linker script support.

## 8) Functional test checklist (on hardware)

Using your serial terminal at 115200: - You should see a short startup tip (if enabled) - When you type 8 bytes, the device echoes 8 bytes back (if echo enabled)

If echo is disabled (FP_FEATURE_ECHO=0): - device receives but does not echo (RX-only mode)

## 9) Troubleshooting guide

### Problem: Build fails with unresolved DebugConsole symbols

- Ensure:

  - Debug console is disabled for Release_Footprint

  - BOARD_InitDebugConsole() is compiled to a no-op when disabled

  - Your application code does not include/call PRINTF

### Problem: Size doesn’t improve much

- Ensure:

  - -ffunction-sections and -fdata-sections are enabled

  - -Wl,--gc-sections is enabled

  - -specs=nano.specs is applied

- Check largest_symbols_* outputs to see what remains large.

### Problem: UART output not visible

- Confirm serial port selection and settings

- Confirm the board uses the expected UART instance for the example

- Verify the terminal is 115200 8N1

### Problem: Map file not generated

- Confirm linker “Map file” option is enabled

- Confirm the map path is not write-protected

- Rebuild after changing linker options

## 10) Completion rubric (what “done” means)

You are done when: 1. Both builds run on the board (baseline and optimized). 2. footprint_artifacts/ contains: - README_FOOTPRINT.md - diff_summary.txt - baseline.map - optimized.map - baseline_size.txt - optimized_size.txt - largest_symbols_baseline.txt - largest_symbols_optimized.txt 3. README includes a clear statement of: - Flash delta (text+rodata) - RAM delta (data+bss) 4. Map file shows expected symbol placement for s_rxBuffer and s_txBuffer.

## 11) Optional extensions (if you want more practice)

- Further reduce .rodata:

  - Make the tip string optional and default-off in Release_Footprint.

- Switch to polling mode (if allowed) and compare:

  - fewer interrupt/handle symbols vs flexibility tradeoff.

- Add a minimal binary protocol:

  - send fixed-size frames without any string formatting.

- Add a compile-time “diagnostics” module:

  - verify it is fully discarded in Release_Footprint by linker GC.

## 12) What to submit

Submit the entire footprint_artifacts/ folder (plus your code changes in the project).