## Hands-on Exercise: “Footprint-First UART App” (i.MX RT1050 EVKB)

Goal: Starting from an NXP MCUXpresso SDK UART interrupt example, produce a measurably smaller Flash + RAM footprint build that is suitable for an avionics-style codebase: deterministic, minimal dependencies, and with clear evidence (map file + size output) of what changed and why.
(Reference doc: )

### 1) Starting point (provided code base)

- Import/open the MCUXpresso SDK example project:

  - Start project: boards/evkbimxrt1050/driver_examples/lpuart/interrupt_transfer

- Build it as-is in Debug configuration.

- Capture a baseline measurement of:

  - Flash sections: .text, .rodata

  - RAM sections: .data, .bss, plus any heap/stack allocations your build system reports

  - Linker map file for section placement verification

### 2) Required constraints (treat these as “avionics rules”)

Implement the optimization work under these constraints:

- No dynamic allocation during runtime (no “steady-state” malloc/free). If the example uses any heap, refactor to fixed storage so that runtime behavior is bounded.

- Add at least one compile-time switch that completely removes a feature (e.g., verbose logging, self-test mode, or debug tracing) from the production binary (so the linker can discard it).

- All large read-only tables/strings must be truly const so they remain in Flash/XIP and don’t inflate .data.

### 3) Footprint reduction tasks (what you must change)

#### A. Create a “RELEASE” build that enables dead-code elimination

In MCUXpresso, clone the Debug configuration into a new config (e.g., Release_Footprint), then apply:

- Compiler flags that favor size + section granularity:

  - -Os -ffunction-sections -fdata-sections ... (size-oriented, section-based)

- Linker flags enabling garbage collection of unused sections:

  - -Wl,--gc-sections -Wl,--cref

EXERCISE AGENDA: ensure the project actually produces a smaller binary and prove it with before/after measurements.

#### B. Switch to smaller libc and prevent accidental “printf bloat”

Apply:

- -specs=nano.specs to use a smaller C library variant

- Confirm you are not pulling float printf support unless explicitly required (don’t opt-in to float formatting unless you have a reason)

#### C. Remove the NXP debug console dependency from production

- Disable the SDK debug console:

  - #define SDK_DEBUGCONSOLE DEBUGCONSOLE_DISABLE (or equivalent build define)

- Replace any PRINTF() usage with a minimal output routine (or remove output entirely in the production configuration).

Acceptance intent: This step alone should remove a noticeable chunk of Flash usage because formatted printing pulls in heavy formatting code.

#### D. Make buffer placement explicit (DMA-safe, cache-aware)

Even if the starter UART example uses interrupts (not DMA), update at least one RX/TX staging buffer so it is:

- Placed in a non-cacheable region, and

- Aligned to cache line size (32 bytes),
using the SDK macros:

- AT_NONCACHEABLE_SECTION_ALIGN(var, 32)

Then prove via the map file that the object landed in the intended NonCacheable section.

### 4) Measurement and evidence requirements (non-negotiable)

Must produce a repeatable “evidence bundle”:

- Before/after size reports using arm-none-eabi-size -A -d <elf>

- A list of the largest symbols (top offenders) using:

  - arm-none-eabi-nm --size-sort -C -r <elf> | tail -n 50

- Two linker map files (baseline + optimized) and an explanation of:

  - Which sections shrank (.text, .rodata, .data, .bss)

  - Whether anything unexpectedly moved from Flash to RAM

- A short “configuration delta log” listing exactly what changed:

  - toolchain flags

  - debug console setting

  - any compile-time feature gates added

### 5) Success criteria (what “done” means)

Exercise considered successful if all of the following are true:

- Flash reduction: you can show a clear reduction in total Flash-related sections (primarily .text and .rodata) after applying -Os, --gc-sections, nano libc, and removing debug console.

- RAM control: .data does not grow, and any large tables/strings are verified to be in .rodata due to const usage.

- Determinism: no runtime heap usage remains (or it is strictly init-only and proven/documented, per your chosen rule set).

- Map-file proof: your non-cacheable aligned buffer is shown in the correct memory section in the map file.

- Evidence bundle produced: the “before vs after” artifacts are complete and repeatable.

### 6) Deliverables checklist (what you hand in)

- README_FOOTPRINT.md containing:

  - baseline sizes

  - optimized sizes

  - explanations of the key deltas

- baseline.map and optimized.map

- baseline_size.txt and optimized_size.txt

- largest_symbols_baseline.txt and largest_symbols_optimized.txt

- A short “diff summary” (bullet list) of what you changed in MCUXpresso project settings and code