# Hardware Fault Injection & Signal‑Glitch–Induced Resets

## Platform: NXP i.MX RT1050‑EVKB (Cortex‑M7) • MCUXpresso SDK 25.06.00

This canvas contains: 1) A handout that captures the essence of the EVKB hardware user guide (PDF) and the fault‑injection/reset module (DOCX). 2) ONE hands‑on exercise (single lab) with code‑level solutions, including a generic demo and an avionics‑style (ARINC‑429 recovery) simulation.

# Part A — Handout (Essence of the PDFs / DOCX)

## A1) EVKB board essentials (what matters for fault‑injection + resets)

### Power & bring‑up

- Power inputs: EVKB can be powered from the 5V DC jack (J2), or via other board power entry points depending on jumper selection.

- Power switch (SW1): ON connects main 5V to the board; OFF removes power.

- Power management sequence: SNVS/RTC domain powers first; then PMIC_REQ_ON enables the rest of the rails.

### Reset controls (you will use these in the lab)

- SW3 = Power‑On Reset (POR) button: Forces a system power reset (except SNVS domain). This is the closest thing to “board‑level power reset” without removing the supply.

- SW4 = Reset button: Standard reset button.

- Watchdog output can also reset the system (important when you enable RTWDOG).

### Debug and console

- OpenSDA (DAP‑Link / CMSIS‑DAP): single USB connection can provide debug + a virtual COM port.

- Debug console on this board is typically LPUART1 to OpenSDA (used by hello_world).

### Boot selection (only the practical bits)

- Boot mode is selected by SW7‑3 & SW7‑4 (BOOT_MODE bits).

- Typical boot device selection uses SW7‑1..SW7‑4 combinations.

### Arduino expansion header (useful for inter‑board links)

- Arduino header includes UART RX/TX (D0/D1) pins you can use for a second UART (we will map these to LPUART3 for the avionics simulation).

## A2) Fault injection & glitch‑induced resets (core ideas you will practice)

### Definitions (plain language)

- Hardware fault injection (HFI): applying (or encountering) physical disturbances—power droops, reset‑pin pulses, clock disturbance, EMI/EFT—that can push the MCU into unintended behavior.

- Signal glitch: short, unintended transient pulse on a signal (reset line, supply rails, clocks, or I/O) that can cause resets or faults.

### Reset taxonomy (how you’ll prove what happened)

On i.MX RT1050, the System Reset Controller (SRC) exposes sticky status flags that describe why the last reset happened. Typical sources: - Power‑On reset / power detection - External reset pins - Watchdog resets (including RTWDOG/WDOG3) - Software reset (SYSRESETREQ) - CPU lockup reset - JTAG‑initiated resets

### Persisting evidence across resets

- Use SRC GPR (General Purpose Registers) as tiny “sticky scratchpads” to store:

  - Monotonic boot count

  - Last reset flags

  - A small “marker” or sequence number

### Resilience patterns (what “good” looks like)

- Every reset is attributed and logged early in boot.

- Watchdog is windowed and only refreshed by healthy software paths.

- Boot‑loop detection can force a safe mode.

- In avionics‑style comms (ARINC‑429), recovery avoids corrupting label timing by restarting at safe boundaries and re‑handshaking downstream.

### Safety boundary

This lab is reliability and recovery only. Stay within component ratings. No security bypass attempts.

# Part B — One Hands‑On Exercise (with code‑level solutions)

## Exercise: “Fault Injection Telemetry + Windowed Watchdog + Avionics‑Style Recovery”

### What you will build

A single application (based on SDK hello_world) that: 1) Reads/prints reset cause from SRC on every boot. 2) Maintains a boot counter across resets (SRC GPR). 3) Demonstrates windowed RTWDOG with controlled fault injection. 4) (Optional, strongly recommended) Runs a two‑board avionics simulation: - Board A = “RDC” (Remote Data Concentrator) that streams scheduled “ARINC labels” over UART. - Board B = “Bridge/FCC” that acknowledges and checks for missing words. - When Board A resets (SW4/SW3 or watchdog), it re‑handshakes and resumes at label boundaries.

### Hardware you need

- 1 EVKB for the generic lab.

- 2 EVKBs for the avionics simulation (you have 4 boards, so you’re covered).

- 2 jumper wires + shared ground for UART link.

### Wiring for the avionics UART link (Board‑to‑Board)

We use LPUART3 on Arduino header pins: - Board A (RDC) TX → Board B (Bridge) RX - Board A (RDC) RX ← Board B (Bridge) TX - GND ↔ GND

Arduino header mapping (EVKB): - J22[2] = UART_TX (LPUART3_TXD) - J22[1] = UART_RX (LPUART3_RXD)

Important: Do NOT connect to the OpenSDA UART pins; those are already used for debug console.

## B1) MCUXpresso IDE steps (practical workflow)

### Step 1 — Install/point MCUXpresso to the SDK

- Extract the SDK zip somewhere on disk (or keep as a folder you can point to).

- In MCUXpresso, use the MCUXpresso SDK import workflow (varies slightly by version):

  - Import SDK example projects and select the EVKB IMXRT1050 board.

  - Choose demo_apps → hello_world as the base.

### Step 2 — Build and run hello_world first

- Confirm you can program + open the console (virtual COM from OpenSDA).

### Step 3 — Add the lab source files

- In the imported hello_world project:

  - Add the source files below into your project (for example under source/).

  - Add the #include and function calls shown in the “Integration” section.

### Step 4 — Build two variants (for the avionics sim)

- RDC build: define ARINC_ROLE_RDC=1 (and ARINC_ROLE_BRIDGE=0).

- BRIDGE build: define ARINC_ROLE_RDC=0 (and ARINC_ROLE_BRIDGE=1).

How to set a preprocessor define in MCUXpresso: - Project → Properties → C/C++ Build → Settings → Tool Settings → MCU C Compiler → Preprocessor - Add the macro under “Defined symbols”.

# Part C — Solution Code (drop‑in files)

These files use only MCUXpresso SDK drivers and CMSIS. They are designed to be dropped into the EVKB hello_world project.

## C1) reset_diag.h

#ifndef RESET_DIAG_H_
#define RESET_DIAG_H_

#include <stdint.h>

#ifdef __cplusplus
extern "C" {
#endif

typedef struct ResetDiag_Info
{
    uint32_t resetFlags;
    uint32_t bootCount;
} ResetDiag_Info_t;

/* Call once very early after clocks + debug console init. */
ResetDiag_Info_t ResetDiag_RunEarly(void);

/* Convert reset flags to a short human-readable string list. */
void ResetDiag_PrintFlags(uint32_t flags);

#ifdef __cplusplus
}
#endif

#endif /* RESET_DIAG_H_ */

## C2) reset_diag.c

#include "reset_diag.h"

#include "fsl_debug_console.h"
#include "fsl_src.h"

/* GPR indexes: index 0 == GPR1, index 1 == GPR2, etc. */
#define GPR_BOOTCOUNT_INDEX   (0U)
#define GPR_LASTFLAGS_INDEX   (1U)

static void print_if_set(uint32_t flags, uint32_t mask, const char *name)
{
    if ((flags & mask) != 0U)
    {
        PRINTF(" - %s\r\n", name);
    }
}

ResetDiag_Info_t ResetDiag_RunEarly(void)
{
    ResetDiag_Info_t info = {0};

    /* SRC reset status is sticky; read it first thing after console init. */
    const uint32_t flags = SRC_GetResetStatusFlags(SRC);

    uint32_t bootCount = SRC_GetGeneralPurposeRegister(SRC, GPR_BOOTCOUNT_INDEX);
    bootCount++;

    SRC_SetGeneralPurposeRegister(SRC, GPR_BOOTCOUNT_INDEX, bootCount);
    SRC_SetGeneralPurposeRegister(SRC, GPR_LASTFLAGS_INDEX, flags);

    info.resetFlags = flags;
    info.bootCount  = bootCount;

    PRINTF("\r\n=== Reset Diagnostics (SRC) ===\r\n");
    PRINTF("BootCount (SRC GPR1): %lu\r\n", (unsigned long)bootCount);
    PRINTF("ResetFlags (SRC SRSR): 0x%08lx\r\n", (unsigned long)flags);

    ResetDiag_PrintFlags(flags);

    /* Clear only the flags we observed. */
    if (flags != 0U)
    {
        SRC_ClearResetStatusFlags(SRC, flags);
    }

    return info;
}

void ResetDiag_PrintFlags(uint32_t flags)
{
    if (flags == 0U)
    {
        PRINTF(" - (no flags set)\r\n");
        return;
    }

    /* NOTE: Availability depends on silicon feature macros; these are the names used by this SDK. */
#ifdef kSRC_PowerOnResetFlag
    print_if_set(flags, kSRC_PowerOnResetFlag, "Power-On Reset (POR)");
#endif
#ifdef kSRC_IppResetPinFlag
    print_if_set(flags, kSRC_IppResetPinFlag, "IPP_RESET_B qualified reset (board-level reset path)");
#endif
#ifdef kSRC_IppUserResetFlag
    print_if_set(flags, kSRC_IppUserResetFlag, "IPP_USER_RESET_B (external reset pin event)");
#endif
#ifdef kSRC_Wdog3ResetFlag
    print_if_set(flags, kSRC_Wdog3ResetFlag, "RTWDOG / WDOG3 timeout reset");
#endif
#ifdef kSRC_WatchdogResetFlag
    print_if_set(flags, kSRC_WatchdogResetFlag, "WDOG1/WDOG2 timeout reset");
#endif
#ifdef kSRC_SoftwareResetFlag
    print_if_set(flags, kSRC_SoftwareResetFlag, "Software SYSRESETREQ reset");
#endif
#ifdef kSRC_CoreLockupResetFlag
    print_if_set(flags, kSRC_CoreLockupResetFlag, "Core LOCKUP reset");
#endif
#ifdef kSRC_LockupSysResetFlag
    print_if_set(flags, kSRC_LockupSysResetFlag, "LOCKUP or SYSRESETREQ (combined)");
#endif
#ifdef kSRC_JTAGSystemResetFlag
    print_if_set(flags, kSRC_JTAGSystemResetFlag, "JTAG system reset");
#endif
#ifdef kSRC_JTAGSoftwareResetFlag
    print_if_set(flags, kSRC_JTAGSoftwareResetFlag, "JTAG software reset (SJC)");
#endif
#ifdef kSRC_JTAGGeneratedResetFlag
    print_if_set(flags, kSRC_JTAGGeneratedResetFlag, "JTAG generated reset (EXTEST/HIGHZ)");
#endif
#ifdef kSRC_TemperatureSensorResetFlag
    print_if_set(flags, kSRC_TemperatureSensorResetFlag, "Temperature sensor software reset");
#endif
}

## C3) wdog_window_demo.h

#ifndef WDOG_WINDOW_DEMO_H_
#define WDOG_WINDOW_DEMO_H_

#include <stdbool.h>

#ifdef __cplusplus
extern "C" {
#endif

/*
 * Demonstrates windowed RTWDOG:
 *  - normal refresh for N cycles
 *  - optional early refresh violation
 *  - optional missed refresh timeout
 */
void WdogWindowDemo_Run(bool triggerEarlyViolation, bool triggerMissedRefresh);

#ifdef __cplusplus
}
#endif

#endif /* WDOG_WINDOW_DEMO_H_ */

## C4) wdog_window_demo.c

#include "wdog_window_demo.h"

#include "fsl_debug_console.h"
#include "fsl_rtwdog.h"
#include "fsl_common.h"

static void delay_ms(uint32_t ms)
{
    /* SDK helper: delays at least the requested time. */
    SDK_DelayAtLeastUs((uint64_t)ms * 1000ULL, SystemCoreClock);
}

void WdogWindowDemo_Run(bool triggerEarlyViolation, bool triggerMissedRefresh)
{
    rtwdog_config_t cfg;
    RTWDOG_GetDefaultConfig(&cfg);

    cfg.enableRtwdog      = true;
    cfg.enableUpdate      = true;
    cfg.enableInterrupt   = false;
    cfg.enableWindowMode  = true;

    /* Make debugging friendlier: allow halting without instant watchdog reset. */
    cfg.workMode.enableDebug = true;

    /*
     * Windowed watchdog behavior (counter counts down):
     *  - If you refresh too early (counter still above windowValue) => reset
     *  - If you refresh too late (counter reaches 0) => reset
     *
     * Choose conservative values for lab demonstration.
     */
    cfg.timeoutValue = 0xB000U; /* overall timeout */
    cfg.windowValue  = 0x6000U; /* earliest allowed refresh */

    PRINTF("\r\n[WDOG] Initializing RTWDOG windowed mode (window=0x%04x, timeout=0x%04x)\r\n",
           cfg.windowValue, cfg.timeoutValue);

    RTWDOG_Init(RTWDOG, &cfg);

    /* Normal operation: refresh inside the allowed window for a few cycles. */
    for (int i = 0; i < 5; i++)
    {
        /* Wait until we are inside the refresh window. */
        while (RTWDOG_GetCounterValue(RTWDOG) > (uint32_t)cfg.windowValue)
        {
            /* spin */
        }
        RTWDOG_Refresh(RTWDOG);
        PRINTF("[WDOG] Refreshed inside window (cycle %d)\r\n", i);
        delay_ms(50);
    }

    if (triggerEarlyViolation)
    {
        PRINTF("[WDOG] Triggering EARLY refresh violation now (expect reset) ...\r\n");
        /* Refresh immediately while counter is high -> violation. */
        RTWDOG_Refresh(RTWDOG);
        while (1) { /* should reset */ }
    }

    if (triggerMissedRefresh)
    {
        PRINTF("[WDOG] Triggering MISSED refresh timeout (expect reset) ...\r\n");
        /* Do not refresh anymore; just wait. */
        while (1)
        {
            delay_ms(200);
            PRINTF("[WDOG] waiting... CNT=0x%04lx\r\n", (unsigned long)RTWDOG_GetCounterValue(RTWDOG));
        }
    }

    PRINTF("[WDOG] Demo finished without forced reset.\r\n");
}

## C5) UART3 pins + UART3 init for the avionics simulation

### board_uart3_pins.h

#ifndef BOARD_UART3_PINS_H_
#define BOARD_UART3_PINS_H_

#ifdef __cplusplus
extern "C" {
#endif

/* Configure IOMUXC for LPUART3 on Arduino UART pins (J22[2]/J22[1]). */
void BOARD_InitUart3Pins(void);

#ifdef __cplusplus
}
#endif

#endif /* BOARD_UART3_PINS_H_ */

### board_uart3_pins.c

#include "board_uart3_pins.h"

#include "fsl_common.h"
#include "fsl_iomuxc.h"
#include "fsl_clock.h"

void BOARD_InitUart3Pins(void)
{
    CLOCK_EnableClock(kCLOCK_Iomuxc);

    /* Arduino header UART pins: J22[2]=TX, J22[1]=RX */
    IOMUXC_SetPinMux(IOMUXC_GPIO_AD_B1_06_LPUART3_TXD, 0U);
    IOMUXC_SetPinMux(IOMUXC_GPIO_AD_B1_07_LPUART3_RXD, 0U);

    /* Match the style used by the SDK examples for LPUART pin config. */
    IOMUXC_SetPinConfig(IOMUXC_GPIO_AD_B1_06_LPUART3_TXD, 0x10B0U);
    IOMUXC_SetPinConfig(IOMUXC_GPIO_AD_B1_07_LPUART3_RXD, 0x10B0U);
}

## C6) Avionics simulation protocol over LPUART3

### Protocol (ASCII, line-based)

- RDC → Bridge: HELLO <bootCount> <flagsHex>

- Bridge → RDC: READY

- RDC → Bridge: TX <seq> <labelHex> <dataHex>

- Bridge → RDC: ACK <seq>

### arinc_sim.h

#ifndef ARINC_SIM_H_
#define ARINC_SIM_H_

#include <stdint.h>
#include <stdbool.h>

#ifdef __cplusplus
extern "C" {
#endif

void ArincSim_InitUart3(uint32_t baudrate);

/* Roles */
void ArincSim_RunRdc(uint32_t bootCount, uint32_t lastResetFlags);
void ArincSim_RunBridge(void);

#ifdef __cplusplus
}
#endif

#endif /* ARINC_SIM_H_ */

### arinc_sim.c

#include "arinc_sim.h"

#include "fsl_debug_console.h"
#include "fsl_lpuart.h"
#include "fsl_common.h"
#include "board.h"
#include "board_uart3_pins.h"

#define ARINC_UART            LPUART3
#define ARINC_UART_CLK_FREQ   BOARD_DebugConsoleSrcFreq()

#define LINE_MAX              96

static volatile uint32_t g_msTicks;

void SysTick_Handler(void)
{
    g_msTicks++;
}

static void timebase_init_1ms(void)
{
    g_msTicks = 0;
    (void)SysTick_Config(SystemCoreClock / 1000U);
}

static uint32_t now_ms(void)
{
    return g_msTicks;
}

static void uart3_write_str(const char *s)
{
    while (*s)
    {
        LPUART_WriteByte(ARINC_UART, (uint8_t)*s++);
        while ((LPUART_GetStatusFlags(ARINC_UART) & kLPUART_TxDataRegEmptyFlag) == 0U) { }
    }
}

static bool uart3_read_line(char *out, uint32_t outMax, uint32_t timeoutMs)
{
    uint32_t idx = 0;
    const uint32_t t0 = now_ms();

    while ((now_ms() - t0) < timeoutMs)
    {
        if ((LPUART_GetStatusFlags(ARINC_UART) & kLPUART_RxDataRegFullFlag) != 0U)
        {
            const char ch = (char)LPUART_ReadByte(ARINC_UART);

            if ((ch == '\n') || (ch == '\r'))
            {
                if (idx == 0U)
                {
                    continue;
                }
                out[idx] = '\0';
                return true;
            }

            if (idx + 1U < outMax)
            {
                out[idx++] = ch;
            }
        }
    }

    return false;
}

void ArincSim_InitUart3(uint32_t baudrate)
{
    BOARD_InitUart3Pins();

    lpuart_config_t cfg;
    LPUART_GetDefaultConfig(&cfg);
    cfg.baudRate_Bps = baudrate;
    cfg.enableTx     = true;
    cfg.enableRx     = true;

    LPUART_Init(ARINC_UART, &cfg, ARINC_UART_CLK_FREQ);

    timebase_init_1ms();
}

static bool bridge_wait_ready(uint32_t bootCount, uint32_t flags)
{
    char line[LINE_MAX];
    char hello[LINE_MAX];

    (void)snprintf(hello, sizeof(hello), "HELLO %lu %08lx\r\n",
                   (unsigned long)bootCount, (unsigned long)flags);

    for (int attempt = 0; attempt < 10; attempt++)
    {
        uart3_write_str(hello);
        if (uart3_read_line(line, sizeof(line), 200U))
        {
            if (strcmp(line, "READY") == 0)
            {
                return true;
            }
        }
        SDK_DelayAtLeastUs(100000ULL, SystemCoreClock); /* 100 ms */
    }

    return false;
}

void ArincSim_RunRdc(uint32_t bootCount, uint32_t lastResetFlags)
{
    PRINTF("\r\n[ARINC-SIM:RDC] Init UART3 on Arduino pins.\r\n");
    ArincSim_InitUart3(115200U);

    PRINTF("[ARINC-SIM:RDC] Handshaking...\r\n");
    if (!bridge_wait_ready(bootCount, lastResetFlags))
    {
        PRINTF("[ARINC-SIM:RDC] ERROR: Bridge not READY. Check wiring + that the other board runs BRIDGE build.\r\n");
        return;
    }

    PRINTF("[ARINC-SIM:RDC] READY received. Starting scheduled label stream.\r\n");
    PRINTF("[ARINC-SIM:RDC] Tip: press SW4 (Reset) or SW3 (POR reset) to simulate a fault. After reboot, RDC re-handshakes.\r\n");

    uint32_t seq = 0;
    uint32_t lastTxMs = now_ms();

    while (1)
    {
        const uint32_t label = 0x123U; /* placeholder label */
        const uint32_t data  = (seq & 0xFFFFU);

        char tx[LINE_MAX];
        (void)snprintf(tx, sizeof(tx), "TX %lu %03lx %04lx\r\n",
                       (unsigned long)seq, (unsigned long)label, (unsigned long)data);

        uart3_write_str(tx);

        /* Optional: wait for ACK briefly (not strictly required for ARINC, but helps the lab). */
        char line[LINE_MAX];
        if (uart3_read_line(line, sizeof(line), 50U))
        {
            /* expected: ACK <seq> */
        }

        /* Fixed schedule: send every 50 ms (20 Hz) */
        const uint32_t now = now_ms();
        const uint32_t elapsed = now - lastTxMs;
        if (elapsed < 50U)
        {
            SDK_DelayAtLeastUs((uint64_t)(50U - elapsed) * 1000ULL, SystemCoreClock);
        }
        lastTxMs = now_ms();

        seq++;
    }
}

void ArincSim_RunBridge(void)
{
    PRINTF("\r\n[ARINC-SIM:BRIDGE] Init UART3 on Arduino pins.\r\n");
    ArincSim_InitUart3(115200U);

    PRINTF("[ARINC-SIM:BRIDGE] Waiting for HELLO...\r\n");

    uint32_t lastWordMs = now_ms();
    uint32_t missedCount = 0;

    while (1)
    {
        char line[LINE_MAX];
        if (!uart3_read_line(line, sizeof(line), 200U))
        {
            /* No line within 200ms -> treat as a gap */
            missedCount++;
            if (missedCount == 3U)
            {
                PRINTF("[ARINC-SIM:BRIDGE] WARNING: intermittent source (3 consecutive gaps).\r\n");
            }
            continue;
        }

        /* Got a line; reset gap counter */
        missedCount = 0;

        /* Measure timing gaps */
        const uint32_t now = now_ms();
        const uint32_t gap = now - lastWordMs;
        lastWordMs = now;

        if (gap > 120U)
        {
            PRINTF("[ARINC-SIM:BRIDGE] Gap detected: %lu ms\r\n", (unsigned long)gap);
        }

        if (strncmp(line, "HELLO ", 6) == 0)
        {
            PRINTF("[ARINC-SIM:BRIDGE] %s\r\n", line);
            uart3_write_str("READY\r\n");
            continue;
        }

        if (strncmp(line, "TX ", 3) == 0)
        {
            /* Parse TX <seq> <label> <data> (best-effort) */
            unsigned long seq = 0, label = 0, data = 0;
            (void)sscanf(line, "TX %lu %lx %lx", &seq, &label, &data);

            /* Acknowledge */
            char ack[LINE_MAX];
            (void)snprintf(ack, sizeof(ack), "ACK %lu\r\n", seq);
            uart3_write_str(ack);
            continue;
        }

        /* Unknown line; ignore but print for visibility */
        PRINTF("[ARINC-SIM:BRIDGE] RX: %s\r\n", line);
    }
}

## C7) Integrate into hello_world (hello_world.c / your main file)

### Add includes

#include "reset_diag.h"
#include "wdog_window_demo.h"
#include "arinc_sim.h"

### Add these macros (choose role)

#ifndef ARINC_ROLE_RDC
#define ARINC_ROLE_RDC     1
#endif

#ifndef ARINC_ROLE_BRIDGE
#define ARINC_ROLE_BRIDGE  0
#endif

### In main() after BOARD_InitDebugConsole()

ResetDiag_Info_t info = ResetDiag_RunEarly();

PRINTF("\r\nPress keys to inject faults:\r\n");
PRINTF("  1 = Software reset (SYSRESETREQ)\r\n");
PRINTF("  2 = RTWDOG missed-refresh timeout reset\r\n");
PRINTF("  3 = RTWDOG early-refresh (window violation) reset\r\n");
PRINTF("  a = Start ARINC simulation role\r\n");
PRINTF("\r\nAlso try hardware: SW4 (Reset) and SW3 (POR reset).\r\n");

while (1)
{
    const char ch = (char)GETCHAR();

    if (ch == '1')
    {
        PRINTF("\r\n[FAULT] Triggering software reset now...\r\n");
        NVIC_SystemReset();
    }
    else if (ch == '2')
    {
        WdogWindowDemo_Run(false, true);
    }
    else if (ch == '3')
    {
        WdogWindowDemo_Run(true, false);
    }
    else if (ch == 'a')
    {
#if ARINC_ROLE_RDC
        ArincSim_RunRdc(info.bootCount, info.resetFlags);
#elif ARINC_ROLE_BRIDGE
        ArincSim_RunBridge();
#else
        PRINTF("ARINC roles disabled.\r\n");
#endif
    }
}

# Part D — What to do in the lab (procedure + expected observations)

## D1) Baseline reset attribution

- Flash the app.

- Open the UART console.

- Observe: boot counter increments each reboot; SRC flags printed.

Try these and record what you see: - Power cycle (SW1 OFF→ON or unplug/plug): expect POR/power-on style flags. - SW4 Reset: expect external reset pin / user reset style flag(s). - SW3 POR reset: often looks closer to a power reset path.

## D2) Controlled fault injection via watchdog

- Press 2 → stop refreshing watchdog → expect RTWDOG reset.

- Press 3 → early refresh violation → expect RTWDOG reset.

- After each reboot, confirm SRC reports watchdog‑related flags.

## D3) Avionics simulation (two boards)

- Build BRIDGE on Board B and run it.

- Build RDC on Board A and run it.

- Press a on both boards:

  - Bridge waits for HELLO, replies READY.

  - RDC begins TX stream at 20 Hz.

- Inject a fault on RDC:

  - Press SW4 (reset) or SW3 (POR reset), or use watchdog menu.

- Observe on Bridge:

  - You will see a short gap and possibly “intermittent source” warning if gaps persist.

- Observe on RDC after reboot:

  - It prints last reset cause and boot count, re‑handshakes, and resumes streaming.

# Part E — Notes for real avionics robustness (apply after the lab)

- Add an external reset supervisor to qualify reset pulses and hold reset long enough.

- Improve power integrity (bulk + local decoupling) and separate noisy loads.

- Add EMC protections (TVS at connectors, routing discipline, bonding/shielding).

- Make recovery deterministic: re‑init peripherals idempotently; restart streams at safe boundaries.

# Appendix — Troubleshooting

- If you always see “multiple/unknown flags”: you forgot to clear flags or multiple sources asserted.

- If watchdog resets happen while debugging: ensure cfg.workMode.enableDebug = true (already set in the demo).

- If ARINC sim doesn’t handshake:

  - Confirm UART wiring (TX↔RX, GND↔GND)

  - Confirm both boards run correct roles

  - Confirm pin mux file BOARD_InitUart3Pins() is compiled and linked