Here’s a complete, working 2-firmware solution:

- Board A (Injector): a UART-controlled digital function / pulse generator that outputs open-drain active-low pulses (safe for pulling another board’s RESET line low).

- Board B (Target): prints SRC reset cause flags + a persistent boot counter so you can prove your reset pulses are being seen as external reset pin events, then runs a small “avionics-like” periodic frame streamer (UART simulation).

This implementation follows the reset-attribution + controlled-pulse methodology described in your uploaded fault-injection notes, and uses the EVKB hardware behavior documented in the board HUG.

## Hardware setup (2 boards, safe default)

### Wiring

- GND(A) ↔ GND(B) (common ground)

- Board A output → Board B RESET through ≥10 kΩ series resistor

  - Board A output pin: Arduino SPI header J24[4] (MOSI) = GPIO_SD_B0_02 / GPIO3_IO14 (open-drain)

  - Board B reset pin: Arduino header J25 “RESET”

### Notes (important)

- Inject into RESET (SW4 / IPP_USER_RESET_B), not POR (SW3).

- Keep everything at 3.3 V logic.

- The injector output is open-drain: “high” = released (Hi-Z). The target’s reset pull-up provides the high level. If you test standalone, add a 4.7k–10k pull-up to 3.3 V.

## What you get (features)

### Injector (Board A) – “Function Generator”

- Generates arbitrary pulse widths inside very long periods (e.g., 100 µs low every 5 seconds), which is exactly what you need for reset fault injection.

- Supports:

  - continuous pulse train

  - burst mode (N pulses then stop)

  - one-shot pulse

  - automated sweep of pulse width (for threshold finding)

### Target (Board B) – Reset attribution + avionics simulation

- On every boot, prints:

  - boot counter stored in SRC GPR (persists across warm resets)

  - SRC reset flags, including IPP_USER_RESET_B

- Then runs a periodic “label frame” transmitter over UART to simulate an avionics-style periodic output stream (conceptually similar to a reset-recovery requirement in Airbus-class systems).

## MCUXpresso IDE instructions (copy-in, build, flash)

### 1) Import SDK example projects (recommended path)

In MCUXpresso IDE:

- File → Import… → MCUXpresso IDE → Import SDK example(s)

- Point it at your SDK: SDK_25_06_00_EVKB-IMXRT1050

- Import two projects:

  - Injector base: boards/evkbimxrt1050/driver_examples/pit

  - Target base: boards/evkbimxrt1050/demo_apps/hello_world (or demo_apps/new_project)

### 2) Copy the provided sources into each project

Unzip the package and copy:

- For the injector project, copy everything from:

  - funcgen_injector/source/ → your project’s source/ (replace main.c if present)

- For the target project, copy everything from:

  - reset_target/source/ → your project’s source/ (replace main.c if present)

### 3) Build + flash both boards

- Project → Build Project

- Run → Debug to program via the onboard DAP-Link

### 4) Open UART terminals

- 115200, 8-N-1

- One terminal per board

## Using the injector (Board A) – commands

Type help on the injector console.

Common workflow to reproduce your reset injection approach:

- Start gentle: 10 ms low pulse every 5 seconds (0.2 Hz)

  - set 5000000 10000

  - burst 3

- Then sweep narrower to find the minimum pulse width that still resets:

  - sweep 10000 100 -100 5000000 1

  - (10,000 µs down to 100 µs, step −100 µs, one pulse at each step)

You should see the target reboot each time, printing IPP_USER_RESET_B when the reset was caused by your injected low pulse.

## Expected target behavior (Board B)

On each reset you’ll see something like:

- Boot count incrementing (SRC GPR)

- Reset flags including IPP_USER_RESET_B (external reset pin)

Then it resumes periodic “TX:…” frames on UART.

## Why this design works well for reset fault injection

A classic PWM peripheral often can’t do 0.2 Hz with 100 µs pulses cleanly due to counter width / duty-cycle resolution. This solution uses the RT1050 PIT (32-bit timer) to schedule low and high phases precisely over very long periods, which is ideal for the “short glitch inside long idle” pattern used in reset fault injection experiments (and aligns with NXP Semiconductors’s RT SDK timer model).