# Exercise 1 GPT “Tick”: Measure + Reduce Interrupt Latency & Jitter

Target HW: EVKB‑IMXRT1050 (1 board)

What you’ll build: a strictly periodic 10 kHz interrupt “tick” (think: flight‑control timebase) using GPT2 Output Compare 1. You’ll measure latency and jitter in timer ticks + core cycles, then reduce worst‑case outliers using the techniques from the “Minimizing Interrupt Latency and Jitter” handout:

- Priority + masking discipline (BASEPRI > PRIMASK) so a lifeline ISR stays serviceable

- Make memory deterministic (ITCM/DTCM placement for the timing‑critical ISR + hot data)

- Keep ISR tiny/branch‑light (timestamp + enqueue, no logging)

- Avoid sporadic penalties (guard against lazy FPU stacking outliers)

## 1) Learning objectives

By the end you can:

- Quantify interrupt entry latency (now − expected compare time) and jitter (max − min) under:

  - idle

  - “bad” masking (global disable)

  - background ISR load

- Apply 3 practical fixes:

  - strict periodic scheduling (no drift)

  - priority plan + BASEPRI

  - move critical code/data into ITCM/DTCM (SDK “quick access” sections)

## 2) Required tools

- MCUXpresso IDE (any 11.x)

- EVKB‑IMXRT1050 board + micro‑USB cable

- Optional but recommended: logic analyzer / scope probe

Probe signal: the project uses the EVKB USER LED pin (GPIO1_IO09) as a “probe” that is set high at ISR entry and low at ISR exit. Even without a scope, you’ll still get measured stats printed over the debug console.

## 3) Import the starting point (SDK example)

You will start from the SDK example:

boards/evkbimxrt1050/driver_examples/gpt/timer

### MCUXpresso import steps

- File → Import… → MCUXpresso IDE → Import SDK example(s)

- Select your EVKB‑IMXRT1050 SDK installation.

- Find gpt → timer and import it.

- Build (hammer icon) and Debug/Run once to confirm the original example works.

## 4) Hands‑on tasks

### Task A — Baseline measurement (strict periodic compare)

- Replace the example’s gpt_timer.c with the Solution Code below.

- Build + run.

- Observe printed stats:

  - min/avg/max latency (timer ticks and microseconds)

  - jitter (max − min)

  - optional p99.9 (approx)

Expected: small jitter in idle, larger once we introduce stress.

### Task B — Inject jitter: “bad” masking and background ISR load

Enable one or both switches in the code:

- USE_BAD_GLOBAL_MASKING = 1 → uses PRIMASK (global disable) in a periodic critical section

- ENABLE_SYSTICK_STRESS = 1 → adds a low‑priority SysTick ISR that burns CPU for a fixed time

Run again and compare worst‑case latency/jitter.

### Task C — Reduce jitter with the PDF techniques

Enable:

- USE_BAD_GLOBAL_MASKING = 0 and USE_BASEPRI_MASKING = 1

- PLACE_ISR_AND_DATA_IN_QUICKACCESS = 1

- DISABLE_LAZY_FPU_STACKING = 1

Rebuild and compare results.

You should see fewer long outliers (smaller worst‑case latency and narrower jitter band).

## 5) Avionics simulation context

Treat GPT2 OC1 interrupt as the 10 kHz flight‑control scheduler tick:

- ISR = “hard real‑time” timebase capture (timestamp + quick enqueue)

- Main loop = “soft real‑time” work (sensor fusion, comms, logging)

The experiment shows how a single long global interrupt‑disable window or mis‑prioritized background work can create rare timing outliers — the exact failure mode that makes jitter more damaging than mean latency in certified control loops.

## 6) Solution code (drop‑in replacement for gpt_timer.c)

Paste this into boards/evkbimxrt1050/driver_examples/gpt/timer/gpt_timer.c.

/*
 * Exercise 1 — Minimizing Interrupt Latency & Jitter
 * Target: EVKB-IMXRT1050 (Cortex-M7)
 *
 * Start from SDK example: boards/evkbimxrt1050/driver_examples/gpt/timer
 * Replace gpt_timer.c with this file.
 */

#include <stdint.h>
#include <stdbool.h>
#include <stdlib.h>

#include "fsl_debug_console.h"
#include "board.h"
#include "app.h"
#include "fsl_gpt.h"
#include "fsl_common_arm.h"   /* AT_QUICKACCESS_SECTION_* macros */

/*******************************************************************************
 * Lab configuration switches
 ******************************************************************************/
#define TICK_HZ                           (10000U)  /* 10 kHz flight-control tick */
#define SAMPLE_COUNT                      (2048U)   /* samples per report */

#define ENABLE_SYSTICK_STRESS             (1U)  /* background ISR load */
#define USE_BAD_GLOBAL_MASKING            (1U)  /* PRIMASK: demonstrates jitter spikes */
#define USE_BASEPRI_MASKING               (1U)  /* BASEPRI: recommended fix */

#define PLACE_ISR_AND_DATA_IN_QUICKACCESS (1U)  /* ITCM/DTCM via CodeQuickAccess/DataQuickAccess */
#define DISABLE_LAZY_FPU_STACKING         (1U)  /* remove sporadic FP first-use penalty */

/* Priority plan (numerically smaller = higher priority on Cortex-M)
 * Keep the timebase ISR very high.
 */
#define GPT2_ISR_PRIORITY                 (1U)
#define SYSTICK_ISR_PRIORITY              (5U)

/*******************************************************************************
 * DWT helpers (core-cycle timing)
 ******************************************************************************/
static inline void DWT_Init(void)
{
    CoreDebug->DEMCR |= CoreDebug_DEMCR_TRCENA_Msk;
    DWT->CYCCNT = 0U;
    DWT->CTRL |= DWT_CTRL_CYCCNTENA_Msk;
}

static inline void DWT_DelayCycles(uint32_t cycles)
{
    uint32_t start = DWT->CYCCNT;
    while ((uint32_t)(DWT->CYCCNT - start) < cycles)
    {
        __NOP();
    }
}

/*******************************************************************************
 * BASEPRI masking helper
 * Masks interrupts with numerical priority >= threshold.
 * (Lower numbers are higher priority.)
 ******************************************************************************/
static inline uint32_t EnterCritical_BASEPRI(uint32_t prioThreshold)
{
    uint32_t old = __get_BASEPRI();

    /* Convert priority number to BASEPRI field value. CMSIS uses 8-bit priority fields.
     * __NVIC_PRIO_BITS comes from CMSIS.
     */
    uint32_t basepri = (prioThreshold << (8U - __NVIC_PRIO_BITS)) & 0xFFU;
    __set_BASEPRI(basepri);
    __DSB();
    __ISB();
    return old;
}

static inline void ExitCritical_BASEPRI(uint32_t oldBasepri)
{
    __set_BASEPRI(oldBasepri);
    __DSB();
    __ISB();
}

/*******************************************************************************
 * Measurement storage
 ******************************************************************************/
#if PLACE_ISR_AND_DATA_IN_QUICKACCESS
AT_QUICKACCESS_SECTION_DATA_ALIGN(static volatile uint32_t s_latencyTicks[SAMPLE_COUNT], 32);
AT_QUICKACCESS_SECTION_DATA_ALIGN(static volatile uint32_t s_isrCycles[SAMPLE_COUNT], 32);
#else
static volatile uint32_t s_latencyTicks[SAMPLE_COUNT];
static volatile uint32_t s_isrCycles[SAMPLE_COUNT];
#endif

static volatile uint32_t s_wr = 0U;
static volatile bool s_batchReady = false;

static volatile uint32_t s_expectedCompare = 0U;
static volatile uint32_t s_periodTicks = 0U;
static volatile uint32_t s_timerHz = 0U;

/*******************************************************************************
 * Optional stress ISR (SysTick)
 * Runs at low priority; burns fixed cycles to simulate background load.
 ******************************************************************************/
#if ENABLE_SYSTICK_STRESS
void SysTick_Handler(void)
{
    /* Fixed-time burn. Keep deterministic.
     * Tune this if you want harsher contention.
     */
    DWT_DelayCycles(SystemCoreClock / 20000U); /* ~50 us at 1 GHz; scaled by actual core */
}
#endif

/*******************************************************************************
 * GPT2 ISR — the timing-critical tick
 ******************************************************************************/
#if PLACE_ISR_AND_DATA_IN_QUICKACCESS
AT_QUICKACCESS_SECTION_CODE(void EXAMPLE_GPT_IRQHandler(void))
#else
void EXAMPLE_GPT_IRQHandler(void)
#endif
{
    /* Probe high at ISR entry: GPIO1_IO09 (USER LED pin on EVKB) */
    GPIO_PortSet(BOARD_USER_LED_GPIO, (1U << BOARD_USER_LED_GPIO_PIN));

    uint32_t startCycles = DWT->CYCCNT;

    /* Clear interrupt flag first (keep ISR branch-light) */
    GPT_ClearStatusFlags(EXAMPLE_GPT, kGPT_OutputCompare1Flag);

    /* Measure latency relative to expected compare */
    uint32_t now = GPT_GetCurrentTimerCount(EXAMPLE_GPT);
    uint32_t lat = now - s_expectedCompare; /* wrap-safe unsigned subtraction */

    uint32_t idx = s_wr;
    if (idx < SAMPLE_COUNT)
    {
        s_latencyTicks[idx] = lat;
        s_isrCycles[idx] = (uint32_t)(DWT->CYCCNT - startCycles);
        s_wr = idx + 1U;

        /* Strict periodic schedule: next = previous_expected + period */
        s_expectedCompare += s_periodTicks;
        GPT_SetOutputCompareValue(EXAMPLE_GPT, kGPT_OutputCompare_Channel1, s_expectedCompare);

        if (s_wr >= SAMPLE_COUNT)
        {
            s_batchReady = true;
            /* Stop further IRQ storm while main processes stats */
            GPT_DisableInterrupts(EXAMPLE_GPT, kGPT_OutputCompare1InterruptEnable);
        }
    }

    GPIO_PortClear(BOARD_USER_LED_GPIO, (1U << BOARD_USER_LED_GPIO_PIN));

    /* ARM errata barrier used in SDK examples */
    SDK_ISR_EXIT_BARRIER;
}

/*******************************************************************************
 * Stats helpers
 ******************************************************************************/
static int CmpU32(const void *a, const void *b)
{
    uint32_t ua = *(const uint32_t *)a;
    uint32_t ub = *(const uint32_t *)b;
    if (ua < ub) return -1;
    if (ua > ub) return 1;
    return 0;
}

static void PrintStats(const volatile uint32_t *arr, uint32_t n, const char *label, uint32_t denomHz)
{
    /* Copy to a local buffer so we can sort without touching ISR data */
    static uint32_t tmp[SAMPLE_COUNT];
    for (uint32_t i = 0; i < n; i++)
        tmp[i] = arr[i];

    qsort(tmp, n, sizeof(tmp[0]), CmpU32);

    uint64_t sum = 0;
    for (uint32_t i = 0; i < n; i++)
        sum += tmp[i];

    uint32_t minv = tmp[0];
    uint32_t maxv = tmp[n - 1U];
    uint32_t p99 = tmp[(uint32_t)((n * 99U) / 100U)];
    uint32_t p999 = tmp[(uint32_t)((n * 999U) / 1000U)];

    double avg = (double)sum / (double)n;

    /* Convert to microseconds using denomHz if provided */
    if (denomHz != 0U)
    {
        double min_us = ((double)minv * 1000000.0) / (double)denomHz;
        double avg_us = (avg * 1000000.0) / (double)denomHz;
        double max_us = ((double)maxv * 1000000.0) / (double)denomHz;
        double jit_us = ((double)(maxv - minv) * 1000000.0) / (double)denomHz;

        PRINTF("\r\n[%s] min=%u (%.3fus)  avg=%.1f (%.3fus)  max=%u (%.3fus)  jitter=%u (%.3fus)",
               label, minv, min_us, avg, avg_us, maxv, max_us, (maxv - minv), jit_us);
        PRINTF("\r\n[%s] p99=%u  p99.9=%u", label, p99, p999);
    }
    else
    {
        PRINTF("\r\n[%s] min=%u  avg=%.1f  max=%u  jitter=%u", label, minv, avg, maxv, (maxv - minv));
        PRINTF("\r\n[%s] p99=%u  p99.9=%u", label, p99, p999);
    }
}

/*******************************************************************************
 * Main
 ******************************************************************************/
int main(void)
{
    gpt_config_t cfg;

    BOARD_InitHardware();
    DWT_Init();

#if DISABLE_LAZY_FPU_STACKING
#if (__FPU_PRESENT == 1) && (__FPU_USED == 1)
    /* Make FP stacking deterministic: keep ASPEN=1, clear LSPEN to disable lazy stacking */
    FPU->FPCCR |= FPU_FPCCR_ASPEN_Msk;
    FPU->FPCCR &= ~FPU_FPCCR_LSPEN_Msk;
#endif
#endif

    USER_LED_INIT(LOGIC_LED_OFF);

    /* Configure GPT2 */
    GPT_GetDefaultConfig(&cfg);
    cfg.enableFreeRun = true;
    GPT_Init(EXAMPLE_GPT, &cfg);

    /* Choose a timer tick near 1 MHz for easy microsecond math */
    uint32_t srcHz = EXAMPLE_GPT_CLK_FREQ;
    uint32_t divider = srcHz / 1000000U;
    if (divider < 1U) divider = 1U;
    if (divider > 4096U) divider = 4096U;

    GPT_SetClockDivider(EXAMPLE_GPT, divider);
    s_timerHz = srcHz / divider;

    s_periodTicks = s_timerHz / TICK_HZ;
    if (s_periodTicks < 10U) s_periodTicks = 10U; /* sanity */

    /* Prime first compare strictly from current timer */
    uint32_t now = GPT_GetCurrentTimerCount(EXAMPLE_GPT);
    s_expectedCompare = now + s_periodTicks;
    GPT_SetOutputCompareValue(EXAMPLE_GPT, kGPT_OutputCompare_Channel1, s_expectedCompare);

    /* IRQ priority plan */
    NVIC_SetPriority(GPT_IRQ_ID, GPT2_ISR_PRIORITY);

#if ENABLE_SYSTICK_STRESS
    NVIC_SetPriority(SysTick_IRQn, SYSTICK_ISR_PRIORITY);
    SysTick_Config(SystemCoreClock / 1000U);
#endif

    GPT_EnableInterrupts(EXAMPLE_GPT, kGPT_OutputCompare1InterruptEnable);
    EnableIRQ(GPT_IRQ_ID);

    PRINTF("\r\n--- Exercise 1: GPT 10kHz tick latency & jitter ---");
    PRINTF("\r\nTimerHz=%u, periodTicks=%u (target %u Hz)", s_timerHz, s_periodTicks, TICK_HZ);

#if USE_BAD_GLOBAL_MASKING
    PRINTF("\r\nMasking mode: PRIMASK (BAD) enabled");
#else
#if USE_BASEPRI_MASKING
    PRINTF("\r\nMasking mode: BASEPRI (recommended) enabled");
#else
    PRINTF("\r\nMasking mode: none");
#endif
#endif

#if PLACE_ISR_AND_DATA_IN_QUICKACCESS
    PRINTF("\r\nPlacement: CodeQuickAccess/DataQuickAccess enabled (ITCM/DTCM if linker maps them)");
#else
    PRINTF("\r\nPlacement: default sections (likely XIP flash for code)");
#endif

    PRINTF("\r\nPress any key to start collecting %u samples...", SAMPLE_COUNT);
    GETCHAR();

    /* Start timer */
    GPT_StartTimer(EXAMPLE_GPT);

    /* Main loop: optionally introduce periodic critical sections */
    uint32_t iter = 0U;
    while (1)
    {
        if (s_batchReady)
        {
            /* Stop the timer while printing stats */
            GPT_StopTimer(EXAMPLE_GPT);

            PRINTF("\r\n\r\nBatch complete.");
            PrintStats(s_latencyTicks, SAMPLE_COUNT, "LatencyTicks", s_timerHz);
            PrintStats(s_isrCycles, SAMPLE_COUNT, "ISR_Cycles", SystemCoreClock);

            /* Reset for next run */
            s_wr = 0U;
            s_batchReady = false;

            /* Re-arm next compare from current time */
            uint32_t t = GPT_GetCurrentTimerCount(EXAMPLE_GPT);
            s_expectedCompare = t + s_periodTicks;
            GPT_SetOutputCompareValue(EXAMPLE_GPT, kGPT_OutputCompare_Channel1, s_expectedCompare);

            GPT_EnableInterrupts(EXAMPLE_GPT, kGPT_OutputCompare1InterruptEnable);
            GPT_StartTimer(EXAMPLE_GPT);

            PRINTF("\r\n\r\nCollecting next batch... (press USER RESET to stop)");
        }

        /* Periodically create a critical section to demonstrate masking effects */
        if ((iter++ % 5000U) == 0U)
        {
#if USE_BAD_GLOBAL_MASKING
            __disable_irq();
            DWT_DelayCycles(SystemCoreClock / 50000U); /* ~20 us */
            __enable_irq();
#else
#if USE_BASEPRI_MASKING
            /* Mask low/medium priorities only; GPT (priority 1) still preempts */
            uint32_t old = EnterCritical_BASEPRI(3U); /* mask priority 3 and lower */
            DWT_DelayCycles(SystemCoreClock / 50000U);
            ExitCritical_BASEPRI(old);
#endif
#endif
        }

        __WFI();
    }
}

## 7) What to look for (expected observations)

- Idle run: latency band is tight; jitter is small.

- PRIMASK enabled: you will see rare latency spikes that align with the global masking windows.

- BASEPRI: spikes shrink dramatically because the high‑priority GPT ISR is not blocked by low‑priority critical sections.

- QuickAccess placement: further reduces variability by removing XIP/cache fetch effects for the ISR/hot data.

## 8) Troubleshooting

### If build fails with “CodeQuickAccess/DataQuickAccess section not found”

Your linker script doesn’t define these sections.

Fast workaround: - Set PLACE_ISR_AND_DATA_IN_QUICKACCESS to 0.

Proper fix: - In the project linker script, ensure sections CodeQuickAccess and DataQuickAccess are mapped to ITCM/DTCM (common in NXP RT MCUXpresso templates).

### If UART/console output is missing

- Ensure the project’s debug console is configured for the EVKB OpenSDA virtual COM.

- Use 115200 8‑N‑1 in your terminal.

## 9) Optional extensions

- Move the vector table to a deterministic memory region (RAM/ITCM) and point SCB->VTOR to it.

- Add a second timer ISR with intentionally wrong priority to demonstrate how a “medium” ISR can starve the lifeline ISR.