# Exercise 1 —GPT “Tick”: Measure + Reduce Interrupt Latency & Jitter

Target HW: EVKB‑IMXRT1050 (1 board)

What you’ll build: a strictly periodic 10 kHz interrupt “tick” (think: flight‑control timebase) using GPT2 Output Compare 1. You’ll measure latency and jitter in timer ticks + core cycles, then reduce worst‑case outliers using the techniques from the “Minimizing Interrupt Latency and Jitter” handout:

- Priority + masking discipline (BASEPRI > PRIMASK) so a lifeline ISR stays serviceable

- Make memory deterministic (ITCM/DTCM placement for the timing‑critical ISR + hot data)

- Keep ISR tiny/branch‑light (timestamp + enqueue, no logging)

- Avoid sporadic penalties (guard against lazy FPU stacking outliers)

## 1) Learning objectives

By the end you can:

- Quantify interrupt entry latency (now − expected compare time) and jitter (max − min) under:

  - idle

  - “bad” masking (global disable)

  - background ISR load

- Apply 3 practical fixes:

  - strict periodic scheduling (no drift)

  - priority plan + BASEPRI

  - move critical code/data into ITCM/DTCM (SDK “quick access” sections)

## 2) Required tools

- MCUXpresso IDE (any 11.x)

- EVKB‑IMXRT1050 board + micro‑USB cable

- Optional but recommended: logic analyzer / scope probe

Probe signal: the project uses the EVKB USER LED pin (GPIO1_IO09) as a “probe” that is set high at ISR entry and low at ISR exit. Even without a scope, you’ll still get measured stats printed over the debug console.

## 3) Import the starting point (SDK example)

You will start from the SDK example:

boards/evkbimxrt1050/driver_examples/gpt/timer

### MCUXpresso import steps

- File → Import… → MCUXpresso IDE → Import SDK example(s)

- Select your EVKB‑IMXRT1050 SDK installation.

- Find gpt → timer and import it.

- Build (hammer icon) and Debug/Run once to confirm the original example works.

## 4) Hands‑on tasks

### Task A — Baseline measurement (strict periodic compare)

- Replace the example’s gpt_timer.c with the Solution Code below.

- Build + run.

- Observe printed stats:

  - min/avg/max latency (timer ticks and microseconds)

  - jitter (max − min)

  - optional p99.9 (approx)

Expected: small jitter in idle, larger once we introduce stress.

### Task B — Inject jitter: “bad” masking and background ISR load

Enable one or both switches in the code:

- USE_BAD_GLOBAL_MASKING = 1 → uses PRIMASK (global disable) in a periodic critical section

- ENABLE_SYSTICK_STRESS = 1 → adds a low‑priority SysTick ISR that burns CPU for a fixed time

Run again and compare worst‑case latency/jitter.

### Task C — Reduce jitter with the PDF techniques

Enable:

- USE_BAD_GLOBAL_MASKING = 0 and USE_BASEPRI_MASKING = 1

- PLACE_ISR_AND_DATA_IN_QUICKACCESS = 1

- DISABLE_LAZY_FPU_STACKING = 1

Rebuild and compare results.

You should see fewer long outliers (smaller worst‑case latency and narrower jitter band).

## 5) Avionics simulation context

Treat GPT2 OC1 interrupt as the 10 kHz flight‑control scheduler tick:

- ISR = “hard real‑time” timebase capture (timestamp + quick enqueue)

- Main loop = “soft real‑time” work (sensor fusion, comms, logging)

The experiment shows how a single long global interrupt‑disable window or mis‑prioritized background work can create rare timing outliers — the exact failure mode that makes jitter more damaging than mean latency in certified control loops.

## 6) Solution code (drop‑in replacement for gpt_timer.c)

Paste this into boards/evkbimxrt1050/driver_examples/gpt/timer/gpt_timer.c.

/*
 * Exercise 1 — Minimizing Interrupt Latency & Jitter
 * Target: EVKB-IMXRT1050 (Cortex-M7)
 *
 * Start from SDK example: boards/evkbimxrt1050/driver_examples/gpt/timer
 * Replace gpt_timer.c with this file.
 */

#include <stdint.h>
#include <stdbool.h>
#include <stdlib.h>

#include "fsl_debug_console.h"
#include "board.h"
#include "app.h"
#include "fsl_gpt.h"
#include "fsl_common_arm.h"   /* AT_QUICKACCESS_SECTION_* macros */

/*******************************************************************************
 * Lab configuration switches
 ******************************************************************************/
#define TICK_HZ                           (10000U)  /* 10 kHz flight-control tick */
#define SAMPLE_COUNT                      (2048U)   /* samples per report */

#define ENABLE_SYSTICK_STRESS             (1U)  /* background ISR load */
#define USE_BAD_GLOBAL_MASKING            (1U)  /* PRIMASK: demonstrates jitter spikes */
#define USE_BASEPRI_MASKING               (1U)  /* BASEPRI: recommended fix */

#define PLACE_ISR_AND_DATA_IN_QUICKACCESS (1U)  /* ITCM/DTCM via CodeQuickAccess/DataQuickAccess */
#define DISABLE_LAZY_FPU_STACKING         (1U)  /* remove sporadic FP first-use penalty */

/* Priority plan (numerically smaller = higher priority on Cortex-M)
 * Keep the timebase ISR very high.
 */
#define GPT2_ISR_PRIORITY                 (1U)
#define SYSTICK_ISR_PRIORITY              (5U)

/*******************************************************************************
 * DWT helpers (core-cycle timing)
 ******************************************************************************/
static inline void DWT_Init(void)
{
    CoreDebug->DEMCR |= CoreDebug_DEMCR_TRCENA_Msk;
    DWT->CYCCNT = 0U;
    DWT->CTRL |= DWT_CTRL_CYCCNTENA_Msk;
}

static inline void DWT_DelayCycles(uint32_t cycles)
{
    uint32_t start = DWT->CYCCNT;
    while ((uint32_t)(DWT->CYCCNT - start) < cycles)
    {
        __NOP();
    }
}

/*******************************************************************************
 * BASEPRI masking helper
 * Masks interrupts with numerical priority >= threshold.
 * (Lower numbers are higher priority.)
 ******************************************************************************/
static inline uint32_t EnterCritical_BASEPRI(uint32_t prioThreshold)
{
    uint32_t old = __get_BASEPRI();

    /* Convert priority number to BASEPRI field value. CMSIS uses 8-bit priority fields.
     * __NVIC_PRIO_BITS comes from CMSIS.
     */
    uint32_t basepri = (prioThreshold << (8U - __NVIC_PRIO_BITS)) & 0xFFU;
    __set_BASEPRI(basepri);
    __DSB();
    __ISB();
    return old;
}

static inline void ExitCritical_BASEPRI(uint32_t oldBasepri)
{
    __set_BASEPRI(oldBasepri);
    __DSB();
    __ISB();
}

/*******************************************************************************
 * Measurement storage
 ******************************************************************************/
#if PLACE_ISR_AND_DATA_IN_QUICKACCESS
AT_QUICKACCESS_SECTION_DATA_ALIGN(static volatile uint32_t s_latencyTicks[SAMPLE_COUNT], 32);
AT_QUICKACCESS_SECTION_DATA_ALIGN(static volatile uint32_t s_isrCycles[SAMPLE_COUNT], 32);
#else
static volatile uint32_t s_latencyTicks[SAMPLE_COUNT];
static volatile uint32_t s_isrCycles[SAMPLE_COUNT];
#endif

static volatile uint32_t s_wr = 0U;
static volatile bool s_batchReady = false;

static volatile uint32_t s_expectedCompare = 0U;
static volatile uint32_t s_periodTicks = 0U;
static volatile uint32_t s_timerHz = 0U;

/*******************************************************************************
 * Optional stress ISR (SysTick)
 * Runs at low priority; burns fixed cycles to simulate background load.
 ******************************************************************************/
#if ENABLE_SYSTICK_STRESS
void SysTick_Handler(void)
{
    /* Fixed-time burn. Keep deterministic.
     * Tune this if you want harsher contention.
     */
    DWT_DelayCycles(SystemCoreClock / 20000U); /* ~50 us at 1 GHz; scaled by actual core */
}
#endif

/*******************************************************************************
 * GPT2 ISR — the timing-critical tick
 ******************************************************************************/
#if PLACE_ISR_AND_DATA_IN_QUICKACCESS
AT_QUICKACCESS_SECTION_CODE(void EXAMPLE_GPT_IRQHandler(void))
#else
void EXAMPLE_GPT_IRQHandler(void)
#endif
{
    /* Probe high at ISR entry: GPIO1_IO09 (USER LED pin on EVKB) */
    GPIO_PortSet(BOARD_USER_LED_GPIO, (1U << BOARD_USER_LED_GPIO_PIN));

    uint32_t startCycles = DWT->CYCCNT;

    /* Clear interrupt flag first (keep ISR branch-light) */
    GPT_ClearStatusFlags(EXAMPLE_GPT, kGPT_OutputCompare1Flag);

    /* Measure latency relative to expected compare */
    uint32_t now = GPT_GetCurrentTimerCount(EXAMPLE_GPT);
    uint32_t lat = now - s_expectedCompare; /* wrap-safe unsigned subtraction */

    uint32_t idx = s_wr;
    if (idx < SAMPLE_COUNT)
    {
        s_latencyTicks[idx] = lat;
        s_isrCycles[idx] = (uint32_t)(DWT->CYCCNT - startCycles);
        s_wr = idx + 1U;

        /* Strict periodic schedule: next = previous_expected + period */
        s_expectedCompare += s_periodTicks;
        GPT_SetOutputCompareValue(EXAMPLE_GPT, kGPT_OutputCompare_Channel1, s_expectedCompare);

        if (s_wr >= SAMPLE_COUNT)
        {
            s_batchReady = true;
            /* Stop further IRQ storm while main processes stats */
            GPT_DisableInterrupts(EXAMPLE_GPT, kGPT_OutputCompare1InterruptEnable);
        }
    }

    GPIO_PortClear(BOARD_USER_LED_GPIO, (1U << BOARD_USER_LED_GPIO_PIN));

    /* ARM errata barrier used in SDK examples */
    SDK_ISR_EXIT_BARRIER;
}

/*******************************************************************************
 * Stats helpers
 ******************************************************************************/
static int CmpU32(const void *a, const void *b)
{
    uint32_t ua = *(const uint32_t *)a;
    uint32_t ub = *(const uint32_t *)b;
    if (ua < ub) return -1;
    if (ua > ub) return 1;
    return 0;
}

static void PrintStats(const volatile uint32_t *arr, uint32_t n, const char *label, uint32_t denomHz)
{
    /* Copy to a local buffer so we can sort without touching ISR data */
    static uint32_t tmp[SAMPLE_COUNT];
    for (uint32_t i = 0; i < n; i++)
        tmp[i] = arr[i];

    qsort(tmp, n, sizeof(tmp[0]), CmpU32);

    uint64_t sum = 0;
    for (uint32_t i = 0; i < n; i++)
        sum += tmp[i];

    uint32_t minv = tmp[0];
    uint32_t maxv = tmp[n - 1U];
    uint32_t p99 = tmp[(uint32_t)((n * 99U) / 100U)];
    uint32_t p999 = tmp[(uint32_t)((n * 999U) / 1000U)];

    double avg = (double)sum / (double)n;

    /* Convert to microseconds using denomHz if provided */
    if (denomHz != 0U)
    {
        double min_us = ((double)minv * 1000000.0) / (double)denomHz;
        double avg_us = (avg * 1000000.0) / (double)denomHz;
        double max_us = ((double)maxv * 1000000.0) / (double)denomHz;
        double jit_us = ((double)(maxv - minv) * 1000000.0) / (double)denomHz;

        PRINTF("\r\n[%s] min=%u (%.3fus)  avg=%.1f (%.3fus)  max=%u (%.3fus)  jitter=%u (%.3fus)",
               label, minv, min_us, avg, avg_us, maxv, max_us, (maxv - minv), jit_us);
        PRINTF("\r\n[%s] p99=%u  p99.9=%u", label, p99, p999);
    }
    else
    {
        PRINTF("\r\n[%s] min=%u  avg=%.1f  max=%u  jitter=%u", label, minv, avg, maxv, (maxv - minv));
        PRINTF("\r\n[%s] p99=%u  p99.9=%u", label, p99, p999);
    }
}

/*******************************************************************************
 * Main
 ******************************************************************************/
int main(void)
{
    gpt_config_t cfg;

    BOARD_InitHardware();
    DWT_Init();

#if DISABLE_LAZY_FPU_STACKING
#if (__FPU_PRESENT == 1) && (__FPU_USED == 1)
    /* Make FP stacking deterministic: keep ASPEN=1, clear LSPEN to disable lazy stacking */
    FPU->FPCCR |= FPU_FPCCR_ASPEN_Msk;
    FPU->FPCCR &= ~FPU_FPCCR_LSPEN_Msk;
#endif
#endif

    USER_LED_INIT(LOGIC_LED_OFF);

    /* Configure GPT2 */
    GPT_GetDefaultConfig(&cfg);
    cfg.enableFreeRun = true;
    GPT_Init(EXAMPLE_GPT, &cfg);

    /* Choose a timer tick near 1 MHz for easy microsecond math */
    uint32_t srcHz = EXAMPLE_GPT_CLK_FREQ;
    uint32_t divider = srcHz / 1000000U;
    if (divider < 1U) divider = 1U;
    if (divider > 4096U) divider = 4096U;

    GPT_SetClockDivider(EXAMPLE_GPT, divider);
    s_timerHz = srcHz / divider;

    s_periodTicks = s_timerHz / TICK_HZ;
    if (s_periodTicks < 10U) s_periodTicks = 10U; /* sanity */

    /* Prime first compare strictly from current timer */
    uint32_t now = GPT_GetCurrentTimerCount(EXAMPLE_GPT);
    s_expectedCompare = now + s_periodTicks;
    GPT_SetOutputCompareValue(EXAMPLE_GPT, kGPT_OutputCompare_Channel1, s_expectedCompare);

    /* IRQ priority plan */
    NVIC_SetPriority(GPT_IRQ_ID, GPT2_ISR_PRIORITY);

#if ENABLE_SYSTICK_STRESS
    NVIC_SetPriority(SysTick_IRQn, SYSTICK_ISR_PRIORITY);
    SysTick_Config(SystemCoreClock / 1000U);
#endif

    GPT_EnableInterrupts(EXAMPLE_GPT, kGPT_OutputCompare1InterruptEnable);
    EnableIRQ(GPT_IRQ_ID);

    PRINTF("\r\n--- Exercise 1: GPT 10kHz tick latency & jitter ---");
    PRINTF("\r\nTimerHz=%u, periodTicks=%u (target %u Hz)", s_timerHz, s_periodTicks, TICK_HZ);

#if USE_BAD_GLOBAL_MASKING
    PRINTF("\r\nMasking mode: PRIMASK (BAD) enabled");
#else
#if USE_BASEPRI_MASKING
    PRINTF("\r\nMasking mode: BASEPRI (recommended) enabled");
#else
    PRINTF("\r\nMasking mode: none");
#endif
#endif

#if PLACE_ISR_AND_DATA_IN_QUICKACCESS
    PRINTF("\r\nPlacement: CodeQuickAccess/DataQuickAccess enabled (ITCM/DTCM if linker maps them)");
#else
    PRINTF("\r\nPlacement: default sections (likely XIP flash for code)");
#endif

    PRINTF("\r\nPress any key to start collecting %u samples...", SAMPLE_COUNT);
    GETCHAR();

    /* Start timer */
    GPT_StartTimer(EXAMPLE_GPT);

    /* Main loop: optionally introduce periodic critical sections */
    uint32_t iter = 0U;
    while (1)
    {
        if (s_batchReady)
        {
            /* Stop the timer while printing stats */
            GPT_StopTimer(EXAMPLE_GPT);

            PRINTF("\r\n\r\nBatch complete.");
            PrintStats(s_latencyTicks, SAMPLE_COUNT, "LatencyTicks", s_timerHz);
            PrintStats(s_isrCycles, SAMPLE_COUNT, "ISR_Cycles", SystemCoreClock);

            /* Reset for next run */
            s_wr = 0U;
            s_batchReady = false;

            /* Re-arm next compare from current time */
            uint32_t t = GPT_GetCurrentTimerCount(EXAMPLE_GPT);
            s_expectedCompare = t + s_periodTicks;
            GPT_SetOutputCompareValue(EXAMPLE_GPT, kGPT_OutputCompare_Channel1, s_expectedCompare);

            GPT_EnableInterrupts(EXAMPLE_GPT, kGPT_OutputCompare1InterruptEnable);
            GPT_StartTimer(EXAMPLE_GPT);

            PRINTF("\r\n\r\nCollecting next batch... (press USER RESET to stop)");
        }

        /* Periodically create a critical section to demonstrate masking effects */
        if ((iter++ % 5000U) == 0U)
        {
#if USE_BAD_GLOBAL_MASKING
            __disable_irq();
            DWT_DelayCycles(SystemCoreClock / 50000U); /* ~20 us */
            __enable_irq();
#else
#if USE_BASEPRI_MASKING
            /* Mask low/medium priorities only; GPT (priority 1) still preempts */
            uint32_t old = EnterCritical_BASEPRI(3U); /* mask priority 3 and lower */
            DWT_DelayCycles(SystemCoreClock / 50000U);
            ExitCritical_BASEPRI(old);
#endif
#endif
        }

        __WFI();
    }
}

## 7) What to look for (expected observations)

- Idle run: latency band is tight; jitter is small.

- PRIMASK enabled: you will see rare latency spikes that align with the global masking windows.

- BASEPRI: spikes shrink dramatically because the high‑priority GPT ISR is not blocked by low‑priority critical sections.

- QuickAccess placement: further reduces variability by removing XIP/cache fetch effects for the ISR/hot data.

## 8) Troubleshooting

### If build fails with “CodeQuickAccess/DataQuickAccess section not found”

Your linker script doesn’t define these sections.

Fast workaround: - Set PLACE_ISR_AND_DATA_IN_QUICKACCESS to 0.

Proper fix: - In the project linker script, ensure sections CodeQuickAccess and DataQuickAccess are mapped to ITCM/DTCM (common in NXP RT MCUXpresso templates).

### If UART/console output is missing

- Ensure the project’s debug console is configured for the EVKB OpenSDA virtual COM.

- Use 115200 8‑N‑1 in your terminal.

## 9) Optional extensions

- Move the vector table to a deterministic memory region (RAM/ITCM) and point SCB->VTOR to it.

- Add a second timer ISR with intentionally wrong priority to demonstrate how a “medium” ISR can starve the lifeline ISR.

## Exercise 2 — UART RX Interrupt Storm vs EDMA Ring Buffer: impact on GPT tick jitter

Target HW: EVKB-IMXRT1050 (2 boards) + jumper wires

What you’ll build: an avionics-style “data bus flood” and a time-critical scheduler tick running in parallel:

- Board A (TX): continuously transmits fixed-size “ARINC-like words” on LPUART3 (Arduino header pins)

- Board B (RX): runs the 10 kHz GPT tick (same concept as Exercise 1) and receives UART data in two ways:

  - Per-byte RX interrupt (high interrupt rate → jitter)

  - EDMA ring buffer (DMA moves bytes; CPU does bulk reads → lower jitter)

This exercise demonstrates the  core idea: jitter outliers often come from “unrelated” interrupts and memory effects (high-frequency per-byte ISRs + cache/DMA coherency). Using DMA + non-cacheable buffers makes timing more deterministic.

## 1) Wiring (Arduino header J22)

On both boards, use the Arduino header J22:

- D1 (UART_TX) → D0 (UART_RX)

- GND → GND

That is a unidirectional link (TX→RX). If you want bidirectional later, also wire the reverse TX/RX.

Tip: Keep wires short. Set both boards to 3.3V logic (default).

## 2) Import the SDK starting points

You will create three projects (TX, RX-IRQ, RX-EDMA). All are SDK examples so you’re not fighting clocks/pinmux from scratch.

### Project A: UART3 Transmitter (Board A)

Start from:

boards/evkbimxrt1050/driver_examples/lpuart/polling_seven_bits

We’ll reuse its LPUART3 pin mux on Arduino header and replace the app with a continuous transmitter.

### Project B: UART RX Interrupt Receiver + GPT jitter monitor (Board B)

Start from:

boards/evkbimxrt1050/driver_examples/lpuart/interrupt

We’ll switch the demo UART from LPUART1 (debug port) to LPUART3 and merge in the GPT jitter monitor.

### Project C: UART RX EDMA Ring Buffer Receiver + GPT jitter monitor (Board B)

Start from:

boards/evkbimxrt1050/driver_examples/lpuart/edma_rb_transfer

We’ll switch the demo UART to LPUART3 (and update DMA request mux constants) and merge in the same GPT jitter monitor.

## 3) Hands-on tasks

### Task A — Baseline tick jitter (no bus traffic)

- Flash Project B (RX-IRQ) to Board B.

- Don’t power Board A yet.

- Observe baseline tick jitter report (expect tight band).

### Task B — Create the interrupt storm

- Flash Project A (TX) to Board A.

- Power both boards.

- Re-run Board B and observe:

  - tick max latency spikes

  - tick jitter growth

  - UART overflow counters (if your ring buffer can’t keep up)

### Task C — Fix it with EDMA ring buffer

- Flash Project C (RX-EDMA) to Board B.

- Keep TX running.

- Compare jitter:

  - per-byte interrupt mode vs EDMA

You should see significantly fewer outliers in EDMA mode, because most bytes arrive without executing a CPU ISR per byte.

### Task D — Priority planning (optional but recommended)

In the receiver projects, flip these priorities and measure:

- BAD: UART priority higher than GPT tick priority (jitter gets worse)

- GOOD: GPT tick priority higher (UART cannot block the lifeline tick)

## 4) Avionics simulation context

- Board A = “remote I/O concentrator” sending a steady stream of decoded words.

- Board B = “flight control computer” with a hard real-time 10 kHz scheduler tick.

The lesson is exactly what avionics RT engineers care about: unbounded ISR work (per-byte RX) is toxic to deterministic schedule ticks. Use DMA, ring buffers, and priority discipline.

## 5) Solution code — Project A (Board A TX)

Replace the main C file in lpuart/polling_seven_bits with this (e.g., lpuart_tx_flood.c) and update the project to build it.

/*
 * Exercise 2 — Board A (TX): UART3 continuous transmitter flood
 * Base project: boards/evkbimxrt1050/driver_examples/lpuart/polling_seven_bits
 */

#include <stdint.h>
#include <stdbool.h>

#include "fsl_debug_console.h"
#include "board.h"
#include "app.h"
#include "fsl_lpuart.h"

#define TX_BAUDRATE     (921600U)
#define FRAME_BYTES     (32U)

static void FillFrame(uint8_t *buf, uint32_t len, uint32_t seq)
{
    /* Simple ARINC-like word stream: [sync][seq][payload..][crc-ish]
     * Not a real ARINC-429 encoding; it’s an avionics-flavored traffic pattern.
     */
    if (len < 8U) return;
    buf[0] = 0xA5U; /* sync */
    buf[1] = (uint8_t)(seq & 0xFFU);
    buf[2] = (uint8_t)((seq >> 8) & 0xFFU);
    buf[3] = (uint8_t)((seq >> 16) & 0xFFU);

    uint8_t c = 0U;
    for (uint32_t i = 4; i < (len - 1U); i++)
    {
        buf[i] = (uint8_t)(i + seq);
        c ^= buf[i];
    }
    buf[len - 1U] = (uint8_t)(c ^ 0x5AU);
}

int main(void)
{
    lpuart_config_t cfg;
    uint8_t frame[FRAME_BYTES];
    uint32_t seq = 0U;

    BOARD_InitHardware();

    /* LPUART3 pins are already configured in this base project (Arduino header).
     * If you changed base project, ensure pin mux sets:
     *  - IOMUXC_GPIO_AD_B1_06_LPUART3_TXD
     *  - IOMUXC_GPIO_AD_B1_07_LPUART3_RXD
     */

    LPUART_GetDefaultConfig(&cfg);
    cfg.baudRate_Bps = TX_BAUDRATE;
    cfg.enableTx     = true;
    cfg.enableRx     = false;

    LPUART_Init(DEMO_LPUART, &cfg, DEMO_LPUART_CLK_FREQ);

    PRINTF("
[TX] Flooding UART3 at %u bps, frame=%u bytes
", TX_BAUDRATE, FRAME_BYTES);

    while (1)
    {
        FillFrame(frame, FRAME_BYTES, seq++);
        LPUART_WriteBlocking(DEMO_LPUART, frame, FRAME_BYTES);
    }
}

## 6) Solution code — Project B (Board B RX per-byte interrupt + GPT jitter)

### Step B1 — Switch UART instance to LPUART3

In boards/.../lpuart/interrupt/app.h, change:

- #define DEMO_LPUART LPUART1 → LPUART3

- #define DEMO_LPUART_IRQn LPUART1_IRQn → LPUART3_IRQn

- #define DEMO_LPUART_IRQHandler LPUART1_IRQHandler → LPUART3_IRQHandler

(Keep PRINTF on the debug console as-is.)

### Step B2 — Ensure LPUART3 pin mux is set

In the project’s pin_mux.c, add the LPUART3 mux (or copy from the UART3 example):

IOMUXC_SetPinMux(IOMUXC_GPIO_AD_B1_06_LPUART3_TXD, 0U);
IOMUXC_SetPinMux(IOMUXC_GPIO_AD_B1_07_LPUART3_RXD, 0U);
IOMUXC_SetPinConfig(IOMUXC_GPIO_AD_B1_06_LPUART3_TXD, 0x10B0U);
IOMUXC_SetPinConfig(IOMUXC_GPIO_AD_B1_07_LPUART3_RXD, 0x10B0U);

### Step B3 — Replace lpuart_interrupt.c with this integrated receiver

/*
 * Exercise 2 — Board B (RX): per-byte RX interrupt storm + GPT jitter monitor
 * Base project: boards/evkbimxrt1050/driver_examples/lpuart/interrupt
 */

#include <stdint.h>
#include <stdbool.h>
#include <stdlib.h>

#include "fsl_debug_console.h"
#include "board.h"
#include "app.h"
#include "fsl_lpuart.h"
#include "fsl_gpt.h"
#include "fsl_common_arm.h"

#define RX_BAUDRATE               (921600U)
#define DEMO_RING_BUFFER_SIZE     (512U)

#define TICK_HZ                   (10000U)
#define SAMPLE_COUNT              (2048U)

#define GPT_ISR_PRIORITY          (1U)
#define UART_ISR_PRIORITY         (3U)

/* --- DWT cycle counter --- */
static inline void DWT_Init(void)
{
    CoreDebug->DEMCR |= CoreDebug_DEMCR_TRCENA_Msk;
    DWT->CYCCNT = 0U;
    DWT->CTRL |= DWT_CTRL_CYCCNTENA_Msk;
}

/* --- Measurement buffers --- */
static volatile uint32_t s_latencyTicks[SAMPLE_COUNT];
static volatile uint32_t s_wr = 0U;
static volatile bool s_batchReady = false;

static volatile uint32_t s_expectedCompare = 0U;
static volatile uint32_t s_periodTicks = 0U;
static volatile uint32_t s_timerHz = 0U;

/* --- UART ring buffer (interrupt-driven) --- */
static volatile uint8_t demoRingBuffer[DEMO_RING_BUFFER_SIZE];
static volatile uint16_t rxIndex = 0U;
static volatile uint16_t txIndex = 0U;
static volatile uint32_t s_overflow = 0U;
static volatile uint32_t s_bytesRx = 0U;

/* --- Stats helpers --- */
static int CmpU32(const void *a, const void *b)
{
    uint32_t ua = *(const uint32_t *)a;
    uint32_t ub = *(const uint32_t *)b;
    if (ua < ub) return -1;
    if (ua > ub) return 1;
    return 0;
}

static void PrintLatencyStats(void)
{
    static uint32_t tmp[SAMPLE_COUNT];
    for (uint32_t i = 0; i < SAMPLE_COUNT; i++) tmp[i] = s_latencyTicks[i];
    qsort(tmp, SAMPLE_COUNT, sizeof(tmp[0]), CmpU32);

    uint32_t minv = tmp[0];
    uint32_t maxv = tmp[SAMPLE_COUNT - 1U];
    uint64_t sum = 0;
    for (uint32_t i = 0; i < SAMPLE_COUNT; i++) sum += tmp[i];
    double avg = (double)sum / (double)SAMPLE_COUNT;

    double min_us = ((double)minv * 1000000.0) / (double)s_timerHz;
    double avg_us = (avg * 1000000.0) / (double)s_timerHz;
    double max_us = ((double)maxv * 1000000.0) / (double)s_timerHz;
    double jit_us = ((double)(maxv - minv) * 1000000.0) / (double)s_timerHz;

    uint32_t p99  = tmp[(SAMPLE_COUNT * 99U) / 100U];
    uint32_t p999 = tmp[(SAMPLE_COUNT * 999U) / 1000U];

    PRINTF("
LatencyTicks: min=%u(%.3fus) avg=%.1f(%.3fus) max=%u(%.3fus) jitter=%u(%.3fus)",
           minv, min_us, avg, avg_us, maxv, max_us, (maxv - minv), jit_us);
    PRINTF("
LatencyTicks: p99=%u p99.9=%u", p99, p999);
}

/* --- GPT ISR (10kHz lifeline tick) --- */
void EXAMPLE_GPT_IRQHandler(void)
{
    /* Probe high at ISR entry */
    GPIO_PortSet(BOARD_USER_LED_GPIO, (1U << BOARD_USER_LED_GPIO_PIN));

    GPT_ClearStatusFlags(EXAMPLE_GPT, kGPT_OutputCompare1Flag);

    uint32_t now = GPT_GetCurrentTimerCount(EXAMPLE_GPT);
    uint32_t lat = now - s_expectedCompare;

    uint32_t idx = s_wr;
    if (idx < SAMPLE_COUNT)
    {
        s_latencyTicks[idx] = lat;
        s_wr = idx + 1U;

        /* Strict periodic schedule */
        s_expectedCompare += s_periodTicks;
        GPT_SetOutputCompareValue(EXAMPLE_GPT, kGPT_OutputCompare_Channel1, s_expectedCompare);

        if (s_wr >= SAMPLE_COUNT)
        {
            s_batchReady = true;
            GPT_DisableInterrupts(EXAMPLE_GPT, kGPT_OutputCompare1InterruptEnable);
        }
    }

    GPIO_PortClear(BOARD_USER_LED_GPIO, (1U << BOARD_USER_LED_GPIO_PIN));
    SDK_ISR_EXIT_BARRIER;
}

/* --- UART RX ISR: per-byte interrupt load --- */
void DEMO_LPUART_IRQHandler(void)
{
    uint8_t data;
    uint16_t tmprxIndex = rxIndex;
    uint16_t tmptxIndex = txIndex;

    if ((kLPUART_RxDataRegFullFlag)&LPUART_GetStatusFlags(DEMO_LPUART))
    {
        data = LPUART_ReadByte(DEMO_LPUART);
        s_bytesRx++;

        /* Ring buffer insert (drop on overflow) */
        if (((tmprxIndex + 1U) % DEMO_RING_BUFFER_SIZE) != tmptxIndex)
        {
            demoRingBuffer[rxIndex] = data;
            rxIndex++;
            rxIndex %= DEMO_RING_BUFFER_SIZE;
        }
        else
        {
            s_overflow++;
        }
    }

    SDK_ISR_EXIT_BARRIER;
}

int main(void)
{
    BOARD_InitHardware();
    DWT_Init();

    USER_LED_INIT(LOGIC_LED_OFF);

    /* ---- UART3 (demo) init ---- */
    lpuart_config_t cfg;
    LPUART_GetDefaultConfig(&cfg);
    cfg.baudRate_Bps = RX_BAUDRATE;
    cfg.enableTx     = false;
    cfg.enableRx     = true;

    LPUART_Init(DEMO_LPUART, &cfg, DEMO_LPUART_CLK_FREQ);

    NVIC_SetPriority(DEMO_LPUART_IRQn, UART_ISR_PRIORITY);
    LPUART_EnableInterrupts(DEMO_LPUART, kLPUART_RxDataRegFullInterruptEnable);
    EnableIRQ(DEMO_LPUART_IRQn);

    /* ---- GPT2 10kHz tick init ---- */
    gpt_config_t gcfg;
    GPT_GetDefaultConfig(&gcfg);
    gcfg.enableFreeRun = true;
    GPT_Init(EXAMPLE_GPT, &gcfg);

    uint32_t srcHz = EXAMPLE_GPT_CLK_FREQ;
    uint32_t divider = srcHz / 1000000U;
    if (divider < 1U) divider = 1U;
    if (divider > 4096U) divider = 4096U;

    GPT_SetClockDivider(EXAMPLE_GPT, divider);
    s_timerHz = srcHz / divider;

    s_periodTicks = s_timerHz / TICK_HZ;
    uint32_t now = GPT_GetCurrentTimerCount(EXAMPLE_GPT);
    s_expectedCompare = now + s_periodTicks;
    GPT_SetOutputCompareValue(EXAMPLE_GPT, kGPT_OutputCompare_Channel1, s_expectedCompare);

    NVIC_SetPriority(GPT_IRQ_ID, GPT_ISR_PRIORITY);
    GPT_EnableInterrupts(EXAMPLE_GPT, kGPT_OutputCompare1InterruptEnable);
    EnableIRQ(GPT_IRQ_ID);

    PRINTF("
[RX-IRQ] UART3 per-byte IRQ @%u bps + GPT 10kHz tick", RX_BAUDRATE);
    PRINTF("
Priorities: GPT=%u (high), UART=%u (lower)
", GPT_ISR_PRIORITY, UART_ISR_PRIORITY);

    PRINTF("
Press any key to start collecting %u tick samples...", SAMPLE_COUNT);
    GETCHAR();

    GPT_StartTimer(EXAMPLE_GPT);

    while (1)
    {
        if (s_batchReady)
        {
            GPT_StopTimer(EXAMPLE_GPT);

            PRINTF("

Batch complete. bytesRx=%u overflow=%u", (unsigned)s_bytesRx, (unsigned)s_overflow);
            PrintLatencyStats();

            /* Reset batch */
            s_wr = 0U;
            s_batchReady = false;

            uint32_t t = GPT_GetCurrentTimerCount(EXAMPLE_GPT);
            s_expectedCompare = t + s_periodTicks;
            GPT_SetOutputCompareValue(EXAMPLE_GPT, kGPT_OutputCompare_Channel1, s_expectedCompare);

            GPT_EnableInterrupts(EXAMPLE_GPT, kGPT_OutputCompare1InterruptEnable);
            GPT_StartTimer(EXAMPLE_GPT);

            /* Reset counters per batch */
            s_bytesRx = 0U;
            s_overflow = 0U;
        }

        __WFI();
    }
}

## 7) Solution code — Project C (Board B RX EDMA ring buffer + GPT jitter)

### Step C1 — Switch EXAMPLE_LPUART to LPUART3 and update DMA request mux

In boards/.../lpuart/edma_rb_transfer/app.h, change:

- #define EXAMPLE_LPUART LPUART1 → LPUART3

- kDmaRequestMuxLPUART1Tx/Rx → kDmaRequestMuxLPUART3Tx/Rx

- IRQ handler macro: LPUART3_IRQHandler

Keep DMAMUX/DMA base as-is (DMAMUX + DMA0).

### Step C2 — Ensure LPUART3 pin mux is set

Same as Project B (add the LPUART3 mux/config in pin_mux.c).

### Step C3 — Replace lpuart_edma_rb_transfer.c with this integrated receiver

/*
 * Exercise 2 — Board B (RX): EDMA ring buffer + GPT jitter monitor
 * Base project: boards/evkbimxrt1050/driver_examples/lpuart/edma_rb_transfer
 */

#include <stdint.h>
#include <stdbool.h>
#include <stdlib.h>

#include "fsl_debug_console.h"
#include "board.h"
#include "app.h"
#include "fsl_lpuart.h"
#include "fsl_gpt.h"
#include "fsl_edma.h"
#include "fsl_dmamux.h"
#include "fsl_common_arm.h"

#define UART_BAUDRATE           (921600U)
#define EXAMPLE_RING_BUFFER_SIZE (512U)

#define TICK_HZ                 (10000U)
#define SAMPLE_COUNT            (2048U)

#define GPT_ISR_PRIORITY        (1U)
#define EDMA_ISR_PRIORITY       (3U)

/* --- GPT jitter measurement --- */
static volatile uint32_t s_latencyTicks[SAMPLE_COUNT];
static volatile uint32_t s_wr = 0U;
static volatile bool s_batchReady = false;
static volatile uint32_t s_expectedCompare = 0U;
static volatile uint32_t s_periodTicks = 0U;
static volatile uint32_t s_timerHz = 0U;

/* --- EDMA / ring buffer (non-cacheable to avoid coherency jitter) --- */
AT_NONCACHEABLE_SECTION_INIT(uint8_t g_ringBuffer[EXAMPLE_RING_BUFFER_SIZE]) = {0};
AT_NONCACHEABLE_SECTION_ALIGN(static edma_tcd_t tcdMemoryPoolPtr[1], sizeof(edma_tcd_t));

static lpuart_edma_handle_t g_lpuartEdmaHandle;
static edma_handle_t g_lpuartTxEdmaHandle;
static edma_handle_t g_lpuartRxEdmaHandle;

static volatile uint32_t ringBufferIndex = 0U;
static volatile uint32_t s_bytesRx = 0U;

static void EXAMPLE_InitEDMA(void)
{
    edma_config_t config;

    DMAMUX_Init(EXAMPLE_LPUART_DMAMUX_BASEADDR);

    DMAMUX_SetSource(EXAMPLE_LPUART_DMAMUX_BASEADDR, LPUART_TX_DMA_CHANNEL, LPUART_TX_DMA_REQUEST);
    DMAMUX_SetSource(EXAMPLE_LPUART_DMAMUX_BASEADDR, LPUART_RX_DMA_CHANNEL, LPUART_RX_DMA_REQUEST);
    DMAMUX_EnableChannel(EXAMPLE_LPUART_DMAMUX_BASEADDR, LPUART_TX_DMA_CHANNEL);
    DMAMUX_EnableChannel(EXAMPLE_LPUART_DMAMUX_BASEADDR, LPUART_RX_DMA_CHANNEL);

    EDMA_GetDefaultConfig(&config);
    EDMA_Init(EXAMPLE_LPUART_DMA_BASEADDR, &config);

    EDMA_CreateHandle(&g_lpuartTxEdmaHandle, EXAMPLE_LPUART_DMA_BASEADDR, LPUART_TX_DMA_CHANNEL);
    EDMA_CreateHandle(&g_lpuartRxEdmaHandle, EXAMPLE_LPUART_DMA_BASEADDR, LPUART_RX_DMA_CHANNEL);

    NVIC_SetPriority(EXAMPLE_LPUART_DMA_TX_IRQn, EDMA_ISR_PRIORITY);
    NVIC_SetPriority(EXAMPLE_LPUART_DMA_RX_IRQn, EDMA_ISR_PRIORITY);

    /* Create LPUART DMA handle (Tx callback unused here) */
    LPUART_TransferCreateHandleEDMA(EXAMPLE_LPUART, &g_lpuartEdmaHandle, NULL, NULL,
                                    &g_lpuartTxEdmaHandle, &g_lpuartRxEdmaHandle);
}

static void EXAMPLE_InitLPUART(void)
{
    lpuart_config_t cfg;

    LPUART_GetDefaultConfig(&cfg);
    cfg.rxIdleType   = kLPUART_IdleTypeStopBit;
    cfg.rxIdleConfig = kLPUART_IdleCharacter2;
    cfg.baudRate_Bps = UART_BAUDRATE;
    cfg.enableTx     = false;
    cfg.enableRx     = true;

    LPUART_Init(EXAMPLE_LPUART, &cfg, EXAMPLE_LPUART_CLK_FREQ);
}

static void EXAMPLE_StartRingBufferEDMA(void)
{
    edma_transfer_config_t xferConfig;

    EDMA_InstallTCDMemory(&g_lpuartRxEdmaHandle, (edma_tcd_t *)&tcdMemoryPoolPtr[0], 1U);

    EDMA_PrepareTransfer(&xferConfig,
                         (void *)(uint32_t *)LPUART_GetDataRegisterAddress(EXAMPLE_LPUART),
                         sizeof(uint8_t),
                         (void *)g_ringBuffer,
                         sizeof(uint8_t),
                         sizeof(uint8_t),
                         EXAMPLE_RING_BUFFER_SIZE,
                         kEDMA_PeripheralToMemory);

    g_lpuartRxEdmaHandle.tcdUsed = 1U;
    g_lpuartRxEdmaHandle.tail    = 0U;

#if defined FSL_EDMA_DRIVER_EDMA4 && FSL_EDMA_DRIVER_EDMA4
    EDMA_TcdResetExt(g_lpuartRxEdmaHandle.base, &g_lpuartRxEdmaHandle.tcdPool[0U]);
    EDMA_TcdSetTransferConfigExt(g_lpuartRxEdmaHandle.base, &g_lpuartRxEdmaHandle.tcdPool[0U], &xferConfig,
                                 tcdMemoryPoolPtr);
    EDMA_TcdEnableInterruptsExt(g_lpuartRxEdmaHandle.base, &g_lpuartRxEdmaHandle.tcdPool[0U],
                                kEDMA_MajorInterruptEnable);
#else
    EDMA_TcdReset(&g_lpuartRxEdmaHandle.tcdPool[0U]);
    EDMA_TcdSetTransferConfig(&g_lpuartRxEdmaHandle.tcdPool[0U], &xferConfig, tcdMemoryPoolPtr);
    EDMA_TcdEnableInterrupts(&g_lpuartRxEdmaHandle.tcdPool[0U], kEDMA_MajorInterruptEnable);
#endif

    EDMA_SubmitTransfer(&g_lpuartRxEdmaHandle, &xferConfig);

    /* Enable LPUART Rx DMA request */
    LPUART_EnableRxDMA(EXAMPLE_LPUART, true);

    EDMA_StartTransfer(&g_lpuartRxEdmaHandle);
}

/* RX EDMA IRQ handler: count bytes in major loop chunks (low frequency vs per-byte IRQ) */
void EXAMPLE_LPUART_DMA_RX_IRQHandler(void)
{
    EDMA_HandleIRQ(&g_lpuartRxEdmaHandle);

    /* One major loop completed => EXAMPLE_RING_BUFFER_SIZE bytes moved */
    ringBufferIndex += EXAMPLE_RING_BUFFER_SIZE;
    s_bytesRx += EXAMPLE_RING_BUFFER_SIZE;
}

/* --- GPT ISR (10kHz lifeline tick) --- */
void EXAMPLE_GPT_IRQHandler(void)
{
    GPIO_PortSet(BOARD_USER_LED_GPIO, (1U << BOARD_USER_LED_GPIO_PIN));

    GPT_ClearStatusFlags(EXAMPLE_GPT, kGPT_OutputCompare1Flag);

    uint32_t now = GPT_GetCurrentTimerCount(EXAMPLE_GPT);
    uint32_t lat = now - s_expectedCompare;

    uint32_t idx = s_wr;
    if (idx < SAMPLE_COUNT)
    {
        s_latencyTicks[idx] = lat;
        s_wr = idx + 1U;

        s_expectedCompare += s_periodTicks;
        GPT_SetOutputCompareValue(EXAMPLE_GPT, kGPT_OutputCompare_Channel1, s_expectedCompare);

        if (s_wr >= SAMPLE_COUNT)
        {
            s_batchReady = true;
            GPT_DisableInterrupts(EXAMPLE_GPT, kGPT_OutputCompare1InterruptEnable);
        }
    }

    GPIO_PortClear(BOARD_USER_LED_GPIO, (1U << BOARD_USER_LED_GPIO_PIN));
    SDK_ISR_EXIT_BARRIER;
}

static int CmpU32(const void *a, const void *b)
{
    uint32_t ua = *(const uint32_t *)a;
    uint32_t ub = *(const uint32_t *)b;
    if (ua < ub) return -1;
    if (ua > ub) return 1;
    return 0;
}

static void PrintLatencyStats(void)
{
    static uint32_t tmp[SAMPLE_COUNT];
    for (uint32_t i = 0; i < SAMPLE_COUNT; i++) tmp[i] = s_latencyTicks[i];
    qsort(tmp, SAMPLE_COUNT, sizeof(tmp[0]), CmpU32);

    uint32_t minv = tmp[0];
    uint32_t maxv = tmp[SAMPLE_COUNT - 1U];
    uint64_t sum = 0;
    for (uint32_t i = 0; i < SAMPLE_COUNT; i++) sum += tmp[i];
    double avg = (double)sum / (double)SAMPLE_COUNT;

    double min_us = ((double)minv * 1000000.0) / (double)s_timerHz;
    double avg_us = (avg * 1000000.0) / (double)s_timerHz;
    double max_us = ((double)maxv * 1000000.0) / (double)s_timerHz;
    double jit_us = ((double)(maxv - minv) * 1000000.0) / (double)s_timerHz;

    uint32_t p99  = tmp[(SAMPLE_COUNT * 99U) / 100U];
    uint32_t p999 = tmp[(SAMPLE_COUNT * 999U) / 1000U];

    PRINTF("
LatencyTicks: min=%u(%.3fus) avg=%.1f(%.3fus) max=%u(%.3fus) jitter=%u(%.3fus)",
           minv, min_us, avg, avg_us, maxv, max_us, (maxv - minv), jit_us);
    PRINTF("
LatencyTicks: p99=%u p99.9=%u", p99, p999);
}

int main(void)
{
    BOARD_InitHardware();
    USER_LED_INIT(LOGIC_LED_OFF);

    EXAMPLE_InitEDMA();
    EXAMPLE_InitLPUART();
    EXAMPLE_StartRingBufferEDMA();

    /* ---- GPT2 10kHz tick init ---- */
    gpt_config_t gcfg;
    GPT_GetDefaultConfig(&gcfg);
    gcfg.enableFreeRun = true;
    GPT_Init(EXAMPLE_GPT, &gcfg);

    uint32_t srcHz = EXAMPLE_GPT_CLK_FREQ;
    uint32_t divider = srcHz / 1000000U;
    if (divider < 1U) divider = 1U;
    if (divider > 4096U) divider = 4096U;

    GPT_SetClockDivider(EXAMPLE_GPT, divider);
    s_timerHz = srcHz / divider;

    s_periodTicks = s_timerHz / TICK_HZ;
    uint32_t now = GPT_GetCurrentTimerCount(EXAMPLE_GPT);
    s_expectedCompare = now + s_periodTicks;
    GPT_SetOutputCompareValue(EXAMPLE_GPT, kGPT_OutputCompare_Channel1, s_expectedCompare);

    NVIC_SetPriority(GPT_IRQ_ID, GPT_ISR_PRIORITY);
    GPT_EnableInterrupts(EXAMPLE_GPT, kGPT_OutputCompare1InterruptEnable);
    EnableIRQ(GPT_IRQ_ID);

    PRINTF("
[RX-EDMA] UART3 EDMA ring buffer @%u bps + GPT 10kHz tick", UART_BAUDRATE);
    PRINTF("
Priorities: GPT=%u (high), EDMA=%u (lower)
", GPT_ISR_PRIORITY, EDMA_ISR_PRIORITY);

    PRINTF("
Press any key to start collecting %u tick samples...", SAMPLE_COUNT);
    GETCHAR();

    GPT_StartTimer(EXAMPLE_GPT);

    while (1)
    {
        if (s_batchReady)
        {
            GPT_StopTimer(EXAMPLE_GPT);

            PRINTF("

Batch complete. bytesRx(approx)=%u", (unsigned)s_bytesRx);
            PrintLatencyStats();

            /* Reset batch */
            s_wr = 0U;
            s_batchReady = false;
            s_bytesRx = 0U;

            uint32_t t = GPT_GetCurrentTimerCount(EXAMPLE_GPT);
            s_expectedCompare = t + s_periodTicks;
            GPT_SetOutputCompareValue(EXAMPLE_GPT, kGPT_OutputCompare_Channel1, s_expectedCompare);

            GPT_EnableInterrupts(EXAMPLE_GPT, kGPT_OutputCompare1InterruptEnable);
            GPT_StartTimer(EXAMPLE_GPT);
        }

        __WFI();
    }
}

## 8) What to expect (results interpretation)

- RX-IRQ (per-byte ISR): jitter increases and worst-case latency spikes show up, especially as baud rate rises.

- RX-EDMA: much lower ISR rate (major-loop completion interrupts) → far fewer timing outliers.

If you still see big outliers in EDMA mode: - verify DMA buffers are non-cacheable (the AT_NONCACHEABLE_SECTION_* macros) - ensure GPT tick priority is higher than EDMA/UART

## 9) MCUXpresso build/run checklist

For each project: 1. Import SDK example. 2. Build. 3. Use Debug to flash. 4. Open UART terminal on the debug console COM port for printed stats. 5. Wire J22 pins between boards.