## 0) What you’re building and demonstrating

You will run firmware on the i.MX RT1050 EVKB that:

- Receives UART frames carrying ARINC-429 words (from the ADK-8582 bridge).

- Validates them in layers:

  - UART frame checksum (CHK)

  - ARINC odd parity

  - (optional) label/allow-list plausibility

- Runs a deterministic fault campaign with named “fault sites”:

  - RX_CORRUPT (byte corruption on UART RX path)

  - TX_API_FAIL (synthetic UART TX API failures)

  - A429_LABEL_TAMPER and A429_PARITY_TAMPER (field-aware corruption)

- Provides a tiny console to re-arm faults and dump the event log.

This matches the module’s intent: controlled corruption + controlled driver/API failures, with reproducible evidence.

## 1) UART frame contract (what the EVKB expects)

The exercise assumes the ADK-8582 emits each ARINC word as a UART frame:

[0xA5][LEN=4][W0][W1][W2][W3][CHK]

- W0..W3 = 32-bit ARINC word bytes (choose endianness and keep consistent)

- CHK = 8-bit sum of payload bytes (your solution checks this before ARINC checks)

## 2) Physical wiring checklist (EVKB ↔ ADK-8582 UART)

### EVKB UART pins you can use quickly (Arduino header)

The EVKB user guide shows the Arduino expansion port includes UART signals:

- D0 = UART_RX

- D1 = UART_TX

### Wiring rules

- ADK UART TX → EVKB D0 (UART_RX)

- ADK UART RX ← EVKB D1 (UART_TX) (only if your ADK side expects responses/acks)

- GND ↔ GND (mandatory)

### Electrical level sanity check (important)

Before connecting TX/RX:

- Confirm the ADK-8582 UART IO voltage level (3.3V vs 5V).

- If it’s not 3.3V TTL compatible, use a level shifter.

(I did not find the ADK demo-board “UART header pinout” details in the searchable excerpts here, so you’ll need to use your ADK8582 demo board schematic/user docs to locate the correct UART header/connector on that side.)

### UART settings

Use 115200-8-N-1 (typical for this bridge approach).

## 3) MCUXpresso IDE: create the EVKB project from the SDK (no guessing APIs)

The module explicitly starts from the NXP LPUART interrupt example for EVKB-IMXRT1050.

### 3.1 Import the SDK example project

- Launch MCUXpresso IDE and choose a workspace folder.

- Use Quickstart Panel (left side) → Import SDK example(s)

  - If you don’t see it: File → Import → MCUXpresso IDE → Import SDK example projects.

- Point MCUXpresso at the extracted SDK folder for SDK_25_06_00_EVKB-IMXRT1050 (same baseline mentioned in the module).

- Filter for EVKBIMXRT1050 + lpuart and import the closest match to:

  - boards/evkbimxrt1050/driver_examples/lpuart/interrupt/ (module baseline)

  - If your code solution used nonblocking transfers/handle callbacks, importing an interrupt_transfer example is also fine (still an LPUART driver example).

### 3.2 Build once before adding your fault campaign code

- Project → Build Project

- Fix any toolchain/SDK path issues now, while it’s still “vanilla.”

## 4) Drop in the code solution modules (from the canvas)

In the imported example project, add the source files from the code solution you already have (canvas). Conceptually, they fall into these buckets:

### A) Fault Injection harness (FI core)

- Implements deterministic sites, counters, event log, and corruption helpers.

- Default seed + deterministic arming pattern (example: corrupt every N hits) aligns with the module’s guidance.

### B) UART frame parser + ARINC checks

- Parses [0xA5][LEN=4]…[CHK]

- Computes CHK, builds 32-bit word, checks odd parity, extracts label/fields.

### C) Fault sites integrated at the right places

- RX data corruption injection happens before a received byte is committed (ring buffer / receive pipeline).

- TX API failure injection wraps the driver boundary so send calls can return synthetic “busy/fail” and exercise retry logic.

### D) Minimal runtime console

- Supports at least:

  - fi dump

  - fi arm <site> every <N> [bits=<M>]

### How to add files in MCUXpresso

- In Project Explorer → right click project → New → Source Folder (optional, e.g., app/)

- Right click folder → Import… → General → File System (or drag-drop if enabled)

- Ensure the new .c/.h are included in the build:

  - Right click project → Properties → C/C++ Build → Settings

  - Confirm include paths (where your new headers live)

## 5) Pin mux and UART instance selection (so debug console ≠ data UART)

The module allows using LPUART1 (default debug UART) or a second LPUART.

Recommendation for sanity:

- Keep debug console UART on the default (what the example already uses).

- Use a second LPUART for the ADK data stream if your wiring would otherwise collide with the debug port.

### If you must change UART pins/instances

Use the MCUXpresso Pins tool:

- Open pins_config / Config Tools → Pins

- Assign the chosen LPUART TX/RX signals to the header pins you physically use (e.g., Arduino D0/D1 if they map cleanly in your board routing).

- Rebuild; verify no peripheral conflicts.

## 6) Build + flash + open the console

### 6.1 Flash

- Click Debug (green bug) to program the board.

- Once it halts at main(), click Resume or Terminate (either is fine as long as the image is programmed and running).

### 6.2 Console

Open a serial terminal on the EVKB’s debug COM port:

- 115200-8-N-1

- You should see the startup banner from your solution (including “SFI enabled” style indicator if you included it—recommended by the module).

## 7) Bring-up procedure (do this in order)

### Step 1 — Verify UART traffic without fault injection noise

- Start the ADK-8582 UART stream (or a UART generator).

- Confirm:

  - rx_ok increments for valid frames

  - rx_bad stays low/near zero initially

This matches the “parse and validate, increment counters, don’t forward on failure” flow.

### Step 2 — Enable RX_CORRUPT deterministically

Arm the corruption site (either compile-time default or via console):

- Example target behavior: corrupt 3 bits every 200 received bytes.

Expected observation

- rx_bad increases at the expected cadence

- Console shows RX_CORRUPT events

- LED toggles per injection (if you wired that in)

- System stays responsive for extended runtime (module suggests >10 minutes).

### Step 3 — Enable ARINC field-aware faults

Arm:

- A429_LABEL_TAMPER (e.g., EVERY_N = 37)

- A429_PARITY_TAMPER (e.g., EVERY_N = 53)

The module’s intent is: checksum catches some faults, parity catches parity tamper, allow-list catches label tamper; invalid values must not propagate.

### Step 4 — Enable TX_API_FAIL and confirm recovery behavior

Arm the API failure so that:

- every ~100th TX request returns “busy/fail” to exercise retry logic.

Expected observation

- Your TX progress counter does not stall beyond the deadline (module example: >100 ms stall is a fail).

- Event log shows TX_API_FAIL injections.

## 8) Using the event log for evidence (what to capture)

### fi dump

The module’s FI log dump format includes event index, site name, hit count, args, and “peek bytes”.

What you should save

- Terminal capture showing:

  - Counters (rx_ok, rx_bad, parity rejects, label rejects, tx retries)

  - fi dump output at multiple times

- A short “pass/fail matrix” (the module’s suggested deliverable style).

## 9) Troubleshooting (fast fixes)

### If rx_ok never increments

- Recheck TX↔RX swap and GND.

- Confirm the ADK side is emitting the 0xA5 header and correct CHK.

- Confirm 115200-8-N-1.

### If the debug console becomes garbage

- You’re on the wrong COM port or wrong baud (115200).

- Possible UART instance conflict: move the ADK data link to a second LPUART.

### If the board resets or becomes sluggish during injections

- Keep ISR work bounded; avoid heavy printing from interrupts (module explicitly warns to defer heavy output).

## 10) End condition (what “done” looks like)

You’re finished when:

- All injected faults are detected at the correct stage (CHK/parity/allow-list)

- No corrupted ARINC word updates your “good output”

- Your firmware shows recovery from TX API failures without deadlock/stall

- The campaign is reproducible (fixed seed + deterministic EVERY_N), and you can prove it via logs.