# Line-by-line Walkthrough — Deterministic SFI Campaign Codebase (EVKB-IMXRT1050)

This walkthrough explains the reference implementation file-by-file and then function-by-function / line-by-line, explicitly tying each line to the exercise goals:

- Fault sites: RX corruption, ARINC label/parity tamper, TX API failure, TX stall simulation.

- Determinism: fixed seed, per-site hit counters, repeatable triggers.

- Barriers: CHK → odd parity → plausibility (allow-list + rate limit).

- Recovery: bounded retry on TxBusy, PIT-based stall monitor recovery.

- Observability: LED toggle/solid error, counters, CLI.

Note: I describe “lines” in the order they appear in each file. Where many lines serve a single purpose (e.g., struct fields), I still explain each item in sequence.

## File: source/app.h

### Purpose

Holds compile-time configuration: which LPUART is debug console vs data link, which PIT channel is used for the 10ms tick.

### Walkthrough

#define DEMO_LPUART          LPUART1

- Keeps the original SDK example’s debug console UART on LPUART1 so PRINTF and the CLI continue to work.

#define DEMO_LPUART_CLK_FREQ BOARD_DebugConsoleSrcFreq()

- Uses the SDK board helper to obtain the clock used for the debug console.

#define DATA_LPUART          LPUART3

- Chooses a second UART for the ADK/HI-8582 data stream. This prevents collisions with the debug console.

#define DATA_LPUART_IRQn     LPUART3_IRQn
#define DATA_LPUART_IRQHandler LPUART3_IRQHandler

- Names the interrupt line and handler symbol used by this LPUART instance.

#define DATA_LPUART_CLK_FREQ BOARD_DebugConsoleSrcFreq()

- Uses the same clock-frequency helper. If your data UART is clocked differently, you must change this to the correct clock getter.

#define DATA_LPUART_BAUDRATE (115200U)

- Enforces the exercise requirement: keep the UART link at 115200.

#define DEMO_PIT_BASEADDR PIT
#define DEMO_PIT_CHANNEL  kPIT_Chnl_0
#define PIT_IRQ_ID        PIT_IRQn
#define PIT_IRQ_HANDLER   PIT_IRQHandler
#define PIT_SOURCE_CLOCK  CLOCK_GetFreq(kCLOCK_OscClk)

- Selects the PIT peripheral and channel used to create a 10ms periodic tick.

- This tick drives (a) deterministic timestamps in FI logs and (b) TX stall recovery.

## File: source/fi.h

### Purpose

Declares the fault injection framework API, including: - Site enumeration - Trigger policies - Per-site configuration (hit counts, every-N/window) - Event logging

### Walkthrough

#ifndef SFI_ENABLED
#define SFI_ENABLED 1
#endif

- Allows you to compile a clean build by setting SFI_ENABLED=0 in preprocessor symbols.

typedef enum { ... } fi_site_t;

- Declares the named injection points:

  - FI_SITE_RX_CORRUPT: corrupt incoming bytes

  - FI_SITE_A429_LABEL_TAMPER: flip label field after parse

  - FI_SITE_A429_PARITY_TAMPER: flip parity bit after parse

  - FI_SITE_TX_API_FAIL: return TxBusy without calling driver

  - FI_SITE_TX_STALL: simulate lost TX completion (stall)

typedef enum { FI_POLICY_DISABLED, FI_POLICY_EVERY_N, FI_POLICY_WINDOW } fi_policy_t;

- Defines how each site is triggered:

  - DISABLED: never fires

  - EVERY_N: fires on hit N, 2N, 3N…

  - WINDOW: fires when hit count is inside [start, end)

typedef struct { ... } fi_site_cfg_t;

Line-by-line fields: - policy: which trigger policy is active - every_n: parameter for EVERY_N - window_start, window_end: parameters for WINDOW - hit_count: increments every time the code reaches that injection site - fire_count: increments every time a fault is actually injected - bits_to_flip: only meaningful for RX_CORRUPT (how many bits per byte)

typedef struct { ... } fi_event_t;

Event record fields in sequence: - ts_ms: timestamp when event was logged - site: which fault site - fired: 1 if fault injected, 0 if only recorded - hit: site hit count at that time - extra: a small numeric parameter (e.g., length) - data_peek: a small sample of data (e.g., byte or word)

Function declarations: - FI_Init(seed): fixed seed gives reproducibility - FI_SetNowMs(): connects to PIT timebase - FI_SiteCfg(): returns pointer to the per-site config for runtime tuning - FI_ShouldFire(): increments hit count and returns fire/no-fire based on policy - FI_CorruptByteDeterministic(): flips bits using deterministic RNG - FI_Log(): pushes event into ring buffer

## File: source/fi.c

### Purpose

Implements deterministic fault triggering and logging.

### Walkthrough (top-of-file)

#if SFI_ENABLED

- All real logic is compiled only if SFI is enabled.

#define FI_LOG_DEPTH 128

- Size of event log ring buffer.

static volatile uint32_t g_now_ms;

- Global timestamp (ms). Updated by PIT tick.

static fi_site_cfg_t g_sites[FI_SITE_COUNT];

- One config struct per fault site.

static fi_event_t g_log[FI_LOG_DEPTH];
static volatile uint32_t g_log_wr;

- Ring buffer storage plus write index.

static uint32_t g_rng;

- Deterministic PRNG state.

### Function: xorshift32

x ^= x << 13;
x ^= x >> 17;
x ^= x << 5;

- Minimal deterministic PRNG transform. Same seed → same random sequence.

### Function: FI_Init(seed)

g_now_ms = 0;

- Start time at zero.

g_rng = (seed == 0U) ? 0xA5A5A5A5U : seed;

- Ensures seed is never 0 (xorshift32 degenerates with 0).

for (...) { ... }

- Initializes each site:

  - policy disabled

  - hit/fire counters cleared

  - default bits_to_flip=1

Default arming block:

g_sites[FI_SITE_RX_CORRUPT].policy = FI_POLICY_EVERY_N;
g_sites[FI_SITE_RX_CORRUPT].every_n = 200;
g_sites[FI_SITE_RX_CORRUPT].bits_to_flip = 3;

- Implements “corrupt 3 bits every 200 bytes” campaign.

g_sites[FI_SITE_TX_API_FAIL].policy = FI_POLICY_EVERY_N;
g_sites[FI_SITE_TX_API_FAIL].every_n = 100;

- Implements “return TxBusy every 100th TX request”.

g_sites[FI_SITE_TX_STALL].policy = FI_POLICY_WINDOW;
g_sites[FI_SITE_TX_STALL].window_start = 500;
g_sites[FI_SITE_TX_STALL].window_end   = 520;

- Implements the stall window campaign: between hit 500 and 519 inclusive (end exclusive), stall behavior is active.

g_sites[FI_SITE_A429_LABEL_TAMPER].policy = FI_POLICY_DISABLED;
g_sites[FI_SITE_A429_PARITY_TAMPER].policy = FI_POLICY_DISABLED;

- Starts label/parity tamper disabled so you can arm them intentionally via CLI.

g_log_wr = 0;

- Starts log write index at 0.

### Functions: FI_SetNowMs, FI_NowMs

- Simple setters/getters for the timebase.

### Function: FI_SiteCfg(site)

if ((int)site < 0 || site >= FI_SITE_COUNT) return 0;
return &g_sites[site];

- Defensive bounds check and returns config pointer.

### Function: FI_DisableAll()

for (...) g_sites[i].policy = FI_POLICY_DISABLED;

- Used by CLI fi off to shut down all injections at runtime.

### Function: FI_Rand32()

g_rng = xorshift32(g_rng);
return g_rng;

- Advances and returns PRNG state.

### Function: FI_ShouldFire(site)

cfg->hit_count++;

- This is the key “determinism counter”: every reach increments hit count.

Switch on policy: - DISABLED: always false - EVERY_N: true when hit_count % every_n == 0 - WINDOW: true when hit_count is within [start, end)

if (fire) cfg->fire_count++;

- Separately counts how many actual injections occurred.

### Function: FI_CorruptByteDeterministic(in)

fi_site_cfg_t *cfg = FI_SiteCfg(FI_SITE_RX_CORRUPT);
uint8_t out = in;

- Uses RX_CORRUPT config (especially bits_to_flip).

const uint8_t flips = (cfg && cfg->bits_to_flip) ? cfg->bits_to_flip : 1U;

- Ensures at least 1 bit flip if misconfigured.

Loop:

uint32_t r = FI_Rand32();
uint8_t bit = (uint8_t)(r & 0x7U);
out ^= (uint8_t)(1U << bit);

- Picks bit index 0–7 deterministically and flips it.

### Function: FI_Log(site, fired, hit, extra, data_peek)

uint32_t idx = g_log_wr++;
fi_event_t *e = &g_log[idx % FI_LOG_DEPTH];

- Ring buffer indexing: overwrites oldest when full.

e->ts_ms = FI_NowMs();
...

- Copies event metadata for later dump.

### Function: site_name()

- Converts enum to printable name.

### Function: FI_DumpLog()

- Kept minimal in this reference; the CLI prints site counters as “dump.”

- If you want a full dump, you’d iterate g_log and print entries.

### #else /* SFI_ENABLED == 0 */

- Stubs out all functions so the build becomes “clean flight build.”

## File: source/a429_frame.h

### Purpose

Defines UART-frame parsing state machine and ARINC checks.

### Walkthrough

typedef enum { A429_PARSE_NONE, A429_PARSE_WORD_OK, A429_PARSE_BAD_CHK, A429_PARSE_BAD_LEN } a429_parse_result_t;

- Parser can output:

  - nothing yet

  - completed good word

  - bad checksum

  - bad length

typedef struct { uint8_t st; uint8_t len; uint8_t payload[4]; ... } a429_uart_parser_t;

- Holds state-machine variables:

  - st: current state

  - len: payload length (expect 4)

  - payload: the 4 bytes

  - pay_idx: which payload byte is next

  - chk: received checksum

- And parser-level stats.

Function declarations: - A429_ParserFeed: called per received byte. - A429_CheckOddParity: barrier #2. - A429_Label: used for plausibility allow-list.

## File: source/a429_frame.c

### Purpose

Implements UART frame parser and ARINC parity/label extraction.

### State constants

#define ST_WAIT_SYNC  0
#define ST_LEN        1
#define ST_PAYLOAD    2
#define ST_CHK        3

- Defines a simple 4-state parser.

### Function: chk8(len, payload)

uint16_t s = (uint16_t)len;
s = (uint16_t)(s + payload[0] + payload[1] + payload[2] + payload[3]);
return (uint8_t)(s & 0xFFU);

- Computes the 8-bit checksum required by the exercise.

### Function: A429_ParserInit(p)

- Resets state to WAIT_SYNC and clears stats.

### Function: A429_ParserFeed(p, byte, out_word)

State: WAIT_SYNC

if (byte == 0xA5U) p->st = ST_LEN;

- Waits for frame sync 0xA5.

State: LEN

p->len = byte;
...
if (p->len != 4U) { bad_len++; st=WAIT_SYNC; return BAD_LEN; }

- Enforces fixed 4-byte payload.

State: PAYLOAD

p->payload[p->pay_idx++] = byte;
if (p->pay_idx >= 4U) p->st = ST_CHK;

- Stores bytes W0..W3.

State: CHK

p->chk = byte;
uint8_t exp = chk8(p->len, p->payload);
if (exp != p->chk) return BAD_CHK;

- Barrier #1: checksum verification.

uint32_t w = payload[0] | payload[1]<<8 | payload[2]<<16 | payload[3]<<24;
*out_word = w;

- Assembles a 32-bit word in little-endian order.

p->frames_ok++;
return A429_PARSE_WORD_OK;

- Signals completion.

### Function: popcount32

- Portable bit-count to implement parity check.

### Function: A429_CheckOddParity(word)

return (popcount32(word) & 0x1U) == 1U;

- Odd parity across all 32 bits (including the parity bit).

### Function: A429_Label(word)

return (uint8_t)(word & 0xFFU);

- Label is taken as the lowest byte (consistent with our assembly choice).

## File: source/data_uart.h

### Purpose

Wraps the SDK LPUART nonblocking transfer APIs and adds fault injection at the driver boundary.

### Walkthrough

#include "fsl_lpuart.h"
#include "fi.h"

- Uses NXP SDK LPUART handle-based APIs + FI framework.

typedef struct { LPUART_Type *base; lpuart_handle_t handle; ... } data_uart_t;

Fields explained in order: - base: which LPUART instance - handle: SDK handle used for nonblocking transfers - rxOnGoing: true while a receive transfer is active - txOnGoing: true while a send transfer is active - rxByteReady: becomes true when one byte completes - rxByte: storage for 1-byte receive - txStallActive: when true, callback refuses to clear txOnGoing

Functions: - DataUart_Init: initialize UART + handle + callback - DataUart_StartReceive1: arms a 1-byte receive (nonblocking) - DataUart_SendNonBlocking_FI: sends with possible injected failures - DataUart_GetSendCount: used by stall monitor - DataUart_AbortSend: stall recovery

## File: source/data_uart.c

### Purpose

Implements: - callback-based RX/TX completion - injected “lost TX completion” behavior - injected “TxBusy” return without touching hardware

### Callback: DataUart_Callback(...)

if (status == kStatus_LPUART_RxIdle) { rxOnGoing=false; rxByteReady=true; }

- When one byte finishes receiving, mark it ready for main loop.

else if (status == kStatus_LPUART_TxIdle)

- Normal TX completion.

if (u->txStallActive) { return; }

- TX stall injection: simulate “lost interrupt/completion” by refusing to clear txOnGoing.

- PIT monitor must detect no progress and call AbortSend.

u->txOnGoing = false;

- Normal completion path clears the flag.

### Function: DataUart_Init(u, base, clkHz, baud)

LPUART_GetDefaultConfig(&cfg);
...
LPUART_Init(base, &cfg, clkHz);

- Standard NXP SDK initialization.

LPUART_TransferCreateHandle(base, &u->handle, DataUart_Callback, (void *)u);

- Creates a handle with our callback, giving us completion events.

### Function: DataUart_StartReceive1(u)

if (u->rxOnGoing) return kStatus_LPUART_RxBusy;

- Prevents overlapping receives.

xfer.data = &u->rxByte;
xfer.dataSize = 1U;

- Receive exactly 1 byte each time.

return LPUART_TransferReceiveNonBlocking(...);

- Arms the transfer.

### Function: DataUart_SendNonBlocking_FI(u, data, len)

Fault site: TX API fail

bool fire = FI_ShouldFire(FI_SITE_TX_API_FAIL);
...
if (fire) return kStatus_LPUART_TxBusy;

- This is the key injected API failure: returns busy without calling into the driver.

Fault site: TX stall window

bool fire = FI_ShouldFire(FI_SITE_TX_STALL);
u->txStallActive = fire;

- If active, completion callback will stall.

Normal send

u->txOnGoing = true;
return LPUART_TransferSendNonBlocking(...);

- Starts nonblocking send.

### Function: DataUart_GetSendCount

- Used by stall monitor to see whether hardware is making progress.

### Function: DataUart_AbortSend

LPUART_TransferAbortSend(...);
u->txOnGoing=false; u->txStallActive=false;

- Stops stalled transfers and resets state.

## File: source/main.c

### Purpose

This is the “exercise harness”: - Initializes board, PIT, UARTs - Runs RX parser with barrier checks - Applies FI sites (RX corrupt + label/parity tamper) - Sends ACKs back over UART to exercise TX (API fail + stall) - Provides CLI to re-arm sites

### Includes

#include "fsl_debug_console.h"

- Used for CLI and telemetry printing.

#include "fsl_pit.h"

- PIT provides deterministic 10ms tick.

#include "fi.h" "a429_frame.h" "data_uart.h"

- Connects the exercise components.

### ACK protocol

#define ACK_SYNC 0xACU

- Small 2-byte response format: [0xAC][reason].

- This is optional but useful to:

  - confirm receiver decisions

  - continuously exercise the TX path

typedef enum { ACK_OK, ACK_BAD_CHK, ... } ack_reason_t;

- Enumerates why a frame was rejected.

### ACK queue

#define ACK_Q_DEPTH 32
static ack_msg_t g_ackQ[ACK_Q_DEPTH];
static volatile uint32_t g_ackWr = 0, g_ackRd = 0;

- Nonblocking ring queue so RX processing never blocks on TX.

#### Function: AckQ_Push(r)

if ((wr - rd) >= ACK_Q_DEPTH) return false;

- Overflow protection.

m->b[0] = ACK_SYNC; m->b[1] = (uint8_t)r; m->len = 2;

- Writes the outgoing ACK message.

#### Function: AckQ_Peek(out)

- Reads next message without popping it.

#### Function: AckQ_Pop()

- Advances read pointer when TX starts successfully.

### PIT timebase

static volatile uint32_t g_ms = 0;

- Global milliseconds counter.

#### ISR: PIT_IRQHandler

PIT_ClearStatusFlags(...);
g_ms += 10U;
FI_SetNowMs(g_ms);

- Every 10ms: increment ms and feed FI timebase.

#### Pit10ms_Init()

PIT_SetTimerPeriod(... USEC_TO_COUNT(10000U, ...));

- 10,000 microseconds = 10ms.

### Plausibility barrier

static uint8_t g_allowLabel[256];
static uint32_t g_lastLabelMs[256];

- Allow-list plus last accepted timestamp per label.

static const uint32_t g_minLabelIntervalMs = 20U; /* 50 Hz */

- Rate limit requirement.

#### Plausibility_Init()

memset(g_allowLabel, 0, ...);
...
g_allowLabel[0x01] = 1; ...

- Clears and then enables a few example labels.

#### Plausibility_Accept(label)

if (!g_allowLabel[label]) return false;

- Rejects unknown labels.

if ((now - last) < g_minLabelIntervalMs) return false;

- Rejects too-frequent updates.

g_lastLabelMs[label] = now;
return true;

- Accepts and records timestamp.

### CLI

Global CLI buffer:

static char g_cliLine[96];
static uint32_t g_cliLen = 0;

- Stores user input line.

#### cli_print_cfg()

- Prints per-site policy/hit/fire counters so you can verify determinism.

#### cli_handle_line(line)

- Tokenizes input.

- Supports:

  - fi dump

  - fi off

  - fi arm rx every N bits M

  - fi arm txbusy every N

  - fi arm txstall window S E

  - fi arm label every N

  - fi arm parity every N

Key FI coupling:

fi_site_cfg_t *c = FI_SiteCfg(site);

- Gets mutable config pointer.

c->policy = FI_POLICY_EVERY_N; c->every_n = ...;

- Arms deterministically.

c->policy = FI_POLICY_WINDOW; c->window_start = ...; c->window_end = ...;

- Arms stall window.

#### cli_poll()

- Uses DbgConsole_TryGetchar so the main loop never blocks.

### Telemetry printing

if ((g_ms - last) >= 1000U) PRINTF(... counters ...);

- Prints 1-second summary:

  - rx_ok, rx_bad_chk, rx_bad_parity, rx_bad_plaus, rx_bad_len

  - tx_retries, tx_failures, tx_recoveries, tx_drops

### TX stall monitor

if (!g_dataUart.txOnGoing) return;

- Only monitor if a send is in progress.

DataUart_GetSendCount(..., &cnt)

- If send count increases, progress is being made.

if ((g_ms - g_tx_last_progress_ms) > 100U) { AbortSend(); }

- If no progress for >100ms, treat as stall.

- This is the recovery mechanism required by the exercise.

### main() initialization

BOARD_InitHardware();
USER_LED_INIT(LOGIC_LED_OFF);

- Initializes EVKB and sets LED to OFF.

PRINTF("SFI BUILD ...");
FI_Init(0xC0FFEE01U);

- Fixed seed means identical injection schedule across runs.

A429_ParserInit(&g_parser);
Plausibility_Init();
Pit10ms_Init();

- Prepares parsing + barrier #3 + timebase.

DataUart_Init(&g_dataUart, DATA_LPUART, ...);
EnableIRQ(DATA_LPUART_IRQn);

- Initializes the data UART used to talk to ADK/HI-8582.

(void)DataUart_StartReceive1(&g_dataUart);

- Arms the first byte receive.

### Main loop (core of the exercise)

#### 1) CLI polling

cli_poll();

- Allows runtime arming without stopping the campaign.

#### 2) RX “pump” (ensures continuous reception)

if (!rxOnGoing && !rxByteReady) StartReceive1();

- Always keep a 1-byte receive active.

#### 3) Byte ready path

uint8_t b = g_dataUart.rxByte;
g_dataUart.rxByteReady = false;

- Takes the received byte from driver.

##### Fault site: RX corruption (byte-level)

bool fire = FI_ShouldFire(FI_SITE_RX_CORRUPT);
if (fire) { USER_LED_TOGGLE(); b = FI_CorruptByteDeterministic(b); }

- This injects corruption before the parser/CHK barrier.

- LED toggle is immediate observability.

##### Frame parsing + barrier #1 (CHK)

a429_parse_result_t pr = A429_ParserFeed(&g_parser, b, &word);

- Feeds byte into the UART-frame parser.

if (pr == A429_PARSE_BAD_CHK) { rx_bad_chk++; USER_LED_ON(); Ack(BAD_CHK); }

- Detects corruption at checksum stage; turns LED solid ON to indicate error.

##### Completed word path

else if (pr == A429_PARSE_WORD_OK) { ... }

- Parser produced a word that passed CHK.

###### Fault site: label tamper

bool fire = FI_ShouldFire(FI_SITE_A429_LABEL_TAMPER);
if (fire) word ^= 0x00000080U;

- Flips a label bit to push it out of allow-list.

###### Fault site: parity tamper

bool fire = FI_ShouldFire(FI_SITE_A429_PARITY_TAMPER);
if (fire) word ^= 0x80000000U;

- Flips parity bit (bit31) to force parity failure.

##### Barrier #2: odd parity

if (!A429_CheckOddParity(word)) { rx_bad_parity++; USER_LED_ON(); Ack(BAD_PARITY); }

- Rejects parity-tampered words.

##### Barrier #3: plausibility

uint8_t label = A429_Label(word);
if (!Plausibility_Accept(label)) { rx_bad_plaus++; USER_LED_ON(); Ack(BAD_PLAUS); }

- Rejects unknown labels or too-fast updates.

##### Accept path

rx_ok++; last_good_word = word; last_good_valid = true; USER_LED_OFF(); Ack(OK);

- Implements “last-known-good” concept:

  - only updates on valid word

  - LED OFF indicates healthy acceptance

#### 4) TX path (exercise API failures)

if (!g_dataUart.txOnGoing) { if (AckQ_Peek(&m)) { st = DataUart_SendNonBlocking_FI(...); ... } }

- Only sends next ACK when UART isn’t busy.

if (st == kStatus_LPUART_TxBusy) { tx_retries++; g_dataUart.txOnGoing=false; }

- When FI injects TxBusy, we do not deadlock.

- We count a retry and allow loop to try again later.

else if (st != kStatus_Success) { tx_failures++; AckQ_Pop(); }

- Unexpected driver errors are counted and the ACK is dropped.

#### 5) Stall monitor + telemetry

tx_stall_monitor_10ms();
telemetry_print_1s();

- Ensures we recover if TX_STALL is active.

- Prints ongoing evidence.

## How each exercise requirement maps to code (quick index)

- Determinism

- FI_Init(seed) with fixed seed

- FI_ShouldFire() increments hit_count and uses EVERY_N/WINDOW

- Data corruption fault injection

- In main loop: FI_SITE_RX_CORRUPT applied to received byte before parser

- API failure injection

- DataUart_SendNonBlocking_FI() returns kStatus_LPUART_TxBusy when FI_SITE_TX_API_FAIL fires

- TX stall injection + recovery

- Callback refuses to clear txOnGoing when txStallActive

- tx_stall_monitor_10ms() aborts transfer after 100ms of no progress

- Barriers

- A429_ParserFeed() checksum barrier

- A429_CheckOddParity() parity barrier

- Plausibility_Accept() label/rate barrier

- Observability

- LED toggle on injection; solid ON on detected errors

- Per-second telemetry counters

- CLI for arming and dumping counters

## If you want an even more literal “one line per explanation” mode

Tell me which exact code file you want to focus on first (main.c, fi.c, data_uart.c, a429_frame.c). I can rewrite that file into an annotated version where every single line is preceded by an explanation comment.