## Hands-On Exercise: Deterministic Fault-Injection Campaign on UART-Bridged ARINC-429 Words (Data Corruption + API Failures)

### 1) Exercise intent (what you are proving)

You will build (and then run) a self-contained Software Fault Injection harness inside bare-metal EVKB firmware to validate FDIR-style behaviors:

- Data corruption: bit flips/malformed frames on the UART receive path and inside the accepted ARINC-429 word fields.

- API failures: synthetic driver failures (busy, fail codes, lost completion/interrupt behavior) injected at the UART driver boundary to force retry/timeout/recovery paths.

You must show that faults do not escape as failures, i.e., the system detects the corruption/failure, isolates it, and stays in a defined safe behavior. (The module explicitly frames this as fault → error → failure and using SFI to prevent externally observable failures.)

### 2) Hardware + software setup constraints

Target hardware

- NXP EVKB-IMXRT1050.

External avionics link used by this exercise

- EVKB LPUART ↔ ADK-8582 over UART (ADK-8582 represents the ARINC-429 side and emits words toward EVKB).

UART constraint

- You must run at 115200-8-N-1 (or adapter-compatible equivalent), and throttle traffic to stay well within UART capacity. The module assumes typical 115200-8-N-1 for this bridge.

  - Use the module’s example framing size (7 bytes total per word frame; see below). At 115200 bps with 10 bits/byte, the theoretical max is about:

    - bytes/s = 115200/10 = 11520 bytes/s

    - frames/s = 11520 / 7 ≈ 1645 frames/s

  - For realism and easier observation, set your generator to something like 50–200 frames/s and implement a simple rate limit in firmware (required below).

Baseline code starting point

- Start from the NXP SDK example:
boards/evkbimxrt1050/driver_examples/lpuart/interrupt/

### 3) UART word framing (your receive contract)

Implement a UART frame parser that accepts this framing (as specified in the module):

[0xA5][LEN=4][W0][W1][W2][W3][CHK]

- W0..W3 are the 4 bytes of the 32-bit ARINC word (endianness is your choice, but must be consistent).

- CHK is an 8-bit sum of all payload bytes (interpretation: sum over LEN and W0..W3 unless you explicitly document otherwise; your implementation must be self-consistent and tested).

Receive validation pipeline must perform checks in this order (three barriers):

- UART frame checksum CHK

- ARINC odd parity check (bit-level parity on the word)

- Label/value plausibility (allow-list + simple rate/monotonicity constraints)

### 4) The SFI harness requirements (what you must implement)

You will integrate a small library (conceptually fi.h/fi.c) into the SDK example, with these properties:

#### 4.1 Compile-time isolation

- A compile-time flag (e.g., SFI_ENABLED) must allow you to build:

  - SFI build (faults enabled + console enabled)

  - Flight/clean build (SFI removed/disabled)
The module explicitly requires isolating SFI from production builds and showing a startup banner indicating SFI builds.

#### 4.2 Determinism + repeatability

- Every fault site must be deterministic and reproducible:

  - fixed seed(s)

  - per-site counters (“hits”)

  - logged events sufficient to reproduce the campaign exactly

#### 4.3 Fault Sites you must support (minimum set)

Create at least these named “Fault Sites” and wire them into the firmware:

- RX_CORRUPT

  - Inject bit flips on incoming UART bytes before they enter your RX ring buffer (i.e., in the receive interrupt path). The module shows this exact intent: corrupt on trigger, log, toggle LED.

- A429_LABEL_TAMPER

  - After a full UART frame is parsed into a 32-bit word (but before application acceptance), corrupt the Label field by inverting a bit of the label.

- A429_PARITY_TAMPER

  - After parse, corrupt the parity bit to force the ARINC parity check to fail.

- TX_API_FAIL

  - Wrap UART transmit API usage so that on trigger it returns a realistic UART driver error such as TxBusy, and logs the event (size, code, etc.). The module explicitly targets SDK driver status_t return handling and specifically calls out kStatus_LPUART_TxBusy.

#### 4.4 Trigger policies you must implement

Each fault site must support at least these trigger policies (as implied by the module’s “every N”, “window” concepts):

- EVERY_N (fire every Nth hit)

- WINDOW [K1, K2) (fire only when hit count is in a specific window)

Default arming (required for your demo run):

- RX_CORRUPT: every 200 bytes, flip 3 bits (or equivalent “burst” corruption).

- TX_API_FAIL: every 100th transmit request returns TxBusy.

- TX empty interrupt “lost event” simulation: enable a WINDOW event where the TX path stalls (e.g., suppress TX empty behavior) between hits 500 and 520; recovery must detect and re-prime.

### 5) Observability requirements (how you prove what happened)

You must implement all of the following observability mechanisms:

- Event log ring buffer

  - Each injection produces an event record containing at least: site name, hit counter, and a small “peek” into the affected data.

- Minimal runtime console

  - On the EVKB debug UART, implement at least these commands:

    - fi dump → dumps recent injection events

    - fi arm <site> every <N> [bits=<M>] → re-arm sites without rebuilding (at minimum for RX_CORRUPT)

- LED behavior

  - Toggle the user LED on each injection event.

  - Set LED to “solid fault” (or equivalent) when corruption/API error is detected by your validation logic.

- Counters

  - Maintain and periodically print:

    - rx_ok (valid frames accepted)

    - rx_bad_chk (failed CHK)

    - rx_bad_parity (failed parity)

    - rx_bad_plaus (failed allow-list/rate checks)

    - tx_retries, tx_failures, tx_recoveries (for API failure behavior)

### 6) Application behavior requirements (what “safe” means here)

Your application must enforce the following safety-style behaviors under injected faults:

#### 6.1 Acceptance rules

- A word is only accepted if it passes:

  - CHK, 2) odd parity, 3) plausibility.

#### 6.2 Plausibility barrier (must include these two checks)

- Allow-list: only accept a configured set of Labels/SDIs.

- Rate limit: per label (or per label+SDI), reject updates that exceed a defined maximum frequency (e.g., 50 Hz unless you intentionally configure higher).

#### 6.3 Safe output behavior

- On any failure in the pipeline (CHK/parity/plausibility/API failure beyond retry deadline):

  - Do not propagate the bad value to the rest of the application.

  - Hold the last-known-good value and mark it as failed/stale in an application status field (e.g., “SSM = Failure” concept). The module states the expected observation: checks catch injected errors and the app maintains safe outputs (freeze last-known-good with failure status).

### 7) API failure recovery requirements (TX path)

Your transmit path must be non-blocking and include a recovery monitor:

- Retry behavior: On TxBusy, back off and retry, but with a firm deadline (no infinite loops).

- Stall detection via PIT: Use a periodic timer tick (e.g., 10 ms) and a progress counter so you can detect “no forward progress” for >100 ms and then recover by canceling and re-priming the send.

- Lost interrupt simulation: You must demonstrate recovery during the defined WINDOW where TX empty behavior is suppressed (hits 500–520).

### 8) End-to-end fault campaign you must run (test procedure)

You will execute the following campaign and collect evidence:

- Clean baseline (SFI disabled)

  - Build with SFI_ENABLED=0.

  - Show that rx_bad_* stays near zero (aside from real wiring noise) and all nominal words pass to the application.

- Default SFI campaign (SFI enabled)

  - Enable SFI and arm the default policies described above (RX_CORRUPT every 200 bytes, TX_API_FAIL every 100 transmits, plus the TX stall window).

  - Run continuously for ≥ 10 minutes without deadlock/reset while faults are being injected. The module’s lab success criteria explicitly expects long-run responsiveness and no watchdog reset in the RX corruption lab.

- Runtime re-arming

  - Use the console to change RX corruption cadence (e.g., every 50 bytes, bits=2) and show the system remains stable and the counters/events reflect the new arming.

- Barrier demonstration (negative test)

  - Temporarily disable one of the barriers (e.g., parity check or checksum check) and re-run a short campaign to show that corrupted data can escape and cause visible misbehavior—then re-enable it and show the misbehavior disappears. (This is explicitly called out as a required learning outcome in the module.)

### 9) Acceptance criteria (pass/fail)

Your submission is considered “PASS” if all are true:

- Detection works at the correct barrier

  - RX_CORRUPT predominantly triggers CHK failures (or whichever layer you corrupt), and those frames do not reach the app.

  - A429_LABEL_TAMPER is rejected by allow-list/plausibility.

  - A429_PARITY_TAMPER is rejected by parity.

- Recovery works

  - TX_API_FAIL produces retries with bounded latency and no deadlock.

  - The PIT stall monitor detects and recovers from the “lost interrupt” window.

- System safety behavior

  - No invalid word updates the application state.

  - Output stays at last-known-good with a “failure/stale” indicator when needed.

- Evidence quality

  - fi dump output shows deterministic event cadence (counters and site names).

  - Logs are sufficient to reproduce the run.

### 10) What you must submit (deliverables)

Provide the following artifacts (exactly aligned with the module’s deliverables section):

- Code diffs against the SDK example showing:

  - SFI harness integration

  - driver wrappers / injection points

  - console + LED + counters

- Event logs from FI_Dump() captured during the campaign.

- Pass/fail matrix

  - rows = fault sites (RX_CORRUPT, LABEL_TAMPER, PARITY_TAMPER, TX_API_FAIL, TX stall window)

  - columns = detection barrier, recovery action, expected counters, observed counters

- Short narrative (1–2 pages)

  - Explain what broke when you disabled one barrier, and how restoring it prevented the failure.

### 11) MCUXpresso IDE workflow requirements (no solutions, just process)

For this exercise, your steps in MCUXpresso IDE must be:

- Install/import the EVKB-IMXRT1050 SDK and import the example project located at
boards/evkbimxrt1050/driver_examples/lpuart/interrupt/.

- Add new source files for your SFI harness and ARINC/UART framing parser to the project (as separate compilation units).

- Build and flash to the EVKB.

- Open two terminals if possible:

  - terminal A: debug UART console (for fi dump / fi arm)

  - terminal B (optional): independent observation stream (or use GPIO/LED) to avoid confusing “test output” with “system output”.