# CPU Load Profiling & Bottleneck Analysis

Board & tools assumed:

- NXP i.MX RT1050 EVKB (Cortex‑M7 @ up to 600 MHz), MCUXpresso SDK (EVKB‑IMXRT1050), console over LPUART1 (PRINTF), no RTOS (bare‑metal).

- For ARINC 429 topics we interface the EVKB to ADK‑8582 (ARINC 429 adapter kit) via UART (Universal Asynchronous Receiver/Transmitter).

- This module goes from fundamentals to advanced techniques.

- Every section includes:

  - plain‑language explanation,

  - a generic example,

  - a realistic avionics use case, and

  - hands‑on exercises based on the EVKB MCUXpresso SDK you uploaded (/boards/evkbimxrt1050/...).

## 1) Why CPU load profiling matters

- CPU load is the fraction of available core time consumed by useful work (and overhead) over an interval.

- On a real‑time avionics controller, meeting deadlines is more important than raw throughput.

- Load tells you how much margin remains before deadlines are missed.

- Instantaneous load: utilization at a specific moment (often noisy).

- Averaged load: utilization integrated over a window (e.g., 10 ms, 1 s).

- Headroom:

  - 100% – average load (safety margin).

  - In avionics, headroom is sized to worst‑case execution time (WCET) and DAL (Design Assurance Level) objectives under DO‑178C (RTCA Document 178C – Software Considerations in Airborne Systems and Equipment Certification).

Bottleneck analysis

- locates the code path or hardware resource that limits performance (e.g., Flash XIP latency,

- Cache misses, unbounded ISR work, UART burst handling, memory copies).

- You always measure first, then narrow the hot spot, then iterate: measure → change one thing → measure again.

## 2) i.MX RT1050 features that help profiling

- Cortex‑M7 DWT (Data Watchpoint and Trace): CYCCNT cycle counter + event counters (CPICNT, EXCCNT, SLEEPCNT, LSUCNT, FOLDCNT).

  - On the i.MX RT1050 (Cortex-M7), these counters live in the DWT (Data Watchpoint and Trace) block. DWT provides:

    - one cycle counter (CYCCNT), and several 8-bit “event” counters (CPICNT, EXCCNT, SLEEPCNT, LSUCNT, FOLDCNT). (PJRC)

    - They’re mainly used for cycle-accurate profiling and for diagnosing why cycles were burned (memory stalls, interrupt overhead, sleep time, etc.).

- CYCCNT — core cycle counter (the “stopwatch”)

  - What it counts

  - A free-running counter that increments by 1 every core clock cycle (when enabled).

  - It does not count while the core is halted in debug.

  - It wraps on overflow (32-bit on most Cortex-M implementations).

What it’s good for

Benchmarking a function: cycles = end - start

Measuring jitter / worst-case timing of an ISR

Building a fast “microsecond-ish” delay (careful: depends on core frequency and caches)

The 8-bit event counters (why your cycles increased)

- These are “event” counters, but several of them actually count cycles spent in a condition.

- Each uses only the low 8 bits and overflows every 256 counts (optionally generating a DWT event on overflow). (PJRC)

- CPICNT — CPI “extra cycles” + fetch stalls

What it counts

Total number of instruction cycles beyond the first cycle, for most instructions, plus all instruction fetch stalls. (Keil)

It does not include the extra cycles for load/store operations that are accounted in LSUCNT. (Keil)

Interpretation

High CPICNT often points to instruction-side stalls: flash wait states, I-cache miss, ITCM vs AXI fetch differences, branch refill effects, etc.

- EXCCNT — interrupt/exception overhead cycles

What it counts

Total cycles spent in interrupt processing overhead: entry stacking, return unstacking, pre-emption, etc. (Keil)

Interpretation

Useful to quantify “how expensive are my interrupts” independent of the actual ISR body.

- SLEEPCNT — cycles while sleeping

What it counts

Number of cycles during which the processor is sleeping. (Keil)

Note: it’s clocked by FCLK, and that clock might be reduced during sleep, so converting counts → time depends on the sleep clock frequency. (Keil)

Interpretation

Great for “CPU load” style measurements (sleep vs awake), but only if you understand what clock is running during sleep.

- LSUCNT — extra cycles for Load/Store Unit operations

What it counts

Cycles where the processor is processing an LSU operation beyond the first cycle. (Keil)

Example from the TRM:

an LDR that takes 2 cycles increments LSUCNT by 1

an LDR that stalls and takes 4 cycles increments LSUCNT by 3 (Keil)

Interpretation

High LSUCNT usually means data-side penalties: D-cache miss, slow peripheral/bus, unaligned access penalties, contention on AXI/AHB, etc.

- FOLDCNT — folded instructions (0-cycle instructions)

What it counts

Total number of folded instructions (instructions that take 0 cycles). (Keil)

Interpretation

A “good news” counter: more folding can indicate the pipeline is keeping up and some instructions are effectively free.

How they fit together

Think of CYCCNT as your total budget, then the event counters help explain where time went:

EXCCNT: cycles burned on exception mechanics (stack/unstack/preempt), not your code’s logic.

SLEEPCNT: cycles not executing because you were asleep.

LSUCNT: extra cycles because loads/stores took longer than 1 cycle.

CPICNT: extra cycles for everything else beyond 1 cycle + instruction fetch stalls.

FOLDCNT: number of “free” folded instructions (often correlates with efficiency, but it’s not directly “saved cycles” in a simple linear way).

Minimal enable/use snippet (C, CMSIS style)

#include "core_cm7.h"

static inline void dwt_enable_counters(void)

{

// Enable trace (required for DWT)

CoreDebug->DEMCR |= CoreDebug_DEMCR_TRCENA_Msk;

// Reset counters

DWT->CYCCNT   = 0;

DWT->CPICNT   = 0;

DWT->EXCCNT   = 0;

DWT->SLEEPCNT = 0;

DWT->LSUCNT   = 0;

DWT->FOLDCNT  = 0;

// Enable the cycle counter

DWT->CTRL |= DWT_CTRL_CYCCNTENA_Msk;

// Enable event counters you care about (bit names exist in CMSIS)

DWT->CTRL |= DWT_CTRL_CPIEVTENA_Msk

|  DWT_CTRL_EXCEVTENA_Msk

|  DWT_CTRL_SLEEPEVTENA_Msk

|  DWT_CTRL_LSUEVTENA_Msk

|  DWT_CTRL_FOLDEVTENA_Msk;

}

static inline uint32_t dwt_cycles_now(void) { return DWT->CYCCNT; }

Practical tip: because CPICNT/EXCCNT/SLEEPCNT/LSUCNT/FOLDCNT are 8-bit, read them frequently (or handle overflow events) if you’re measuring longer windows.

- ITM (Instrumentation Trace Macrocell) / SWO: high‑speed event logging (optional; UART PRINTF is simpler but intrusive).

- SysTick: 24‑bit down counter; convenient 1 kHz timebase.

- PIT/GPT (Periodic/GPIO Timers): long‑period, low‑overhead timekeeping or input capture.

- GPIO: scope/LA‑visible strobes to time code or ISR sections.

- Memory hierarchy: ITCM/DTCM (tightly‑coupled, deterministic), OCRAM, and FlexSPI XIP from external QSPI Flash with cache and prefetch. Many bottlenecks are actually XIP wait states or cache misses.

## 3) Load and time measurement models

- Idle‑time method (system‑wide load)
Let idle_ticks be work done only when the system is idle. Sample it periodically with SysTick or PIT.
[ = 100 (1 - )]
The baseline is the increment when the CPU does nothing but idle.

- Cycle counting (section‑level)
Enclose code with DWT CYCCNT reads: cycles → time using CLOCK_GetFreq(kCLOCK_CpuClk).

- GPIO timing (ISR/latency)
Raise a pin on entry and lower on exit; measure pulse width on logic analyzer. Zero software overhead for timing, but needs equipment.

- Event counters
EXCCNT shows exception/ISR activity, SLEEPCNT shows time in WFI (Wait‑For‑Interrupt), LSUCNT hints at load/store stalls (e.g., unaligned, cache).

## 4) Generic walk‑through: from “how busy” to “what’s slow”

- Implement an idle loop meter with SysTick (1 kHz).

- Use DWT to bracket suspected hot functions (serialization, parsing, filtering).

- If time spikes occur in interrupts, add a GPIO strobe or CYCCNT inside the ISR.

- If section time is stable but slow, check where code executes (XIP vs ITCM), data placement (DTCM vs OCRAM), and cache effects.

- Move only the hot path (e.g., a parser) to ITCM/DTCM; verify improvement.

- For bursty peripherals (UART from ARINC 429 adapter), convert blocking processing to interrupt + ring buffer and defer heavy work to the main loop; confirm reduced ISR time and smoother load.

## 5) Avionics scenario: ARINC 429 message burst

- ARINC 429 (Aeronautical Radio, Incorporated 429) provides 32‑bit words at 100 kbps (high‑speed) or 12.5 kbps (low‑speed) on a unidirectional, self‑clocking bus.

- Many aircraft subsystems (e.g., ADC – Air Data Computer, AHRS – Attitude and Heading Reference System) broadcast labels that your controller must parse, scale, sanity‑check, and publish over other links.

Setup:

- The ADK‑8582 converts an ARINC 429 channel to a byte stream over a UART. During mode changes (e.g., sensor self‑test), the adapter can burst several hundred words quickly.

- If the EVKB parses each byte inside the UART ISR, the ISR monopolizes the core, starving other deadlines (e.g., a control loop at 1 kHz).

- The symptom is sporadic deadline misses; the cause is an ISR bottleneck.

Fix path:

- Measure ISR time with DWT/GPIO, prove it’s too long, then switch to: quick ISR → push to ring buffer → parse in main loop slices.

- Move the hot parser to ITCM and verify the load drop. Keep the UART at a baud rate with headroom (e.g., 460 800 Bd) and set a FIFO watermark to reduce IRQ rate.

## 6) Hands‑on 1 — System‑wide CPU load via SysTick + idle loop (bare‑metal)

Goal:

- Compute average CPU load every 100 ms using the idle‑time method.

- Uses the EVKB SDK: board.h, clock_config.h, fsl_debug_console.h.

Where in SDK: Start from boards/evkbimxrt1050/project_template/. Add the source below to source/cpu_load.c (or your main file) and enable the debug console.

#include "fsl_common.h"
#include "fsl_debug_console.h"
#include "board.h"
#include "clock_config.h"
#include "pin_mux.h"

#ifndef SYSTICK_HZ
#define SYSTICK_HZ (1000U)
#endif

static volatile uint32_t idle_counter = 0;          // increments only in idle
static volatile uint32_t idle_baseline = 1;         // measured at startup
static volatile uint32_t ms_accum = 0;              // elapsed ms in current window
static volatile uint32_t last_idle_sample = 0;      // last idle snapshot

static inline void idle_touch(void) { idle_counter++; __NOP(); }

static void measure_idle_baseline(uint32_t ms)
{
    uint32_t start = idle_counter;
    uint32_t target = ms;
    ms_accum = 0;
    while (ms_accum < target) { idle_touch(); }
    idle_baseline = idle_counter - start; // idle increments for 'ms' when fully idle
    if (idle_baseline == 0) idle_baseline = 1;
}

void SysTick_Handler(void)
{
    ms_accum++;
}

static void setup_systick(uint32_t cpu_hz)
{
    SysTick->LOAD  = (cpu_hz / SYSTICK_HZ) - 1U;
    SysTick->VAL   = 0;
    SysTick->CTRL  = SysTick_CTRL_CLKSOURCE_Msk | SysTick_CTRL_TICKINT_Msk | SysTick_CTRL_ENABLE_Msk;
}

int main(void)
{
    BOARD_InitBootPins();
    BOARD_InitBootClocks();
    BOARD_InitDebugConsole();

    const uint32_t cpu_hz = CLOCK_GetFreq(kCLOCK_CpuClk);
    setup_systick(cpu_hz);

    PRINTF("\r\nCPU=%lu Hz\r\n", (unsigned long)cpu_hz);

    /* Establish baseline: how fast idle_counter increments when *only* idling for 100 ms */
    measure_idle_baseline(100U);
    PRINTF("Idle baseline=%lu counts/100ms\r\n", (unsigned long)idle_baseline);

    uint32_t window_ms = 100U;  // report period
    uint32_t last_ms = 0;

    for (;;)
    {
        /* ---- Application work would go here (timers, polling, etc.) ---- */

        /* Idle touch marks spare cycles. Keep it cheap. */
        idle_touch();

        /* Periodic reporting from main loop (non‑ISR). */
        uint32_t now_ms = ms_accum;
        if ((now_ms - last_ms) >= window_ms)
        {
            uint32_t idle_now = idle_counter;
            uint32_t idle_delta = idle_now - last_idle_sample;
            last_idle_sample = idle_now;
            last_ms = now_ms;

            /* Scale idle delta to baseline window */
            uint32_t scaled_idle = (idle_delta * 100U) / (idle_baseline == 0 ? 1U : idle_baseline);
            uint32_t load = (scaled_idle >= 100U) ? 0U : (100U - scaled_idle);
            PRINTF("Load=%u %%  (idle_delta=%lu)\r\n", (unsigned)load, (unsigned long)idle_delta);
        }
    }
}

What to observe –

- With no extra work, Load≈0% (some non‑zero due to interrupts).

- Add a busy loop inside for(;;) (e.g., a CRC or for(volatile int i=0;i<20000;i++);)—load should increase.

- Change window_ms to 10 ms and see noisier readings (shorter window).

Avionics mapping

- Idle budget reflects the headroom for deterministic loops like FCC (Flight Control Computer) health monitoring or BIT (Built‑In Test).

- Document the minimum observed headroom.

## 7) Hands‑on 2 — Fine‑grained timing with DWT CYCCNT

Goal: Time code sections with cycle precision and compute µs. Uses CMSIS registers directly; MCUXpresso SDK already enables clock helpers.

#include "fsl_common.h"
#include "fsl_debug_console.h"
#include "board.h"
#include "clock_config.h"

static inline void dwt_init(void)
{
    /* Enable DWT & CYCCNT */
#if (__CM_CMSIS_VERSION_MAIN >= 6U)
    DCB->DEMCR |= DCB_DEMCR_TRCENA_Msk;
#else
    CoreDebug->DEMCR |= CoreDebug_DEMCR_TRCENA_Msk;
#endif
    DWT->CYCCNT = 0; // reset
    DWT->CTRL |= DWT_CTRL_CYCCNTENA_Msk;
}

static inline uint32_t dwt_cycles(void) { return DWT->CYCCNT; }

static uint32_t dummy_crc32(const uint8_t *buf, size_t len)
{
    uint32_t crc = 0xFFFFFFFFu;
    for (size_t i = 0; i < len; ++i)
    {
        crc ^= buf[i];
        for (int b = 0; b < 8; ++b)
            crc = (crc >> 1) ^ (0xEDB88320u & (-(int)(crc & 1)));
    }
    return ~crc;
}

int main(void)
{
    BOARD_InitBootPins();
    BOARD_InitBootClocks();
    BOARD_InitDebugConsole();
    dwt_init();

    const uint32_t cpu_hz = CLOCK_GetFreq(kCLOCK_CpuClk);

    uint8_t buf[512];
    for (size_t i = 0; i < sizeof buf; ++i) buf[i] = (uint8_t)i;

    uint32_t c0 = dwt_cycles();
    (void)dummy_crc32(buf, sizeof buf);
    uint32_t c1 = dwt_cycles();

    uint32_t cycles = c1 - c0;
    uint32_t us = (uint32_t)((uint64_t)cycles * 1000000ull / cpu_hz);
    PRINTF("CRC32: %lu cycles, ~%lu us\r\n", (unsigned long)cycles, (unsigned long)us);

    for(;;) {}
}

What to observe

- Re‑run with the function placed in ITCM (see Hands‑on 4) and compare cycles.

- Run the same code from XIP Flash (default).

- Expect more cycles due to Flash latency mitigated by cache/prefetch, but still slower than ITCM.

Avionics mapping

- Use this to time label parsing and sanity checks for ARINC 429 words (e.g., label 203 Airspeed (BNR – Binary Number Representation), label 204 Magnetic Heading (BCD – Binary‑Coded Decimal)).

## 8) Hands‑on 3 — ISR profiling: UART burst (proxy for ARINC 429 via ADK‑8582)

Goal: Convert a potentially heavy UART ISR into a short ISR that pushes bytes to a ring buffer and measure ISR time. Replace LPUARTx/pins with the port connected to ADK‑8582.

The EVKB debug console uses LPUART1. For the ARINC adapter, pick another instance (e.g., LPUART3/4) and route pins in pin_mux.c. The SDK driver is devices/MIMXRT1052/drivers/fsl_lpuart.h.

#include "fsl_common.h"
#include "fsl_lpuart.h"
#include "fsl_gpio.h"
#include "board.h"

#define ARINC_UART       LPUART3
#define ARINC_UART_IRQn  LPUART3_IRQn
#define ARINC_UART_BAUD  460800U

#define RBUF_SIZE 1024
static volatile uint8_t rbuf[RBUF_SIZE];
static volatile uint16_t rhead = 0, rtail = 0;

/* Simple GPIO strobe on USER LED pin to scope ISR duration (active‑low LED) */
static inline void isr_strobe_hi(void){ GPIO_PortSet(GPIO1, 1u << 9); }
static inline void isr_strobe_lo(void){ GPIO_PortClear(GPIO1, 1u << 9); }

static inline void rbuf_push(uint8_t b)
{
    uint16_t n = (uint16_t)(rhead + 1u);
    if (n == RBUF_SIZE) n = 0;
    if (n != rtail) { rbuf[rhead] = b; rhead = n; } // drop if full
}

void LPUART3_IRQHandler(void)
{
    isr_strobe_hi();
    uint32_t status = LPUART_GetStatusFlags(ARINC_UART);
    if (status & kLPUART_RxDataRegFullFlag)
    {
        uint8_t b = LPUART_ReadByte(ARINC_UART);
        rbuf_push(b);
    }
    LPUART_ClearStatusFlags(ARINC_UART, kLPUART_RxOverrunFlag);
    SDK_ISR_EXIT_BARRIER; // MCUXpresso SDK macro
    isr_strobe_lo();
}

static void uart_init(void)
{
    lpuart_config_t cfg;
    LPUART_GetDefaultConfig(&cfg);
    cfg.baudRate_Bps = ARINC_UART_BAUD;
    cfg.enableTx = true;
    cfg.enableRx = true;
    LPUART_Init(ARINC_UART, &cfg, CLOCK_GetFreq(kCLOCK_OscClk));
    LPUART_EnableInterrupts(ARINC_UART, kLPUART_RxDataRegFullInterruptEnable | kLPUART_RxOverrunInterruptEnable);
    EnableIRQ(ARINC_UART_IRQn);
}

/* In main(): call BOARD_InitBootPins/Clocks/DebugConsole; mux pins for LPUART3; then uart_init().
   In the main loop, drain rbuf and run the ARINC word parser there.
*/

Measurements
- On a scope/logic analyzer, probe GPIO1_IO09 (USER LED) to see ISR pulse width under steady and burst traffic.
- With DWT, bracket the parser in the main loop and confirm ISR time drops while total throughput remains.

Avionics mapping
At 100 kbps ARINC 429, worst‑case bursts from an AHRS (Attitude & Heading Reference System) can cluster labels; the ring buffer prevents ISR monopolization, making control‑loop jitter acceptable.

## 9) Hands‑on 4 — Moving hot code to ITCM / hot data to DTCM

Goal: Demonstrate that placing hot paths in tightly‑coupled memory reduces cycles and jitter.

- Place function in ITCM
In GCC/MCUXpresso, add a section and attribute:

__attribute__((section(".itcm_func")))
static uint32_t parse_label_203(const uint8_t *w)
{
    /* convert ARINC429 BNR Airspeed word into fixed‑point knots, range checks, etc. */
    /* ... */
}

- Linker file: ensure .itcm_func is mapped to ITCM region. MCUXpresso EVKB linker scripts already define ITCM/DTCM; add a new section mapping.

- Measure with the DWT code from Hands‑on 2 before and after relocation.

Avionics mapping
Pin the label decode + sanity check path in ITCM, keep bulk buffers in OCRAM. Demonstrate reduced worst‑case latency during UART bursts.

## 10) Advanced counters: DWT event insight

You can reset and read additional DWT counters to understand why it’s slow:

static inline void dwt_counters_reset(void)
{
    DWT->CYCCNT = 0; DWT->CPICNT = 0; DWT->EXCCNT = 0; DWT->SLEEPCNT = 0; DWT->LSUCNT = 0; DWT->FOLDCNT = 0;
}

static inline void dwt_counters_read(uint32_t *cyc, uint8_t *cpi, uint8_t *exc, uint8_t *slp, uint8_t *lsu, uint8_t *fld)
{
    *cyc = DWT->CYCCNT; *cpi = DWT->CPICNT; *exc = DWT->EXCCNT; *slp = DWT->SLEEPCNT; *lsu = DWT->LSUCNT; *fld = DWT->FOLDCNT;
}

- High EXCCNT → too many interrupts; increase FIFO watermark or coalesce work.

- High SLEEPCNT with low load → good headroom (lots of WFI).

- High LSUCNT → memory stalls; move to ITCM/DTCM or align data.

## 11) Bottleneck patterns on i.MX RT1050 (Cortex‑M7)

- XIP from QSPI Flash: Even with cache/prefetch, random branches or large hot sets exceed cache → extra wait states. Symptom: sections fast when placed in ITCM but not in Flash.

- Large/long ISRs: Byte‑wise parsing or PRINTF inside ISRs. Fix: buffer in ISR, process in main; never print from ISR.

- Busy‑wait polling of peripherals: Replace with IRQ + DMA where available.

- Unaligned or 8‑bit accesses in tight loops: Use 32‑bit aligned buffers; copy once, process aligned.

- Cache management with DMA: If later using DMA, maintain cache coherence (clean/invalidate); bad coherence appears as random spikes.

- FPU (Floating‑Point Unit) context: Heavy float in ISR causes lazy stacking or long save/restore; prefer fixed‑point in ISR.

## 12) Best practices checklist

- Measure with minimal perturbation: logging over UART changes timing; prefer cycle counts or GPIO strobes for critical sections.

- Stabilize the clock: Know CLOCK_GetFreq(kCLOCK_CpuClk) and keep it constant during runs.

- One change at a time: Keep diffs small and attributable.

- Pin hot code/data: ITCM for code, DTCM for working sets; leave bulk buffers in OCRAM.

- Keep ISRs short: Move parsing/validation to main loop or deferred handlers. Avoid dynamic allocation and PRINTF in ISRs.

- Watch heap/stack: Overruns cause unpredictable slowdowns and faults.

- Document WCET: Record worst‑case cycles and the test that produced them; keep artifacts for audit (aligns with DO‑178C evidence needs).

- Use SDK helpers: SDK_ISR_EXIT_BARRIER avoids re‑entry glitches on fast cores vs peripheral clocks.

## 13) Capstone avionics lab — ARINC 429 pipeline profiling (EVKB + ADK‑8582 over UART)

Objective:

- Prove end‑to‑end timing budget from UART byte arrival to parsed/validated word publish within 1 ms deadline at 100 kbps ARINC 429 bursts.

Steps

  - Wire ADK‑8582 UART to EVKB chosen LPUART (e.g., LPUART3), pins via pin_mux.c (Arduino header or J24 per your schematic).

  - Implement Hands‑on 3 ISR ring buffer; set UART baud (e.g., 460 800 Bd) and RX FIFO watermark to reduce IRQ rate.

  - In main loop, periodically drain the buffer, reassemble 32‑bit ARINC words, verify parity, decode known labels (203/204/364 etc.), and run range/sanity checks.

  - Use DWT to time each phase: (A) ISR, (B) buffer→word assembly, (C) parse+scale, (D) publish.

  - Toggle a GPIO around the whole pipeline for external measurement.

  - Move phases (B) and (C) to ITCM; re‑measure.

  - Increase adapter’s output rate to emulate bursts; confirm margin remains (e.g., <40% average load, no deadlines missed).

Deliverables

- Table of cycles/µs for each phase (min/avg/max over 1000 words).

- Scope captures of ISR and pipeline GPIO pulses under nominal and burst traffic.

- Notes explaining any cache/ITCM effects observed.

## 14) Troubleshooting tips

- Load reads >100% or negative: Baseline recalibration needed; ensure no extra work runs during baseline.

- No DWT counts: Ensure TRCENA set and DWT_CTRL_CYCCNTENA enabled after clocks.

- UART overrun: Raise baud, increase RX FIFO watermark, process faster in main loop, or enlarge ring buffer.

- Jittery times: Disable other noisy sources temporarily; confirm caches/branch predictor enabled (board files do this by default).

## 15) What to keep for certification evidence (DO‑178C)

- Exact test binaries and linker maps showing ITCM/DTCM placement.

- Measured WCET of each safety‑relevant function and the test setup (clock, tools, stimulus).

- Rationale that residual load/headroom meets system‑level timing budgets with assumed fault hypotheses.

## Appendix A — Pin, clock, and macros (from SDK)

- USER LED is GPIO1_IO09 on EVKB. In board.h:

  - USER_LED_INIT(LOGIC_LED_OFF), USER_LED_TOGGLE().

- Debug console defaults to LPUART1 @ 115200 (BOARD_InitDebugConsole()).

- Useful SDK bits: CLOCK_GetFreq(kCLOCK_CpuClk), SDK_ISR_EXIT_BARRIER, LPUART_* API.

All exercises above follow the MCUXpresso SDK style and include headers you have in boards/evkbimxrt1050/... and devices/MIMXRT1052/drivers/.

## Appendix B — Minimal ARINC 429 word framing (main‑loop side)

Sketch only; adapt framing to your ADK‑8582 UART protocol.

static bool try_pop_byte(uint8_t *out)
{
    uint16_t t = rtail; if (t == rhead) return false; *out = rbuf[t]; rtail = (uint16_t)((t + 1u) % RBUF_SIZE); return true;
}

static inline bool arinc_parity_ok(uint32_t w)
{
    /* ARINC 429 odd parity over bits 1..31; bit 32 is parity bit */
    uint32_t data = w & 0x7FFFFFFFu; // 31 bits
    uint32_t p = __builtin_popcount(data);
    uint32_t parity_bit = (w >> 31) & 1u;
    return ((p + parity_bit) & 1u) == 1u; // odd
}

static bool try_get_word(uint32_t *w)
{
    /* Example: adapter sends 4 bytes MSB first per word */
    static uint8_t acc[4]; static int n = 0; uint8_t b;
    while (try_pop_byte(&b))
    {
        acc[n++] = b; if (n == 4) { *w = ((uint32_t)acc[0]<<24)|((uint32_t)acc[1]<<16)|((uint32_t)acc[2]<<8)|acc[3]; n = 0; return true; }
    }
    return false;
}

static void process_words_slice(void)
{
    uint32_t w; while (try_get_word(&w)) { if (!arinc_parity_ok(w)) continue; /* decode labels, scale, publish */ }
}

Run process_words_slice() periodically in the main loop between other duties; measure with DWT.

### Wrap‑up

You now have:

- a system‑wide load meter,

- micro‑timing for any section,

- ISR‑safe UART handling, and

- a repeatable workflow to find and fix bottlenecks—all on the i.MX RT1050 EVKB with SDK‑compliant code.

Apply the same pattern to other peripherals and maintain WCET evidence for your avionics safety case.