# Fault Injection Techniques (Bare‑Metal) for Airbus — i.MX RT1050 EVKB + ARINC 429 via ADK‑8582

## 1) Why Fault Injection?

- In avionics, we must demonstrate that software detects, contains, and recovers from faults with a rigor consistent with DO‑178C (Software Considerations in Airborne Systems and Equipment Certification) and aligned with ARP4761 (Guidelines and Methods for Conducting the Safety Assessment Process on Civil Airborne Systems and Equipment) and ARP4754A (Guidelines for Development of Civil Aircraft and Systems).

- Fault injection is a structured way to provoke misbehaviour—bit flips, timing overruns, bus protocol violations—so we can verify monitors, Built‑In Tests (BIT), graceful degradation, and annunciation paths.

- This module covers software‑implemented fault injection (SWIFI) you can run directly on the EVKB‑IMXRT1050 board and interface‑level injection for ARINC 429 using the ADK‑8582 instrument via UART commands.

## 2) Clear Definitions

- Fault: The cause of an error (e.g., flipped SRAM bit, corrupted pointer, malformed ARINC (Aeronautical Radio, Incorporated) word).

- Error: The system state deviating from correct state (e.g., wrong altitude value stored).

- Failure: External deviation from required service (e.g., incorrect display to pilot).

- FI (Fault Injection): Techniques to deliberately introduce faults to observe error detection, containment, and recovery.

- SWIFI (Software Implemented Fault Injection): Using code to corrupt memory, stall timing, trigger exceptions.

- MPU (Memory Protection Unit): ARM core feature creating memory regions with access permissions to provoke MemManage faults on violations.

- NVIC (Nested Vectored Interrupt Controller): ARM exception controller, routes HardFault, BusFault, UsageFault, MemManage.

- SCB (System Control Block): ARM system registers providing Fault Status registers (CFSR, HFSR, BFAR, MMFAR).

On Cortex-M (including the i.MX RT1050’s Cortex-M7), the SCB (System Control Block) is the core’s “system registers hub” for exceptions, interrupt control, and fault reporting.

The fault-related registers are what you read inside a HardFault/MemManage/BusFault/UsageFault handler to answer two key questions:

What kind of fault happened?

What address (if any) caused it?

These registers are part of the architectural definition from Arm and are memory-mapped (SCB base typically 0xE000ED00 via CMSIS SCB->...).

How faults are organized (big picture)

Cortex-M has several fault classes:

- MemManage fault (MPU / permission / execute-never style issues)

- BusFault (bus errors: instruction/data access problems, stacking/unstacking bus errors)

- UsageFault (illegal instruction usage: undefined instruction, unaligned, divide-by-zero, etc.)

- HardFault (catch-all; also the escalation target when other faults aren’t enabled or when a fault occurs during fault handling)

- Three of those (MemManage/BusFault/UsageFault) are “configurable faults”. Their status bits live in CFSR.

- HardFault summary lives in HFSR.

SCB fault status registers

1) CFSR — Configurable Fault Status Register

What it is: A 32-bit register that packs three sub-status registers:

MMFSR (MemManage Fault Status): bits [7:0]

BFSR (BusFault Status): bits [15:8]

UFSR (UsageFault Status): bits [31:16]

What it tells you: exactly which condition triggered the fault.

Common, high-value bits (names are CMSIS-style; exact bit positions matter in code):

MemManage (MMFSR, low 8 bits)

IACCVIOL — instruction fetch violation (execute permission / MPU)

DACCVIOL — data access violation (MPU / permission)

MSTKERR / MUNSTKERR — stacking / unstacking fault on exception entry/return

MMARVALID — MMFAR contains a valid fault address

BusFault (BFSR, next 8 bits)

IBUSERR — instruction bus error (prefetch/fetch)

PRECISERR — precise data bus error (BFAR typically valid)

IMPRECISERR — imprecise data bus error (harder to attribute)

STKERR / UNSTKERR — stacking/unstacking bus error

BFARVALID — BFAR contains a valid fault address

UsageFault (UFSR, upper 16 bits)

UNDEFINSTR — undefined instruction

INVSTATE — invalid EPSR/IT state

INVPC — invalid PC on exception return

NOCP — no coprocessor (e.g., FPU instruction when FPU disabled)

UNALIGNED — unaligned access trap (if trapping enabled)

DIVBYZERO — divide by zero trap (if enabled)

Clearing behavior: Most CFSR bits are write-1-to-clear (W1C). (In practice: read CFSR, log it, then write the same value back to clear the set bits.)

Super important behavior: Even if MemManage/BusFault/UsageFault exceptions are not enabled, the condition may still set bits in CFSR and then escalate to HardFault. So CFSR is often still informative even when you only see HardFault.

2) HFSR — HardFault Status Register

What it is: a 32-bit register with summary flags for HardFault causes.

The two flags you care about most:

FORCED — a “configurable fault” (MemManage/BusFault/UsageFault) happened but escalated into HardFault (commonly because that fault handler wasn’t enabled, or fault occurred during exception processing).

VECTTBL — a fault occurred during vector table read (e.g., bad vector table address, bus error fetching handler address)

How to use it:

If HFSR.FORCED = 1, then CFSR almost always contains the real “root cause” bits you should decode next.

3) BFAR — BusFault Address Register

What it is: holds the faulting address for certain BusFaults.

When it’s valid: only trust BFAR if CFSR.BFSR.BFARVALID = 1.
In many “nice” cases:

PRECISERR + BFARVALID → BFAR is the exact address of the failing load/store.

When it may be useless:

IMPRECISERR often means the pipeline/buffer delayed the fault; BFAR might be unrelated or invalid.

4) MMFAR — MemManage Fault Address Register

What it is: holds the address that caused a MemManage fault (MPU/permission/execute faults).

When it’s valid: only trust MMFAR if CFSR.MMFSR.MMARVALID = 1.

Typical use: MPU violations where you want the exact address that was accessed/fetched.

Practical decoding flow in a fault handler

Read and log:

SCB->HFSR

SCB->CFSR

SCB->BFAR (only if BFARVALID)

SCB->MMFAR (only if MMARVALID)

If HFSR.FORCED, decode CFSR for the real reason.

Use BFAR/MMFAR for the address if valid.

Optionally clear fault flags (W1C) before reset.

Minimal C snippet to capture these in RT1050 firmware

#include "core_cm7.h"

#include <stdint.h>

typedef struct {

uint32_t hfsr;

uint32_t cfsr;

uint32_t bfar;

uint32_t mmfar;

} fault_info_t;

volatile fault_info_t g_fault;

static inline void capture_fault_info(void)

{

g_fault.hfsr  = SCB->HFSR;

g_fault.cfsr  = SCB->CFSR;

// Only meaningful if valid bits are set

g_fault.bfar  = SCB->BFAR;   // check (g_fault.cfsr & SCB_CFSR_BFARVALID_Msk)

g_fault.mmfar = SCB->MMFAR;  // check (g_fault.cfsr & SCB_CFSR_MMARVALID_Msk)

}

If you want these faults to not automatically become HardFault, enable the configurable fault handlers:

SCB->SHCSR |= SCB_SHCSR_MEMFAULTENA_Msk

|  SCB_SHCSR_BUSFAULTENA_Msk

|  SCB_SHCSR_USGFAULTENA_Msk;

Quick intuition: what each register answers

HFSR: “Was this a forced escalation into HardFault?” / “Vector table fetch problem?”

CFSR: “Exactly what fault condition happened?”

BFAR: “What address caused the bus access fault?” (only if BFARVALID)

MMFAR: “What address caused the MPU/memory-management fault?” (only if MMARVALID)

- WDOG (Window Watchdog) and RTWDOG (Real‑Time Watchdog): i.MX RT watchdog peripherals that reset or interrupt on missed service.

- CMSIS (Cortex Microcontroller Software Interface Standard): HAL/headers used by the MCUXpresso SDK.

- SSM (Sign/Status Matrix): ARINC 429 bits [31:30] encoding status (Normal, No Computed Data, Functional Test, Failure).

- SDI (Source/Destination Identifier): ARINC 429 bits [10:9] identifying source or destination channel.

## 3) Platform map: EVKB‑IMXRT1050 (ARM Cortex‑M7)

- Core: ARM Cortex‑M7 with HardFault, BusFault, MemManage, UsageFault, NMI (Non‑Maskable Interrupt), Systick.

- On‑chip memory: OCRAM (On‑Chip RAM). Execute‑in‑place from external QSPI via FlexSPI.

- Relevant peripherals in SDK: LPUART (for ADK‑8582), WDOG/RTWDOG, EDMA (Enhanced DMA), TRNG (True Random Number Generator), GPIO, FLEXPWM (for the SDK’s PWM fault demo).

SDK references (paths in your uploaded package): - boards/evkbimxrt1050/driver_examples/lpuart/interrupt_transfer/ - boards/evkbimxrt1050/driver_examples/wdog/ and .../rtwdog/ - boards/evkbimxrt1050/driver_examples/edma/ - boards/evkbimxrt1050/driver_examples/trng/random/ - boards/evkbimxrt1050/demo_apps/pwm_fault/ (peripheral fault input demo)

## 4) Taxonomy of Fault Injection Covered Here

- Memory/Data corruption
Flip bits in SRAM buffers; over‑/under‑write; CRC mismatch; stale data reuse.

- Protection violations & exceptions
Use MPU to mark a region as read‑only then attempt a write → MemManage; dereference invalid addresses → BusFault; divide by zero → UsageFault; escalate to HardFault.

- Timing/scheduling faults
Inject jitter/delay; starve service routines; miss WDOG window to force system reset; stress NVIC priority inversions.

- Interface‑level faults (ARINC 429)
Bad parity (ARINC 429 is odd parity), illegal SSM, inconsistent SDI, wrong word rates/bursts (at instrument level), label mismatches.

- DMA/Peripheral faults
Mis‑configure EDMA to write unexpected sizes; starve UART RX to simulate dropped words.

- Security‑motivated fault tests
Brief note on FIH (Fault‑Injection Hardening) patterns (e.g., double‑checked states as seen in MCUboot), useful to guard safety monitors.

## 5) A generic worked example (end‑to‑end)

Scenario:

- A simple “sensor pipeline” reads a 32‑byte buffer, checks a CRC (Cyclic Redundancy Check), and publishes a value.

- We inject a single‑bit flip and verify the CRC monitor catches it, then make sure the system does not crash (graceful handling + annunciation).

- Normal path: Buffer → CRC32 → OK → publish.

- Injected fault: Flip bit k in buffer using SWIFI utility → CRC32 fails → publish is inhibited; a BIT (Built‑In Test) counter is incremented; an ECAM (Electronic Centralized Aircraft Monitor)/maintenance log line is printed via PRINTF.

- Recovery: After 3 consecutive CRC failures, WDOG reset is triggered deliberately to emulate a “go‑safe” policy.

## 6) A strong avionics use case

Use case:

- IRU (Inertial Reference Unit) air data fusion sanity monitoring on a Display Computer.

- The Display Computer (DC) receives ARINC 429 words for pressure altitude and indicated airspeed from an ADC (Air Data Computer).

- The DC independently computes a consistency check using an internal inertial solution.

When we inject:

- Wrong parity on altitude words → DC shall reject the word and maintain last‑known‑good for that label; increment a word‑level error counter; flag SSM = Failure when generating outputs to downstream busses.

- SSM = “Failure” while the numeric field looks valid → DC shall trust SSM and discard the numeric value; annunciate ADC A FAIL.

- Label mismatch (airspeed word sent under an altitude label) → DC’s label filter shall reject it and record label discipline violation.

- Timing burst (no inter‑word gap) → DC’s input driver shall detect overrun and report ARINC input saturation.

We will implement these tests using the ADK‑8582 (driving ARINC 429) with commands issued over LPUART.

The i.MX RT1050 executes monitors that count, log, and react.

Where the ADK‑8582 command strings differ, adapt the adk429_send_*() wrappers accordingly.

## 7) i.MX RT1050 exception handling primer (Cortex‑M7)

- MemManage Fault: MPU access violation (bad permissions or execute‑never).

- BusFault: Precise/imprecise AXI/AHB bus error (e.g., invalid address).

- UsageFault: Undefined instruction, divide by zero (if enabled), unaligned access (if trapped).

- HardFault: Escalation or system error not handled by the above.

- SCB registers: SCB->CFSR (Configurable Fault Status), SCB->HFSR (HardFault Status), SCB->BFAR, SCB->MMFAR.

- Recommendation: Always implement verbose handlers that log then reset (or halt in lab mode). See §10 code.

## 8) Lab topology and toolchain

- Board: EVKB‑IMXRT1050 via USB debug (CMSIS‑DAP/J‑Link) and UART (virtual or external).

- Instrument: ADK‑8582 ARINC 429 generator/analyzer connected to DUT ARINC 429 RX; command/control via LPUART from the EVKB.

- SDK: Use the uploaded SDK. Start from boards/evkbimxrt1050/driver_examples/lpuart/interrupt_transfer/ for UART and .../wdog/ for watchdog patterns.

- Build: MCUXpresso IDE or equivalent; include board.c, clock_config.c, pin_mux.c, and enable PRINTF through fsl_debug_console.h.

Safety note:

- Perform all ARINC 429 injections on a bench harness, never on aircraft wiring. Use opto‑isolated transceivers where applicable.

- Ensure the watchdog’s final reset does not drive unintended IO states.

## 9) Step‑by‑step labs (with solutions)

### Lab 1 — Memory bit‑flip and CRC detection (SWIFI)

Goal:

- Demonstrate data integrity monitors catch single‑bit errors.

Steps:

- Create a new project by copying boards/evkbimxrt1050/driver_examples/trng/random/ as a template (it already initializes board/clock/pins).

- Add fi_crc32.c/.h and fi_mutate.c/.h from §10 to your project.
Call FI_FlipRandomBit() on a 32‑byte buffer seeded by TRNG (True Random Number Generator); compute CRC using FI_CRC32().

- If CRC mismatches, log via PRINTF, increment a BIT counter, and deliberately do not publish the buffer.

Success criteria:

- CRC failures are detected deterministically; no exceptions or crashes; counters/logs increment.

### Lab 2 — MPU‑guarded region and MemManage fault

Goal: Use the MPU (Memory Protection Unit) to catch stray writes.

Steps:

- Add fi_mpu.c/.h from §10, which uses CMSIS (core_cm7.h and m-profile/armv7m_mpu.h).

- Call FI_MPU_EnableReadOnlyRegion(buf, 32U); then attempt a write to buf[0].

- Observe MemManage fault; see the SCB logs printed by HardFault_Handler()/MemManage_Handler().

Success criteria:

- Violation triggers handler; status registers printed; system resets cleanly after log.

### Lab 3 — UsageFault via divide‑by‑zero and recovery

Goal:

- Show UsageFault handling and graceful recovery path.

Steps:

- Enable divide‑by‑zero trapping: SCB->CCR |= SCB_CCR_DIV_0_TRP_Msk;.

- Deliberately divide by zero in a lab function guarded by #if LAB_MODE.

- Handler logs the event and returns to a safe main loop.

Success criteria:

- No board lock‑up; fault count increments; watchdog remains serviced.

### Lab 4 — Missed WDOG service → autonomous reset

Goal:

- Show that the WDOG policy resets the system on missed refresh.

Steps:

- Start from boards/evkbimxrt1050/driver_examples/wdog/.

- Configure an interrupt time (config.interruptTimeValue) and timeout short enough for lab (e.g., ~2 s).

- Intentionally skip WDOG_Refresh(); on interrupt, log urgent context (e.g., last error codes), then allow timeout reset.

Success criteria:

- Board resets; boot log shows reset reason as WDOG timeout.

### Lab 5 — ARINC 429 parity/SSM injection via ADK‑8582

Goal:

- Verify ARINC input filters reject malformed words.

Steps:

- Start from boards/evkbimxrt1050/driver_examples/lpuart/interrupt_transfer/ for UART to ADK‑8582. Wire EVKB LPUARTx TX/RX to ADK‑8582 control UART.

- Add adk429_uart.c/.h and arinc429_word.c/.h from §10.

- Send a valid altitude word ARINC429_BuildWord(...) → ADK‑8582 pass‑through to DUT ARINC; your DUT monitor should accept it.

- Resend with parityOverride = EVEN (ARINC requires odd parity).

- Resend with SSM = Failure but plausible numeric field.

- Resend with label set to a different parameter while keeping data field from the first (label discipline violation).

Success criteria:

- Each injected fault increments the right counters; invalid words are dropped; normal operation resumes after valid traffic.

### Lab 6 — UART burst/overrun simulation (timing fault on interface)

Goal:

- Emulate bus saturation; ensure driver handles overrun.

Steps:

  - Modify adk429_uart_send_burst() to stream back‑to‑back commands with minimal inter‑byte gap (use EDMA or a tight loop).

  - Verify LPUART RX overrun flags are detected (kLPUART_RxOverrunFlag), counters increment, and recovery occurs.

Success criteria:

- Overruns logged, no deadlocks; flow control (if used) is honoured.

## 10) Code (built on your SDK)

Conventions: All examples include board.h, clock_config.h, pin_mux.h, and fsl_debug_console.h. UART/WDOG APIs follow the SDK style you can see in the examples listed earlier. Handlers print SCB status then either reset or loop in lab mode.

### 10.1 fi_crc32.h / fi_crc32.c — lightweight CRC32 & utilities

// fi_crc32.h
#pragma once
#include <stdint.h>
#include <stddef.h>

uint32_t FI_CRC32(const void *data, size_t len);

// fi_crc32.c
#include "fi_crc32.h"

uint32_t FI_CRC32(const void *data, size_t len)
{
    const uint8_t *p = (const uint8_t*)data;
    uint32_t crc = 0xFFFFFFFFu;
    for (size_t i = 0; i < len; ++i) {
        crc ^= p[i];
        for (int k = 0; k < 8; ++k) {
            uint32_t mask = -(crc & 1u);
            crc = (crc >> 1) ^ (0xEDB88320u & mask);
        }
    }
    return ~crc;
}

### 10.2 fi_mutate.h / fi_mutate.c — bit flip & random delay

// fi_mutate.h
#pragma once
#include <stdint.h>
#include <stddef.h>
#include <stdbool.h>

void FI_FlipBit(uint8_t *buf, size_t len, uint32_t bitIndex);
void FI_FlipRandomBit(uint8_t *buf, size_t len);
void FI_RandomDelayUs(uint32_t minUs, uint32_t maxUs);

// fi_mutate.c
#include "fi_mutate.h"
#include "fsl_trng.h"
#include "fsl_common.h"

extern TRNG_Type * const TRNG0; // provided by SDK device headers

static uint32_t rng32(void)
{
    uint32_t w;
    TRNG_GetRandomData(TRNG0, &w, sizeof(w));
    return w;
}

void FI_FlipBit(uint8_t *buf, size_t len, uint32_t bitIndex)
{
    uint32_t byte = bitIndex / 8u;
    uint8_t  mask = 1u << (bitIndex % 8u);
    if (byte < len) buf[byte] ^= mask;
}

void FI_FlipRandomBit(uint8_t *buf, size_t len)
{
    uint32_t totalBits = (uint32_t)len * 8u;
    FI_FlipBit(buf, len, rng32() % totalBits);
}

void FI_RandomDelayUs(uint32_t minUs, uint32_t maxUs)
{
    uint32_t span = (maxUs > minUs) ? (maxUs - minUs) : 0u;
    uint32_t delay = minUs + (span ? (rng32() % span) : 0u);
    /* Busy wait: for lab only. Use DWT cycle counter if enabled. */
    for (volatile uint32_t t = 0; t < delay * 100; ++t) { __NOP(); }
}

Hook: Initialize TRNG as shown in boards/evkbimxrt1050/driver_examples/trng/random/trng_random.c before using these utilities.

### 10.3 Exception/MPU: fi_faults.c — enable traps and verbose handlers

// fi_faults.c (compile as part of your project)
#include "fsl_debug_console.h"
#include "fsl_common.h"
#include "board.h"
#include "core_cm7.h"
#include "m-profile/armv7m_mpu.h"

/* Enable divide-by-zero trap for UsageFault tests */
void FI_EnableCoreTraps(void)
{
    SCB->CCR |= SCB_CCR_DIV_0_TRP_Msk;     // trap divide-by-zero
    SCB->SHCSR |= SCB_SHCSR_MEMFAULTENA_Msk | SCB_SHCSR_BUSFAULTENA_Msk | SCB_SHCSR_USGFAULTENA_Msk;
}

/* Create a read-only MPU region over [addr, addr+len) to trigger MemManage on write */
void FI_MPU_EnableReadOnlyRegion(void *addr, uint32_t len)
{
    uint32_t base = (uint32_t)addr;
    /* region size must be power-of-two and aligned; round up to next power */
    uint32_t size = 32; while (size < len) size <<= 1;
    uint32_t region = 3; // pick a free region index for lab

    __DMB();
    ARM_MPU_Disable();
    MPU->RNR  = region;
    MPU->RBAR = (base & 0xFFFFFFE0u) | MPU_RBAR_VALID_Msk | region; // 32B align minimal; adjust for your size
    /* AP=110 (RO for priv/unpriv), TEX/S/C/B=0, SRD=0, XN=0, SIZE=log2(size)-1 */
    uint32_t rasr = (6u << MPU_RASR_AP_Pos) | (((31 - __CLZ(size)) - 1) << MPU_RASR_SIZE_Pos) | MPU_RASR_ENABLE_Msk;
    MPU->RASR = rasr;
    ARM_MPU_Enable(MPU_CTRL_PRIVDEFENA_Msk);
    __DSB(); __ISB();
}

/* Minimal verbose fault handlers (lab mode). Avoid PRINTF in production ISRs. */
void HardFault_Handler(void)
{
    PRINTF("\n[HardFault] HFSR=0x%08lx CFSR=0x%08lx BFAR=0x%08lx MMFAR=0x%08lx\r\n",
           SCB->HFSR, SCB->CFSR, SCB->BFAR, SCB->MMFAR);
    NVIC_SystemReset();
}
void MemManage_Handler(void)
{
    PRINTF("\n[MemManage] CFSR=0x%08lx MMFAR=0x%08lx\r\n", SCB->CFSR, SCB->MMFAR);
    NVIC_SystemReset();
}
void BusFault_Handler(void)
{
    PRINTF("\n[BusFault] CFSR=0x%08lx BFAR=0x%08lx\r\n", SCB->CFSR, SCB->BFAR);
    NVIC_SystemReset();
}
void UsageFault_Handler(void)
{
    PRINTF("\n[UsageFault] CFSR=0x%08lx\r\n", SCB->CFSR);
    NVIC_SystemReset();
}

Note: The exact MPU helper macros are from CMSIS included in your SDK under CMSIS/Core/Include/. The code above uses raw register fields and ARM_MPU_Enable/Disable from CMSIS for clarity.

### 10.4 WDOG policy snippet (based on SDK example)

#include "fsl_wdog.h"

static WDOG_Type * const s_wdog = WDOG1; // per board

void FI_WDOG_InitAndArm(void)
{
    wdog_config_t config;
    WDOG_GetDefaultConfig(&config);
    config.timeoutValue = 0x400u;          // short for lab
    config.interruptTimeValue = 0x200u;    // early warning
    WDOG_Init(s_wdog, &config);
}

void FI_WDOG_Service(void)
{
    WDOG_Refresh(s_wdog);
}

Compare with boards/evkbimxrt1050/driver_examples/wdog/wdog.c for interrupts and reset cause printing.

### 10.5 ARINC 429 word builder and ADK‑8582 UART wrappers

// arinc429_word.h
#pragma once
#include <stdint.h>
#include <stdbool.h>

typedef enum { ARINC429_SSM_NORMAL=0, ARINC429_SSM_NCD=1, ARINC429_SSM_FT=2, ARINC429_SSM_FAIL=3 } arinc429_ssm_t;

uint32_t ARINC429_BuildWord(uint8_t label, uint8_t sdi, uint32_t data21, arinc429_ssm_t ssm, bool forceEvenParity);

// arinc429_word.c
#include "arinc429_word.h"

static inline uint8_t parity_odd32(uint32_t w)
{
    /* Returns 1 if odd number of ones in bits 0..30 */
    w ^= w >> 16; w ^= w >> 8; w ^= w >> 4; w ^= w >> 2; w ^= w >> 1;
    return (uint8_t)(~w & 1u);
}

uint32_t ARINC429_BuildWord(uint8_t label, uint8_t sdi, uint32_t data21, arinc429_ssm_t ssm, bool forceEvenParity)
{
    data21 &= 0x1FFFFFu;  // 21 bits
    sdi    &= 0x3u;       // 2 bits
    uint32_t word = 0;
    /* Bit allocation (logical view): [31:30]=SSM, [29:11]=DATA(21), [10:9]=SDI, [8:1]=LABEL, [32]=PARITY] */
    word |= ((uint32_t)ssm & 0x3u) << 30;
    word |= (data21 & 0x1FFFFFu) << 11;
    word |= ((uint32_t)sdi & 0x3u) << 9;
    word |= (uint32_t)label; // 8 bits
    /* Compute odd parity over bits 0..30. If forceEvenParity, flip it to be wrong. */
    uint8_t odd = parity_odd32(word);
    uint8_t parity = forceEvenParity ? (odd ^ 1u) : odd;
    if (parity) word |= (1u << 31); // place in MSB logically; instrument handles wire order
    return word;
}

// adk429_uart.h
#pragma once
#include <stdint.h>
#include <stdbool.h>
#include "fsl_lpuart.h"

status_t ADK429_InitUart(LPUART_Type *base, uint32_t baudrate);
status_t ADK429_SendWord(LPUART_Type *base, uint8_t label, uint8_t sdi, uint32_t data21, int ssm, bool badParity);
status_t ADK429_SendBurst(LPUART_Type *base, const uint32_t *words, uint32_t count);

// adk429_uart.c  (illustrative; adapt to your ADK-8582 command set)
#include "adk429_uart.h"
#include "arinc429_word.h"
#include <stdio.h>

status_t ADK429_InitUart(LPUART_Type *base, uint32_t baudrate)
{
    lpuart_config_t cfg;
    LPUART_GetDefaultConfig(&cfg);
    cfg.baudRate_Bps = baudrate;
    cfg.enableRx = true; cfg.enableTx = true;
    return LPUART_Init(base, &cfg, CLOCK_GetFreq(kCLOCK_OscClk));
}

static status_t send_ascii(LPUART_Type *base, const char *s)
{
    return LPUART_WriteBlocking(base, (const uint8_t*)s, (size_t)strlen(s));
}

status_t ADK429_SendWord(LPUART_Type *base, uint8_t label, uint8_t sdi, uint32_t data21, int ssm, bool badParity)
{
    uint32_t w = ARINC429_BuildWord(label, sdi, data21, (arinc429_ssm_t)ssm, badParity);
    /* Example ASCII command — replace with actual ADK-8582 format: "TX <hexword>\r\n" */
    char line[32];
    (void)snprintf(line, sizeof(line), "TX %08lX\r\n", (unsigned long)w);
    return send_ascii(base, line);
}

status_t ADK429_SendBurst(LPUART_Type *base, const uint32_t *words, uint32_t count)
{
    for (uint32_t i = 0; i < count; ++i) {
        char line[32];
        (void)snprintf(line, sizeof(line), "TX %08lX\r\n", (unsigned long)words[i]);
        status_t s = send_ascii(base, line);
        if (s) return s;
    }
    return kStatus_Success;
}

Important: The TX ASCII is a placeholder. Replace with the actual ADK‑8582 command set (word rate, channel select, loop, etc.). The LPUART_WriteBlocking usage follows the style in boards/evkbimxrt1050/driver_examples/lpuart/interrupt_transfer/lpuart_interrupt_transfer.c.

### 10.6 Putting it together — main.c (sketch)

#include "board.h"
#include "pin_mux.h"
#include "clock_config.h"
#include "fsl_debug_console.h"
#include "fsl_lpuart.h"
#include "fi_crc32.h"
#include "fi_mutate.h"

#define LAB_MODE 1

static uint8_t g_buf[32];
static volatile uint32_t g_crcOkCnt = 0, g_crcBadCnt = 0, g_faultCnt = 0;

int main(void)
{
    BOARD_ConfigMPU();     // generated by MCUXpresso config tool; keeps caches coherent
    BOARD_InitBootPins();
    BOARD_InitBootClocks();
    BOARD_InitDebugConsole();

    PRINTF("\r\nFault Injection Lab start\r\n");

    FI_EnableCoreTraps();
    FI_WDOG_InitAndArm();

    /* Fill buffer and compute CRC */
    for (int i=0;i<32;i++) g_buf[i]=(uint8_t)i;
    uint32_t crc = FI_CRC32(g_buf, sizeof g_buf);

    /* Inject a random bit flip */
    FI_FlipRandomBit(g_buf, sizeof g_buf);
    uint32_t crc2 = FI_CRC32(g_buf, sizeof g_buf);
    if (crc2 != crc) {
        ++g_crcBadCnt;
        PRINTF("CRC mismatch as expected after bit flip (good=%08x bad=%08x)\r\n", crc, crc2);
    } else {
        ++g_crcOkCnt; // extremely unlikely
    }

#if LAB_MODE
    /* Provoke divide-by-zero UsageFault */
    volatile int z = 0; volatile int x = 10 / z; (void)x;
#endif

    while (1) {
        FI_WDOG_Service();
        // Your ARINC test sequences here, e.g., ADK429_SendWord(...)
    }
}

## 11) Test design patterns to maximise coverage

- One‑fault‑at‑a‑time: Isolate fault types to attribute detection to a single mechanism.

- Seeded randomness: When using TRNG for bit selection, also log a software seed so the sequence can be reproduced (or capture the flipped index).

- State checkpointing: On fault, dump minimal state (active label, last counters, SCB fault registers) into a reserved SRAM log (guard with MPU execute‑never to avoid code execution from logs).

- Watchdog policy table: Define when to continue, when to re‑initialise a subsystem, and when to reset the whole system.

- Graceful degradation: For ARINC reception, drop to last‑known‑good with SSM propagated downstream.

- Fault campaigns: Automate sequences: parity→SSM→label→burst→timing to prove monitors don’t interfere with each other.

## 12) Best practices (avionics‑grade)

- Traceability to requirements: Each injected fault maps to a low‑level requirement (LLR) and a safety objective from the SSA (System Safety Assessment).

- Segregation: Keep FI utilities out of production by #if LAB_MODE and separate libraries/binaries.

- Deterministic logging: Use ring buffers; don’t PRINTF inside tight ISRs in production (acceptable in lab).

- Re‑entrancy & priority: Be mindful that faults often occur at awkward times; test with interrupts enabled and realistic NVIC priorities.

- MPU everywhere: Protect stacks, constant tables (RO), and a poison region at address 0 to trap null dereferences.

- Data guards: CRC or HMAC (Hash‑Based Message Authentication Code) on critical tables; dual‑compute then compare for FIH‑style robustness of safety monitors.

- Watchdog reason codes: On boot, read/reset reason and surface it to maintenance logs (compare WDOG examples).

- Interface discipline: For ARINC 429, enforce label allow‑lists, SDI checks, SSM semantics, and rate policing before values reach the application.

- Bench safety: Current‑limit supplies, use IO clamps; have a physical kill‑switch; isolate ARINC wiring.

- Certification evidence: Keep procedures, objective evidence (test results, logs), and independence per DO‑178C. If tools drive fault scripts, assess under DO‑330 (Software Tool Qualification Considerations).

## 13) What to collect as evidence

- Test procedure IDs linked to LLRs.

- The exact fault parameters (e.g., bit index, label, SSM, parity flag) and timestamps.

- SCB register dumps for exception faults.

- WDOG reset cause and reboot times.

- Before/after counters for ARINC input filters.

- Pass/fail verdict and rationale.

## 14) Troubleshooting

- No MemManage fault when writing RO region? Ensure region size/alignment follow MPU rules and SCB->SHCSR enables MemManage.

- PRINTF not visible in handlers? Use SWO/ITM or buffer the log to SRAM and print on next boot.

- ADK‑8582 not reacting? Verify UART baud, newline, command syntax, and instrument channel enable.

- Board resets unexpectedly during FI runs? Likely WDOG timeout—tune timeoutValue or ensure service calls in your loops.

## 15) Wrap‑up

You now have a progressive set of SWIFI and ARINC 429 interface‑level injections on i.MX RT1050, built directly on the MCUXpresso SDK you provided. The labs aim to exercise the exception architecture (HardFault/MemManage/BusFault/UsageFault), MPU protections, WDOG policy, and ARINC input discipline. With these in place, you can build automated fault campaigns and gather certification‑ready evidence.

## 16) Appendix — Mapping to SDK examples (for quick copy‑start)

- LPUART (for ADK‑8582 control): boards/evkbimxrt1050/driver_examples/lpuart/interrupt_transfer/

- WDOG/RTWDOG: boards/evkbimxrt1050/driver_examples/wdog/ and .../rtwdog/

- TRNG (for randomized bit flips): boards/evkbimxrt1050/driver_examples/trng/random/

- EDMA (for burst stress): boards/evkbimxrt1050/driver_examples/edma/

- PWM Fault demo (peripheral fault concept): boards/evkbimxrt1050/demo_apps/pwm_fault/

Keep the acronym expansion policy in your documents: expand at first use (as done here).