# Hardware Fault Injection & Signal‑Glitch–Induced Resets

Platform focus: NXP i.MX RT1050‑EVKB (Cortex‑M7)

I/O context for avionics labs: EVKB interfaced to Holt ADK‑8582 (HI‑8582/HI‑8583) ARINC 429 evaluation board via UART console bridge.

## 1) Why this module exists (learning outcomes)

By the end of this module, you will be able to:

1. Distinguish power, clock, reset‑pin and electromagnetic/transient fault mechanisms and explain how they precipitate system resets.

2. Read, decode and persist reset cause on the i.MX RT1050 using the System Reset Controller (SRC); maintain a monotonic boot counter across resets.

3. Configure RTWDOG (windowed watchdog) safely; prove, with experiments, that watchdog and reset sources are correctly attributed.

4. Design board‑ and firmware‑level glitch resilience (reset supervision, power integrity, EMI/ESD countermeasures, software fallbacks).

5. In an avionics context (ARINC 429), build a fault‑tolerant data path; recover from microcontroller resets without corrupting bus timing or labels.

## 2) Definitions

Hardware fault injection (HFI):

- Any deliberate or accidental physical perturbation—voltage, clock, reset, electromagnetic, or environmental—that changes the microcontroller’s intended behavior.

- In safety‑critical work our goal is resilience (avoid unsafe states), not bypassing protections.

Signal glitch:

- A short‑duration, unintended pulse or transient on a digital or power signal.

- On reset lines it can cause spurious POR/warm resets;

- on power rails it can cause brown‑outs;

- on clocks it can cause loss‑of‑lock or instruction fetch faults.

Reset taxonomy (as seen by i.MX RT1050 SRC): -

- POR (Power‑On Reset): Triggered by the POR_B circuit when power ramps. –

- External reset pin events: Short pulses on IPP_RESET_B/IPP_USER_RESET_B qualified by on‑chip logic. –

- Watchdog resets: From RTWDOG/WDOG3 or WDOG1/WDOG2 timeouts. - Software/CPU lockup resets: Via SYSRESETREQ or lockup. –

- JTAG‑initiated resets: System, software, or certain IR code selections.

Glitch sources you’ll meet in the lab: -

- Power droop/brown‑out: Supply impedance + step load → rails dip → POR logic asserts.

- Reset‑pin bounce: Mechanical button bounce, crosstalk, or radiated bursts coupled onto the trace.

- Clock disturbance: RF/EMI near the crystal/PLL or bad decoupling.

- EMI/EFT (Electro‑Magnetic Interference / Electrical Fast Transient): Cable injection (e.g., ARINC 429 cabling near static discharges) coupling into MCU nets.

Important:

- In all exercises we stay strictly within component absolute‑maximum ratings.

- We never attempt security bypass;

- we explore reliability and recovery only.

## 3) The i.MX RT1050 reset architecture in practice

### 3.1 What the SRC (System Reset Controller) tells you

On each boot, the SRC exposes sticky flags indicating the last reset source.

You will:

- Read SRC_GetResetStatusFlags() (MCUXpresso SDK) at the start of main().

- Map bits such as Watchdog, WDOG3 (RTWDOG), JTAG, IPP_RESET_B, IPP_USER_RESET_B, CPU lockup, to human‑readable strings.

- Clear the flags you’ve consumed so the next reset is unambiguous.

### 3.2 GPRs survive resets

- Use SRC GPRx (General Purpose Registers) as tiny, always‑on “scratchpads” across resets to keep a boot counter, last reset cause, and a sequence number for post‑mortem correlation.

### 3.3 Board‑level reset and supervision

- On the EVKB the pushbutton asserts reset via the board’s reset path.

- In production avionics hardware, add an external reset supervisor IC to qualify POR_B and guarantee minimum reset pulse width and release timing relative to power rails.

- Pair this with clean routing (short reset trace, strong pull‑up, optional RC for switch bounce), and robust decoupling near VDD pins.

## 4) How signal glitches become resets (failure physics)

- Supply droop → POR comparator trips → SOC goes through cold or warm reset. Typical trigger: motor/relay actuation, RF PA burst, or ARINC 429 line drivers starting up.

- Reset‑pin transients → Threshold crossing for just long enough to qualify as reset. Typical trigger: ESD, cable hot‑plug, or long reset trace as an antenna.

- Clock/PLL disturbance → Instruction fetch or bus faults → software triggers system reset or watchdog times out.

- EMI/EFT bursts on I/O → Internal protection diodes inject current → local ground bounce → unintended resets.

## 5) Generic example (non‑avionics)

A handheld industrial scanner intermittently resets when a high‑current barcode LED strobe fires.

Root cause:

- Shared 3.3 V rail with insufficient bulk capacitance;

- 1.8 V LDO momentarily falls out of regulation; POR asserts.

- Fixes: add 22 µF bulk + 0.1 µF local ceramics per IC, move LED driver to its own buck, add a 180 ms reset supervisor with proper release sequencing, and implement windowed watchdog for graceful recovery.

## 6) Specific avionics use case (strong, realistic scenario)

Platform:

- A Remote Data Concentrator (RDC) collecting discrete inputs and forwarding status over ARINC 429 (Aeronautical Radio, Incorporated 429) to a Flight Control Computer (FCC).

- The RDC is built on i.MX RT1050; ARINC 429 Tx/Rx are handled by a Holt HI‑8582 on the ADK‑8582 evaluation board.

- A cabin maintenance event produces EFT/ESD bursts on a harness near the RDC bay.

Observed symptom:

- Sporadic resets every few hours, each causing a 300 ms gap in label transmission.

- Downstream LRUs (Line‑Replaceable Units) flag “intermittent source” faults when three consecutive words are missed.

Root‑cause chain:

- Harness‑coupled burst → transient on MCU reset net (long, unterminated trace) → SRC shows external IPP_RESET_B resets;

- Boot counter increments with no watchdog flags.

Remediation deployed: -

Hardware:

- Add reset supervisor IC (≥140 ms), route reset trace away from high‑dv/dt lines;

- Tighten VDD decoupling around ARINC line driver;

- Add TVS at connector;

- Lift pull‑up to 10 kΩ and add 100 nF RC to button path only (not POR_B);

- improve chassis bonding. –

- Firmware:

  - Early‑boot SRC logging, windowed RTWDOG (no feeding during boot stalls),

  - ARINC 429 stream state machine that resumes at label boundaries after reset;

  - UART heartbeat to the ADK‑8582 bridge to re‑negotiate stream parameters on reboot.

- Operations:

  - DO‑160 (Environmental Conditions and Test Procedures for Airborne Equipment) radiated and conducted susceptibility sweeps repeated;

  - no further resets.

## 7) Hands‑on exercises (EVKB + MCUXpresso SDK 25.06.00)

Project base:

- Start from the EVKB hello_world (XIP or SDRAM) example in the SDK you provided.

- Add the files below and enable fsl_debug_console for logging over OpenSDA UART.

### Exercise 1 — “Know your resets”: decode and persist reset causes

Goal:

- On every boot, print the last reset cause, increment a boot counter in SRC GPR1, and clear the flags.

Key ideas:

- SRC_GetResetStatusFlags(),

- SRC_ClearResetStatusFlags(),

- SRC_{Get,Set}GeneralPurposeRegister().

/* File: reset_diag.c  (add to your app and call ResetDiag_RunEarly()) */
#include "fsl_src.h"
#include "fsl_debug_console.h"

static const char *flag_to_str(uint32_t f)
{
    switch (f) {
    case kSRC_Wdog3ResetFlag:          return "RTWDOG/WDOG3 timeout";
    case kSRC_WatchdogResetFlag:       return "WDOG1/WDOG2 timeout";
    case kSRC_IppResetPinFlag:         return "IPP_RESET_B (power-up sequence)";
    case kSRC_IppUserResetFlag:        return "IPP_USER_RESET_B (external reset pin)";
    case kSRC_JTAGSystemResetFlag:     return "JTAG system reset";
    case kSRC_JTAGSoftwareResetFlag:   return "JTAG software reset";
    case kSRC_JTAGGeneratedResetFlag:  return "JTAG generated reset (EXTEST/HIGHZ)";
    case kSRC_LockupSysResetFlag:      return "CPU lockup / SYSRESETREQ";
    case kSRC_TemperatureSensorResetFlag: return "On-chip temperature sensor reset";
    default: return "Unknown/Multiple";
    }
}

void ResetDiag_RunEarly(void)
{
    uint32_t flags = SRC_GetResetStatusFlags(SRC);
    uint32_t bootCount = SRC_GetGeneralPurposeRegister(SRC, 0); /* GPR1 index 0 */
    bootCount++;
    SRC_SetGeneralPurposeRegister(SRC, 0, bootCount);

    PRINTF("\r\n=== Reset diagnostic ===\r\n");
    PRINTF("Boot count (GPR1): %lu\r\n", (unsigned long)bootCount);

    if (flags == 0) {
        PRINTF("Reset flags: none (cold power-up or flags already cleared)\r\n");
    } else {
        PRINTF("Reset flags raw: 0x%08lx\r\n", (unsigned long)flags);
        /* Print all flags that are set (may be more than one). */
        if (flags & kSRC_Wdog3ResetFlag)         PRINTF(" - %s\r\n", flag_to_str(kSRC_Wdog3ResetFlag));
        if (flags & kSRC_WatchdogResetFlag)      PRINTF(" - %s\r\n", flag_to_str(kSRC_WatchdogResetFlag));
        if (flags & kSRC_IppResetPinFlag)        PRINTF(" - %s\r\n", flag_to_str(kSRC_IppResetPinFlag));
        if (flags & kSRC_IppUserResetFlag)       PRINTF(" - %s\r\n", flag_to_str(kSRC_IppUserResetFlag));
        if (flags & kSRC_JTAGSystemResetFlag)    PRINTF(" - %s\r\n", flag_to_str(kSRC_JTAGSystemResetFlag));
        if (flags & kSRC_JTAGSoftwareResetFlag)  PRINTF(" - %s\r\n", flag_to_str(kSRC_JTAGSoftwareResetFlag));
        if (flags & kSRC_JTAGGeneratedResetFlag) PRINTF(" - %s\r\n", flag_to_str(kSRC_JTAGGeneratedResetFlag));
        if (flags & kSRC_LockupSysResetFlag)     PRINTF(" - %s\r\n", flag_to_str(kSRC_LockupSysResetFlag));
        if (flags & kSRC_TemperatureSensorResetFlag) PRINTF(" - %s\r\n", flag_to_str(kSRC_TemperatureSensorResetFlag));
    }

    /* Clear all observed flags so the next reset cause is unambiguous. */
    SRC_ClearResetStatusFlags(SRC, flags);
}

Integration:

- Call ResetDiag_RunEarly();

- at the very top of main() right after clocks and console init, e.g.:

int main(void)
{
    BOARD_ConfigMPU();
    BOARD_InitBootPins();
    BOARD_InitBootClocks();
    BOARD_InitDebugConsole();

    ResetDiag_RunEarly();

    PRINTF("System up.\r\n");
    while (1) { __NOP(); }
}

What you should see:

- Press the EVKB reset button → IPP_USER_RESET_B appears.

- If you purposely cause a watchdog timeout in Exercise 2, you’ll see RTWDOG/WDOG3 timeout flagged.

### Exercise 2 — Configure a windowed watchdog and verify attribution

Goal:

- Configure RTWDOG (aka WDOG3) in windowed mode.

- Demonstrate three behaviors:

(a) correct periodic refresh;

(b) early refresh → reset;

(c) missed refresh → reset.

- Verify SRC flags each time.

/* File: wdog_window.c */
#include "fsl_rtwdog.h"
#include "fsl_debug_console.h"
#include "board.h"

static void busy_delay(volatile uint32_t loops) { while (loops--) __NOP(); }

void Wdog_Demo_Run(void)
{
    rtwdog_config_t cfg;
    RTWDOG_GetDefaultConfig(&cfg);
    cfg.enableUpdate   = true;                   /* allow reconfig without reset */
    cfg.enableInterrupt = false;                 /* simple reset behavior */
    cfg.enableWindowMode = true;                 /* enable windowing */
    cfg.windowValue    = 0x4000;                 /* earliest allowed refresh */
    cfg.timeoutValue   = 0x9000;                 /* reset if not refreshed by then */
    cfg.prescaler      = kRTWDOG_ClockPrescalerDivide1;

    PRINTF("Configuring RTWDOG (window:0x%X, timeout:0x%X)\r\n", cfg.windowValue, cfg.timeoutValue);
    RTWDOG_Init(RTWDOG, &cfg);

    /* Case (a): good refresh cadence (between window and timeout) */
    for (int i = 0; i < 5; ++i) {
        while (RTWDOG_GetCounterValue(RTWDOG) > cfg.windowValue) { /* wait to enter window */ }
        RTWDOG_Refresh(RTWDOG);
        PRINTF("Refreshed in window, iteration %d\r\n", i);
    }

    /* Case (b): early refresh → intentional violation */
    PRINTF("Refreshing too early to trigger window violation reset...\r\n");
    RTWDOG_Refresh(RTWDOG); /* out-of-window refresh → reset */

    /* You will not reach here on violation; after reset, Exercise 1 prints WDOG3 flag. */
}

Integration:

- Call Wdog_Demo_Run();

- from main() after ResetDiag_RunEarly().

- To demonstrate missed refresh (case c), comment out the refresh loop so the timeout expires.

Safety note:

- RTWDOG uses its own low‑frequency clock domain.

- Do not disable it globally in SystemInit if you want reliable reset attribution.

### Exercise 3 — Reset‑pin pulse qualification (safe, controlled)

Goal:

- Empirically determine what minimum reset pulse width on the EVKB’s external reset input produces a reset, and demonstrate that adding a supervisor or RC qualification raises the immunity margin.

Method (safe lab pattern): -

- Use a lab function generator configured for 3.3 V logic levels (or lower with a buffer) and AC‑coupled via a series resistor (≥10 kΩ) into the board’s reset test point (not the POR_B pin).

- Start with clean 5–10 ms pulses at 0.2 Hz and progressively shorten to 1 ms/500 µs/100 µs. –

- Observe console output after each pulse.

- Exercise 1 prints IPP_USER_RESET_B when the pulse is long enough to qualify. –

- If your hardware has a reset supervisor mod, repeat the sweep;

- you should see the threshold lengthen (i.e., short pulses are ignored), which is the intended effect.

Boundaries:

- Never inject negative voltages, never exceed VDDIO, and never connect a low‑impedance source directly to the reset pin.

- You are testing qualification, not abusing the net.

### Exercise 4 — ARINC 429 continuity across MCU resets (EVKB ↔ ADK‑8582 via UART bridge)

Goal:

- Maintain bus integrity and rapid recovery of ARINC 429 transmissions when the EVKB resets, without corrupting labels or data on the 429 bus.

Topology:

- EVKB UART (115 200 bps, 8‑N‑1) ↔ simple UART bridge (on the ADK‑8582 kit or a small intermediary MCU if required) ↔ HI‑8582 parallel interface driving the ARINC 429 line driver.

- The UART side accepts framed commands like TX:<LABEL>:<SDI>:<DATA>:<SCHED> and returns health/ack frames.

Firmware tasks on EVKB:

- Handshake: On boot, send HELLO <bootCount> <lastResetCause>. Expect READY from the bridge.

- Stream restart: Resume label schedule only after READY; this guarantees label boundaries after a reset.

- Heartbeat: Send PING n every second; if 3 consecutive timeouts occur, pause and re‑handshake.

- Logging: Store last 32 events (reset, handshake, timeouts) in OCRAM ring for post‑flight analysis.

Skeleton UART framing (EVKB side):

/* Pseudocode sketch for clarity; use SDK LPUART driver on EVKB */
void Arinc_Bridge_OnBoot(uint32_t bootCount, const char *cause)
{
    printf("HELLO %lu %s\r\n", (unsigned long)bootCount, cause);
    if (!WaitForReply("READY", 100 /*ms*/)) {
        /* Retry/backoff; do not emit ARINC traffic until READY received */
    }
}

Validation:

- While streaming labels, press the EVKB reset button (Exercise 1 & 2 active).

- You should see a short gap on the ARINC bus (the line driver keeps its last good state), then automatic resynchronization after READY.

## 8) Best‑practices checklist (avionics‑grade)

Architecture & requirements –

- Allocate reset behavior in the system safety assessment (ARP4754A / ARP4761):

  - define maximum permitted data gap on ARINC 429 for your function and ensure watchdog/reset strategy cannot produce a hazardous latent fault.

- Distinguish degraded mode (reduced label set) from fail‑silent (no transmit) on recovery.

Hardware

- Add an external reset supervisor with ≥140 ms hold‑time;

- route reset short, isolated from high‑dv/dt nets;

- use Schmitt‑trigger where a manual button is present.

- Power integrity: low‑ESR bulk + local 0.1 µF decouplers per VDD pin;

- split noisy loads (ARINC line drivers) from MCU core rails;

- keep ground returns tight.

- EMC: TVS on I/O at the connector, common‑mode chokes on long runs, proper shielding/grounding, cable clamp bonding to chassis.

Firmware

- Read and clear SRC reset flags first thing; persist boot counter in SRC GPR1. - Use windowed RTWDOG with an independent clock;

- refresh from a healthy‑system task only (not a bare SysTick) so stalled systems time out.

- Implement boot‑loop detection: if bootCount increased N times within T seconds, enter ‘safe mode’ (minimal label set, heavy logging) and report via maintenance channel.

- Make drivers idempotent: all I/O (UART/ARINC bridge) must re‑initialize cleanly on any reset cause.

Verification –

- Reproduce faults with controlled pulses and DO‑160 style susceptibility tests; capture before/after immunity margins. –

- Include reset‑storm tests in CI hardware loops: random watchdog violations, external reset pulses, power rail brown‑out simulations using programmable supplies (within ratings).

## 9) Troubleshooting guide (what you’ll actually see)

- “Unknown/Multiple” reset causes: You forgot to clear flags after reading, or more than one source asserted. Clear and repeat.

- WDOG resets when debugging: You halted the core without disabling watchdog in debug mode. Set workMode.enableDebug = true or disable RTWDOG during interactive sessions.

- Resets when ARINC bursts start: Check line‑driver inrush → beef up local decoupling; add soft‑start if available.

- Resets only with long UART cables: Common‑mode noise → add CM choke and improve bonding; ensure UART ground reference is solid.

## 10) Wrap‑up (what “good” looks like)

- Every reset is attributed and logged with a monotonic counter.

- Watchdog is windowed and feeds only when the system is actually healthy.

- Short reset pulses and moderate EMI no longer produce spurious resets thanks to supervision and routing.

- ARINC 429 streams resume deterministically after EVKB resets without corrupting label timing.

## Appendix A — Turn‑key file list (drop‑in to your SDK tree)

- source/reset_diag.c (Exercise 1)

- source/wdog_window.c (Exercise 2)

- source/arinc_bridge.c (Exercise 4, UART framing)

- Add includes: #include "fsl_src.h", #include "fsl_rtwdog.h", #include "fsl_debug_console.h" in main.c and link the corresponding drivers in your MCUXpresso project settings.

## Appendix B — Acronyms (expanded at first use in text)

- ARINC 429 — Aeronautical Radio, Incorporated 429 serial data bus standard

- EMI — Electromagnetic Interference

- EFT — Electrical Fast Transient

- POR — Power‑On Reset

- RDC — Remote Data Concentrator

- FCC — Flight Control Computer

- LRU — Line‑Replaceable Unit

- SRC — System Reset Controller

- RTWDOG/WDOG3 — Real‑Time Watchdog Timer

- GPR — General Purpose Register (in SRC)

- DO‑160 — Environmental Conditions and Test Procedures for Airborne Equipment (RTCA)

- ARP4754A / ARP4761 — Guidelines for development of civil aircraft and systems / Safety assessment

### Notes on kit connectivity

The Holt ADK‑8582 kit exposes the HI‑8582/HI‑8583 ARINC 429 device. Where a native UART bridge is unavailable, insert a small microcontroller bridge that accepts UART frames from EVKB and services the HI‑8582’s parallel interface; the exercise remains unchanged from the EVKB perspective.