# Training Module: Software Fault Injection — Data Corruption & API Failures

Target hardware: NXP i.MX RT1050 EVKB (EVKB‑IMXRT1050)
SDK baseline: SDK_25_06_00_EVKB‑IMXRT1050
Avionics link for labs: EVKB‑IMXRT1050 LPUART ↔ ADK‑8582 (ARINC 429 development kit) over UART

## 1) Why this module exists

- Software Fault Injection (SFI) is the deliberate introduction of faults into a running program to validate Fault Detection, Isolation and Recovery (FDIR) mechanisms and to harden error‑handling paths that rarely execute in nominal testing.

- In avionics, this supports objectives typically derived from ARP4754A (Guidelines for Development of Civil Aircraft and Systems) and DO‑178C (Software Considerations in Airborne Systems and Equipment Certification), by demonstrating that the software behaves safely when inputs are corrupted or when lower‑layer Application Programming Interface (API) calls fail.

This module builds a practical SFI harness you can embed into bare‑metal firmware on the i.MX RT1050.

You will exercise two fault classes end‑to‑end:

- Data corruption: bit‑flips and malformed frames on internal buffers and on the UART receive path.

- API failures: synthetic error returns, timeouts, and partial operations injected at driver boundaries.

- The labs culminate in an avionics scenario where ARINC 429 words from an external ARINC 429 (Aeronautical Radio, Incorporated) link are fed to the EVKB through a UART bridge (ADK‑8582), and software corruption/failure is injected to verify monitors and fail‑safe behaviors.

## 2) Foundations and precise definitions

Before we inject anything, align on terminology used in safety engineering:

- Fault: the hypothesized root cause introduced in the system (e.g., a flipped bit in a buffer, or an API returning a failure code).

- Error: the part of the system state that becomes incorrect because of a fault (e.g., a corrupted ARINC 429 word in the receive ring buffer).

- Failure: an externally observable deviation of service (e.g., the application outputs an invalid attitude to the bus, or crashes).

SFI (Software Fault Injection)

- Act of adding code that creates controlled faults so that you can observe whether your detection and recovery logic prevents those errors from escaping as failures.

API (Application Programming Interface) failure

- Means the software boundary between your application and a driver or service (e.g., NXP LPUART driver).

- We synthetically return error codes, truncate transfers, or delay/tamper with completion notifications to exercise retry/timeout paths.

ARINC 429 is a unidirectional, self‑clocking, two‑wire databus standard widely used in transport aircraft.

Each 32‑bit word contains an 8‑bit Label (bits 1..8),

Source/Destination Identifier (SDI) (bits 9..10), 19‑bit Data (bits 11..29),

Sign/Status Matrix (SSM) (bits 30..31), and an odd parity bit (bit 32).

In this module we will not generate electrical ARINC 429 directly from the EVKB; instead, an external ADK‑8582 provides ARINC I/O and exposes a simple UART framing toward the EVKB, which we exercise and corrupt.

## 3) Fault models we will implement on EVKB‑IMXRT1050

We keep the models simple, reproducible, and relevant to airborne systems:

### 3.1 Data corruption models

- Single‑bit flip: invert one bit at a configured offset within a buffer (typical Single Event Upset).

- Multi‑bit burst: invert N random bits within a packet or ring‑buffer window.

- Stuck‑at: force a bit to 0 or 1 regardless of written value.

- Field‑aware tamper: for ARINC 429 frames: mis‑label (change Label), corrupt Sign/Status Matrix (SSM), or flip parity.

### 3.2 API failure models

- Synchronous error returns: driver function returns kStatus_Fail, kStatus_InvalidArgument, or a synthetic code while doing nothing.

- Busy/partial operations: nonblocking send reports kStatus_LPUART_TxBusy or only transfers part of the buffer.

- Timeouts: completion IRQ or DMA callback is intentionally suppressed or delayed beyond application watchdog thresholds.

All models are deterministically controllable by a small runtime harness with a seedable pseudo‑random number generator (PRNG).

You can run campaigns that are repeatable for debugging and certification evidence.

## 4) Architecture of the SFI harness

- The harness is a tiny library (fi.h/fi.c) compiled into the firmware with #define SFI_ENABLED 1. A Fault Site is a named point in code where a fault might be injected.

- Each site has a trigger policy (every Nth hit, probability p, one‑shot after count K, or active within a window), an optional parameter (e.g., how many bits to flip), and a per‑site PRNG state so that sites are independent.

The harness provides three primitives:

- bool FI_ShouldFault(fi_site_t *s): increments a site counter and decides whether to fire.

- void FI_CorruptBytes(uint8_t *buf, size_t len, uint32_t nbits): flips nbits in place using the site PRNG.

- Driver wrappers: thin functions/macros that either call into SDK drivers or synthesize an error/partial behavior when FI_ShouldFault fires.

- Every injection emits an event record into a small ring buffer (site name, monotonic counter, argument, first few bytes of the affected buffer).

- A shell prompt on the debug UART dumps this ring on demand so you retain evidence of what was injected and when.

### 4.1 Observability on the EVKB

- Console: the EVKB’s default debug UART (LPUART1 in the SDK examples) prints injection events and accepts simple commands (e.g., fi arm RX_BURST every 100 bits=3).

- LED: the board user LED (GPIO1 pin 9 via BOARD_USER_LED_GPIO/BOARD_USER_LED_PIN) is toggled on each injection and set solid if the application detects data corruption or an API error.

- Watchdog: use RTWDOG or WDOG1 to demonstrate that unhandled failure paths lead to a controlled reset.

## 5) Implementation: the fault injector library (fi.h / fi.c)

Note: Code below is written to the NXP SDK style and builds directly inside an SDK example application. Headers from the SDK used here: fsl_common.h, fsl_gpio.h, fsl_lpuart.h, and the board layer (board.h).

### 5.1 fi.h

#ifndef FI_H
#define FI_H

#include <stdint.h>
#include <stdbool.h>
#include <stddef.h>

#ifdef __cplusplus
extern "C" {
#endif

#ifndef SFI_ENABLED
#define SFI_ENABLED 1
#endif

/* Trigger modes */
typedef enum {
    FI_MODE_NEVER = 0,          /* disabled */
    FI_MODE_EVERY_N,            /* fire every Nth hit */
    FI_MODE_PROB_1_IN_N,        /* fire with probability 1/N */
    FI_MODE_AFTER_K_THEN_ONCE,  /* fire once after K hits */
    FI_MODE_WINDOW              /* fire when K1 <= hit < K2 */
} fi_mode_t;

/* Per‑site descriptor */
typedef struct {
    const char *name;
    fi_mode_t mode;
    uint32_t paramN;     /* N for EVERY_N/PROB; K for AFTER_K; K1 for WINDOW */
    uint32_t paramM;     /* unused except: WINDOW end K2; or bits to flip when used for data sites */
    uint32_t hits;       /* incremented on every probe */
    uint32_t rng;        /* xorshift32 state (non‑zero) */
} fi_site_t;

/* Event log entry */
typedef struct {
    const char *name;
    uint32_t hits;
    uint32_t arg0;
    uint32_t arg1;
    uint8_t  peek[8];
} fi_event_t;

void FI_Init(uint32_t global_seed);
void FI_Arm(fi_site_t *s, const char *name, fi_mode_t mode, uint32_t pN, uint32_t pM, uint32_t seed);
bool FI_ShouldFault(fi_site_t *s);
void FI_CorruptBytes(fi_site_t *s, uint8_t *buf, size_t len, uint32_t nbits);

/* Optional helpers for driver/API sites */
bool FI_ShouldFail_API(fi_site_t *s, int32_t *out_synthetic_status);

/* Event logging */
void FI_Log(const char *name, uint32_t hits, uint32_t a0, uint32_t a1, const uint8_t *peek, size_t peek_len);
size_t FI_Dump(char *out, size_t maxlen); /* returns bytes written into out */

#ifdef __cplusplus
}
#endif

#endif /* FI_H */

### 5.2 fi.c

#include "fi.h"
#include <string.h>

#ifndef FI_MAX_EVENTS
#define FI_MAX_EVENTS 128u
#endif

static fi_event_t s_events[FI_MAX_EVENTS];
static volatile uint32_t s_evt_head;
static uint32_t s_global_seed = 0xA5A5A5A5u;

static inline uint32_t xorshift32(uint32_t *state)
{
    uint32_t x = (*state) ? *state : 0x6D2B79F5u; /* avoid zero */
    x ^= x << 13; x ^= x >> 17; x ^= x << 5; *state = x; return x;
}

void FI_Init(uint32_t global_seed)
{
    s_global_seed = (global_seed) ? global_seed : 0x1u;
    s_evt_head = 0u;
    memset((void*)s_events, 0, sizeof(s_events));
}

void FI_Arm(fi_site_t *s, const char *name, fi_mode_t mode, uint32_t pN, uint32_t pM, uint32_t seed)
{
    s->name = name; s->mode = mode; s->paramN = pN; s->paramM = pM; s->hits = 0u; s->rng = seed ? seed : (name ? (uint32_t)name : s_global_seed);
    if (!s->rng) s->rng = 0xCAFEBABEu;
}

bool FI_ShouldFault(fi_site_t *s)
{
    if (!SFI_ENABLED || !s) return false;
    uint32_t h = ++(s->hits);
    switch (s->mode) {
        case FI_MODE_NEVER: return false;
        case FI_MODE_EVERY_N: return (s->paramN && (h % s->paramN) == 0u);
        case FI_MODE_PROB_1_IN_N: return (s->paramN && (xorshift32(&s->rng) % s->paramN) == 0u);
        case FI_MODE_AFTER_K_THEN_ONCE: return (h == s->paramN);
        case FI_MODE_WINDOW: return (h >= s->paramN && h < s->paramM);
        default: return false;
    }
}

void FI_CorruptBytes(fi_site_t *s, uint8_t *buf, size_t len, uint32_t nbits)
{
    if (!SFI_ENABLED || !buf || !len || !nbits) return;
    for (uint32_t i = 0; i < nbits; ++i)
    {
        uint32_t r = xorshift32(&s->rng);
        size_t idx = r % len; uint8_t bit = (1u << (r % 8u));
        buf[idx] ^= bit;
    }
}

bool FI_ShouldFail_API(fi_site_t *s, int32_t *out_synthetic_status)
{
    if (!SFI_ENABLED) return false;
    if (FI_ShouldFault(s)) { if (out_synthetic_status) *out_synthetic_status = -1; return true; }
    return false;
}

void FI_Log(const char *name, uint32_t hits, uint32_t a0, uint32_t a1, const uint8_t *peek, size_t peek_len)
{
    uint32_t i = s_evt_head++ % FI_MAX_EVENTS;
    s_events[i].name = name; s_events[i].hits = hits; s_events[i].arg0 = a0; s_events[i].arg1 = a1;
    if (peek && peek_len) {
        size_t n = (peek_len > sizeof(s_events[i].peek)) ? sizeof(s_events[i].peek) : peek_len;
        memcpy(s_events[i].peek, peek, n);
    } else {
        memset(s_events[i].peek, 0, sizeof(s_events[i].peek));
    }
}

size_t FI_Dump(char *out, size_t maxlen)
{
    size_t w = 0; uint32_t start = (s_evt_head > FI_MAX_EVENTS) ? (s_evt_head - FI_MAX_EVENTS) : 0;
    for (uint32_t k = start; k < s_evt_head; ++k) {
        uint32_t i = k % FI_MAX_EVENTS; const fi_event_t *e = &s_events[i];
        if (!e->name) continue;
        int n = snprintf(out ? out + w : NULL, out ? (int)(maxlen - w) : 0,
                         "[%06u] %-16s hits=%lu a0=%lu a1=%lu peek=%02X %02X %02X %02X\r\n",
                         (unsigned)k, e->name, (unsigned long)e->hits,
                         (unsigned long)e->arg0, (unsigned long)e->arg1,
                         e->peek[0], e->peek[1], e->peek[2], e->peek[3]);
        if (n < 0) break; w += (size_t)n; if (out && w >= maxlen) break;
    }
    return w;
}

## 6) Integrating with the NXP LPUART examples from the SDK

We start from the SDK example boards/evkbimxrt1050/driver_examples/lpuart/interrupt/ which sets up DEMO_LPUART (LPUART1) and an RX ring buffer in its DEMO_LPUART_IRQHandler.

We will add two fault sites:

- RX_CORRUPT: corrupt bytes arriving from the ADK‑8582 UART before storing into the ring.

- TX_API_FAIL: make LPUART_TransferSendNonBlocking return a synthetic error or behave as if busy.

Below is a trimmed patch you can apply to the example’s ISR and transmit path.

### 6.1 Header and site declarations (add to lpuart_interrupt.c)

#include "fi.h"

static fi_site_t s_rx_corrupt;
static fi_site_t s_tx_api_fail;

static void FI_Bringup(void)
{
    /* Seed with a constant for reproducibility; change at runtime from console */
    FI_Init(0x1050A429u);
    /* Corrupt 3 bits every 200 received bytes */
    FI_Arm(&s_rx_corrupt, "RX_CORRUPT", FI_MODE_EVERY_N, 200, 3, 0);
    /* Fail every 100th API call */
    FI_Arm(&s_tx_api_fail, "TX_API_FAIL", FI_MODE_EVERY_N, 100, 0, 0);
}

- Call FI_Bringup(); once in main() after board init and before entering the transmit/receive loop.

### 6.2 Injecting data corruption in the RX interrupt

- Locate in DEMO_LPUART_IRQHandler the standard code that reads a byte and pushes it into the ring buffer:

if ((kLPUART_RxDataRegFullFlag)&LPUART_GetStatusFlags(DEMO_LPUART))
{
    data = LPUART_ReadByte(DEMO_LPUART);
    /* If ring buffer not full, add data to ring */
    if (((tmprxIndex + 1) % DEMO_RING_BUFFER_SIZE) != tmptxIndex)
    {
        demoRingBuffer[rxIndex] = data;
        rxIndex++;
        rxIndex %= DEMO_RING_BUFFER_SIZE;
    }
}

Replace the assignment with a corrupt‑on‑trigger block:

if (((tmprxIndex + 1) % DEMO_RING_BUFFER_SIZE) != tmptxIndex)
    {
        uint8_t tmp = data;
        if (FI_ShouldFault(&s_rx_corrupt)) {
            FI_CorruptBytes(&s_rx_corrupt, &tmp, 1u, s_rx_corrupt.paramM /*bits*/);
            FI_Log("RX_CORRUPT", s_rx_corrupt.hits, (uint32_t)tmp, 0, &tmp, 1);
            USER_LED_TOGGLE();
        }
        demoRingBuffer[rxIndex] = tmp;
        rxIndex = (rxIndex + 1u) % DEMO_RING_BUFFER_SIZE;
    }

### 6.3 Injecting API failure on transmit path

- If your app uses LPUART_TransferSendNonBlocking(), wrap it as follows; otherwise, for the basic example that uses LPUART_WriteByte(), we simulate a drop (no transmit) on trigger.

static status_t LPUART_Send_WithFI(LPUART_Type *base, lpuart_handle_t *handle, lpuart_transfer_t *xfer)
{
    int32_t syn = 0;
    if (FI_ShouldFail_API(&s_tx_api_fail, &syn)) {
        /* Synthetic failure: pretend TX is busy; nothing sent */
        FI_Log("TX_API_FAIL", s_tx_api_fail.hits, (uint32_t)(xfer ? xfer->dataSize : 0), (uint32_t)syn, xfer ? xfer->data : NULL, xfer ? (xfer->dataSize>4?4:xfer->dataSize):0);
        USER_LED_TOGGLE();
        return kStatus_LPUART_TxBusy; /* or kStatus_Fail */
    }
    return LPUART_TransferSendNonBlocking(base, handle, xfer);
}

- Now route your application’s transmit calls through LPUART_Send_WithFI() instead of calling the SDK directly.

## 7) A generic worked example: corrupting a CRC‑protected message

- Imagine two tasks on the EVKB exchanging fixed‑size messages over a queue, each message ending with a CRC (Cyclic Redundancy Check).

- Insert a Fault Site on the producer’s memcpy into the queue.

- On every 50th hit, flip 2 bits.

- The consumer’s CRC check must detect the corruption, increment an error counter, and not act on the message.

Expected observation:

- console shows RX_CORRUPT events at deterministic intervals;

- error counter increments;

- system stays alive and responsive;

- watchdog does not fire.

If you temporarily disable the CRC check, the corrupted payload escapes and you will see the consumer misbehave—this demonstrates the purpose of the check.

## 8) Avionics use case: ARINC 429 word intake over UART from ADK‑8582

### 8.1 Framing and decode on the EVKB UART

- For the lab we assume the ADK‑8582 emits ARINC 429 words into UART frames like:

[0xA5][LEN=4][W0][W1][W2][W3][CHK]

- Where W0..W3 is the 32‑bit ARINC word (LSB first or MSB first per your adapter; pick one and keep it consistent), and CHK is a simple 8‑bit sum of all payload bytes.

- On the EVKB we parse the frame into a 32‑bit word, verify CHK, then verify odd parity on the word itself. Utilities below help construct/decode and intentionally corrupt fields.

/* arinc429.h */
static inline uint8_t arinc_label(uint32_t w)  { return (uint8_t)(w & 0xFFu); }
static inline uint8_t arinc_sdi(uint32_t w)    { return (uint8_t)((w >> 8) & 0x3u); }
static inline uint32_t arinc_data(uint32_t w)  { return (w >> 10) & 0x7FFFFu; }
static inline uint8_t arinc_ssm(uint32_t w)    { return (uint8_t)((w >> 29) & 0x3u); }
static inline uint8_t arinc_parity(uint32_t w) { return (uint8_t)((w >> 31) & 0x1u); }

static inline uint8_t arinc_parity_odd(uint32_t w)
{
    uint32_t v = w & 0x7FFFFFFFu; /* drop existing parity */
    v ^= v >> 16; v ^= v >> 8; v ^= v >> 4; v &= 0xFu; /* 1‑bit parity of 31 bits */
    uint8_t p = (0x6996u >> v) & 1u; /* even parity of 4 bits */
    return (uint8_t)(p ^ 1u); /* odd parity */
}

static inline uint32_t arinc_make(uint8_t label, uint8_t sdi, uint32_t data, uint8_t ssm)
{
    uint32_t w = 0u;
    w |= (uint32_t)label;
    w |= ((uint32_t)(sdi & 0x3u))  << 8;
    w |= ((uint32_t)(data & 0x7FFFFu)) << 10;
    w |= ((uint32_t)(ssm & 0x3u)) << 29;
    w |= ((uint32_t)arinc_parity_odd(w)) << 31; /* set odd parity */
    return w;
}

### 8.2 Injected faults specific to ARINC 429

We place two Fault Sites around the point where the 32‑bit word is accepted from the UART frame:

- A429_LABEL_TAMPER: when triggered, invert bit 0 of the Label.

- A429_PARITY_TAMPER: when triggered, invert the parity bit.

The receiver performs these checks in order:

- UART frame checksum CHK,

- ARINC odd parity,

- label/value plausibility (e.g., only accept a configured set of labels/SDI, and enforce monotonicity or rate limits on numeric data).

On any failure, the software increments a health counter, sets the LED, and does not propagate the value to the application.

Expected observation:

- All three detection stages catch the different injected errors, and your application maintains safe outputs (e.g., freezes to last‑known‑good value with SSM = Failure).

## 9) API failure injection pattern at driver boundary

- The i.MX RT1050 SDK uses status_t returns for most drivers.

- Your application must treat them as contracts.

- The FI wrappers replace calls to functions such as LPUART_TransferSendNonBlocking() or LPUART_TransferReceiveNonBlocking() with behavior that returns error codes or withholds callbacks.

Exercise behavior:

- Every 100th transmit request returns kStatus_LPUART_TxBusy. The application should back off and retry, bounded by a deadline.

- A window between hits 500 and 520 suppresses the TX empty interrupt, simulating an ISR lost event. Your application must detect the stall via a PIT (Periodic Interrupt Timer) tick and recover by re‑priming the transfer.

Implementation follows the wrapper in §6.3 and a small modification in the TX Empty ISR path: guard the call to LPUART_WriteByte() with if (!FI_ShouldFault(&s_tx_api_fail)).

## 10) Minimal console to control the fault campaign

Add a thin console on the debug UART so you can adjust sites without rebuilding.

Two commands are enough for the labs:

- fi dump — prints the last N injection events using FI_Dump().

- fi arm <site> every <N> [bits=<M>] — re‑arms a site at runtime. For example: fi arm RX_CORRUPT every 50 bits=2.

A simple parser can reuse the SDK’s lpuart_polling example for I/O.

## 11) Step‑by‑step labs (hands‑on)

### Lab 1 — Data corruption on the UART receive path

Objective. Prove that your input validation detects corrupted bytes before they can be decoded as valid ARINC 429 data.

Setup.

- Start from boards/evkbimxrt1050/driver_examples/lpuart/interrupt/.

- Add fi.h/fi.c to the project; call FI_Bringup() in main().

- Connect the ADK‑8582 UART to EVKB LPUART1 (default debug UART) or a second LPUART instance as available. Configure baud/format to match the adapter (typical 115200‑8‑N‑1 for bridging frames).

- Implement the simple frame parser and ARINC checks from §8.1. On successful decode, increment rx_ok; on failure, increment rx_bad and do not forward the value.

Injection. Arm s_rx_corrupt with EVERY_N=200 and bits=3.

Success criteria. rx_bad increases at the expected cadence; console shows RX_CORRUPT events; no invalid ARINC words pass parity + plausibility; LED toggles on each injection. The system remains responsive for >10 minutes without WDT reset.

### Lab 2 — API failure on transmit

Objective.

- Demonstrate that the application’s UART transmit code retries on kStatus_LPUART_TxBusy and does not deadlock when TX empty interrupts are suppressed for a bounded window.

Setup.

- Start from the same example, but switch transmit to LPUART_TransferSendNonBlocking() with a handle and callback (see SDK driver examples under driver_examples/lpuart/* for reference).

- Route all sends through LPUART_Send_WithFI().

- Add a PIT tick at 10 ms; in the tick, check a TX progress counter and if stalled for >100 ms, cancel and re‑prime the send (recovery path under test).

Injection.

- Arm s_tx_api_fail to EVERY_N=100 (synthetic busy), and once to WINDOW [500,520) (suppress TX empty ISR).

Success criteria.

- Application retries successfully; progress counter never stalls >100 ms; event log shows both types of injections.

### Lab 3 — ARINC 429 field‑aware corruption

Objective. Catch label and parity tampering and enforce safe output behavior.

Setup.

- In your UART frame handler, after assembling uint32_t word, check arinc_parity_odd(word) equals the parity bit. Then decode label, sdi, data, ssm.

- Maintain an allow‑list of labels/SDIs and a per‑label rate limit (e.g., do not accept more than 50 Hz unless expected).

- On plausibility failure, set an internal health flag and hold last‑known‑good value.

Injection.

- Add two new sites A429_LABEL_TAMPER and A429_PARITY_TAMPER; arm them with EVERY_N=37 and EVERY_N=53 respectively. Implement the mutation as:

if (FI_ShouldFault(&s_label)) { word ^= 0x01u; FI_Log("A429_LABEL_TAMPER", s_label.hits, word, 0, (uint8_t*)&word, 4); }
if (FI_ShouldFault(&s_par))   { word ^= (1u << 31); FI_Log("A429_PARITY_TAMPER", s_par.hits, word, 0, (uint8_t*)&word, 4); }

Success criteria.

- Neither tampered frame updates the output value; parity errors rejected; mis‑label rejected by allow‑list; LED set and health counters reflect events; system continues nominal operation.

## 12) Best practices tailored for avionics projects

- Trace every injection to a requirement or hazard. Each site should map to a low‑level requirement (LLR) derived from safety analysis (e.g., “The receiver shall reject frames that fail parity or plausibility checks”).

- Make injections deterministic and reportable. Fix seeds and record every event with counters and site names so that a failing campaign can be reproduced exactly.

- Isolate SFI from flight builds. Compile with SFI_ENABLED=0 and remove the console in production configurations. A visible banner on startup should indicate SFI builds.

- Exercise realistic magnitudes. For data corruption, inject single‑bit and burst patterns aligned with expected failure modes (e.g., SEU). For API failures, choose codes actually returned by the SDK (kStatus_LPUART_TxBusy, kStatus_Fail).

- Measure recovery latencies. Use PIT timestamps to demonstrate that the system meets detection/recovery deadlines consistent with aircraft function timing budgets.

- Use independent observation. When possible, monitor outputs on a second UART or GPIO pin to confirm the application did not mis‑actuate during injected faults.

- Review and lock down fault sites. The set of sites is configuration‑controlled; do not add ad‑hoc sites without analysis and review.

- Keep the harness tiny and audited. Simplicity reduces the risk that SFI code itself becomes a source of defects.

## 13) What to submit (lab deliverables)

- Code diffs against the NXP SDK examples showing fi.* integration and driver wrappers.

- Event logs from FI_Dump() correlating with your test plan.

- Pass/fail matrix per lab showing detection points and recovery paths.

- Short narrative (1–2 pages) explaining what failed when you intentionally disabled a check (to demonstrate the value of each barrier).

## 14) Appendix — SDK integration notes

- The EVKB user LED is exposed via BOARD_USER_LED_GPIO and BOARD_USER_LED_PIN (GPIO1/9 in the SDK demo). Macros USER_LED_ON/OFF/TOGGLE exist in board.h under the LED demos.

- The LPUART interrupt example macro‑configures the peripheral as:

- #define DEMO_LPUART            LPUART1
#define DEMO_LPUART_CLK_FREQ   BOARD_DebugConsoleSrcFreq()
#define DEMO_LPUART_IRQn       LPUART1_IRQn
#define DEMO_LPUART_IRQHandler LPUART1_IRQHandler

- Keep this mapping unless your hardware wiring requires a different instance.

- Keep ISR code bounded: the corruption logic above only flips a few bits and logs a small event; the heavy console output is deferred to the main loop to avoid ISR floods.

## 15) Additional

- How to  inject timing jitter? Add a FI_MODE_WINDOW site around a busy‑wait or PIT delay to stretch it and verify timeouts.

- How to flip DMA descriptors?  start with byte‑level corruption of the payload; descriptor corruption can crash the SoC and is better done later with a debugger halt/resume experiment.

- How to use ARM Debug (DAP) to patch memory at runtime? On i.MX RT1050, with an external probe; but for this module we keep SFI self‑contained in firmware.

### End of module