# Hands-on Exercise: Interfacing CY14B101Q2A-SXI nvSRAM with IMXRT1050-EVKB

This exercise shows how to connect and use the CY14B101Q2A-SXI 1 Mbit SPI nvSRAM as an external memory for the EVKB-IMXRT1050 board. You will:

- Wire the nvSRAM to the EVKB via Arduino SPI pins.

- Create an MCUXpresso project based on the lpspi_loopback SDK example.

- Implement SPI routines to write and read nvSRAM.

- Implement an avionics-style configuration/health record stored in nvSRAM.

## 1. Hardware hookup – CY14B101Q2A-SXI to IMXRT1050-EVKB

We will use the Arduino SPI header on the EVKB.

### EVKB Arduino SPI Pins (LPSPI1)

- D10 → GPIO_SD_B0_01 → SPI_SS → LPSPI1_PCS0 (chip select)

- D11 → GPIO_SD_B0_02 → SPI_SOUT (MOSI) → LPSPI1_SDO

- D12 → GPIO_SD_B0_03 → SPI_SIN (MISO) → LPSPI1_SDI

- D13 → GPIO_SD_B0_00 → SPI_CLK → LPSPI1_SCK

These are already configured for LPSPI1 in the SDK lpspi_loopback example.

### CY14B101Q2A-SXI Overview

- Capacity: 1 Mbit = 128K × 8 (0x20000 bytes).

- Address space: 0x00000 – 0x1FFFF.

- SPI commands:

  - READ (0x03) / WRITE (0x02) with 3-byte address (A16..A0).

  - WREN (0x06) must be issued before any write.

  - Other commands: RDSR, WRSR, STORE, RECALL, etc.

- Package: 8-pin SOIC with AutoStore (Q2A variant), using VCAP instead of WP.

### Pin Connections

Assuming your CY14B101Q2A-SXI is on an SOIC-8 breakout, connect:

| CY14B101Q2A Pin | Function | Connect to EVKB | Notes |
| --- | --- | --- | --- |
| 1 | HOLD# | 3.3V | Tie HIGH to avoid pausing SPI transfers. |
| 2 | SCK | Arduino D13 | SPI clock (LPSPI1_SCK). |
| 3 | SI (MOSI) | Arduino D11 | Master output to nvSRAM input. |
| 4 | VSS | GND | Common ground. |
| 5 | SO (MISO) | Arduino D12 | nvSRAM output to master input. |
| 6 | CS# | Arduino D10 | Chip select (LPSPI1_PCS0). |
| 7 | VCC | 3.3V | Supply (2.7–3.6 V). |
| 8 | VCAP | 0.1 µF to GND | Required for AutoStore. |

Additional notes:

- Keep SPI signals at 3.3V.

- Use short wires; twist with ground where possible for signal integrity.

- You can duplicate this wiring on multiple EVKBs for multi-channel experiments.

## 2. MCUXpresso Project Setup (using LPSPI example)

We reuse the lpspi_loopback example from the EVKB-IMXRT1050 SDK so all the pin mux and clocks for LPSPI1 are already configured.

### Step 1 – Ensure SDK is installed

Install or verify the SDK for EVKB-IMXRT1050 (matching your SDK_25_06_00_EVKB-IMXRT1050 ZIP) in MCUXpresso.

### Step 2 – Import the LPSPI loopback example

- Open MCUXpresso IDE.

- In the Quickstart Panel, click Import SDK example(s).

- Select board: EVKB-IMXRT1050.

- From the examples list choose:

  - demo_apps → lpspi_loopback

- Finish the wizard. A project like evkbimxrt1050_lpspi_loopback appears.

### Step 3 – Build and test the stock example

- Build the project to ensure it compiles.

- Optionally run it once; it performs an internal LPSPI1 loopback test.

### Step 4 – Replace the main source file

- Open source/lpspi_loopback.c in the project.

- Replace the entire file content with the code from section 3 below.

- Do not modify:

  - app.h (contains BOARD_EEPROM_LPSPI_BASEADDR, PCS macros, clock frequency).

  - pin_mux.c (configures LPSPI1 pins on GPIO_SD_B0_00..03).

  - board.c, clock_config.c, etc.

### Step 5 – Rebuild

- Build the project again with the new source.

### Step 6 – Debug configuration

- Connect the EVKB to your PC via the OpenSDA USB connector.

- Use the default CMSIS-DAP debug configuration that MCUXpresso creates.

- Click the Debug button (green bug icon) to flash and run.

### Step 7 – Serial output

- The example uses PRINTF via the board’s debug UART (OpenSDA virtual COM).

- Open a terminal on the OpenSDA COM port:

  - Baud rate: 115200, 8 data bits, no parity, 1 stop bit (115200 8-N-1).

## 3. C Code – CY14B101Q2A nvSRAM SPI Demo

Copy-paste this code into lpspi_loopback.c in the imported example project.

/*
 * NVSRAM (CY14B101Q2A-SXI) SPI interface demo for EVKB-IMXRT1050
 *
 * This file is intended to replace lpspi_loopback.c in the
 * 'lpspi_loopback' MCUXpresso SDK example project.
 *
 * It reuses:
 *   - BOARD_InitHardware() from board.c
 *   - LPSPI1 pin mux and clock setup from pin_mux.c / clock_config.c
 *   - app.h macros for LPSPI1 base address and PCS selection
 */

#include "fsl_device_registers.h"
#include "fsl_lpspi.h"
#include "fsl_debug_console.h"
#include "board.h"
#include "app.h"

#include <string.h>

/*******************************************************************************
 * Definitions
 ******************************************************************************/

/* CY14B101Q2A-SXI command opcodes */
#define NVSRAM_CMD_WREN     (0x06u) /* Write enable */
#define NVSRAM_CMD_WRDI     (0x04u) /* Write disable */
#define NVSRAM_CMD_RDSR     (0x05u) /* Read status register */
#define NVSRAM_CMD_WRSR     (0x01u) /* Write status register */
#define NVSRAM_CMD_READ     (0x03u) /* Read memory array */
#define NVSRAM_CMD_WRITE    (0x02u) /* Write memory array */
#define NVSRAM_CMD_FSTRD    (0x0Bu) /* Fast read */
#define NVSRAM_CMD_STORE    (0x3Cu) /* Software STORE to non-volatile */
#define NVSRAM_CMD_RECALL   (0x60u) /* Software RECALL from non-volatile */
#define NVSRAM_CMD_ASENB    (0x59u) /* AutoStore enable */
#define NVSRAM_CMD_ASDISB   (0x19u) /* AutoStore disable */
#define NVSRAM_CMD_SLEEP    (0xB9u) /* Enter low power / STORE + sleep */

/* 1 Mbit = 128 K x 8 = 0x20000 bytes */
#define NVSRAM_SIZE_BYTES   (0x20000u)
#define NVSRAM_MAX_XFER     (64u)   /* demo helper: max payload bytes per call */

/* Simple avionics configuration record stored in nvSRAM */
typedef struct
{
    uint32_t magic;           /* 0xA5A5A5A5 indicates valid struct */
    uint32_t firmwareVersion; /* arbitrary firmware version */
    uint32_t flightHours;     /* simulated accumulated flight hours */
    uint16_t lastFaultCode;   /* simulated last fault code */
    uint16_t reserved;
    uint32_t checksum;        /* simple additive checksum over first 5 fields */
} avionics_config_t;

/*******************************************************************************
 * Prototypes
 ******************************************************************************/

/* Reuse the same LPSPI1 initialization style as lpspi_loopback demo */
static void LPSPI_Init(void);

/* Low-level transfer helper */
static status_t NVSRAM_LpspiTransfer(uint8_t *txData,
                                     uint8_t *rxData,
                                     size_t dataSize);

/* nvSRAM protocol helpers */
static status_t NVSRAM_WriteEnable(void);
static status_t NVSRAM_WriteBytes(uint32_t address,
                                  const uint8_t *data,
                                  size_t length);
static status_t NVSRAM_ReadBytes(uint32_t address,
                                 uint8_t *data,
                                 size_t length);
static status_t NVSRAM_ReadStatus(uint8_t *status);

/* Demo helpers */
static void run_generic_pattern_test(void);
static void run_avionics_use_case_demo(void);
static uint32_t simple_checksum32(const uint32_t *data, size_t wordCount);

/*******************************************************************************
 * Code
 ******************************************************************************/

static void LPSPI_Init(void)
{
    lpspi_master_config_t masterConfig;

    /* Master config – based on lpspi_loopback demo, baudrate at 500 kHz
     * which is well below the 40 MHz limit of standard READ/WRITE opcodes. */
    LPSPI_MasterGetDefaultConfig(&masterConfig);
    masterConfig.baudRate                      = 500000U;
    masterConfig.bitsPerFrame                  = 8;
    masterConfig.cpol                          = kLPSPI_ClockPolarityActiveHigh;
    masterConfig.cpha                          = kLPSPI_ClockPhaseFirstEdge;
    masterConfig.direction                     = kLPSPI_MsbFirst;
    masterConfig.whichPcs                      = BOARD_LPSPI_PCS_FOR_INIT;
    masterConfig.pcsActiveHighOrLow            = kLPSPI_PcsActiveLow;
    masterConfig.pinCfg                        = kLPSPI_SdiInSdoOut;
    masterConfig.dataOutConfig                 = kLpspiDataOutTristate;
    masterConfig.pcsToSckDelayInNanoSec        = 1000000000 / masterConfig.baudRate;
    masterConfig.lastSckToPcsDelayInNanoSec    = 1000000000 / masterConfig.baudRate;
    masterConfig.betweenTransferDelayInNanoSec = 1000000000 / masterConfig.baudRate;
    masterConfig.enableInputDelay              = false;

    LPSPI_MasterInit(BOARD_EEPROM_LPSPI_BASEADDR, &masterConfig, BOARD_LPSPI_CLK_FREQ);
}

/* Low-level LPSPI blocking transfer wrapper */
static status_t NVSRAM_LpspiTransfer(uint8_t *txData,
                                     uint8_t *rxData,
                                     size_t dataSize)
{
    lpspi_transfer_t xfer;

    memset(&xfer, 0, sizeof(xfer));
    xfer.txData      = txData;
    xfer.rxData      = rxData;
    xfer.dataSize    = dataSize;
    xfer.configFlags = BOARD_LPSPI_PCS_FOR_TRANSFER | kLPSPI_MasterPcsContinuous;

    return LPSPI_MasterTransferBlocking(BOARD_EEPROM_LPSPI_BASEADDR, &xfer);
}

/* Send WREN (Write Enable) command */
static status_t NVSRAM_WriteEnable(void)
{
    uint8_t cmd = NVSRAM_CMD_WREN;

    return NVSRAM_LpspiTransfer(&cmd, NULL, 1u);
}

/* Read Status Register (RDSR) */
static status_t NVSRAM_ReadStatus(uint8_t *status)
{
    uint8_t tx[2] = { NVSRAM_CMD_RDSR, 0xFFu };
    uint8_t rx[2] = { 0 };

    status_t result = NVSRAM_LpspiTransfer(tx, rx, sizeof(tx));
    if (result == kStatus_Success)
    {
        *status = rx[1]; /* first received byte is dummy, second is status */
    }
    return result;
}

/* Write up to NVSRAM_MAX_XFER bytes to nvSRAM */
static status_t NVSRAM_WriteBytes(uint32_t address,
                                  const uint8_t *data,
                                  size_t length)
{
    status_t result;
    uint8_t  localBuf[4 + NVSRAM_MAX_XFER];

    if ((address + length) > NVSRAM_SIZE_BYTES)
    {
        return kStatus_InvalidArgument;
    }

    if (length > NVSRAM_MAX_XFER)
    {
        return kStatus_InvalidArgument;
    }

    /* Write Enable must precede any WRITE instruction */
    result = NVSRAM_WriteEnable();
    if (result != kStatus_Success)
    {
        return result;
    }

    /* Command + 3-byte address (A16..A0) */
    localBuf[0] = NVSRAM_CMD_WRITE;
    localBuf[1] = (uint8_t)((address >> 16) & 0x01u); /* A16 in bit0 */
    localBuf[2] = (uint8_t)((address >> 8) & 0xFFu);  /* A15..A8 */
    localBuf[3] = (uint8_t)(address & 0xFFu);         /* A7..A0 */

    memcpy(&localBuf[4], data, length);

    return NVSRAM_LpspiTransfer(localBuf, NULL, 4u + length);
}

/* Read up to NVSRAM_MAX_XFER bytes from nvSRAM */
static status_t NVSRAM_ReadBytes(uint32_t address,
                                 uint8_t *data,
                                 size_t length)
{
    status_t result;
    uint8_t  txBuf[4 + NVSRAM_MAX_XFER];
    uint8_t  rxBuf[4 + NVSRAM_MAX_XFER];

    if ((address + length) > NVSRAM_SIZE_BYTES)
    {
        return kStatus_InvalidArgument;
    }

    if (length > NVSRAM_MAX_XFER)
    {
        return kStatus_InvalidArgument;
    }

    /* Command + 3-byte address */
    txBuf[0] = NVSRAM_CMD_READ;
    txBuf[1] = (uint8_t)((address >> 16) & 0x01u);
    txBuf[2] = (uint8_t)((address >> 8) & 0xFFu);
    txBuf[3] = (uint8_t)(address & 0xFFu);

    memset(&txBuf[4], 0xFFu, length); /* dummy bytes to clock data out */

    result = NVSRAM_LpspiTransfer(txBuf, rxBuf, 4u + length);
    if (result == kStatus_Success)
    {
        memcpy(data, &rxBuf[4], length);
    }
    return result;
}

/* Simple additive checksum over wordCount 32-bit words */
static uint32_t simple_checksum32(const uint32_t *data, size_t wordCount)
{
    uint32_t sum = 0;
    size_t   i;

    for (i = 0; i < wordCount; i++)
    {
        sum += data[i];
    }
    return sum;
}

/* Generic pattern write/read test */
static void run_generic_pattern_test(void)
{
    status_t result;
    uint8_t  txPattern[16];
    uint8_t  rxPattern[16];
    uint32_t testAddress = 0x000010u;
    uint32_t i;

    PRINTF("\r\n[GENERIC] nvSRAM pattern test starting...\r\n");

    for (i = 0; i < sizeof(txPattern); i++)
    {
        txPattern[i] = (uint8_t)(i * 3u + 0x55u);
        rxPattern[i] = 0u;
    }

    PRINTF("[GENERIC] Writing %d bytes at address 0x%05X...\r\n",
           sizeof(txPattern), testAddress);
    result = NVSRAM_WriteBytes(testAddress, txPattern, sizeof(txPattern));
    if (result != kStatus_Success)
    {
        PRINTF("[GENERIC] Write failed, status = %d\r\n", result);
        return;
    }

    PRINTF("[GENERIC] Reading back...\r\n");
    result = NVSRAM_ReadBytes(testAddress, rxPattern, sizeof(rxPattern));
    if (result != kStatus_Success)
    {
        PRINTF("[GENERIC] Read failed, status = %d\r\n", result);
        return;
    }

    for (i = 0; i < sizeof(txPattern); i++)
    {
        if (txPattern[i] != rxPattern[i])
        {
            PRINTF("[GENERIC] MISMATCH at index %d: wrote 0x%02X, read 0x%02X\r\n",
                   i, txPattern[i], rxPattern[i]);
            PRINTF("[GENERIC] Pattern test FAILED.\r\n");
            return;
        }
    }

    PRINTF("[GENERIC] Pattern test PASSED.\r\n");
}

/* Avionics-style use case: store configuration/health snapshot */
static void run_avionics_use_case_demo(void)
{
    status_t          result;
    avionics_config_t cfg;
    avionics_config_t cfgReadback;
    uint32_t          baseAddress = 0x001000u; /* arbitrary offset in nvSRAM */

    PRINTF("\r\n[AVIONICS] Storing configuration snapshot into nvSRAM...\r\n");

    memset(&cfg, 0, sizeof(cfg));
    cfg.magic           = 0xA5A5A5A5u;
    cfg.firmwareVersion = 0x00010002u; /* v1.2 for example */
    cfg.flightHours     = 1234u;       /* pretend we accumulated 1234 hours */
    cfg.lastFaultCode   = 0x0042u;     /* pretend last fault code */
    cfg.reserved        = 0u;
    cfg.checksum        = simple_checksum32((const uint32_t *)&cfg,
                                            (sizeof(cfg) - sizeof(uint32_t)) / sizeof(uint32_t));

    /* Write struct */
    result = NVSRAM_WriteBytes(baseAddress, (const uint8_t *)&cfg, sizeof(cfg));
    if (result != kStatus_Success)
    {
        PRINTF("[AVIONICS] Write failed, status = %d\r\n", result);
        return;
    }

    /* Read back */
    memset(&cfgReadback, 0, sizeof(cfgReadback));
    result = NVSRAM_ReadBytes(baseAddress, (uint8_t *)&cfgReadback, sizeof(cfgReadback));
    if (result != kStatus_Success)
    {
        PRINTF("[AVIONICS] Read failed, status = %d\r\n", result);
        return;
    }

    /* Verify */
    if (cfgReadback.magic != 0xA5A5A5A5u)
    {
        PRINTF("[AVIONICS] Invalid magic, got 0x%08X\r\n", cfgReadback.magic);
        return;
    }

    {
        uint32_t expectedChecksum =
            simple_checksum32((const uint32_t *)&cfgReadback,
                              (sizeof(cfgReadback) - sizeof(uint32_t)) / sizeof(uint32_t));
        if (expectedChecksum != cfgReadback.checksum)
        {
            PRINTF("[AVIONICS] Checksum mismatch! expected 0x%08X, stored 0x%08X\r\n",
                   expectedChecksum, cfgReadback.checksum);
            return;
        }
    }

    PRINTF("[AVIONICS] Config snapshot verified OK.\r\n");
    PRINTF("[AVIONICS] firmwareVersion = 0x%08X, flightHours = %u, lastFault = 0x%04X\r\n",
           cfgReadback.firmwareVersion,
           cfgReadback.flightHours,
           cfgReadback.lastFaultCode);

    PRINTF("[AVIONICS] To test non-volatility, power-cycle the board and run this demo again.\r\n");
    PRINTF("          The same values should be read back from nvSRAM.\r\n");
}

/* Main entry */
int main(void)
{
    /* Init board hardware (clocks, pins, debug console, etc.). */
    BOARD_InitHardware();

    PRINTF("\r\n*** EVKB-IMXRT1050 + CY14B101Q2A-SXI nvSRAM Demo ***\r\n");

    LPSPI_Init();

    /* Run simple generic pattern test */
    run_generic_pattern_test();

    /* Run avionics-style use-case demo */
    run_avionics_use_case_demo();

    PRINTF("\r\nDemo complete. You can reset the board to repeat.\r\n");

    while (1)
    {
        __NOP();
    }
}

## 4. How to Run & Expected Output

- Power up the EVKB and ensure the nvSRAM wiring is correct.

- Start a debug session from MCUXpresso (using CMSIS-DAP).

- Open your terminal (or use the MCUXpresso console) at 115200 8-N-1.

- Reset/run the program.

You should see output similar to:

*** EVKB-IMXRT1050 + CY14B101Q2A-SXI nvSRAM Demo ***

[GENERIC] nvSRAM pattern test starting...
[GENERIC] Writing 16 bytes at address 0x00010...
[GENERIC] Reading back...
[GENERIC] Pattern test PASSED.

[AVIONICS] Storing configuration snapshot into nvSRAM...
[AVIONICS] Config snapshot verified OK.
[AVIONICS] firmwareVersion = 0x00010002, flightHours = 1234, lastFault = 0x0042

Demo complete. You can reset the board to repeat.

### Non-volatility demonstration

- After a successful run, power off the EVKB (switch off or unplug USB).

- Wait a few seconds.

- Power it on again and run the demo.

- The same firmwareVersion, flightHours, and lastFaultCode are read back, showing that nvSRAM preserved the data across the power cycle.

## 5. Extensions (with 4 boards)

With four EVKB boards and four nvSRAM devices, you can simulate more advanced avionics scenarios:

- Redundant channels

  - Each board represents an independent flight-control channel.

  - Each channel stores its own configuration/health record into its nvSRAM.

  - On boot, each channel validates the record (magic + checksum) and decides whether it is healthy.

- Flight counters and maintenance logging

  - On each simulated flight cycle, increment flightHours or add a flightCount field.

  - Store fault occurrences, last reset reason, or self-test results in nvSRAM.

- Configuration updates

  - Add a command interface (UART menu) to change configuration values.

  - After changing parameters, recompute checksum and write the updated struct to nvSRAM.

- STORE/RECALL and status register

  - Extend the driver with explicit STORE and RECALL commands.

  - Read the status register after STORE to confirm operation.

  - Compare behaviour with and without AutoStore enabled.