# Discrete I/O Driver Design Using Interrupt‑Driven I/O

## 1) Learning outcomes

By the end of this module, you should be able to:

1. Explain what a discrete I/O is, how it’s represented electrically, and why interrupt‑driven I/O is preferred over polling in most avionics contexts.

2. Configure i.MX RT1050 GPIO pins (IOMUXC + pad control) as inputs/outputs and enable edge/level interrupts using the MCUXpresso SDK (fsl_gpio.h).

3. Design and implement a small, reusable Discrete I/O (DIO) driver with: -

- Edge selection (rising/falling/both)

- Debounce/deglitch filtering using PIT

- Safe power‑up default states for outputs

- Event callbacks with timestamps and an event ring buffer.

4. Validate timing, latency, and fault behavior (shorts, bounce, wire‑off) with structured tests.

5. Apply avionics best practices under DO‑178C (Software Considerations in Airborne Systems and Equipment Certification), DO‑254 (Design Assurance Guidance for Airborne Electronic Hardware), ARP4754A (Guidelines for Development of Civil Aircraft and Systems), and ARP4761 (Safety Assessment) contexts.

## 2) Fundamentals

Discrete I/O:

- A binary signal represented by two valid electrical levels (e.g., 0/1, LOW/HIGH).

- In avionics, discretes often follow specific electrical standards (e.g., open‑collector with pull‑ups, 28V discrete with conditioning, opto‑isolated modules).

- On the EVKB we use the microcontroller’s direct GPIO (General‑Purpose Input/Output) pins for learning; in an LRU these would usually be behind input protection and level conditioning.

Interrupt‑driven I/O:

- The microcontroller configures a peripheral (GPIO) to assert an interrupt when a selected event occurs (edge/level).

- The CPU services the event in a short ISR (Interrupt Service Routine) instead of continuously polling.

- Benefits: low latency, low CPU load, time‑stamped events, and deterministic response.

Debounce / deglitch:

- Real mechanical switches bounce and wiring can pick noise; without filtering, the ISR may fire many times for one logical transition.

- Debounce ensures the input is stable for a minimum time (e.g., 10–20 ms) before it is accepted.

Safe state:

- The output level the system must assume at power‑up and during faults to avoid unsafe actuation (e.g., keep a valve closed, keep a relay de‑energized).

- Always design outputs to a known Fail‑Safe or Fail‑Operational strategy depending on the system’s FHA (Functional Hazard Assessment).

Latency:

- Time from the external event to software reaction.

- It includes pad/port latency, interrupt latency, ISR length, and any debounce delay.

- Typicallhy, raw latency to ISR entry is very small (µs‑scale) but application design (filtering, printing) dominates perceived response.

## 3) Hardware and SDK orientation (EVKB‑i.MXRT1050)

- User Button (SW8): Routed to GPIO5, pin 0. Interrupt line: GPIO5_Combined_0_15_IRQn. Board alias in SDK: BOARD_USER_BUTTON_GPIO / BOARD_USER_BUTTON_GPIO_PIN / BOARD_USER_BUTTON_IRQ / BOARD_USER_BUTTON_IRQ_HANDLER.

- User LED (D9): Routed to GPIO1, pin 9. Board alias in SDK: BOARD_USER_LED_GPIO / BOARD_USER_LED_GPIO_PIN. Logical on/off are inverted by hardware (LED on when the GPIO drives low in the EVKB reference design); board macros handle this.

- We will use MCUXpresso SDK drivers shipped with your SDK zip:

  - fsl_gpio.h for GPIO,

  - fsl_iomuxc.h for pin mux/pad control (usually wrapped by generated pin_mux.c),

  - fsl_pit.h for debounce timing,

  - fsl_debug_console.h for UART prints.

- Board init: Use BOARD_InitHardware() from NXP examples to bring up clocks, pins, and console.

We deliberately base all code on the provided EVKB SDK code base so you can drop files into a stock MCUXpresso SDK project and build.

## 4) Driver design overview

We implement a small DIO driver with these responsibilities:

- Describe each discrete channel (GPIO base, pin number, active level, pull configuration, interrupt mode, debounce time).

- Initialize the hardware (pin mux and pad config assumed provided by BOARD_InitHardware() or your pin_mux.c).

- Set up GPIO input with edge‑triggered interrupts and a PIT‑based debounce timer.

- Provide a callback interface to deliver debounced events (pin, new logical state, timestamp, reason).

- Provide output APIs with safe default levels and atomic updates.

- Handle ISR hygiene: fast entry/exit, flag clear, minimal work in ISR, SDK_ISR_EXIT_BARRIER.

### Interrupt model on i.MX RT1050

- Each GPIO port exposes combined interrupt lines (e.g., GPIO5_Combined_0_15_IRQn for pins 0..15 and another for 16..31). The ISR must read/clear the port’s flags and service any matching pins. We illustrate with GPIO5[0] (SW8) but the code supports many pins.

## 5) Implementation (SDK‑based, bare‑metal)

Files you add: dio.h, dio.c, and a sample main.c (or integrate into your board example). All APIs and includes are straight from the SDK, so the code compiles inside a standard EVKB project.

### 5.1 dio.h

#ifndef DIO_H
#define DIO_H

#include <stdint.h>
#include <stdbool.h>
#include "fsl_gpio.h"
#include "fsl_pit.h"

#ifdef __cplusplus
extern "C" {
#endif

// Logical level definition (post‑polarity)
typedef enum { DIO_LEVEL_LOW = 0, DIO_LEVEL_HIGH = 1 } dio_level_t;

typedef enum {
    DIO_EDGE_NONE = 0,
    DIO_EDGE_RISING,
    DIO_EDGE_FALLING,
    DIO_EDGE_BOTH
} dio_edge_t;

// Forward decl for callback context
struct dio_channel_s;

typedef void (*dio_callback_t)(struct dio_channel_s *ch, dio_level_t new_level, uint32_t timestamp_us);

typedef struct dio_channel_s {
    GPIO_Type *base;            // e.g., GPIO5
    uint32_t   pin;             // e.g., 0
    bool       active_high;     // false if active low (maps pad level to logical HIGH)
    dio_edge_t edge;            // which edge(s) generate events
    uint32_t   debounce_us;     // debounce window in microseconds
    dio_callback_t cb;          // user callback (debounced event)
    // internal
    volatile bool irq_armed;    // ISR arms a debounce check
    volatile uint32_t t_irq_us; // time of last IRQ edge
    volatile dio_level_t last_level; // last debounced level
} dio_channel_t;

// Global init for PIT timebase (1 MHz tick) and NVIC, call once at startup.
void dio_timebase_init(void);
uint32_t dio_timebase_get_us(void);

// Initialize a discrete input channel with interrupts+debounce.
void dio_input_init(dio_channel_t *ch);

// Configure an output channel and drive safe level at startup.
void dio_output_init(GPIO_Type *base, uint32_t pin, bool active_high, dio_level_t safe_level);
void dio_write(GPIO_Type *base, uint32_t pin, bool active_high, dio_level_t level);
dio_level_t dio_read(GPIO_Type *base, uint32_t pin, bool active_high);

#ifdef __cplusplus
}
#endif

#endif // DIO_H

### 5.2 dio.c

#include "dio.h"
#include "fsl_common.h"
#include "fsl_debug_console.h"

// === Timebase: PIT channel 0 at 1 MHz (1 us tick) ===
#define DIO_PIT            PIT
#define DIO_PIT_CH         kPIT_Chnl_0
#define DIO_PIT_IRQn       PIT_IRQn
#define DIO_PIT_IRQHandler PIT_IRQHandler

static volatile uint32_t s_time_us = 0;

void dio_timebase_init(void)
{
    pit_config_t cfg;
    PIT_GetDefaultConfig(&cfg);
    PIT_Init(DIO_PIT, &cfg);
    // Run free‑running at 1 MHz (period = 1 us). We reload every 1 us and count in ISR.
    uint32_t src = CLOCK_GetFreq(kCLOCK_OscClk);
    PIT_SetTimerPeriod(DIO_PIT, DIO_PIT_CH, USEC_TO_COUNT(1U, src));
    PIT_EnableInterrupts(DIO_PIT, DIO_PIT_CH, kPIT_TimerInterruptEnable);
    EnableIRQ(DIO_PIT_IRQn);
    PIT_StartTimer(DIO_PIT, DIO_PIT_CH);
}

uint32_t dio_timebase_get_us(void) { return s_time_us; }

void DIO_PIT_IRQHandler(void)
{
    PIT_ClearStatusFlags(DIO_PIT, DIO_PIT_CH, kPIT_TimerFlag);
    s_time_us++;
    SDK_ISR_EXIT_BARRIER; // Prevent spurious re‑entry on fast cores
}

// === Helpers ===
static gpio_interrupt_mode_t map_edge(dio_edge_t e)
{
    switch (e) {
    case DIO_EDGE_RISING:  return kGPIO_IntRisingEdge;
    case DIO_EDGE_FALLING: return kGPIO_IntFallingEdge;
    case DIO_EDGE_BOTH:    return kGPIO_IntRisingOrFallingEdge;
    default:               return kGPIO_NoIntmode;
    }
}

static inline dio_level_t level_from_pad(bool pad_high, bool active_high)
{
    bool logical = active_high ? pad_high : !pad_high;
    return logical ? DIO_LEVEL_HIGH : DIO_LEVEL_LOW;
}

// === Input init (interrupt + debounce) ===
void dio_input_init(dio_channel_t *ch)
{
    // Configure pin as digital input, interrupt per edge setting
    gpio_pin_config_t cfg = {kGPIO_DigitalInput, 0, map_edge(ch->edge)};
    GPIO_PinInit(ch->base, ch->pin, &cfg);

    // Read initial pad and set debounced level
    ch->last_level = level_from_pad(GPIO_PinRead(ch->base, ch->pin), ch->active_high);
    ch->irq_armed = false;

    // Enable per‑pin interrupt at port level
    GPIO_PortEnableInterrupts(ch->base, (1U << ch->pin));
}

// === Output API ===
void dio_output_init(GPIO_Type *base, uint32_t pin, bool active_high, dio_level_t safe_level)
{
    gpio_pin_config_t cfg = {kGPIO_DigitalOutput, 0, kGPIO_NoIntmode};
    GPIO_PinInit(base, pin, &cfg);
    // Drive safe state
    bool pad_high = active_high ? (safe_level == DIO_LEVEL_HIGH) : (safe_level == DIO_LEVEL_LOW);
    if (pad_high) {
        GPIO_PortSet(base, (1U << pin));
    } else {
        GPIO_PortClear(base, (1U << pin));
    }
}

void dio_write(GPIO_Type *base, uint32_t pin, bool active_high, dio_level_t level)
{
    bool pad_high = active_high ? (level == DIO_LEVEL_HIGH) : (level == DIO_LEVEL_LOW);
    if (pad_high) {
        GPIO_PortSet(base, (1U << pin));
    } else {
        GPIO_PortClear(base, (1U << pin));
    }
}

dio_level_t dio_read(GPIO_Type *base, uint32_t pin, bool active_high)
{
    return level_from_pad(GPIO_PinRead(base, pin), active_high);
}

// === GPIO5 combined ISR example ===
// In your app, install this handler under the board’s IRQ name for GPIO5 0..15.
extern dio_channel_t g_sw8; // Declared in main.c for the demo

void GPIO5_Combined_0_15_IRQHandler(void)
{
    uint32_t flags = GPIO_PortGetInterruptFlags(GPIO5);
    // Service SW8 (GPIO5 pin 0) if set
    if (flags & (1U << 0)) {
        // Clear first to avoid retrigger
        GPIO_PortClearInterruptFlags(GPIO5, (1U << 0));

        // Arm debounce window for the demo channel
        if (g_sw8.debounce_us == 0) {
            // No debounce: emit immediately
            bool pad = GPIO_PinRead(g_sw8.base, g_sw8.pin);
            dio_level_t lvl = level_from_pad(pad, g_sw8.active_high);
            if (lvl != g_sw8.last_level) {
                g_sw8.last_level = lvl;
                if (g_sw8.cb) g_sw8.cb(&g_sw8, lvl, dio_timebase_get_us());
            }
        } else {
            g_sw8.irq_armed = true;
            g_sw8.t_irq_us = dio_timebase_get_us();
        }
    }

    SDK_ISR_EXIT_BARRIER;
}

// === Debounce check: called from your main loop ===
// This keeps ISR very short. The main loop periodically calls this helper.
void dio_poll_debounce(dio_channel_t *ch)
{
    if (!ch->irq_armed) return;
    uint32_t now = dio_timebase_get_us();
    if ((now - ch->t_irq_us) >= ch->debounce_us) {
        ch->irq_armed = false;
        bool pad = GPIO_PinRead(ch->base, ch->pin);
        dio_level_t lvl = level_from_pad(pad, ch->active_high);
        if (lvl != ch->last_level) {
            ch->last_level = lvl;
            if (ch->cb) ch->cb(ch, lvl, now);
        }
    }
}

### 5.3 main.c — Generic demo (SW8 interrupt toggles LED with debounce)

#include "fsl_debug_console.h"
#include "board.h"
#include "app.h"       // For BOARD_* aliases from SDK examples
#include "dio.h"

// Demo channel: EVKB SW8 on GPIO5[0], active high, both edges, 20 ms debounce
static void sw8_callback(dio_channel_t *ch, dio_level_t new_level, uint32_t t_us)
{
    PRINTF("[SW8] %s at %u us\r\n", new_level == DIO_LEVEL_HIGH ? "HIGH" : "LOW", (unsigned)t_us);
    // Example action: toggle LED on a rising edge only
    if (new_level == DIO_LEVEL_HIGH) {
        USER_LED_TOGGLE();
    }
}

dio_channel_t g_sw8 = {
    .base = BOARD_USER_BUTTON_GPIO,       // GPIO5
    .pin  = BOARD_USER_BUTTON_GPIO_PIN,   // 0
    .active_high = true,                  // SW8 reads 1 when pressed
    .edge = DIO_EDGE_BOTH,
    .debounce_us = 20000,                 // 20 ms
    .cb = sw8_callback,
};

int main(void)
{
    BOARD_InitHardware();     // clocks, pins, debug console
    dio_timebase_init();      // 1 MHz timebase

    LED_INIT();               // Board macro drives known safe state for LED

    // Configure SW8 interrupt input
    dio_input_init(&g_sw8);

    // Enable NVIC line for the user button (from board/app.h)
    NVIC_SetPriority(BOARD_USER_BUTTON_IRQ, 5); // Medium priority
    EnableIRQ(BOARD_USER_BUTTON_IRQ);

    PRINTF("Discrete I/O driver demo: SW8 -> LED (debounced)\r\n");

    while (1) {
        // Periodically service debounce (keeps ISR minimal)
        dio_poll_debounce(&g_sw8);
        // Do other work...
    }
}

Notes: -

- This pattern keeps the GPIO ISR extremely short: it clears the flag and arms a debounce window.

- The main loop performs the stable‑state check and calls the user callback.

- In an RTOS this would be a deferred ISR (D‑ISR) via queue; in bare‑metal we poll a small flag. - If an input requires no debounce, set debounce_us = 0 and the ISR will invoke the callback immediately.

- For multiple inputs on the same port, extend the ISR to iterate flags and maintain one dio_channel_t per pin.

## 6) Generic example explained

Scenario: A maintenance technician presses the front‑panel pushbutton SW8 to cycle a function. We need reliable event detection without multiple triggers from switch bounce, with a visible LED action and a UART log line.

Walk‑through:

1. BOARD_InitHardware() initializes clocks, IOMUXC, and the UART console (115200 bps by default). The LED macro sets a safe power‑up state.

2. dio_timebase_init() starts the PIT at 1 MHz for microsecond timestamps and debounce windows.

3. dio_input_init(&g_sw8) configures GPIO5[0] as digital input with both‑edge interrupts. The port’s combined IRQ is enabled via EnableIRQ(BOARD_USER_BUTTON_IRQ).

4. On any edge, GPIO5_Combined_0_15_IRQHandler() clears the flag and arms a debounce check (g_sw8.irq_armed = true;).

5. The main loop calls dio_poll_debounce(), which after 20 ms re‑reads the pad and, if changed, invokes sw8_callback() with a timestamp.

Why this matters: The ISR path is deterministic and short; the debounce is time‑based and explicit; the output (LED) demonstrates actuation under control of a debounced discrete.

## 7) Avionics use‑case: Weight‑On‑Wheels (WOW) discrete

Context:

- WOW (Weight‑On‑Wheels) informs many systems (thrust‑reverser logic, ground spoilers, configuration warnings).

- Spurious transitions are safety‑relevant.

- Assume an LRU monitors a conditioned WOW discrete presented as a 3.3 V logic‑level input to the MCU.

Requirements: -

- Latch transitions with timestamps.

- Minimum stable time 50 ms (air/ground logic usually slow‑changing).

- On multiple transitions within 10 ms, flag a FAULT (wiring noise) and ignore until stable for 100 ms.

- Provide a safe default at power‑up: treat as AIR (not on wheels) until a stable ground is seen.

Implementation sketch with our driver: -

- Configure a dio_channel_t for the WOW input: active‑high, DIO_EDGE_BOTH, debounce 50 ms.

- In the callback, implement a small state machine: if the time since the previous event < 10 ms, set FAULT and inhibit further processing until 100 ms stable.

- Log each accepted transition with a timestamp from dio_timebase_get_us(); this provides traceability for maintenance and certification artifacts.

Why interrupt‑driven:

- Polling at (say) 1 ms either wastes CPU or still misses short spikes.

- Interrupts give immediate capture and then software decides what is valid.

## 8) Advanced topics

### 8.1 Edge vs. level interrupts

- Edge: Best for event capture; requires software to read the current level after debounce.

- Level: Can re‑enter ISR until the level changes; only use when you must hold service until the condition clears.

### 8.2 NVIC priorities

- Assign GPIO interrupt priorities lower than hard real‑time buses (e.g., DMA complete) but higher than background.

- Example: set to priority 5 (where 0 is highest on Cortex‑M).

- Never print from ISR at high rates.

### 8.3 Concurrency & memory ordering

- Shared flags between ISR and main must be volatile.

- Keep ISRs short; defer work.

- Use SDK_ISR_EXIT_BARRIER in NXP SDK to avoid re‑entry on fast core/slow bus corner cases.

### 8.4 Input protection in real LRUs

- Real aircraft discretes are often 28 V and come through protection (TVS, dividers, opto).

- Software should assume inversion may occur; that’s why our API has active_high per channel.

### 8.5 Time‑stamping alternatives

- Instead of PIT, you may enable the DWT (Data Watchpoint and Trace) cycle counter for sub‑microsecond timestamps.

- PIT is used here because it’s an SDK‑supported peripheral already present in your SDK tree.

## 9) Best practices (avionics‑oriented)

- Define electrical assumptions in software: pull‑ups, active level, safe level. Keep them in one place (channel table) for easy system safety review.

- Initialize outputs to safe states early (before enabling interrupts).

- Never allocate in ISR / never block: no malloc, no printf loops. Defer using flags or ring buffers.

- Debounce every human‑operated input; log raw vs. debounced if safety assessment requires.

- Bound ISR time and prove it: code review + measurement.

- Fault management: detect oscillation, stuck‑at, wire‑off via plausibility checks and timeouts.

- Configurability: keep debounce and polarity in non‑volatile configuration if allowed by your development assurance level and change control.

- Traceability: link each requirement (e.g., “WOW debounce 50 ms”) to code lines and tests for DO‑178C objectives.

- Deterministic builds: lock SDK version and toolchain; include generated pin_mux.c/h in configuration control.

## 10) Hands‑on exercises (with solutions)

### Exercise A — Basic interrupt input toggles LED

Goal: On SW8 press, toggle the user LED exactly once per press (no bounce).

Steps:

1. Create a new MCUXpresso SDK project for evkbimxrt1050, bare‑metal.
2. Add dio.h/.c and main.c above.
3. Build and run; observe UART prints and LED toggles once per press.

Solution:

Use the provided main.c with debounce_us=20000 and callback toggling LED on DIO_LEVEL_HIGH.

Observations: If you set debounce_us=0, you will see multiple prints per press due to bounce.

### Exercise B — Both‑edge event logger with timestamps

Goal: Log both edges of SW8 with microsecond timestamps and count events.

Modify: In sw8_callback() maintain counters of rising/falling edges, print the counts every 10 events. Keep debounce at 10–20 ms.

Solution snippet:

static uint32_t rises=0, falls=0;
static void sw8_callback(dio_channel_t *ch, dio_level_t new_level, uint32_t t_us)
{
    if (new_level == DIO_LEVEL_HIGH) rises++; else falls++;
    PRINTF("SW8 %s @%u us (R:%u F:%u)\r\n", new_level?"HIGH":"LOW", (unsigned)t_us, rises, falls);
}

### Exercise C — WOW use‑case filter

Goal: Implement the WOW fault rules from Section 7: ignore transitions occurring <10 ms apart, set FAULT, and require 100 ms stable before clearing FAULT.

Hints: Keep static state inside the callback: last_time_us, in_fault, fault_cleared_at_us. Drive an LED ON while in FAULT.

Solution sketch:

static bool in_fault = false;
static uint32_t last_event_us = 0, fault_exit_earliest_us = 0;
static void wow_callback(dio_channel_t *ch, dio_level_t new_level, uint32_t t_us)
{
    if (in_fault) {
        if (t_us >= fault_exit_earliest_us) {
            in_fault = false; USER_LED_OFF(); PRINTF("WOW: FAULT CLEARED\r\n");
        } else {
            return; // Still in fault holdoff
        }
    }
    if ((t_us - last_event_us) < 10000) { // <10 ms
        in_fault = true; USER_LED_ON();
        fault_exit_earliest_us = t_us + 100000; // 100 ms stable window
        PRINTF("WOW: FAULT due to chatter\r\n");
        return;
    }
    last_event_us = t_us;
    PRINTF("WOW: %s\r\n", new_level==DIO_LEVEL_HIGH?"GROUND":"AIR");
}

### Exercise D — Multi‑channel inputs on one port

Goal: Add two more inputs on GPIO5 pins (e.g., pins 1 and 2 if available on your carrier/breadboard) and verify the ISR correctly services multiple flags in one combined interrupt.

Solution approach: Maintain an array of dio_channel_t* for the serviced port; in the ISR, iterate flags bits and arm the corresponding channel’s debounce. This pattern scales to dozens of discrete inputs while using only two NVIC lines per port.

## 11) Testing & verification procedure

- Functional: Press/release SW8 ten times; confirm exactly ten rising‑edge logs and ten LED toggles.

- Bounce characterization: Temporarily set debounce_us=0 and observe multiple ISR firings. Then increase to 5 ms, 10 ms, 20 ms and measure prints per press. Select the minimum value that guarantees a single event on your hardware.

- Latency: With debounce_us=0, connect a logic analyzer to SW8 and LED pin; measure time from edge to LED toggle (µs scale).

- Fault injection: Simulate chatter by rapidly tapping; confirm Exercise C fault logic sets/clears FAULT with the specified timings.

- Power‑up safety: Ensure LED (acting as an actuator proxy) powers up in the declared safe state before any interrupts are enabled.

## 12) Design review checklist

- Inputs

  - For each input: polarity, pull‑up/down, debounce time, safety impact documented.

  - Edge selection justified (rising/falling/both).

  - Debounce deferred out of ISR; ISR clears flags and exits quickly.

- Outputs

  - Safe default for each output documented and enforced before ISR enable.

  - Writes are atomic (read‑modify‑write avoided on shared registers or appropriately masked).

- Timing

  - Interrupt priority consistent with system criticality.

  - Worst‑case ISR time bounded; no prints in ISR.

- Faults

  - Chatter/wire‑off/stuck‑at strategies implemented where required.

  - Event logging with timestamp for maintenance tracing.

- Process

  - Code style and naming aligned with project standards.

  - SDK version locked; pin mux generated files under configuration control.

## 13) Where this leads to

- Output drivers with PWM/hold‑off and interlocks.

- Self‑test (BIT — Built‑In Test) of discrete inputs using internal pull‑ups and loopbacks.

## 14) Appendix: Porting notes

- If you generate pin_mux.c/h with MCUXpresso ConfigTools, keep the signal names consistent and call BOARD_InitPins() early in main() or inside BOARD_InitHardware().

- If you change the timebase, keep the debounce math in microseconds and adjust dio_timebase_get_us() accordingly.

- For higher event rates, replace the PRINTF in callbacks with a ring buffer and a background UART task to meet timing.