# Discrete I/O Driver Design on i.MX RT1050‑EVKB — Integrating I/O into a Bare‑Metal Scheduler

## Learning goals

By the end of this module you should  be able to: -

- Explain what “discrete I/O” means in avionics and why timing/robustness matter. - Configure i.MX RT1050 pads and GPIO blocks (IOMUXC + GPIO) for deterministic discrete inputs/outputs. –

- Build a minimal, deterministic bare‑metal scheduler driven by SysTick and integrate periodic and event‑driven I/O tasks. –

- Implement production‑grade debouncing, edge/level detection, latching, and event queuing. –

- Produce safe, fail‑silent outputs with known power‑up states and interlocks. –

- (Extension) Publish discrete state over UART to an external ARINC 429 adapter (ADK‑8582) as a stepping stone toward label generation.

Hardware & SDK used throughout
• NXP i.MX RT1050‑EVKB MCU board (Cortex‑M7)
• NXP MCUXpresso SDK for EVKB‑i.MXRT1050 (provided)
• On‑board USER LED (via BOARD_USER_LED_* macros) and USER BUTTON SW8 (via BOARD_USER_BUTTON_* macros).
• For the ARINC 429 extension: EVKB‑i.MXRT1050 connected to ADK‑8582 via LPUART1 (3‑wire UART).

## 1. Fundamentals

### 1.1 What is discrete I/O?

Discrete input: a binary signal representing the logical state of an external condition (e.g., door closed, landing gear down, weight‑on‑wheels).
Discrete output: a binary drive provided by the MCU to signal or command external hardware (e.g., lamp on, relay coil drive, inhibit line).

In avionics, discretes are often active‑low, electrically conditioned off‑board, and safety‑relevant. Software must account for polarity, pull‑ups, filtering, and deterministic timing.

### 1.2 Determinism & certification vocabulary

- LRU (Line Replaceable Unit): field‑replaceable avionics box.

- CDD (Component Development Data) and SRS (Software Requirements Specification): artifacts under DO‑178C (Software Considerations in Airborne Systems and Equipment Certification).

- DAL (Design Assurance Level): A through E criticality classification.
We are not implementing a certified design here; however, we adopt patterns (deterministic timing, traceability, defensive coding) that align with DO‑178C expectations.

### 1.3 i.MX RT1050 pin path overview

- IOMUXC (I/O Multiplexer Controller) selects the peripheral function for a physical pad and sets pad electrical properties (pull‑up/down, keeper, hysteresis, drive strength, slew rate).

- GPIO peripheral provides direction control, data registers, and interrupts (edge or level) per pin.
Implication: Correct pad config is as important as the GPIO mode. A “floating” input without pull/keeper will cause chattering and false edges.

## 2. Architecture: Discrete I/O driver inside a bare‑metal system

### 2.1 Layered view

- HAL (Hardware Abstraction Layer): IOMUXC pad config + GPIO init (SDK drivers).

- Driver (DISCRETE): Debounce, polarity, edge/level detect, latching, timestamps, event queue.

- Scheduler: Runs periodic tasks (e.g., every 5 ms for inputs, 10 ms for outputs) and dispatches events from ISRs.

- Application: Implements system logic and avionics use cases.

### 2.2 Scheduler requirements for I/O

- Stable sampling period (e.g., 5 ms for de‑bounce integrators).

- Bounded ISR time: ISRs raise events or snapshot timestamps; heavy work runs in tasks.

- Monotonic timebase: SysTick at 1 ms is adequate for most cockpit‑rate discretes.

## 3. Configuring pads and GPIO (SDK‑centric)

We always use the SDK’s fsl_iomuxc.h and fsl_gpio.h plus the board layer macros. Typical steps: 1. Enable pads & mux with IOMUXC_SetPinMux(PAD, 0U); and set pad electricals with IOMUXC_SetPinConfig(PAD, configVal);
2. Initialize GPIO with gpio_pin_config_t and GPIO_PinInit() (direction, default logic, interrupt mode).
3. For inputs, prefer keeper/pull and hysteresis enabled; for outputs, set drive strength and slew appropriately.

Board convenience macros:
- BOARD_USER_LED_GPIO, BOARD_USER_LED_GPIO_PIN (LED on GPIO1_IO09; active‑low on EVKB).
- BOARD_USER_BUTTON_GPIO, BOARD_USER_BUTTON_GPIO_PIN, BOARD_USER_BUTTON_IRQ (SW8 on GPIO5_IO00; interrupt on GPIO5_Combined_0_15_IRQn).
Use these macros instead of hard‑coding pins to survive board revisions.

## 4. Bare‑metal scheduler design

We implement a cooperative, tick‑driven scheduler using SysTick at 1 ms.

- Tick source: SysTick_Config(SystemCoreClock / 1000U);

- Task table: each task has a period, next‑release time, and a function pointer.

- Determinism: no blocking inside tasks; each task finishes within its budget.

Task classes used here:

- DiscreteIn_Task_5ms(): samples & debounces inputs.
- DiscreteOut_Task_10ms(): applies requested outputs, handles lamp‑test overrides.
- App_Task_10ms(): avionics logic.
- EventPump_Task_1ms(): empties the ISR‑to‑task event queue.

## 5. Discrete input driver design

### 5.1 Features

- Configurable polarity (active‑low or active‑high).

- Debounce integrator (up/down saturating counter) with a settable time constant in samples.

- Edge detect with millisecond timestamps (rise/fall).

- Latching and sticky faults (until explicitly cleared).

- ISR support for hardware edges, but work deferred to the 5 ms task.

### 5.2 Debounce algorithm (integrator)

For each input we maintain an unsigned counter. On each sample: - If raw signal == TRUE, counter++ up to COUNT_MAX; else counter– down to 0.
- Debounced state = TRUE when counter >= THRESHOLD; FALSE when counter == 0.
This avoids time‑window race conditions of “N consecutive samples.”

### 5.3 Event model

- EVENT_RISE / EVENT_FALL: Posted when debounced state changes.

- EVENT_GLITCH (optional): Posted when the ISR detects an edge but the debounced state doesn’t change within a window (diagnostic).

## 6. Discrete output driver design

### 6.1 Features

- Polarity: active‑low or active‑high mapping.

- Failsafe defaults at power‑up (e.g., de‑energize relays).

- Interlocks: optional requires‑true discretes before enabling (e.g., weight‑on‑wheels before deploy).

- Lamp‑test: global override forcing outputs on for ground checks.

### 6.2 Timing

Outputs are requested by the application at any time; the 10 ms output task applies them in a deterministic batch, minimizing pin wiggle and accounting for interlocks.

## 7. Generic example (board‑level): Button toggles LED with robust debouncing

Goal: SW8 toggles USER LED. LED is active‑low, so OFF=logic ‘1’, ON=logic ‘0’. We implement: - 5 ms debounce, edge detection, event queue.
- Press = toggle LED state; long press (>1 s) = lamp‑test mode.

## 8. Avionics use‑case examples

### 8.1 Weight‑On‑Wheels (WOW) discrete consolidation

- Context: Two independent WOW microswitches feed a Single Board Computer (this EVKB) that shares a consolidated WOW status with adjacent LRUs (Line Replaceable Units).

- Requirement: assert WOW_TRUE if both channels are TRUE for ≥40 ms; de‑assert on either channel FALSE for ≥40 ms. Timestamp each transition.

- Usage: Application inhibits thrust‑reverser test in flight, and sends WOW status to a 429 transmitter LRU.

### 8.2 Fire handle lamp drive & latch

- Context: An Engine Fire Handle discrete output should illuminate when EICAS (Engine Indication and Crew Alerting System) signals FIRE, and remain latched until maintenance clears it, unless the lamp‑test override is active.

- Requirement: enforce safe default (lamp off) at power‑up; respect lamp‑test.

### 8.3 Doors closed monitoring with disagree detection

- Context: Two “Door Closed” discretes from independent sensors.

- Requirement: present a single “Doors Closed” discrete to another LRU; raise a sticky fault if sensors disagree for more than 500 ms.

Each example maps cleanly onto our input/output drivers and scheduler periods.

## 9. Best practices (summary)

- Configure pads, not just GPIO: enable keeper/pull and hysteresis for inputs; set drive and slew for outputs.

- Deterministic timing: periodic sampling (5 ms) and bounded ISR time.

- Polarity & defaults: encode at the driver boundary; never scatter active‑low knowledge across the app.

- Debounce with integrators: avoids edge races and handles noise.

- Separate request vs. apply for outputs to allow atomic, rate‑limited updates.

- Event queues crossing ISR→task boundaries; never do heavy work in ISRs.

- Self‑test & lamp‑test hooks wired through the driver, not ad‑hoc.

- Measure: verify periods and jitter with GPIO scope pins and logic analyzer.

- Document assumptions: voltage levels, external pull‑ups, active polarity, time constants, safety defaults.

## 10. Hands‑on Exercises (with SDK‑based solutions)

Project tip: Start from boards/evkbimxrt1050/driver_examples/gpio/led_output in the SDK. Replace the main.c with the files below or add new source files to a clean MCUXpresso project that already includes board.c/.h, pin_mux.c/.h, and clock_config.c/.h.

### Exercise 1 — Minimal bare‑metal scheduler & precise LED blink

Task: Implement a 1 ms SysTick, schedule a 100 ms LED toggle task; measure blink period accuracy.
Acceptance: Average period 200 ms (on/off), jitter < ±2 ms when idle.

Solution (files)

#### scheduler.h

#ifndef SCHEDULER_H
#define SCHEDULER_H
#include <stdint.h>

typedef void (*task_fn_t)(void);

typedef struct {
    task_fn_t fn;
    uint32_t period_ms;
    uint32_t next_ms;
} sched_task_t;

void SCHED_Init(uint32_t tick_hz);
void SCHED_AddTask(task_fn_t fn, uint32_t period_ms);
void SCHED_RunOnce(void);
uint32_t SCHED_Millis(void);

#endif

#### scheduler.c

#include "scheduler.h"
#include "fsl_common.h"
#include "fsl_gpio.h"
#include <string.h>

#define MAX_TASKS 8
static volatile uint32_t g_ms;
static sched_task_t g_tasks[MAX_TASKS];
static uint32_t g_task_count;

void SysTick_Handler(void) { g_ms++; }

void SCHED_Init(uint32_t tick_hz)
{
    g_ms = 0; g_task_count = 0;
    (void)SysTick_Config(SystemCoreClock / tick_hz);
}

uint32_t SCHED_Millis(void) { return g_ms; }

void SCHED_AddTask(task_fn_t fn, uint32_t period_ms)
{
    if (g_task_count < MAX_TASKS) {
        g_tasks[g_task_count].fn = fn;
        g_tasks[g_task_count].period_ms = period_ms;
        g_tasks[g_task_count].next_ms = SCHED_Millis() + period_ms;
        g_task_count++;
    }
}

void SCHED_RunOnce(void)
{
    uint32_t now = SCHED_Millis();
    for (uint32_t i = 0; i < g_task_count; i++) {
        if ((int32_t)(now - g_tasks[i].next_ms) >= 0) {
            g_tasks[i].fn();
            g_tasks[i].next_ms += g_tasks[i].period_ms; // catch‑up, fixed‑period
        }
    }
}

#### main.c (Exercise 1)

#include "board.h"
#include "pin_mux.h"
#include "clock_config.h"
#include "fsl_gpio.h"
#include "scheduler.h"

static void led_task_100ms(void)
{
    // USER_LED_TOGGLE uses board macros (active‑low LED)
    USER_LED_TOGGLE();
}

int main(void)
{
    BOARD_InitBootPins();
    BOARD_InitBootClocks();
    BOARD_InitDebugConsole();

    // LED pin is already configured by board/pin_mux in the SDK example
    USER_LED_OFF(); // set to a known state

    SCHED_Init(1000U); // 1 ms tick
    SCHED_AddTask(led_task_100ms, 100U);

    while (1) {
        SCHED_RunOnce();
        __NOP();
    }
}

### Exercise 2 — Discrete input driver with debouncing and event queue (SW8 → LED)

Task: Sample SW8 every 5 ms, debounce with an integrator, post PRESS/RELEASE events; toggle LED on PRESS. Hold >1 s to enable a lamp‑test state that forces LED on.

Solution (files)

#### discrete.h

#ifndef DISCRETE_H
#define DISCRETE_H
#include <stdbool.h>
#include <stdint.h>

typedef enum { DISCRETE_ACTIVE_LOW=0, DISCRETE_ACTIVE_HIGH=1 } discrete_polarity_t;

typedef struct {
    GPIO_Type *base;
    uint32_t pin;
    discrete_polarity_t pol;
    uint8_t count;      // integrator
    uint8_t thresh;     // threshold to declare TRUE
    uint8_t count_max;  // saturation
    bool debounced;
    bool latched;       // optional sticky
    uint32_t last_change_ms;
} discrete_in_t;

void DISCRETE_IN_Init(discrete_in_t *d, GPIO_Type *base, uint32_t pin,
                      discrete_polarity_t pol, uint8_t thresh, uint8_t count_max);
void DISCRETE_IN_Sample(discrete_in_t *d, uint32_t now_ms);
bool DISCRETE_IN_Get(const discrete_in_t *d);

// Simple lock‑free ring buffer for ISR→task events
typedef enum { EV_NONE=0, EV_PRESS, EV_RELEASE } io_event_t;
void EV_Post(io_event_t ev, uint32_t t_ms);
bool EV_Pop(io_event_t *ev, uint32_t *t_ms);

#endif

#### discrete.c

#include "discrete.h"
#include "fsl_gpio.h"

#define EVQ_SZ 16
static volatile io_event_t ev_q[EVQ_SZ];
static volatile uint32_t ev_t[EVQ_SZ];
static volatile uint8_t ev_head, ev_tail;

static bool raw_read(GPIO_Type *base, uint32_t pin)
{
    // SDK returns pin level as 0/1
    return GPIO_PinRead(base, pin) ? true : false;
}

static bool apply_polarity(bool raw, discrete_polarity_t pol)
{
    return (pol == DISCRETE_ACTIVE_LOW) ? (!raw) : raw;
}

void DISCRETE_IN_Init(discrete_in_t *d, GPIO_Type *base, uint32_t pin,
                      discrete_polarity_t pol, uint8_t thresh, uint8_t count_max)
{
    d->base = base; d->pin = pin; d->pol = pol;
    d->count = 0; d->thresh = thresh; d->count_max = count_max;
    d->debounced = false; d->latched = false; d->last_change_ms = 0U;
}

void DISCRETE_IN_Sample(discrete_in_t *d, uint32_t now_ms)
{
    bool raw = raw_read(d->base, d->pin);
    bool val = apply_polarity(raw, d->pol);

    if (val) { if (d->count < d->count_max) d->count++; }
    else { if (d->count > 0) d->count--; }

    bool prev = d->debounced;
    if (!prev && d->count >= d->thresh) {
        d->debounced = true; d->last_change_ms = now_ms; EV_Post(EV_PRESS, now_ms);
    } else if (prev && d->count == 0) {
        d->debounced = false; d->last_change_ms = now_ms; EV_Post(EV_RELEASE, now_ms);
    }
}

bool DISCRETE_IN_Get(const discrete_in_t *d) { return d->debounced; }

void EV_Post(io_event_t ev, uint32_t t_ms)
{
    uint8_t next = (uint8_t)((ev_head + 1U) % EVQ_SZ);
    if (next != ev_tail) { ev_q[ev_head] = ev; ev_t[ev_head] = t_ms; ev_head = next; }
}

bool EV_Pop(io_event_t *ev, uint32_t *t_ms)
{
    if (ev_tail == ev_head) return false;
    *ev = ev_q[ev_tail]; *t_ms = ev_t[ev_tail];
    ev_tail = (uint8_t)((ev_tail + 1U) % EVQ_SZ); return true;
}

#### main.c (Exercise 2)

#include "board.h"
#include "pin_mux.h"
#include "clock_config.h"
#include "fsl_gpio.h"
#include "fsl_iomuxc.h"
#include "scheduler.h"
#include "discrete.h"

static discrete_in_t sw8;  // SW8 on BOARD_USER_BUTTON_*
static bool lamp_test;
static uint32_t lamp_test_start;

static void io_init(void)
{
    // Button pin already muxed in board/pin_mux for the SDK examples.
    // Ensure it is input with interrupt disabled (we’ll poll in 5 ms task and optionally enable IRQ later)
    gpio_pin_config_t inCfg = { kGPIO_DigitalInput, 0U, kGPIO_NoIntmode };
    GPIO_PinInit(BOARD_USER_BUTTON_GPIO, BOARD_USER_BUTTON_GPIO_PIN, &inCfg);

    // LED: ensure known state
    USER_LED_OFF(); // logical off (drives high on active‑low hardware)

    // Our logical polarity: SW8 is active‑low on EVKB → DISCRETE_ACTIVE_LOW
    DISCRETE_IN_Init(&sw8, BOARD_USER_BUTTON_GPIO, BOARD_USER_BUTTON_GPIO_PIN,
                     DISCRETE_ACTIVE_LOW, /*thresh*/3, /*count_max*/5);
}

static void task_inputs_5ms(void)
{
    DISCRETE_IN_Sample(&sw8, SCHED_Millis());
}

static void task_events_1ms(void)
{
    io_event_t ev; uint32_t t;
    while (EV_Pop(&ev, &t)) {
        if (ev == EV_PRESS) {
            USER_LED_TOGGLE();
            lamp_test = true; lamp_test_start = t; // start long‑press timer
        } else if (ev == EV_RELEASE) {
            lamp_test = false;
        }
    }
}

static void task_outputs_10ms(void)
{
    if (lamp_test && (SCHED_Millis() - lamp_test_start) > 1000U) {
        // Long press → force LED ON regardless of app state
        GPIO_PortClear(BOARD_USER_LED_GPIO, 1U << BOARD_USER_LED_GPIO_PIN);
    }
}

int main(void)
{
    BOARD_InitBootPins();
    BOARD_InitBootClocks();
    BOARD_InitDebugConsole();

    io_init();

    SCHED_Init(1000U);
    SCHED_AddTask(task_events_1ms, 1U);
    SCHED_AddTask(task_inputs_5ms, 5U);
    SCHED_AddTask(task_outputs_10ms, 10U);

    while (1) { SCHED_RunOnce(); __NOP(); }
}

Test: Observe stable debouncing (no chatter). Verify long‑press lamp‑test behavior.

### Exercise 3 — Avionics WOW consolidator with UART publish to ARINC 429 adapter (ADK‑8582)

Task: Model two WOW inputs (here we use SW8 as channel A and a second jumper input as channel B). Consolidate with 40 ms debounce on each channel and logical AND for the final WOW_TRUE. On any change, publish a short ASCII status frame over LPUART1 to the external ADK‑8582 board (e.g., "WOW:1\n" or "WOW:0\n").

Note: The exact ADK‑8582 UART command protocol may differ. We send a simple placeholder token; adjust the command format per your ADK‑8582 documentation to drive a specific ARINC 429 label/SDI as needed.

Solution (delta files)

#### wow.h

#ifndef WOW_H
#define WOW_H
#include <stdbool.h>
#include <stdint.h>
#include "discrete.h"

typedef struct {
    discrete_in_t chA;
    discrete_in_t chB;
    bool wow;              // consolidated
    uint32_t last_change_ms;
} wow_t;

void WOW_Init(wow_t *w, GPIO_Type *a_base, uint32_t a_pin,
              GPIO_Type *b_base, uint32_t b_pin, discrete_polarity_t pol);
void WOW_Task_5ms(wow_t *w, uint32_t now_ms);

#endif

#### wow.c

#include "wow.h"
#include "scheduler.h"
#include "fsl_gpio.h"

void WOW_Init(wow_t *w, GPIO_Type *a_base, uint32_t a_pin,
              GPIO_Type *b_base, uint32_t b_pin, discrete_polarity_t pol)
{
    DISCRETE_IN_Init(&w->chA, a_base, a_pin, pol, 8, 10); // ~40 ms at 5 ms period
    DISCRETE_IN_Init(&w->chB, b_base, b_pin, pol, 8, 10);
    w->wow = false; w->last_change_ms = 0U;
}

void WOW_Task_5ms(wow_t *w, uint32_t now_ms)
{
    DISCRETE_IN_Sample(&w->chA, now_ms);
    DISCRETE_IN_Sample(&w->chB, now_ms);
    bool newWow = DISCRETE_IN_Get(&w->chA) && DISCRETE_IN_Get(&w->chB);
    if (newWow != w->wow) { w->wow = newWow; w->last_change_ms = now_ms; EV_Post(newWow ? EV_PRESS : EV_RELEASE, now_ms); }
}

#### uart_publish.c (LPUART1 send helper)

#include "fsl_lpuart.h"
#include "board.h"

void UART_Init_115200(void)
{
    lpuart_config_t cfg;
    LPUART_GetDefaultConfig(&cfg);
    cfg.baudRate_Bps = 115200U;
    cfg.enableTx = true; cfg.enableRx = true;
    LPUART_Init(LPUART1, &cfg, CLOCK_GetFreq(kCLOCK_OscClk));
}

static void uart_send(const char *s)
{
    while (*s) { LPUART_WriteBlocking(LPUART1, (const uint8_t*)s, 1U); s++; }
}

void UART_SendWOW(bool wow)
{
    uart_send(wow ? "WOW:1\n" : "WOW:0\n");
}

#### main.c (Exercise 3)

#include "board.h"
#include "pin_mux.h"
#include "clock_config.h"
#include "fsl_gpio.h"
#include "scheduler.h"
#include "discrete.h"
#include "wow.h"

void UART_Init_115200(void);
void UART_SendWOW(bool wow);

static wow_t wow;
static bool last_pub;

static void io_init(void)
{
    // Configure two inputs. We reuse SW8 as chA and choose another free GPIO pin for chB.
    gpio_pin_config_t inCfg = { kGPIO_DigitalInput, 0U, kGPIO_NoIntmode };

    GPIO_PinInit(BOARD_USER_BUTTON_GPIO, BOARD_USER_BUTTON_GPIO_PIN, &inCfg); // chA

    // Example chB: use GPIO1_IO10 (adjacent pad to LED on many EVKBs) — update to an actual pin available on your board.
    // Ensure this pin is routed as GPIO in pin_mux.c using MCUXpresso Config Tools.
    IOMUXC_SetPinMux(IOMUXC_GPIO_AD_B0_10_GPIO1_IO10, 0U);
    IOMUXC_SetPinConfig(IOMUXC_GPIO_AD_B0_10_GPIO1_IO10, 0x10B0U);
    GPIO_PinInit(GPIO1, 10U, &inCfg);

    WOW_Init(&wow,
             BOARD_USER_BUTTON_GPIO, BOARD_USER_BUTTON_GPIO_PIN,
             GPIO1, 10U,
             DISCRETE_ACTIVE_LOW);

    USER_LED_OFF();
}

static void task_inputs_5ms(void)
{
    WOW_Task_5ms(&wow, SCHED_Millis());
}

static void task_events_1ms(void)
{
    io_event_t ev; uint32_t t;
    while (EV_Pop(&ev, &t)) {
        if (ev == EV_PRESS || ev == EV_RELEASE) {
            bool now = (ev == EV_PRESS);
            if (now != last_pub) { last_pub = now; UART_SendWOW(now); }
            // Mirror consolidated WOW onto LED (LED ON when WOW TRUE)
            if (now) { GPIO_PortClear(BOARD_USER_LED_GPIO, 1U << BOARD_USER_LED_GPIO_PIN); }
            else { GPIO_PortSet(BOARD_USER_LED_GPIO, 1U << BOARD_USER_LED_GPIO_PIN); }
        }
    }
}

int main(void)
{
    BOARD_InitBootPins();
    BOARD_InitBootClocks();
    BOARD_InitDebugConsole();

    UART_Init_115200();
    io_init();

    SCHED_Init(1000U);
    SCHED_AddTask(task_events_1ms, 1U);
    SCHED_AddTask(task_inputs_5ms, 5U);

    while (1) { SCHED_RunOnce(); __NOP(); }
}

Bench validation 1. Tie GPIO1_IO10 high/low with a jumper to emulate WOW channel B.
2. Toggle SW8 for channel A.
3. Observe UART frames on LPUART1 TX (pins per pin_mux.c) to the ADK‑8582; LED mirrors consolidated WOW.

## 11. Advanced topics (for further study)

- GPIO interrupts vs. polling: For very slow discretes, polling at 5 ms is sufficient; for time stamping, enable edge interrupts (GPIO_PinSetInterruptConfig) and only capture timestamps in the ISR. Mask/unmask in the driver when lamp‑test or maintenance modes run.

- Glitch capture: Maintain a short‑window glitch counter when ISRs report edges that do not survive debouncing; use for harness diagnostics.

- Output rate limiting: If outputs drive external loads, enforce minimum on/off times to avoid relay chatter.

- Time‑deterministic logging: Push events to a pre‑allocated buffer; flush in a low‑priority task to avoid interfering with I/O periods.

## 12. Traceability checklist (mapping to avionics‑style requirements)

- D‑I‑001: The driver shall provide configurable input polarity and a deterministic debounce algorithm.

- D‑I‑002: The driver shall generate events on debounced transitions with timestamps.

- D‑O‑001: The driver shall guarantee a defined power‑up output state.

- S‑K‑001: The scheduler shall provide a 1 ms system tick and run tasks at configured periods.

- APP‑WOW‑001: WOW consolidation shall require both channels TRUE ≥40 ms and de‑assert on either FALSE ≥40 ms.

## 13. Common pitfalls and how to avoid them

- Floating inputs: Always configure pull/keeper and hysteresis; otherwise you will see random events.

- Polarity confusion: Encode active‑low at the driver; expose only logical TRUE/FALSE to the app.

- Doing work in ISRs: Keep ISRs short; defer to tasks via event queues.

- Unbounded tasks: A busy task will starve 5 ms sampling and break debounce guarantees. Budget and measure.

- Not measuring: Use a spare GPIO as a scope pin toggled at the start/end of tasks to visualize jitter.

## 14. What to hand‑in (lab deliverables)

- Source tree with scheduler.*, discrete.*, wow.*, and modified main.c, pin_mux.c.

- Logic analyzer capture showing: 1 ms tick timing, 5 ms input sampling, and LED response.

- UART capture to ADK‑8582 of WOW:0/1 frames on transitions.

## 15. Appendix: Pad configuration cheat‑sheet (EVKB typical)

- IOMUXC_SetPinMux(<PAD>, 0U);

- IOMUXC_SetPinConfig(<PAD>, 0x10B0U); → typical keeper+pull, 100 MHz speed, medium drive; adjust per harness.

- Inputs: prefer HYS=ON, pull‑up/down according to external wiring.

- Outputs: set DSE (drive) and SRE (slew) to meet EMC without over‑driving.