# Discrete I/O Fundamentals — Software Debouncing (i.MX RT1050 EVKB)

## Why this matters in avionics

- A discrete input is a binary signal (logical 0 or 1) typically sourced by a switch, relay, proximity sensor, or an opto‑isolated line.

- In aircraft systems, discrete lines drive flight‑phase logic, interlocks, inhibits, configuration bits, and maintenance modes.

- Examples include WOW (Weight‑On‑Wheels), cargo‑door latch microswitches, fire‑handle positions, and air/ground relays.

- These sources are mechanical or relay‑based and they bounce—their contacts chatter between 0 and 1 for microseconds to tens of milliseconds after a transition.

- If software samples the raw line, the system can misinterpret a single actuation as multiple events.

- In a safety‑critical context governed by DO‑178C (Software Considerations in Airborne Systems and Equipment Certification) and ARP4754A (Guidelines for Development of Civil Aircraft and Systems), this is unacceptable.

- Software debouncing converts a noisy, fast‑changing raw input into a stable logical state and well‑defined edges with guaranteed timing and bounded latency.

## Physical intuition: what is “bounce”?

- When a mechanical contact closes or opens, the metal surfaces approach with elasticity and microscopic asperities.

- They hit, rebound, and scrape until settling.

- The electrical resistance toggles rapidly, producing an apparent square‑wave burst.

- Environmental extremes (temperature, vibration), supply noise, EMI/EMC conditions, and aging all affect bounce duration.

- For typical panel switches and microswitches, design for 5–20 ms worst‑case bounce; for relays or harsh environments, budget up to 50 ms unless component data justifies less.

## Terminology (expanded at first use)

- GPIO — General‑Purpose Input/Output. i.MX RT1050 exposes GPIO1..GPIO5 ports with per‑pin direction and state.

- PIT — Periodic Interrupt Timer. Provides periodic interrupts at programmable rates; ideal for fixed‑rate sampling.

- ISR — Interrupt Service Routine. Code executed in response to an interrupt.

- WOW — Weight‑On‑Wheels discrete indicating aircraft on ground.

- EICAS — Engine Indication and Crew Alerting System (or ECAM on Airbus: Electronic Centralized Aircraft Monitor). Used here as a representative alerting/monitoring consumer of discretes.

## Design goals for a debouncer in airborne systems

- Determinism: fixed sampling period, bounded processing time, bounded edge‑to‑decision latency.

- Monotonicity: one real actuation → at most one debounced rising edge; one real release → at most one debounced falling edge.

- Configurability: thresholds per input to match electrical characteristics and safety requirements.

- Traceability: requirements↔design↔code↔tests under DO‑178C; configuration captured and reviewable.

- Robustness: works across temperature, electrical noise, and timing jitter.

- Observability: counters and status exposed for BIT (Built‑In Test) and maintenance.

## Core software debouncing strategies

### 1) Time‑based lockout (monostable)

- On any detected change, start a timer (e.g., 10 ms).

- Ignore further changes until the timer expires; then re‑sample to confirm.

- This is simple and low‑CPU, but its lockout can delay recognition of a quick legitimate reversal.

### 2) Saturating up/down counter (a.k.a. “integrator”) — Recommended

- Sample at a fixed rate (e.g., 1 kHz). Maintain an N‑bit counter per input. If the raw sample is 1, increment (up to max); if 0, decrement (down to 0).

- Declare the debounced state = 1 only when the counter reaches max; declare 0 only when it reaches 0.

- This behaves like a discrete low‑pass filter, offers excellent immunity to sporadic glitches, and yields tunable latency via the counter size and sample period.

### 3) Majority vote over a sliding window

- Keep the last W samples; the debounced state is the majority bit.

- Useful when noise is symmetric but has higher memory and shift overhead.

### 4) Edge‑qualified state machine

- Trigger on a detected edge, then verify that the input remains stable for T_confirm (e.g., 10 ms) before committing.

- Combines fast detection with a stability check; good for interrupt‑first designs.

We intend to implement strategy (2) with a 1 ms PIT tick and show how to parameterize it to emulate (1) and (3).

## Choosing sampling rate and thresholds

- Let T_s be the sampling period and N the counter range.

- The worst‑case additional latency to recognize a true edge is roughly N × T_s.

- If you budget T_bounce = 10 ms and want lock‑solid immunity with ≤5 ms added latency, choose T_s = 1 ms and N = 5 (≈5 ms).

- If measurements show occasional 15 ms bursts, push N to 15 or reduce T_s (at cost of CPU load).

- Always validate under environmental extremes.

## Generic example : panel pushbutton toggles a status LED

- A front‑panel pushbutton wired to EVKB‑i.MXRT1050 SW8 (GPIO5_IO00) should toggle the USER LED (GPIO1_IO09) exactly once per press.

- Raw bounce should not create multiple toggles.

- We sample at 1 kHz and use a 5‑count integrator.

## Avionics use case example: Weight‑On‑Wheels (WOW) gating ground spoilers

A WOW discrete feeds a Flight Control Computer. On touchdown, ground spoilers should deploy only after WOW = 1 has been stable for ≥ 20 ms to avoid spurious deployment from bounce or runway joints. In air (WOW = 0) the spoilers must be inhibited. For this lab:

- Map the EVKB SW8 to WOW (pressed = on‑ground = 1; released = in‑air = 0).

- Drive a software “spoiler deploy command” when a debounced rising edge occurs and the state remains 1.

- Light the USER LED solid while WOW is 1. Flash at 2 Hz during the 20 ms stabilization window to show the qualifier timing.

- Log events over the debug console for observability (time‑stamp, raw, filtered, edges).

This exercise demonstrates time qualification, edge extraction, and how a consumer subsystem (spoiler logic) must rely only on debounced edges, never raw input.

## Implementation on EVKB‑i.MXRT1050 (bare metal)

We build on the NXP MCUXpresso SDK drivers in this package: - fsl_gpio.h / fsl_gpio.c — GPIO read/write and configuration. - fsl_pit.h / fsl_pit.c — Periodic Interrupt Timer configuration and ISR support. - Board support: board.h, pin_mux.h, clock_config.h, and the example pattern BOARD_InitHardware() from hardware_init.c to enable clocks and the debug console.

Pins used (from EVKB board definitions): - BOARD_USER_BUTTON_GPIO = GPIO5, BOARD_USER_BUTTON_GPIO_PIN = 0 (SW8) - BOARD_USER_LED_GPIO = GPIO1, BOARD_USER_LED_GPIO_PIN = 9 (User LED)

### Debouncer data structure (integrator)

/* debouncer.h */
#include <stdint.h>
#include <stdbool.h>

typedef struct {
    uint8_t count;          /* current integrator value (0..max) */
    uint8_t max;            /* threshold to accept a new state */
    uint8_t state;          /* current debounced state (0 or 1) */
    uint8_t last_state;     /* previous debounced state */
    bool    rose;           /* latched rising edge flag */
    bool    fell;           /* latched falling edge flag */
} debounce_t;

static inline void debounce_init(debounce_t *d, uint8_t max, uint8_t initial)
{
    d->count = initial ? max : 0;
    d->max = max;
    d->state = initial;
    d->last_state = initial;
    d->rose = false;
    d->fell = false;
}

/* Call at fixed rate (e.g., every 1 ms) with the RAW pin value */
static inline void debounce_update(debounce_t *d, uint8_t raw)
{
    /* Saturating up/down integrator */
    if (raw) {
        if (d->count < d->max) d->count++;
    } else {
        if (d->count > 0) d->count--;
    }

    /* Commit new debounced state only at the extremes */
    uint8_t new_state = d->state;
    if (d->count == d->max) new_state = 1U;
    else if (d->count == 0U) new_state = 0U;

    d->rose = (d->last_state == 0U && new_state == 1U);
    d->fell = (d->last_state == 1U && new_state == 0U);
    d->last_state = d->state = new_state;
}

### Board + timer + GPIO glue (uses SDK drivers)

/* debounce_demo.c : EVKB‑i.MXRT1050 + MCUXpresso SDK */
#include "fsl_debug_console.h"
#include "fsl_gpio.h"
#include "fsl_pit.h"
#include "board.h"
#include "pin_mux.h"
#include "clock_config.h"

#include <stdbool.h>
#include <stdint.h>
#include "debouncer.h"

/* PIT configuration (mirrors SDK example style) */
#ifndef DEMO_PIT_BASEADDR
#define DEMO_PIT_BASEADDR PIT
#endif
#ifndef DEMO_PIT_CHANNEL
#define DEMO_PIT_CHANNEL kPIT_Chnl_0
#endif
#ifndef PIT_IRQ_ID
#define PIT_IRQ_ID PIT_IRQn
#endif
#ifndef PIT_SOURCE_CLOCK
#define PIT_SOURCE_CLOCK CLOCK_GetFreq(kCLOCK_OscClk)
#endif

/* Make the button/LED symbols portable via board.h */
#define BTN_GPIO        BOARD_USER_BUTTON_GPIO
#define BTN_PIN         BOARD_USER_BUTTON_GPIO_PIN
#define LED_GPIO        BOARD_USER_LED_GPIO
#define LED_PIN         BOARD_USER_LED_GPIO_PIN

/* 1 kHz sampling → 1 ms per tick; integrator max=5 ⇒ ~5 ms qualification */
#define DEBOUNCE_TICK_HZ   1000U
#define INTEGRATOR_MAX     5U

static volatile bool s_tick = false;
static debounce_t s_wow; /* our single debounced input */

/* Simple time base for prints and LED blinking */
static volatile uint32_t s_ms = 0;

void PIT_IRQHandler(void)
{
    /* Clear interrupt flag */
    PIT_ClearStatusFlags(DEMO_PIT_BASEADDR, DEMO_PIT_CHANNEL, kPIT_TimerFlag);

    s_tick = true;
    s_ms++;
    SDK_ISR_EXIT_BARRIER;
}

static void gpio_init(void)
{
    /* Ensure GPIO clocks are on (GPIO1 for LED, GPIO5 for SW8) */
    CLOCK_EnableClock(kCLOCK_Gpio1);
    CLOCK_EnableClock(kCLOCK_Gpio5);

    /* LED as output */
    gpio_pin_config_t led_cfg = {kGPIO_DigitalOutput, 1}; /* inactive (OFF) */
    GPIO_PinInit(LED_GPIO, LED_PIN, &led_cfg);

    /* Button as input with pull‑up disabled here – rely on board pin_mux */
    gpio_pin_config_t btn_cfg = {kGPIO_DigitalInput, 0};
    GPIO_PinInit(BTN_GPIO, BTN_PIN, &btn_cfg);
}

static void pit_init(void)
{
    pit_config_t pitCfg;
    PIT_GetDefaultConfig(&pitCfg);
    PIT_Init(DEMO_PIT_BASEADDR, &pitCfg);

    /* Set period for 1 kHz */
    uint32_t period_counts = PIT_SOURCE_CLOCK / DEBOUNCE_TICK_HZ;
    PIT_SetTimerPeriod(DEMO_PIT_BASEADDR, DEMO_PIT_CHANNEL, period_counts);

    PIT_EnableInterrupts(DEMO_PIT_BASEADDR, DEMO_PIT_CHANNEL, kPIT_TimerInterruptEnable);
    EnableIRQ(PIT_IRQ_ID);
    PIT_StartTimer(DEMO_PIT_BASEADDR, DEMO_PIT_CHANNEL);
}

int main(void)
{
    BOARD_ConfigMPU();
    BOARD_InitBootPins();
    BOARD_InitBootClocks();
    BOARD_InitDebugConsole();
    gpio_init();
    pit_init();

    PRINTF("\r\n[Debounce demo] 1kHz, integrator=%u (≈%u ms)\r\n", INTEGRATOR_MAX, INTEGRATOR_MAX);

    debounce_init(&s_wow, INTEGRATOR_MAX, 0);

    uint32_t last_print = 0;
    uint8_t last_led = 1; /* OFF */

    while (1) {
        if (s_tick) {
            s_tick = false;

            /* RAW sample: on EVKB SW8 the pressed level is logic 0 or 1 depending on routing; read and normalize */
            uint8_t raw = (uint8_t)GPIO_PinRead(BTN_GPIO, BTN_PIN);
            /* If your hardware inverts the sense, flip here: raw ^= 1; */

            debounce_update(&s_wow, raw);

            /* Drive LED with debounced state for visibility */
            if (s_wow.state != last_led) {
                last_led = s_wow.state;
                if (last_led) GPIO_PortClear(LED_GPIO, 1U << LED_PIN); /* LED ON (active low) */
                else          GPIO_PortSet(LED_GPIO,   1U << LED_PIN); /* LED OFF */
            }

            /* Use the edge latches exactly once */
            if (s_wow.rose) {
                PRINTF("%lu ms : DEBOUNCED RISE\r\n", (unsigned long)s_ms);
                s_wow.rose = false;
            }
            if (s_wow.fell) {
                PRINTF("%lu ms : DEBOUNCED FALL\r\n", (unsigned long)s_ms);
                s_wow.fell = false;
            }

            /* Periodic status */
            if ((s_ms - last_print) >= 500U) {
                last_print = s_ms;
                PRINTF("%lu ms : raw=%u count=%u state=%u\r\n",
                       (unsigned long)s_ms, raw, s_wow.count, s_wow.state);
            }
        }
    }
}

Notes: - The EVKB user LED is active‑low (logic 0 turns it on). We explicitly use GPIO_PortClear/GPIO_PortSet accordingly. - BOARD_InitBootPins() relies on pin_mux.c generated for your SDK project. Ensure SW8 is configured as GPIO5_IO00 with the correct pull state; if unsure, add an external pull‑up/down on your breadboard switch. - If your particular board wiring yields an inverted sense (pressed = 0), flip raw as shown.

## Mapping this to the avionics WOW use case

To enforce a ≥20 ms stability requirement before arming ground spoilers:

- Set INTEGRATOR_MAX = 20 (with a 1 kHz tick → 20 ms).

- Gate any spoiler logic on the debounced rising edge (s_wow.rose) and the current debounced state (s_wow.state == 1).

- Add a small blinking indicator during the qualification window by reading s_wow.count (0..20). When 0 < count < 20, toggle the LED every 250 ms to indicate “validating”; once it hits 20, turn solid.

Because your consumer logic subscribes only to debounced edges, a 5–15 ms contact chatter will not deploy spoilers prematurely.

## Parameterizing the debouncer to emulate other strategies

- Time‑based lockout: after an edge, ignore updates by forcing the counter to stay at 0 or max for L ticks before allowing it to move again.

- Majority vote: store a bitfield of the last W raw samples; every tick, popcount and compare to W/2 to set state.

The provided debounce_update() is the best general default for discrete inputs subject to both chatter and occasional single‑sample spikes.

## Verification on target

- Build the project with the MCUXpresso SDK, enabling modules: GPIO, PIT, DEBUG CONSOLE.

- Start from the SDK PIT driver example (boards/evkbimxrt1050/driver_examples/pit) or GPIO input_interrupt example and replace main() with the demo above, keeping board.c, hardware_init.c, pin_mux.c, and clock_config.c from the example.

- Open a terminal on the EVKB’s debug UART at the baud set by BOARD_InitDebugConsole() (typically 115200 bps). Press and hold SW8; observe one DEBOUNCED RISE print, solid LED, and stable state=1 even if you tap or wiggle the switch. Release SW8; observe one DEBOUNCED FALL.

- Stress: quick double‑presses faster than 5 ms should produce one edge at most. Increase INTEGRATOR_MAX to 20 and verify the designed 20 ms latency.

For formal verification, capture the raw SW8 pin on a logic analyzer in parallel with a GPIO‑toggled test point on each debounced edge, and measure the edge‑to‑decision latency under temperature extremes.

## Best practices (avionics‑oriented)

- Treat debouncing as requirements‑driven. Write an HLR (High‑Level Requirement): “The WOW discrete shall be recognized only after 20 ms of continuous assertion; deassertion shall be recognized only after 20 ms of continuous negation. The edge‑to‑decision latency shall be ≤ 25 ms.” Trace this to LLR, code, and tests.

- Synchronize to the CPU clock domain. External discretes can violate setup/hold at the GPIO sampling flop. Use the fixed‑rate PIT sampling (software synchronization). If you ever use GPIO interrupts directly, add a two‑flop synchronizer in an FPGA or qualify in software before acting.

- Measure your switch population. Do not assume 5 ms; verify the worst case with your exact hardware and harness.

- Bound CPU usage. Keep the ISR constant‑time; do only sampling and debouncer updates there. Push logging/printing to the main loop.

- Avoid long ISRs. Printing (PRINTF) in ISRs can stall the system; never do this in certification‑bound code.

- Fail‑safe defaults. On startup, seed the debouncer to the physically safe state (e.g., WOW=air) until proven otherwise for N ticks.

- Configuration control. Make the thresholds part of a configuration database with checksums and version control.

- Diagnostics. Expose count, state, rose/fell via a maintenance/health page; include a BIT that detects stuck‑at lines.

- EMI/EMC. Software debouncing complements, not replaces, proper input conditioning (RC filters, Schmitt buffers, opto‑isolators, surge protection to RTCA/DO‑160).

## Optional extension: driving an ARINC 429 stub for lab realism

- If you have the ADK‑8582 ARINC 429 interface board connected over UART to the EVKB, extend the demo so that each debounced WOW edge transmits an ARINC 429 label representing air/ground state (e.g., label 053 octal, a common discrete label in some installations).

- The EVKB sends a simple UART command to the ADK‑8582 to put the label into its transmit queue.

- This shows how only debounced edges should trigger avionics bus outputs.

- (The ADK‑8582 specifics are board‑vendor dependent; keep the UART framing and label mapping in a separate module.)

## What you hand in (for the training check‑off)

- Your debouncer.h and debounce_demo.c compiled and running on EVKB‑i.MXRT1050.

- A short capture of the debug log demonstrating single edges per actuation at integrator values 5 and 20.

- A photo or scope trace showing raw bounce vs. debounced edge for at least one transition.

- If attempting the ARINC 429 extension, the UART log of label transmissions aligned to debounced edges.

## Appendix: tuning cheat‑sheet

- Start with 1 kHz sampling; set INTEGRATOR_MAX = T\_qual_ms.

- If the environment is harsher, increase INTEGRATOR_MAX first; only then consider faster sampling to keep latency down.

- For actions that must be fast on release, you can use asymmetric thresholds: separate max_assert and max_deassert counts.

- When multiple discretes arrive together (e.g., gear down + WOW), de‑correlate sampling jitter by reading all raw inputs in the same tick.