Below is every GPIO driver API function that appears in devices/MIMXRT1052/drivers/fsl_gpio.h (your SDK). On i.MX RT1050/RT1052 these APIs target the IGPIO block (GPIO1..GPIOx), where each GPIO instance has 32 pins (pin = 0..31) and “port” operations use a bitmask (mask = 1U << pin).

## Data types these APIs use (quick context)

- gpio_pin_direction_t

  - kGPIO_DigitalInput

  - kGPIO_DigitalOutput

- gpio_interrupt_mode_t

  - kGPIO_NoIntmode, kGPIO_IntLowLevel, kGPIO_IntHighLevel, kGPIO_IntRisingEdge, kGPIO_IntFallingEdge, kGPIO_IntRisingOrFallingEdge

- gpio_pin_config_t

  - direction (input/output)

  - outputLogic (initial output value if output)

  - interruptMode (one of gpio_interrupt_mode_t)

## 1) Initialization & per-pin configuration

### void GPIO_PinInit(GPIO_Type *base, uint32_t pin, const gpio_pin_config_t *Config);

What it does

- Enables the GPIO clock (unless clock-control is disabled in SDK config).

- Disables that pin’s interrupt mask bit first.

- Sets the pin direction:

  - If output: writes initial outputLogic, then sets the direction bit.

  - If input: clears the direction bit.

- Applies Config->interruptMode by calling the interrupt-config API.

When to use

- Normal “one-stop” pin setup after your IOMUX/pinmux has routed the pad to GPIO.

## 2) Output control (single pin)

### void GPIO_PinWrite(GPIO_Type *base, uint32_t pin, uint8_t output);

What it does

- Sets the output latch for a single pin:

  - output == 0 drives output low (uses DR_CLEAR if available, else bit-clears DR)

  - output != 0 drives output high (uses DR_SET if available, else bit-sets DR)

- Asserts pin < 32.

When to use

- Setting an LED, discrete output, or any single-bit output state.

### static inline void GPIO_WritePinOutput(GPIO_Type *base, uint32_t pin, uint8_t output)

What it does

- Alias/wrapper that just calls GPIO_PinWrite().

Why it exists

- Naming consistency across some SDK families.

## 3) Output control (multiple pins / port mask)

### static inline void GPIO_PortSet(GPIO_Type *base, uint32_t mask)

What it does

- Sets all masked pins high in one operation:

  - Uses DR_SET = mask if available; otherwise DR |= mask.

Good for

- Atomically setting multiple outputs, or avoiding read-modify-write when DR_SET exists.

### static inline void GPIO_SetPinsOutput(GPIO_Type *base, uint32_t mask)

What it does

- Alias for GPIO_PortSet().

### static inline void GPIO_PortClear(GPIO_Type *base, uint32_t mask)

What it does

- Clears all masked pins low:

  - Uses DR_CLEAR = mask if available; otherwise DR &= ~mask.

### static inline void GPIO_ClearPinsOutput(GPIO_Type *base, uint32_t mask)

What it does

- Alias for GPIO_PortClear().

### static inline void GPIO_PortToggle(GPIO_Type *base, uint32_t mask)

What it does

- Toggles all masked pins:

  - Uses DR_TOGGLE = mask if available; otherwise DR ^= mask.

Important note

- This toggles the output latch, not necessarily the physical pad state (if something external is forcing the line).

## 4) Reading back state

### static inline uint32_t GPIO_PinRead(GPIO_Type *base, uint32_t pin)

What it does

- Returns the bit from DR (data/output register) at that pin position.

- Meaning: “what value is currently latched in the GPIO data register”.

Use it for

- Reading back what you last drove (or intend to drive) on an output.

### static inline uint32_t GPIO_ReadPinInput(GPIO_Type *base, uint32_t pin)

What it does

- Alias for GPIO_PinRead() (despite the name, it reads DR).

### static inline uint8_t GPIO_PinReadPadStatus(GPIO_Type *base, uint32_t pin)

What it does

- Returns the bit from PSR (pad status register).

- Meaning: “what is the actual sampled level on the pin pad right now”.

Use it for

- Reading inputs (discretes, button, external logic level), or verifying actual pin level.

### static inline uint8_t GPIO_ReadPadStatus(GPIO_Type *base, uint32_t pin)

What it does

- Alias for GPIO_PinReadPadStatus().

## 5) Interrupt configuration & control

### void GPIO_PinSetInterruptConfig(GPIO_Type *base, uint32_t pin, gpio_interrupt_mode_t pinInterruptMode);

What it does

- Programs how a pin generates interrupts by configuring:

  - ICR1 for pins 0–15, ICR2 for pins 16–31 (2 bits per pin)

  - EDGE_SEL for “rising or falling edge” mode

- It does not enable the interrupt mask (IMR) by itself.

Typical flow

- Configure mode with this function

- Enable mask using GPIO_PortEnableInterrupts()

- In ISR: read/clear ISR flags

### static inline void GPIO_SetPinInterruptConfig(GPIO_Type *base, uint32_t pin, gpio_interrupt_mode_t pinInterruptMode)

What it does

- Alias for GPIO_PinSetInterruptConfig().

### static inline void GPIO_PortEnableInterrupts(GPIO_Type *base, uint32_t mask)

What it does

- Enables interrupt generation for masked pins: IMR |= mask.

### static inline void GPIO_EnableInterrupts(GPIO_Type *base, uint32_t mask)

What it does

- Alias for GPIO_PortEnableInterrupts().

### static inline void GPIO_PortDisableInterrupts(GPIO_Type *base, uint32_t mask)

What it does

- Disables interrupt generation for masked pins: IMR &= ~mask.

### static inline void GPIO_DisableInterrupts(GPIO_Type *base, uint32_t mask)

What it does

- Alias for GPIO_PortDisableInterrupts().

## 6) Interrupt flags (status/clear)

### static inline uint32_t GPIO_PortGetInterruptFlags(GPIO_Type *base)

What it does

- Returns the raw interrupt status register: ISR.

### static inline uint32_t GPIO_GetPinsInterruptFlags(GPIO_Type *base)

What it does

- Alias for GPIO_PortGetInterruptFlags().

### static inline void GPIO_PortClearInterruptFlags(GPIO_Type *base, uint32_t mask)

What it does

- Clears pending interrupt flags by writing mask to ISR (write-1-to-clear behavior).

### static inline void GPIO_ClearPinsInterruptFlags(GPIO_Type *base, uint32_t mask)

What it does

- Alias for GPIO_PortClearInterruptFlags().

## Tiny, board-relevant examples (EVKB IMXRT1050)

### Generic: toggle the USER LED (GPIO1 pin 9 on EVKB config)

#include "fsl_gpio.h"

#include "pin_mux.h"     // BOARD_InitBootPins()

#include "clock_config.h"

#include "board.h"

int main(void)

{

BOARD_InitBootPins();   // mux pad to GPIO

BOARD_InitBootClocks();

BOARD_InitDebugConsole();

gpio_pin_config_t ledCfg = {

.direction = kGPIO_DigitalOutput,

.outputLogic = 1U,              // EVKB USER LED is active-low in the demo setup

.interruptMode = kGPIO_NoIntmode

};

GPIO_PinInit(BOARD_USER_LED_GPIO, BOARD_USER_LED_GPIO_PIN, &ledCfg);

while (1)

{

GPIO_PortToggle(BOARD_USER_LED_GPIO, BOARD_USER_LED_GPIO_PIN_MASK);

for (volatile uint32_t i = 0; i < 800000; i++) { __NOP(); }

}

}

### Avionics-style discrete I/O idea (conceptual): “WOW (Weight-on-Wheels)” input + “GEAR UNSAFE” output

- Read actual input level with GPIO_PinReadPadStatus() (PSR)

- Drive output with GPIO_PinWrite() / GPIO_PortSet/Clear()

bool wow = (GPIO_PinReadPadStatus(GPIOx, WOW_PIN) != 0U); // actual pad level

if (wow) {

GPIO_PinWrite(GPIOy, GEAR_UNSAFE_PIN, 0U); // clear warning

} else {

GPIO_PinWrite(GPIOy, GEAR_UNSAFE_PIN, 1U); // assert warning

}

The two layers of interest:

- IOMUXC (pad + mux):

  - Pull-up / pull-down / keeper

  - Open-drain enable

  - Schmitt trigger / hysteresis

  - drive strength, slew rate, speed, etc.
✅ Done with IOMUXC_SetPinMux() and IOMUXC_SetPinConfig()

- GPIO (IGPIO):

  - direction (input/output)

  - write/clear/set/toggle

  - interrupts at GPIO block level
✅ Done with GPIO_PinInit(), GPIO_PinWrite(), GPIO_PortToggle(), etc.

## Exactly which headers define them

### IOMUXC APIs (mux + pad config)

- devices/MIMXRT1052/drivers/fsl_iomuxc.h

  - static inline void IOMUXC_SetPinMux(uint32_t muxRegister, uint32_t muxMode, uint32_t inputRegister, uint32_t inputDaisy, uint32_t configRegister, uint32_t inputOnfield)

  - static inline void IOMUXC_SetPinConfig(uint32_t muxRegister, uint32_t muxMode, uint32_t inputRegister, uint32_t inputDaisy, uint32_t configRegister, uint32_t configValue)

### Pad-control bit macros (pull/up/down, open drain, schmitt…)

- devices/MIMXRT1052/periph/PERI_IOMUXC.h

  - IOMUXC_SW_PAD_CTL_PAD_ODE(x) → Open drain enable

  - IOMUXC_SW_PAD_CTL_PAD_PKE(x) → Pull/Keeper enable

  - IOMUXC_SW_PAD_CTL_PAD_PUE(x) → Pull vs Keeper select

  - IOMUXC_SW_PAD_CTL_PAD_PUS(x) → Pull up/down strength/select

  - IOMUXC_SW_PAD_CTL_PAD_HYS(x) → Hysteresis (Schmitt trigger) enable

## What each one means (practically)

### Pull-up / Pull-down

You typically set both:

- PKE(1) = enable pull/keeper circuitry

- PUE(1) = select pull resistor mode (not keeper)

- PUS(n) chooses the resistor behavior:

  - PUS(0) = ~100K pull-down

  - PUS(1) = ~47K pull-up

  - PUS(2) = ~100K pull-up

  - PUS(3) = ~22K pull-up

### Keeper (instead of pull resistor)

- PKE(1) and PUE(0)
This “holds last value” when the output driver is off, instead of forcing up/down.

### Open drain

- ODE(1)
Then your GPIO output behaves like:

- write 0 → actively pulls low

- write 1 → releases the line (needs an external or internal pull-up)

### Schmitt trigger (hysteresis)

- HYS(1)
Useful for noisy/slow edges (classic avionics discrete inputs, long harness runs).

## How it looks in code (generic + avionics-flavored)

### Example: avionics-style discrete INPUT (pull-up + Schmitt)

#include "fsl_iomuxc.h"

#include "fsl_gpio.h"

#include "PERI_IOMUXC.h"  // (usually included indirectly via device headers, but this is where PAD macros come from)

// Example pin macro: IOMUXC_GPIO_AD_B0_03_GPIO1_IO03 (exists in fsl_iomuxc.h)

#define WOW_PAD_MUX   IOMUXC_GPIO_AD_B0_03_GPIO1_IO03

#define WOW_GPIO      GPIO1

#define WOW_PIN       3U

void WOW_DiscreteInit(void)

{

// 1) Route pad to GPIO function

IOMUXC_SetPinMux(WOW_PAD_MUX, 0U);  // last arg = SION (inputOnfield)

// 2) Electrical pad config: pull-up + Schmitt

uint32_t padCfg =

IOMUXC_SW_PAD_CTL_PAD_PKE(1U) |      // enable pull/keeper

IOMUXC_SW_PAD_CTL_PAD_PUE(1U) |      // select pull resistor

IOMUXC_SW_PAD_CTL_PAD_PUS(2U) |      // 100K pull-up (example)

IOMUXC_SW_PAD_CTL_PAD_HYS(1U);       // Schmitt trigger

IOMUXC_SetPinConfig(WOW_PAD_MUX, padCfg);

// 3) GPIO direction/input setup

gpio_pin_config_t cfg = { .direction = kGPIO_DigitalInput, .outputLogic = 0U, .interruptMode = kGPIO_NoIntmode };

GPIO_PinInit(WOW_GPIO, WOW_PIN, &cfg);

}

### Example: wired-OR “FAULT” output (open drain)

void FaultLine_InitOpenDrain(void)

{

IOMUXC_SetPinMux(IOMUXC_GPIO_AD_B0_04_GPIO1_IO04, 0U);

uint32_t padCfg =

IOMUXC_SW_PAD_CTL_PAD_ODE(1U) |      // open drain

IOMUXC_SW_PAD_CTL_PAD_PKE(1U) |

IOMUXC_SW_PAD_CTL_PAD_PUE(1U) |

IOMUXC_SW_PAD_CTL_PAD_PUS(1U);       // pull-up (if you want internal; often external is better)

IOMUXC_SetPinConfig(IOMUXC_GPIO_AD_B0_04_GPIO1_IO04, padCfg);

gpio_pin_config_t cfg = { .direction = kGPIO_DigitalOutput, .outputLogic = 1U, .interruptMode = kGPIO_NoIntmode };

GPIO_PinInit(GPIO1, 4U, &cfg);

// Drive low (assert fault): GPIO_PinWrite(GPIO1, 4U, 0U);

// Release (deassert):       GPIO_PinWrite(GPIO1, 4U, 1U);

}

## Where this is usually done in MCUXpresso projects

In most SDK examples, the pull/open-drain/schmitt config ends up in pin_mux.c inside:

- BOARD_InitBootPins() (or BOARD_InitPins())

Then your application calls:

- BOARD_InitBootPins();

- GPIO_PinInit(...) / GPIO_PinWrite(...)

## MCUXpresso IDE steps (easy path)

- Open your project → Config Tools → Pins.

- Select the pad you want → set Function = GPIOx_IOyy.

- In the pad settings panel, set:

  - Pull enable / pull select / pull strength (PKE/PUE/PUS)

  - Open drain (ODE)

  - Hysteresis (HYS)

- Click Update Code / Generate.

- Build; you’ll see generated IOMUXC_SetPinMux() / IOMUXC_SetPinConfig() calls in pin_mux.c.

Keeper and pull both use the pad’s weak bias circuitry, but they behave very differently when the pin is not being actively driven.

## Pull (pull-up / pull-down)

- Acts like a weak resistor to VDD (pull-up) or GND (pull-down).

- When nothing drives the pin, it will settle to a defined default (1 for pull-up, 0 for pull-down).

- Good for:

  - buttons/switches

  - open-drain lines (I²C, wired-OR faults)

  - defining a known state during reset / high-Z

## Keeper (a.k.a. bus keeper / weak latch)

- Acts like a very weak “memory”: it tries to hold the last logic level the pin had.

- If the pin was last seen high, it weakly holds high; if last seen low, it weakly holds low.

- It’s not a resistor to VDD/GND; it’s more like a tiny feedback latch.

- Good for:

  - shared buses or signals that sometimes float (to prevent chatter)

  - reducing toggling / noise when a line is temporarily undriven

## Key differences (quick)

- Pull defines a default: forces toward up or down.

- Keeper preserves the last state: forces toward whatever it was.

- With a floating input:

  - Pull → deterministic default

  - Keeper → depends on history (what it was last)

## Avionics-style guidance

- For discrete inputs (WOW, door, squat switch, etc.): prefer pull-up or pull-down (deterministic) + Schmitt (HYS) if noise/slow edges.

- Use keeper mostly for bus-like or shared lines where “hold last” is desired and a fixed default isn’t.

## How it maps to the RT1050 pad bits

- PKE=1 enables the pull/keeper circuit.

- PUE=1 selects pull resistor mode.

- PUE=0 selects keeper mode.

It’s called a “weak memory” because a keeper doesn’t force the pin to a fixed 0 or 1 like a pull-up/pull-down does. Instead, it remembers whatever level the pin had most recently and then weakly reinforces that same level when the pin becomes undriven (high-Z).

Here’s what’s really happening electrically:

### What a keeper is (conceptually)

Inside the pad there’s a tiny feedback element (often implemented like a very weak inverter feedback / latch-like structure) that:

- Senses the current pin level (0 or 1) when something is driving it.

- When the external driver releases the pin (pin floats),

- It provides a very small bias back onto the pin to hold it in that same state.

That “hold last state” behavior is why people describe it as memory.

### Why “weak”?

Because the keeper drive is intentionally very low strength:

- It can be overridden easily by any real driver (external IC, strong pull, etc.).

- It’s not meant to power anything—just to prevent a floating node from drifting and chattering due to noise/leakage.

So it’s “memory,” but only in a gentle, biasing sense—hence weak memory.

### Comparison with pulls (why keeper feels like memory but pull doesn’t)

- Pull-up/pull-down: always biases toward a predefined default (up or down), regardless of history.

- Keeper: biases toward the previous state. If it was last high, it biases high; if it was last low, it biases low.

### Practical example

If a GPIO line is used on a bus or shared signal:

- Without a keeper, when nobody drives it, it can float and randomly toggle → noise, extra interrupts, extra power.

- With a keeper, it tends to stay where it last was, so it doesn’t chatter.

That’s the “memory”: state depends on what happened earlier, not on a fixed default.

In avionics terms, the keeper is “history-dependent biasing.” That can be either helpful (quiet, stable buses) or dangerous (non-deterministic safety inputs). Here’s a practical way to decide.

## Avoid keeper when you need a deterministic, testable default

Use a real pull-up or pull-down (internal or external) instead of keeper when any of these are true:

### 1) Safety-critical discretes must have a defined state at power-up / reset

Examples: WOW (weight-on-wheels), squat switch, fire handle, emergency jettison interlocks, engine shutdown discretes, landing gear unsafe, etc.

- With a keeper, if the line floats during reset, it can stay at the last value (from a prior power state or noise) rather than going to a known safe default.

- With a pull, you get a guaranteed bias (safe default) even if the source is disconnected.

### 2) You rely on “open circuit” detection / fault monitoring

If your system detects wiring faults (open, short to ground, short to VDD) using bias networks:

- Keeper can mask opens by holding a previous value and making a floating wire look “valid.”

- Pull resistors make opens show up as the bias default (and are more predictable for BIT tests).

### 3) Slow/noisy environments where “last state” is unacceptable

Long harnesses, high EMI, or signals that may hover around threshold during transitions:

- Keeper can “help” hold a metastable/noisy level instead of resolving to a safe default.

- Prefer pull + Schmitt (HYS) for robust behavior.

### 4) Any line that should fail to a known safe state if the driver tri-states

Classic fail-safe design: if a remote unit dies or disconnects, the receiving side must go to a known value.

- That’s exactly what pulls are for.

- Keeper defeats that principle because it tries to preserve whatever was last.

### 5) Strap/config pins, reset-related pins, boot mode pins

You usually want a hard default every boot, not something influenced by prior activity.

## Keeper is useful when stability matters, but the default does not

Use keeper when you want the line to not float and not chatter, but you don’t require a fixed “up” or “down” default:

### 1) Shared / bus-like signals that are sometimes undriven

Examples: lines that multiple devices can drive at different times, or a line that is intentionally tri-stated between bursts.

- Keeper helps prevent the node from drifting and causing spurious toggles/interrupts.

- It can reduce EMI and dynamic power from random switching on a floating node.

### 2) Inputs that are valid only when actively driven

If your system treats an input as meaningful only when a transmitter is enabled / a strobe is present, keeper can keep the line quiet between transmissions.

### 3) Internal board-level nets with short traces (low risk of harness faults)

On-board signals that aren’t exposed to long wiring harnesses are less likely to see “open circuit” failure modes; keeper can be a nice “anti-float” option.

### 4) Multiplexed pins / temporary high-Z phases

If the pin changes role (peripheral ↔ GPIO, or multiple functions across modes) and spends time in high-Z, keeper can keep it from bouncing around.

## Quick avionics rule of thumb

- Discretes (especially safety-related / harness-connected): prefer pull-up/down + Schmitt, avoid keeper.

- Bus/shared/multiplexed signals (where “don’t float” is the goal): keeper can be useful.

## How this maps to your i.MX RT1050 pad settings

- PKE=1 enables the bias circuit.

- PUE=1 → pull resistor mode (deterministic default)

- PUE=0 → keeper mode (“weak memory” hold-last)

In avionics, hysteresis (often implemented as a Schmitt trigger input) is there for one big reason: make digital inputs behave predictably in a noisy, slow-edge, long-wiring world.

## Hysteresis vs Schmitt trigger (what they mean)

- Hysteresis = the input has two different thresholds:

  - a higher threshold to recognize a rising signal as “1”

  - a lower threshold to recognize a falling signal as “0”

- A Schmitt trigger is the common circuit that provides that hysteresis.

So in many MCUs (including i.MX RT pads), “enable HYS” effectively means “use a Schmitt-trigger style input.”

## Why avionics benefits so much

Aircraft wiring and environments are tough on clean logic edges:

### 1) Long harnesses + EMI = noisy transitions

A discrete line routed through a harness can pick up:

- radiated EMI (radios, switching power supplies, motor drives)

- coupled noise from adjacent wires

- ground offsets and transients

Without hysteresis, if the signal hovers near a single threshold, the input can chatter (rapid 0/1/0/1 transitions). With hysteresis, once it crosses the “ON” threshold it won’t switch back until it goes below the lower threshold.

### 2) Slow edges (RC filters, opto-isolators, wet contacts)

Avionics interfaces often intentionally slow edges:

- RC filtering for EMI / contact bounce

- opto-couplers or comparators conditioning 28 V discretes down to logic

- long cable capacitance

Slow ramps are exactly where a plain CMOS input is most vulnerable to multiple toggles. Schmitt input fixes that by giving “snap” behavior.

### 3) Preventing false interrupts / false state changes

For inputs that drive interrupts (edge/level), hysteresis greatly reduces:

- spurious interrupts

- false event logging

- nuisance BIT faults (Built-In Test)

### 4) Power and reliability

Chattering inputs can cause extra internal switching, extra ISR load, and weird state-machine behavior. Hysteresis helps keep the system quiet and deterministic.

## Where to use it in avionics-style designs

Strong candidates to enable hysteresis (HYS=1):

- Discrete inputs: WOW, door/gear switches, relays, CB status, overheat switches (after level shifting)

- Any harness-connected GPIO input

- Inputs with RC filters or debounce networks

- Inputs that can be slowly varying or exposed to noise bursts

## Where you might not want it (or it won’t matter)

- High-speed digital interfaces (you usually don’t connect these directly to raw GPIO pads anyway; you use proper peripheral/transceiver)

- Signals where timing is extremely tight and you’re relying on exact threshold behavior (rare for GPIO discretes)

- If you already have a clean, conditioned logic signal from a comparator with hysteresis, enabling MCU hysteresis is usually harmless but may be redundant.

## Practical “avionics discrete input” recipe

For a harness-like discrete input that becomes logic-level at the MCU:

- Pull-up/down (deterministic default) rather than keeper

- Hysteresis enabled (Schmitt) to prevent chatter

- Optional: RC/debounce + software debounce

- Interrupt on edge only after you’ve made the signal clean

On i.MX RT1050 specifically: that’s your pad control HYS bit (Schmitt/hysteresis) in the IOMUXC pad config, not in fsl_gpio.h.