# Task Registration and Scheduling (Bare‑Metal): Scheduler Integration and Priorities

## 1) Why a scheduler on bare metal?

- On a bare‑metal system, a task is simply a function that performs a bounded unit of work, usually periodically or upon an event.

- A scheduler is the policy and mechanism that decides which task runs now.

Without an RTOS (Real‑Time Operating System), you still need determinism:

- Periodic activities (e.g., health monitoring, sensor sampling),

- Sporadic/aperiodic activities (e.g., bytes arriving on UART),

- Background activities (e.g., maintenance logging).

Two common patterns without an RTOS:

- Cyclic executive (time‑triggered):

  - a timer tick sets readiness; the main loop runs ready tasks in fixed priority order.

  - Cooperative (a running task is not preempted by another task), simple, analyzable, and cert‑friendly.

- Interrupt‑driven with deferred work:

  - interrupts do the minimum (buffer, set flags), while the main loop performs heavier processing.

  - Priorities come from NVIC (Nested Vectored Interrupt Controller) for interrupts and from your task‑order policy in the main loop.

For avionics consistent with DO‑178C (Software Considerations in Airborne Systems and Equipment Certification), we prefer: static registration, no dynamic allocation after init, bounded execution per activation, and measurable timing.

## 2) On‑chip building blocks you will use

- PIT (Periodic Interrupt Timer) or GPT (General Purpose Timer) for a base tick. See SDK examples:

  - boards/evkbimxrt1050/driver_examples/pit/pit.c

  - boards/evkbimxrt1050/driver_examples/gpt/timer/gpt_timer.c

- NVIC: prioritizes interrupts (lower numeric priority value ⇒ higher urgency on Cortex‑M). CMSIS provides __NVIC_PRIO_BITS and NVIC_SetPriority.

- LPUART: low‑power UART used to receive ARINC 429 words from ADK‑8582. See SDK examples under driver_examples/lpuart/*.

- DWT (Data Watchpoint and Trace) cycle counter for microsecond timing on Cortex‑M7; excellent for jitter/latency measurement during labs.

## 3) Fixed‑priority scheduling without an RTOS

We implement a static task registry: a table where each entry defines name, function pointer, period (ms), fixed priority (0..255, larger means higher in our policy), and an optional WCET (Worst‑Case Execution Time) hint.

The scheduler provides:

- A 1 kHz (configurable) tick via PIT that marks periodic tasks ready at release times.

- A main dispatcher that continuously scans priorities from high to low, running ready tasks to completion (cooperative). When nothing is ready, the CPU sleeps with WFI (Wait‑For‑Interrupt).

This pattern is deterministic and analyzable provided every task does bounded work per activation.

## 4) Priority assignment you can justify

- For periodic tasks with deadlines equal to periods, RMS (Rate‑Monotonic Scheduling) says: shorter period ⇒ higher priority.

- In avionics, combine RMS with criticality: some safety monitors may sit above parsers even if their period is longer when the safety case demands it.

Important:

- Task priorities (handled by the dispatcher) are entirely separate from NVIC priorities (hardware interrupt preemption).

- ISRs (Interrupt Service Routines) always preempt tasks.

- Keep ISRs short; defer heavy work to tasks.

## 5) Implementation: Bare‑metal scheduler (drop‑in for EVKB‑i.MXRT1050)

Below are production‑style files compatible with the EVKB SDK. They avoid dynamic allocation and rely on fsl_pit.h, fsl_gpt.h, and CMSIS.

### 5.1 bm_sched.h

/* bm_sched.h — Bare-metal fixed-priority scheduler (EVKB-IMXRT1050) */
#pragma once
#include <stdint.h>
#include <stddef.h>

#ifdef __cplusplus
extern "C" {
#endif

typedef void (*bm_task_fn_t)(void);

/* Priority: larger number = higher priority (independent of NVIC) */
typedef struct {
    const char  *name;
    bm_task_fn_t run;
    uint32_t     period_ms;    /* 0 => aperiodic (manually released) */
    uint8_t      prio;         /* 0..255, we’ll scan high→low */
    uint32_t     wcet_us;      /* instrumentation hint (0 if unknown) */
    /* Internal (do not touch after registration) */
    volatile uint8_t ready;
    uint32_t     next_release_tick;
    uint32_t     last_start_us;
    uint32_t     last_finish_us;
    uint32_t     overruns;
} bm_task_t;

void BM_Sched_Init(uint32_t tick_hz);
void BM_Sched_Register(bm_task_t *tasks, size_t count);
void BM_Sched_Run(void);

/* Release an aperiodic task from ISR or main. */
void BM_Sched_Release(bm_task_t *t);

/* Must be called by the 1 kHz timer ISR (PIT). */
void BM_Sched_Tick_ISR(void);

/* Time utils (implemented with GPT/PIT/ARM DWT if enabled) */
uint32_t BM_Ticks(void);          /* ms ticks since init */
uint32_t BM_TimeUs(void);         /* microseconds since boot (monotonic) */

#ifdef __cplusplus
}
#endif

### 5.2 bm_sched.c

/* bm_sched.c — uses PIT for 1kHz tick; uses ARM DWT for microsecond timing if available */
#include "bm_sched.h"
#include "fsl_device_registers.h"
#include "fsl_common.h"
#include "fsl_pit.h"
#include "fsl_gpt.h"
#include "board.h"

#ifndef BM_MAX_TASKS
#define BM_MAX_TASKS 16
#endif

static struct {
    bm_task_t  *tasks[BM_MAX_TASKS];
    size_t      count;
    volatile uint32_t tick_ms;
    uint32_t    tick_hz;
} g_bm;

static inline void dwt_init(void)
{
#if (__CORTEX_M == 7U)
    if ((CoreDebug->DEMCR & CoreDebug_DEMCR_TRCENA_Msk) == 0U)
        CoreDebug->DEMCR |= CoreDebug_DEMCR_TRCENA_Msk;
    DWT->CYCCNT = 0;
    DWT->CTRL |= DWT_CTRL_CYCCNTENA_Msk;
#endif
}

uint32_t BM_Ticks(void) { return g_bm.tick_ms; }

uint32_t BM_TimeUs(void)
{
#if (__CORTEX_M == 7U)
    uint32_t cpu_hz = CLOCK_GetFreq(kCLOCK_CpuClk);
    return (uint32_t)((uint64_t)DWT->CYCCNT * 1000000ULL / cpu_hz);
#else
    /* Fallback: approximate using ms tick */
    return g_bm.tick_ms * 1000U;
#endif
}

void BM_Sched_Init(uint32_t tick_hz)
{
    g_bm.count = 0;
    g_bm.tick_ms = 0;
    g_bm.tick_hz = tick_hz;

    BOARD_InitBootPins();
    BOARD_BootClockRUN();
    BOARD_InitDebugConsole();
    dwt_init();

    /* --- PIT: 1 ms tick (see: boards/evkbimxrt1050/driver_examples/pit/pit.c) --- */
    pit_config_t pitCfg;
    PIT_GetDefaultConfig(&pitCfg);
    PIT_Init(PIT, &pitCfg);

    uint32_t busHz = CLOCK_GetFreq(kCLOCK_BusClk);
    uint32_t period_us = 1000000U / tick_hz; /* tick_hz=1000 → 1000 us */
    PIT_SetTimerPeriod(PIT, kPIT_Chnl_0, USEC_TO_COUNT(period_us, busHz));
    PIT_EnableInterrupts(PIT, kPIT_Chnl_0, kPIT_TimerInterruptEnable);

    /* NVIC: give PIT a moderate urgency (leave room for UART RX) */
    NVIC_SetPriority(PIT_IRQn, 7); /* 0=highest urgency on Cortex-M; tune per system */
    EnableIRQ(PIT_IRQn);

    PIT_StartTimer(PIT, kPIT_Chnl_0);

    /* --- Optional: GPT as free-running microsecond timebase (not strictly needed) --- */
    gpt_config_t gptCfg;
    GPT_GetDefaultConfig(&gptCfg);
    gptCfg.enableFreeRun = true;
    GPT_Init(GPT1, &gptCfg);
    GPT_SetClockDivider(GPT1, 1);
    GPT_StartTimer(GPT1);
}

void BM_Sched_Register(bm_task_t *tasks, size_t count)
{
    for (size_t i = 0; i < count && g_bm.count < BM_MAX_TASKS; ++i) {
        tasks[i].ready = 0;
        tasks[i].next_release_tick = (tasks[i].period_ms ? (BM_Ticks() + tasks[i].period_ms) : 0);
        g_bm.tasks[g_bm.count++] = &tasks[i];
    }
}

/* Called from PIT ISR */
void BM_Sched_Tick_ISR(void)
{
    g_bm.tick_ms++;
    /* Mark periodic tasks ready on their release time */
    for (size_t i = 0; i < g_bm.count; ++i) {
        bm_task_t *t = g_bm.tasks[i];
        if (t->period_ms) {
            if ((int32_t)(g_bm.tick_ms - t->next_release_tick) >= 0) {
                t->ready = 1;
                t->next_release_tick += t->period_ms;
            }
        }
    }
}

void BM_Sched_Release(bm_task_t *t) { t->ready = 1; }

/* Cooperative fixed-priority dispatcher */
void BM_Sched_Run(void)
{
    for (;;) {
        uint8_t ran = 0;
        /* scan priorities high→low each pass */
        for (int pr = 255; pr >= 0; --pr) {
            for (size_t i = 0; i < g_bm.count; ++i) {
                bm_task_t *t = g_bm.tasks[i];
                if (t->ready && t->prio == (uint8_t)pr) {
                    t->ready = 0;
                    t->last_start_us = BM_TimeUs();
                    t->run();
                    t->last_finish_us = BM_TimeUs();
                    if (t->wcet_us && (t->last_finish_us - t->last_start_us) > t->wcet_us) {
                        t->overruns++;
                    }
                    ran = 1;
                }
            }
        }
        if (!ran) { __WFI(); } /* idle until next interrupt */
    }
}

/* PIT IRQ — wire to vector table */
void PIT_IRQHandler(void)
{
    if (PIT_GetStatusFlags(PIT, kPIT_Chnl_0) & kPIT_TimerFlag) {
        PIT_ClearStatusFlags(PIT, kPIT_Chnl_0, kPIT_TimerFlag);
        BM_Sched_Tick_ISR();
    }
    __DSB(); __ISB();
}

## 6) UART RX integration (ADK‑8582) and NVIC priorities

We give LPUART RX a higher urgency (numerically smaller NVIC priority value) than the PIT tick to avoid RX overrun. The ISR only copies bytes into a static ring buffer and returns. A high‑priority task in the main loop (ARINC_RX_SVC) drains and reassembles arinc_word_t structures.

Pick an LPUART instance not used by the debug console on your EVKB build. If your board init uses LPUART1 for the console, select LPUART2/3 for ADK‑8582 and set IOMUXC (Input/Output Multiplexer Controller) accordingly.

### 6.1 arinc_uart.h

#pragma once
#include <stdint.h>
#include <stddef.h>

#ifdef __cplusplus
extern "C" {
#endif

typedef struct {
    uint8_t  label;     /* ARINC 8-bit label (LSB first in bus order) */
    uint8_t  sdi;       /* Source/Destination Identifier (2 bits)     */
    uint32_t data;      /* 19 bits meaningful for many labels         */
    uint8_t  ssm;       /* Sign/Status Matrix (2 bits)                */
    uint8_t  parity_ok; /* odd parity check result                    */
} arinc_word_t;

/* ring buffer interface for RX task */
size_t ARINC_Rx_Drain(uint8_t *dst, size_t maxlen); /* copy raw UART bytes */
void   ARINC_UART_Init(uint32_t baud);               /* initialize LPUART + NVIC */

#ifdef __cplusplus
}
#endif

### 6.2 arinc_uart.c

#include "arinc_uart.h"
#include "fsl_lpuart.h"
#include "fsl_iomuxc.h"
#include "fsl_device_registers.h"
#include "board.h"

/* --- Static ring buffer for UART bytes (ISR <-> task) --- */
#define RX_RING_SIZE 2048
static volatile uint8_t s_rx_ring[RX_RING_SIZE];
static volatile uint32_t s_rx_head = 0, s_rx_tail = 0;

/* Select an LPUART instance dedicated for ADK-8582 */
#define ARINC_LPUART       LPUART1      /* change if LPUART1 is used by console */
#define ARINC_LPUART_IRQn  LPUART1_IRQn

static void arinc_uart_pins_init(void)
{
    /* If board.c already configures your chosen LPUART pins, you may leave this empty.
       Otherwise, set IOMUXC here to route the pins for ARINC UART. */
}

void ARINC_UART_Init(uint32_t baud)
{
    arinc_uart_pins_init();

    lpuart_config_t cfg;
    LPUART_GetDefaultConfig(&cfg);
    cfg.baudRate_Bps = baud;
    cfg.enableRx = true;
    cfg.enableTx = false; /* RX-only path for the bridge */

    LPUART_Init(ARINC_LPUART, &cfg, CLOCK_GetFreq(kCLOCK_UartClk));
    LPUART_EnableInterrupts(ARINC_LPUART, kLPUART_RxDataRegFullInterruptEnable | kLPUART_RxOverrunInterruptEnable);

    /* Give UART higher urgency than PIT (so RX cannot be delayed by the tick) */
    NVIC_SetPriority(ARINC_LPUART_IRQn, 5); /* numerically lower ⇒ higher urgency than PIT’s 7 */
    EnableIRQ(ARINC_LPUART_IRQn);
}

void LPUART1_IRQHandler(void)
{
    uint32_t stat = LPUART_GetStatusFlags(ARINC_LPUART);
    /* Read out all available bytes */
    while (stat & kLPUART_RxDataRegFullFlag) {
        uint8_t b = LPUART_ReadByte(ARINC_LPUART);
        uint32_t next = (s_rx_head + 1U) % RX_RING_SIZE;
        if (next != s_rx_tail) { s_rx_ring[s_rx_head] = b; s_rx_head = next; } /* drop if full */
        stat = LPUART_GetStatusFlags(ARINC_LPUART);
    }
    if (stat & kLPUART_RxOverrunFlag) {
        LPUART_ClearStatusFlags(ARINC_LPUART, kLPUART_RxOverrunFlag);
    }
    __DSB(); __ISB();
}

size_t ARINC_Rx_Drain(uint8_t *dst, size_t maxlen)
{
    size_t n = 0;
    while (n < maxlen && s_rx_tail != s_rx_head) {
        dst[n++] = s_rx_ring[s_rx_tail];
        s_rx_tail = (s_rx_tail + 1U) % RX_RING_SIZE;
    }
    return n;
}

## 7) Application: avionics ARINC 429 use case with realistic priorities

Scenario. The ADK‑8582 converts line‑side ARINC 429 to a UART stream (e.g., 4 bytes per ARINC 32‑bit word, possibly with framing). The EVKB receives bytes via LPUART RX ISR → ring buffer. The ARINC_RX_SVC task (highest priority in dispatcher) drains, reassembles, checks parity, and buffers completed arinc_word_t for the ARINC_PARSE task. A HEALTH_MON task audits deadlines, queue depths, and feeds the watchdog. MAINT_LOG prints occasional status.

NVIC priorities (hardware): LPUART RX IRQ = 5, PIT tick = 7. Task priorities (software): ARINC_RX_SVC > HEALTH_MON > ARINC_PARSE > MAINT_LOG (example set below).

### 7.1 main.c (integration)

#include "bm_sched.h"
#include "arinc_uart.h"
#include "fsl_gpio.h"
#include "fsl_debug_console.h"
#include "board.h"
#include <string.h>

/* -------------------- RX word FIFO between RX_SVC and PARSE -------------------- */
#define RX_WORD_FIFO_LEN 64
static arinc_word_t s_rx_words[RX_WORD_FIFO_LEN];
static volatile uint16_t s_rx_w_head = 0, s_rx_w_tail = 0;

static int fifo_push_word(const arinc_word_t *w)
{
    uint16_t n = (s_rx_w_head + 1U) % RX_WORD_FIFO_LEN;
    if (n == s_rx_w_tail) return -1; /* full */
    s_rx_words[s_rx_w_head] = *w;
    s_rx_w_head = n; return 0;
}
static int fifo_pop_word(arinc_word_t *w)
{
    if (s_rx_w_tail == s_rx_w_head) return -1;
    *w = s_rx_words[s_rx_w_tail];
    s_rx_w_tail = (s_rx_w_tail + 1U) % RX_WORD_FIFO_LEN;
    return 0;
}

/* -------------------- ARINC helpers -------------------- */
static uint8_t arinc_parity_odd(uint32_t word)
{
    /* bit 31 is parity; odd parity across bits [0..30] */
    uint32_t data = word & 0x7FFFFFFFUL;
    uint32_t p = __builtin_popcount(data);
    uint32_t parity_bit = (word >> 31) & 1U;
    return ((p + parity_bit) & 1U) ? 1U : 0U; /* 1 => odd parity OK */
}

static void arinc_unpack(uint32_t raw, arinc_word_t *w)
{
    w->label     = (uint8_t)(raw & 0xFFU);
    w->sdi       = (uint8_t)((raw >> 8) & 0x3U);
    w->data      = (raw >> 9) & 0x7FFFFU;
    w->ssm       = (uint8_t)((raw >> 29) & 0x3U);
    w->parity_ok = arinc_parity_odd(raw);
}

/* -------------------- Tasks -------------------- */
static void TASK_ArincRxSvc(void)
{
    uint8_t buf[128];
    size_t n = ARINC_Rx_Drain(buf, sizeof(buf));
    /* Example framing: 4 bytes per ARINC word, little-endian raw32. Adjust if ADK-8582 adds headers. */
    for (size_t i = 0; i + 3 < n; i += 4) {
        uint32_t raw = (uint32_t)buf[i] |
                       ((uint32_t)buf[i+1] << 8) |
                       ((uint32_t)buf[i+2] << 16) |
                       ((uint32_t)buf[i+3] << 24);
        arinc_word_t w; arinc_unpack(raw, &w);
        (void)fifo_push_word(&w); /* on overflow, drop; HEALTH_MON reports */
    }
}

typedef struct {
    float indicated_airspeed_kt;
    float baro_altitude_ft;
    uint32_t last_update_us;
} avionics_state_t;
static avionics_state_t g_state;

static float arinc_bnr_to_float(uint32_t data_bits, float scale, float offset)
{ return offset + scale * (int32_t)data_bits; }

static void TASK_ArincParse(void)
{
    arinc_word_t w; int processed = 0; uint32_t now = BM_TimeUs();
    while (fifo_pop_word(&w) == 0) {
        if (!w.parity_ok) continue;
        switch (w.label) {
            case 0xCB: g_state.indicated_airspeed_kt = arinc_bnr_to_float(w.data, 0.01f, 0.0f); g_state.last_update_us = now; break; /* demo */
            case 0xCA: g_state.baro_altitude_ft      = arinc_bnr_to_float(w.data, 1.0f,   0.0f); g_state.last_update_us = now; break; /* demo */
            default: break;
        }
        if (++processed > 16) break; /* bound per-activation work */
    }
}

static void TASK_HealthMon(void)
{
    static uint32_t last_ticks;
    uint32_t now_ticks = BM_Ticks();
    if ((now_ticks - last_ticks) > 120) {
        PRINTF("HEALTH: tick slip %lu ms\r\n", now_ticks - last_ticks);
    }
    last_ticks = now_ticks;

    uint16_t depth = (s_rx_w_head >= s_rx_w_tail)
                   ? (s_rx_w_head - s_rx_w_tail)
                   : (RX_WORD_FIFO_LEN - s_rx_w_tail + s_rx_w_head);
    if (depth > (RX_WORD_FIFO_LEN * 3 / 4)) {
        PRINTF("HEALTH: RX FIFO high watermark=%u\r\n", depth);
    }
    /* Feed watchdog here if configured; also toggle a heartbeat GPIO. */
}

static void TASK_MaintLog(void)
{
    PRINTF("LOG: IAS=%.1f kt ALT=%.0f ft @%lu us\r\n",
           (double)g_state.indicated_airspeed_kt,
           (double)g_state.baro_altitude_ft,
           (unsigned long)g_state.last_update_us);
}

/* -------------------- Task registry & boot -------------------- */
static bm_task_t g_tasks[] = {
    { "ARINC_RX_SVC",  TASK_ArincRxSvc,  10,  200,  500, 0,0,0,0,0 },  /* 10 ms, highest priority */
    { "HEALTH_MON",    TASK_HealthMon,  100,  180, 2000, 0,0,0,0,0 },  /* health above parser by policy */
    { "ARINC_PARSE",   TASK_ArincParse,  20,  150, 1000, 0,0,0,0,0 },  /* 20 ms, medium */
    { "MAINT_LOG",     TASK_MaintLog,   500,   50, 3000, 0,0,0,0,0 },  /* 500 ms, low */
};

int main(void)
{
    BM_Sched_Init(1000);          /* 1 kHz tick */
    ARINC_UART_Init(115200U);     /* set to actual ADK-8582 baud */
    BM_Sched_Register(g_tasks, sizeof(g_tasks)/sizeof(g_tasks[0]));
    BM_Sched_Run();               /* never returns */
    while (1) { }
}

## 8) Generic (non‑avionics) example to teach priorities

Design three tasks:

- SENSOR (5 ms, high priority): sample ADC and compute a one‑step filter.

- CONTROL (10 ms, medium priority): compute actuator command using last sensor value.

- LOG (100 ms, low priority): print status.

Inject a temporary 2 ms delay into CONTROL and observe that SENSOR still meets 5 ms releases, while LOG jitters (acceptable). Swap priorities to demonstrate why RMS‑consistent assignments matter.

## 9) Hands‑on exercises (SDK‑based) with acceptance criteria

### Exercise A — Build the cyclic executive and measure jitter

Objective. Compile bm_sched.* and toggle a GPIO at each task start/end to measure execution time and jitter with a logic analyzer.

Steps. 1. Create a new MCUXpresso project; add bm_sched.c/h, arinc_uart.c/h, and main.c. 2. Configure a GPIO as an output (see driver_examples/gpio/); in each task, set it high at entry and low at exit. 3. Capture traces; compute jitter as standard deviation of start offsets from ideal releases.

Acceptance. ARINC_RX_SVC jitter < 50 µs; other tasks within design limits.

### Exercise B — Integrate UART RX ISR with ring buffer

Objective. Wire LPUART to ADK‑8582 and confirm no overrun at nominal baud.

Steps. 1. Choose an LPUART instance not used by the console; adjust IOMUXC pin mux accordingly. 2. Use ARINC_Rx_Drain in ARINC_RX_SVC to drain and reassemble 4‑byte words; print words per second in MAINT_LOG. 3. Increase baud or inject bursts; verify ISR remains short and ring buffer sizing is adequate.

Acceptance. No kLPUART_RxOverrunFlag; RX FIFO < 75% depth during worst‑case bursts.

### Exercise C — Deadline monitoring and BIT (Built‑In Test)

Objective. Add per‑task deadline checks and raise a BIT event on slips >10% of period.

Steps. 1. After each task run, compute: lateness = (finish_us - start_us) - period_us. 2. If lateness > 0.1×period_us, increment a deadline_miss counter and notify HEALTH_MON. 3. On repeated slips, temporarily disable MAINT_LOG to shed load.

Acceptance. Induced parser delays trigger controlled alarms and predictable degradation.

### Exercise D — Priority inversion demo and mitigation (bare metal)

Objective. Demonstrate a low‑priority task holding a shared peripheral delaying a high‑priority task when a medium task is CPU‑busy; fix via time partitioning.

Steps. 1. LOW task holds a pseudo‑bus for 2 ms; HIGH wants it every 1 ms; MEDIUM is CPU heavy at 2 ms. 2. Observe HIGH misses when LOW collides with MEDIUM. 3. Fix: restrict bus usage to a reserved time window (e.g., the 0.5–1.0 ms slot of each 5 ms frame) or shorten the critical section.

Acceptance. HIGH meets its 1 ms deadline post‑fix under the same load.

## 10) Best practices for Airbus‑grade robustness

- No dynamic allocation after init. All buffers and registries are static.

- ISR discipline. ISRs copy bytes and set flags only; avoid parsing/printing inside ISRs.

- Bounded work per activation. Cap processed items (e.g., ≤16 ARINC words per 20 ms).

- NVIC priorities. Assign LPUART RX higher urgency than PIT; document all IRQ priorities in one place.

- Measure everything. Use DWT cycle counter and GPIO toggles to quantify WCET and jitter.

- Time partition shared resources. Prefer fixed windows over ad‑hoc locks in cooperative executives.

- Input validation. Check ARINC parity, SSM (Sign/Status Matrix), SDI (Source/Destination Identifier), label plausibility, and rate limits.

- Fail predictable. On sustained overload, shed non‑critical load (pause logging) before critical tasks suffer.

- Coding standard. Follow MISRA‑C; avoid recursion and unbounded loops.

- Configuration control. Keep a single “Scheduling Table” with task, period, deadline, WCET, priority, and NVIC interactions.

## 11) Scheduling Table template (to maintain under configuration control)

Task Name     Period  Deadline  Priority  WCET(us)  Avg Exec(us)  Jitter(us)  Notes
-------------------------------------------------------------------------------------
ARINC_RX_SVC  10 ms   10 ms     200       300       120           <50         Drains UART ring
HEALTH_MON    100 ms  100 ms    180       500       80            <100        WDT feed, audits
ARINC_PARSE   20 ms   20 ms     150       800       300           <150        Label routing
MAINT_LOG     500 ms  500 ms    50        2000      600           N/A         Background only

## 12) Build & integration notes (MCUXpresso SDK)

- Start from an SDK hello‑world or PIT example and add these files.

- Ensure the chosen LPUART instance is not the debug console. If necessary, move the console to another LPUART or use SWO.

- Configure IOMUXC for the selected UART pins (TX from ADK‑8582 to EVKB RX pin). Keep hardware flow control off unless required by the adapter.

- Keep PIT IRQ priority numerically larger (less urgent) than LPUART RX.

- For lab visibility, enable PRINTF via the SDK debug console and throttle prints (printing is slow).

## 13) Optional advanced: preemption via PendSV (for teaching only)

If you want true task‑level preemption without an RTOS, use the Cortex‑M PendSV exception as a software context switch. SysTick/PIT selects the next runnable task and sets PendSV to perform save/restore of registers and stacks. This is essentially building a micro‑kernel—excellent as a teaching module, but in production prefer a well‑tested RTOS if you need preemption between tasks.

## 14) Troubleshooting

- UART overruns: Lower NVIC priority number for LPUART (increase urgency), increase ring buffer, reduce ISR work, or reduce baud.

- Missed periodic releases: Check PIT clock source and interrupt enable; verify your main loop uses __WFI() and isn’t spinning needlessly.

- Jitter too high: Reduce printing, bound per‑activation work, pre‑compute lookups, and consider moving heavy parsing to slower periods.

- Pin mux issues: Confirm IOMUXC settings and that pins aren’t shared with the debug console.

## 15) What to measure and record for review

- WCET for each task (via DWT cycles → microseconds),

- Jitter of task start times relative to ideal release instants,

- RX ring high‑water mark and word FIFO depth under maximum expected burst,

- IRQ latencies (time from byte arrival to ISR entry),

- Deadline miss counters and recovery behavior.

Maintaining these artifacts alongside the scheduling table supports your safety case and regression testing.