# Middleware Abstraction & Service Layers on i.MX RT1050 (EVKB)

## 1) Why middleware and service layers even on bare metal?

- Bare‑metal means no operating system scheduler: you own the reset vector, startup, interrupts, and the main control loop.

- That does not imply firmware must be a tangle of register pokes.

- A disciplined layered architecture keeps hardware churn contained, enables unit testing, and supports design assurance (DO‑178C) with clear interfaces and traceability.

We will work toward this stack:

Application (mission logic: data acquisition, command/response, health)
└─ Service Layer (logging, time, parameter store, command router, data-bus services)
   └─ Middleware Abstractions (serial port, framed link, checksum, packetizer)
      └─ Drivers/HAL (NXP SDK: fsl_lpuart, fsl_gpio, fsl_iomuxc, clocks)
         └─ BSP (pins, clocks, boot, debug console) + Startup/Linker

Goals

- Isolation: Hardware‑specific code lives behind stable C APIs.

- Determinism: No hidden dynamic allocation or unbounded latency.

- Testability: Middleware/services test on PC with fakes; only driver shim is board‑specific.

- Traceability: Each requirement maps to a function or interface in a designated layer.

## 2) Core definitions (first use expanded)

- BSP (Board Support Package): Board pins, clocks, memory bring‑up, debug console init.

- HAL (Hardware Abstraction Layer): Thin wrappers around registers or vendor drivers, e.g., NXP’s fsl_lpuart.

- Middleware Abstraction: Hardware‑agnostic utilities that compose HAL features (e.g., serial framing with CRC16) and present a clean interface.

- Service Layer: Domain‑oriented components (e.g., ARINC 429 word encode/decode, health monitoring, event logging) that applications call.

- LRU (Line Replaceable Unit): Aircraft hardware module; our firmware should be portable across LRUs that use i.MX RT.

- FCC (Flight Control Computer), FMS (Flight Management System), ADC (Air Data Computer), EICAS (Engine‑Indication and Crew‑Alerting System): Typical avionics participants we will reference in scenarios.

## 3) The EVKB‑IMXRT1050 SDK as the foundation

We base all hardware interactions on the EVKB SDK you provided. The UART driver API we use is from fsl_lpuart.h (e.g., LPUART_Init, LPUART_TransferStartRingBuffer, LPUART_TransferSendNonBlocking, LPUART_TransferReceiveNonBlocking). Board utilities provide BOARD_InitBootPins, BOARD_InitBootClocks, and BOARD_InitDebugConsole. Using these APIs keeps our code aligned with NXP support and example projects under boards/evkbimxrt1050/driver_examples/lpuart/... in the SDK.

Note on instances: We will use the board’s debug LPUART (for logs) and a second LPUART instance for the ARINC converter (ADK‑8582). Pin selection is left in the BSP so the same middleware compiles for different EVKs or carrier cards.

## 4) File‑tree blueprint (drop straight into an SDK project)

src/
  bsp/
    pins.c, pins.h            # IOMUX (fsl_iomuxc), board pin maps
    clocks.c, clocks.h        # BOARD_InitBootClocks(), sources
    board_init.c              # BOARD_InitBootPins/Clocks/DebugConsole
  drivers/
    uart_hw.c, uart_hw.h      # Thin wrapper over fsl_lpuart (HAL)
  middleware/
    serial_port.c, serial_port.h          # Generic serial interface over HAL
    ringbuf.c, ringbuf.h                  # Lockless single-producer/single-consumer
    crc16_ccitt.c, crc16_ccitt.h          # 0x1021 polynomial
    frame_link.c, frame_link.h            # SLIP‑like or custom header+length+CRC
  services/
    log.c, log.h                          # Timestamped records to debug UART
    timebase.c, timebase.h                # SysTick or GPT based ticks
    params.c, params.h                    # Read‑only build‑time parameters
    arinc429.c, arinc429.h                # Pack/unpack 32‑bit A429 words
    a429_service.c, a429_service.h        # High‑level API using UART to ADK‑8582
  app/
    main.c                                # Use services; no direct HAL calls

Each folder compiles as part of your MCUXpresso project; only drivers/ can include fsl_* headers. Everything above relies solely on headers inside our project to enforce separation.

## 5) HAL: a minimal UART shim over fsl_lpuart

Intent: Make a tiny, stable surface that our middleware uses. If we move to a different MCU, only this file changes.

// drivers/uart_hw.h
#pragma once
#include <stddef.h>
#include <stdint.h>
#include <stdbool.h>
#include "fsl_lpuart.h"

typedef struct {
    LPUART_Type *base;
    lpuart_handle_t handle;           // NXP async handle
    uint8_t *rxRing;                  // Provided by caller (middleware)
    size_t rxRingSize;
    volatile bool txBusy;
} uart_hw_t;

// Construction & configuration (no dynamic alloc)
void UART_HW_Init(uart_hw_t *dev, LPUART_Type *base, uint32_t srcClkHz, uint32_t baud);
void UART_HW_AttachRing(uart_hw_t *dev, uint8_t *ring, size_t size);

// Non‑blocking send/receive using NXP transfer APIs
status_t UART_HW_Send(uart_hw_t *dev, const uint8_t *data, size_t len);
status_t UART_HW_Receive(uart_hw_t *dev, uint8_t *dst, size_t len, size_t *got);

// Pump from ISR
void UART_HW_IrqHandler(uart_hw_t *dev);

// drivers/uart_hw.c
#include "uart_hw.h"

static void uart_cb(LPUART_Type *base, lpuart_handle_t *handle, status_t status, void *userData) {
    uart_hw_t *dev = (uart_hw_t*)userData;
    if (status == kStatus_LPUART_TxIdle) dev->txBusy = false;
}

void UART_HW_Init(uart_hw_t *dev, LPUART_Type *base, uint32_t srcClkHz, uint32_t baud) {
    lpuart_config_t cfg;
    LPUART_GetDefaultConfig(&cfg);
    cfg.baudRate_Bps = baud;
    cfg.enableTx = true;
    cfg.enableRx = true;
    dev->base = base;
    dev->txBusy = false;
    LPUART_Init(base, &cfg, srcClkHz);
    LPUART_TransferCreateHandle(base, &dev->handle, uart_cb, dev);
}

void UART_HW_AttachRing(uart_hw_t *dev, uint8_t *ring, size_t size) {
    dev->rxRing = ring; dev->rxRingSize = size;
    LPUART_TransferStartRingBuffer(dev->base, &dev->handle, ring, size);
}

status_t UART_HW_Send(uart_hw_t *dev, const uint8_t *data, size_t len) {
    lpuart_transfer_t x = { .data = (uint8_t*)data, .dataSize = len };
    dev->txBusy = true;
    status_t s = LPUART_TransferSendNonBlocking(dev->base, &dev->handle, &x);
    return s;
}

status_t UART_HW_Receive(uart_hw_t *dev, uint8_t *dst, size_t len, size_t *got) {
    lpuart_transfer_t x = { .rxData = dst, .dataSize = len };
    size_t rec = 0;
    status_t s = LPUART_TransferReceiveNonBlocking(dev->base, &dev->handle, &x, &rec);
    if (got) *got = rec;
    return s; // kStatus_Success even if rec < len (non‑blocking)
}

void UART_HW_IrqHandler(uart_hw_t *dev) {
    LPUART_TransferHandleIRQ(dev->base, &dev->handle);
}

BSP binding: In your app.h, tie the appropriate IRQ to UART_HW_IrqHandler() and provide BOARD_DebugConsoleSrcFreq() for the selected LPUART instance, following the style in SDK examples (boards/evkbimxrt1050/driver_examples/lpuart/interrupt/app.h).

## 6) Middleware #1 — A generic SerialPort facade

Problem: Upper layers should not depend on NXP types.

Solution: Provide a tiny façade with read/write and a poll() hook; implement it over uart_hw_t.

// middleware/serial_port.h
#pragma once
#include <stddef.h>
#include <stdint.h>
#include <stdbool.h>

typedef struct serial_port serial_port_t;

typedef struct {
    size_t (*read)(serial_port_t*, uint8_t *dst, size_t max); // non‑blocking
    size_t (*write)(serial_port_t*, const uint8_t *src, size_t len); // schedule async TX
    void   (*poll)(serial_port_t*); // progress TX queues if needed
} serial_vtbl_t;

struct serial_port { const serial_vtbl_t *v; void *impl; };

// Factory over HAL (implementation in .c)
void SerialPort_fromUart(serial_port_t *sp, void *uart_hw_ptr);

// middleware/serial_port.c
#include "serial_port.h"
#include "uart_hw.h"

static size_t sp_read(serial_port_t* s, uint8_t *dst, size_t max) {
    uart_hw_t *u = (uart_hw_t*)s->impl; size_t got = 0; (void)UART_HW_Receive(u, dst, max, &got); return got;
}
static size_t sp_write(serial_port_t* s, const uint8_t *src, size_t len) {
    uart_hw_t *u = (uart_hw_t*)s->impl; if (UART_HW_Send(u, src, len) != kStatus_Success) return 0; return len;
}
static void sp_poll(serial_port_t* s) { (void)s; /* nothing; async IRQ‑driven */ }

static const serial_vtbl_t V = { sp_read, sp_write, sp_poll };

void SerialPort_fromUart(serial_port_t *sp, void *uart_hw_ptr) { sp->v = &V; sp->impl = uart_hw_ptr; }

## 7) Middleware #2 — Byte ring buffer (for parsing)

Even though the NXP driver offers an RX ring, we keep a light SPSC (single‑producer, single‑consumer) ring for protocol parsing so the framing layer never sees partial multibyte fields.

// middleware/ringbuf.h
#pragma once
#include <stdint.h>
#include <stddef.h>

typedef struct { uint16_t head, tail, cap; uint8_t *buf; } ring8_t;
void rb_init(ring8_t *r, uint8_t *backing, uint16_t cap);
uint16_t rb_avail(const ring8_t *r);
uint16_t rb_space(const ring8_t *r);
uint16_t rb_push(ring8_t *r, const uint8_t *src, uint16_t n);
uint16_t rb_pop(ring8_t *r, uint8_t *dst, uint16_t n);
int rb_peek(const ring8_t *r, uint16_t idx); // -1 if out of range

// middleware/ringbuf.c
#include "ringbuf.h"
void rb_init(ring8_t *r, uint8_t *b, uint16_t c){ r->head=r->tail=0; r->cap=c; r->buf=b; }
uint16_t rb_avail(const ring8_t *r){ return (r->head - r->tail) & (r->cap-1); }
uint16_t rb_space(const ring8_t *r){ return (r->cap-1) - rb_avail(r); }
uint16_t rb_push(ring8_t *r, const uint8_t *s, uint16_t n){ uint16_t k=0; while(k<n && rb_space(r)){ r->buf[r->head++ & (r->cap-1)] = s[k++]; } return k; }
uint16_t rb_pop(ring8_t *r, uint8_t *d, uint16_t n){ uint16_t k=0; while(k<n && rb_avail(r)){ d[k++] = r->buf[r->tail++ & (r->cap-1)]; } return k; }
int rb_peek(const ring8_t *r, uint16_t i){ if(i>=rb_avail(r)) return -1; return r->buf[(r->tail+i) & (r->cap-1)]; }

Determinism: Capacity is a power of two; arithmetic is modulo with masking; functions are O(n) and bounded.

## 8) Middleware #3 — CRC16‑CCITT and framed link

We will use a simple Header + Length + Payload + CRC16 protocol over UART. This is robust on noisy lines and easy to test.

Frame format

Sync:   0xA5 0x5A
Len:    16‑bit little‑endian payload length (N)
Body:   N bytes
CRC16:  CCITT (poly 0x1021, init 0xFFFF) over Len+Body

// middleware/crc16_ccitt.h
#pragma once
#include <stddef.h>
#include <stdint.h>
uint16_t crc16_ccitt(const void *data, size_t len, uint16_t init);

// middleware/crc16_ccitt.c
#include "crc16_ccitt.h"
uint16_t crc16_ccitt(const void *data, size_t len, uint16_t init){
    const uint8_t *p = (const uint8_t*)data; uint16_t crc = init;
    for(size_t i=0;i<len;i++){ crc ^= (uint16_t)p[i] << 8; for(int b=0;b<8;b++){ crc = (crc & 0x8000)? (crc<<1)^0x1021 : (crc<<1); } }
    return crc;
}

// middleware/frame_link.h
#pragma once
#include <stdint.h>
#include <stddef.h>
#include <stdbool.h>
#include "serial_port.h"
#include "ringbuf.h"

typedef struct {
    serial_port_t *sp;
    ring8_t *rx;
} frame_link_t;

void frame_init(frame_link_t *L, serial_port_t *sp, ring8_t *rxbuf);
bool frame_tx(frame_link_t *L, const uint8_t *payload, uint16_t n);
// returns payload length if a valid frame is found; 0 otherwise (non‑blocking)
uint16_t frame_rx_poll(frame_link_t *L, uint8_t *out, uint16_t max);

// middleware/frame_link.c
#include "frame_link.h"
#include "crc16_ccitt.h"

void frame_init(frame_link_t *L, serial_port_t *sp, ring8_t *rx) { L->sp=sp; L->rx=rx; }

bool frame_tx(frame_link_t *L, const uint8_t *p, uint16_t n){
    uint8_t hdr[4] = {0xA5,0x5A,(uint8_t)(n&0xFF),(uint8_t)(n>>8)};
    uint16_t crc = crc16_ccitt(&hdr[2], 2, 0xFFFF); crc = crc16_ccitt(p, n, crc);
    (void)L->sp->v->write(L->sp, hdr, 4);
    (void)L->sp->v->write(L->sp, p, n);
    uint8_t c[2] = {(uint8_t)(crc & 0xFF), (uint8_t)(crc>>8)};
    (void)L->sp->v->write(L->sp, c, 2);
    return true;
}

static int scan_for_sync(const ring8_t *r){
    uint16_t a = rb_avail(r); for(uint16_t i=0;i+1<a;i++){ int b0=rb_peek(r,i); int b1=rb_peek(r,i+1); if(b0==0xA5 && b1==0x5A) return i; }
    return -1;
}

uint16_t frame_rx_poll(frame_link_t *L, uint8_t *out, uint16_t max){
    // Ingest any new bytes from serial into the parsing ring
    uint8_t tmp[64]; size_t got = L->sp->v->read(L->sp, tmp, sizeof tmp);
    if(got){ (void)rb_push(L->rx, tmp, (uint16_t)got); }

    int sync = scan_for_sync(L->rx); if(sync < 0) return 0;
    // Drop junk before sync
    if(sync>0){ uint8_t drop[64]; while(sync){ uint16_t d=(sync>sizeof drop)?sizeof drop:sync; rb_pop(L->rx, drop, d); sync-=d; } }

    if(rb_avail(L->rx) < 4) return 0; // need header+len
    uint8_t header[4]; (void)rb_peek(L->rx,0); // ensure contiguous read
    (void)rb_pop(L->rx, header, 4);
    uint16_t n = (uint16_t)header[2] | ((uint16_t)header[3]<<8);
    if(n > max) { // oversize payload -> resync
        uint8_t dump[2]; rb_pop(L->rx, dump, 2); return 0;
    }
    if(rb_avail(L->rx) < n+2) { // wait for body+CRC
        // Put header back (simple strategy: prepend back)
        // For brevity, not implemented; in practice use a cursor.
        return 0;
    }
    (void)rb_pop(L->rx, out, n);
    uint8_t crcB[2]; (void)rb_pop(L->rx, crcB, 2);
    uint16_t rxCrc = (uint16_t)crcB[0] | ((uint16_t)crcB[1]<<8);
    uint16_t crc = crc16_ccitt(&header[2], 2, 0xFFFF); crc = crc16_ccitt(out, n, crc);
    if(crc != rxCrc) return 0; // bad frame
    return n;
}

## 9) Service #1 — Logging and timebase

Minimal time service using SysTick (or GPT timer if higher resolution is required). Logs go to the debug UART through the same serial_port_t API so the app never touches fsl_debug_console.

// services/timebase.h
#pragma once
#include <stdint.h>
void timebase_init(uint32_t cpuHz); // config SysTick to 1 kHz
uint32_t time_ms(void);

// services/timebase.c
#include "timebase.h"
#include "fsl_common.h"
#include "fsl_device_registers.h"

static volatile uint32_t g_ms;
void SysTick_Handler(void){ g_ms++; }
void timebase_init(uint32_t cpuHz){ SysTick->LOAD = (cpuHz/1000u) - 1u; SysTick->VAL=0; SysTick->CTRL = SysTick_CTRL_CLKSOURCE_Msk|SysTick_CTRL_TICKINT_Msk|SysTick_CTRL_ENABLE_Msk; }
uint32_t time_ms(void){ return g_ms; }

// services/log.h
#pragma once
#include <stdarg.h>
#include "serial_port.h"
void log_bind(serial_port_t *sp);
void log_printf(const char *fmt, ...);

// services/log.c
#include "log.h"
#include <stdio.h>
#include <string.h>
#include "timebase.h"
static serial_port_t *g_log;
void log_bind(serial_port_t *sp){ g_log = sp; }
void log_printf(const char *fmt, ...){ if(!g_log) return; char buf[160]; va_list ap; va_start(ap,fmt); int n=vsnprintf(buf,sizeof buf,fmt,ap); va_end(ap); if(n<0) return; char line[192]; int m=snprintf(line,sizeof line,"[%6lu ms] %.*s\r\n", (unsigned long)time_ms(), n, buf); (void)g_log->v->write(g_log,(const uint8_t*)line,(size_t)m); }

## 10) Service #2 — ARINC 429 word utilities

ARINC 429 encodes aviation parameters in a 32‑bit word over a unidirectional, self‑clocking bus. We will not implement the physical layer here (that’s the ADK‑8582), but we will encode/decode words:

- Label: 8 bits (bit numbers 1–8 in ARINC notation; customarily written in octal).

- SDI (Source/Destination Identifier): 2 bits (bits 9–10).

- Data: 19 bits (bits 11–29), interpretation BNR (Binary Number Representation) or BCD (Binary‑Coded Decimal).

- SSM (Sign/Status Matrix): 2 bits (bits 30–31).

- Parity: 1 bit (bit 32), odd parity over bits 1–31.

// services/arinc429.h
#pragma once
#include <stdint.h>

typedef struct { uint8_t label; uint8_t sdi; uint32_t data; uint8_t ssm; } a429_fields_t;
uint32_t a429_make_word(a429_fields_t f);  // returns 32-bit word (bit 0 = LSB)
int a429_parse_word(uint32_t word, a429_fields_t *out);
uint32_t a429_bnr_from_float(float value, float lsb, unsigned width /*<=19*/);
float    a429_bnr_to_float(uint32_t data, float lsb, unsigned width);

// services/arinc429.c
#include "arinc429.h"

static uint8_t odd_parity32(uint32_t v){ // parity over 31 bits (exclude bit 31: parity)
    v ^= v>>16; v ^= v>>8; v ^= v>>4; v ^= v>>2; v ^= v>>1; return (uint8_t)(~v & 1u);
}

uint32_t a429_make_word(a429_fields_t f){
    uint32_t word = 0;
    word |= ((uint32_t)(f.label & 0xFF)) << 0;       // bits 0..7
    word |= ((uint32_t)(f.sdi   & 0x3))  << 8;       // bits 8..9
    word |= ((uint32_t)(f.data  & 0x7FFFF)) << 10;   // bits 10..28
    word |= ((uint32_t)(f.ssm   & 0x3))  << 29;      // bits 29..30
    // bit 31 is parity (odd over bits 0..30)
    uint32_t p = odd_parity32(word);
    word |= p << 31;
    return word;
}

int a429_parse_word(uint32_t w, a429_fields_t *o){
    uint8_t p = (uint8_t)((w>>31)&1u); uint8_t op = odd_parity32(w & 0x7FFFFFFFu);
    if(p != op) return -1; // parity error
    o->label = (uint8_t)((w>>0)&0xFF);
    o->sdi   = (uint8_t)((w>>8)&0x3);
    o->data  = (uint32_t)((w>>10)&0x7FFFF);
    o->ssm   = (uint8_t)((w>>29)&0x3);
    return 0;
}

uint32_t a429_bnr_from_float(float value, float lsb, unsigned width){
    int32_t q = (int32_t)(value/lsb);
    uint32_t mask = (1u<<width)-1u; return (uint32_t)q & mask;
}
float a429_bnr_to_float(uint32_t data, float lsb, unsigned width){
    // Sign‑extend from width bits
    if(width<32){ uint32_t sign=1u<<(width-1u); data = (data ^ sign) - sign; }
    return (float)((int32_t)data) * lsb;
}

Note: Bit numbering here is least‑significant bit = 0 in C. This maps cleanly to ARINC 429 bits 1..32 when you remember that ARINC’s bit 1 is the LSB of the label field.

## 11) Service #3 — ARINC 429 over UART to ADK‑8582

The ADK‑8582 provides physical ARINC 429 TX/RX. We assume a UART command channel to instruct TX and receive RX captures. We encode our own framed protocol (Section 8) that carries ARINC words so the solution is portable across external adapters.

Command payloads - CMD_TX (0x01): { label(1) sdi(1) data(3) ssm(1) repeat(1) gap_ms(2) } - CMD_RX_SUB (0x02): subscribe to words by label: { count(1) labels[count] } - EV_RX (0x81): device→host indication: { label(1) sdi(1) data(3) ssm(1) }

// services/a429_service.h
#pragma once
#include <stdint.h>
#include <stdbool.h>
#include "frame_link.h"
#include "arinc429.h"

typedef struct { frame_link_t *link; } a429_service_t;
void a429_service_init(a429_service_t *S, frame_link_t *L);
bool a429_tx(a429_service_t *S, a429_fields_t f, uint8_t repeat, uint16_t gap_ms);
int  a429_poll(a429_service_t *S, a429_fields_t *out); // returns 1 if a word received

// services/a429_service.c
#include "a429_service.h"
#include <string.h>

void a429_service_init(a429_service_t *S, frame_link_t *L){ S->link=L; }

bool a429_tx(a429_service_t *S, a429_fields_t f, uint8_t repeat, uint16_t gap_ms){
    uint8_t p[1+1+1+3+1+1+2]; size_t i=0;
    p[i++]=0x01; // CMD_TX
    p[i++]=f.label; p[i++]=f.sdi; p[i++]=(uint8_t)(f.data&0xFF); p[i++]=(uint8_t)((f.data>>8)&0xFF); p[i++]=(uint8_t)((f.data>>16)&0x7F);
    p[i++]=f.ssm; p[i++]=repeat; p[i++]=(uint8_t)(gap_ms&0xFF); p[i++]=(uint8_t)(gap_ms>>8);
    return frame_tx(S->link, p, (uint16_t)i);
}

int a429_poll(a429_service_t *S, a429_fields_t *out){
    uint8_t b[16]; uint16_t n = frame_rx_poll(S->link, b, sizeof b);
    if(n>=1 && b[0]==0x81 && n>=1+1+1+3+1){
        out->label=b[1]; out->sdi=b[2]; out->data=(uint32_t)b[3]|((uint32_t)b[4]<<8)|((uint32_t)(b[5]&0x7F)<<16); out->ssm=b[6]; return 1;
    }
    return 0;
}

## 12) Application example A — Generic framed link: ping/pong

Scenario: During integration, you connect the EVKB to a PC or a second EVKB via UART. You want a deterministic, framed echo test with CRC. This validates the middleware stack without ARINC hardware.

// app/main.c (generic demo)
#include "board.h"
#include "clock_config.h"
#include "pin_mux.h"
#include "uart_hw.h"
#include "serial_port.h"
#include "ringbuf.h"
#include "frame_link.h"
#include "timebase.h"
#include "log.h"

#define RING_CAP 256
static uint8_t rx_storage[RING_CAP]; static ring8_t rx;
static uart_hw_t uart_debug_hw; static serial_port_t sp_debug;
static frame_link_t link;

int main(void){
    BOARD_InitBootPins(); BOARD_InitBootClocks();
    // Debug console clock for chosen LPUART instance
    uint32_t uartClk = BOARD_DebugConsoleSrcFreq();

    UART_HW_Init(&uart_debug_hw, LPUART1, uartClk, 115200);
    UART_HW_AttachRing(&uart_debug_hw, rx_storage, RING_CAP);
    SerialPort_fromUart(&sp_debug, &uart_debug_hw);

    rb_init(&rx, rx_storage, RING_CAP); frame_init(&link, &sp_debug, &rx);
    timebase_init(SystemCoreClock); log_bind(&sp_debug);
    log_printf("Booted framed‑link ping demo");

    const uint8_t ping[] = { 'P','I','N','G' };
    uint32_t t0 = time_ms();
    while(1){
        // send a ping every 1000 ms
        if((time_ms()-t0)>=1000){ frame_tx(&link, ping, sizeof ping); t0 = time_ms(); }
        // if a frame arrives, echo it back
        uint8_t buf[64]; uint16_t n = frame_rx_poll(&link, buf, sizeof buf);
        if(n){ log_printf("RX %u bytes", (unsigned)n); frame_tx(&link, buf, n); }
    }
}

Expected behavior: A PC tool or second board receives a ping frame every second and echoes back. CRC mismatches are dropped silently, demonstrating robustness to noise and framing loss.

## 13) Application example B — Avionics ARINC 429 via UART (ADK‑8582)

Scenario: A Display Unit (DU) needs Indicated Airspeed (IAS) from an Air Data Computer (ADC) on ARINC 429 at 100 kHz. Our EVKB acts as a gateway: it receives IAS words from the ADK‑8582 (as UART events) and republishes them to another LRUs’ maintenance port while also periodically transmitting a Baro‑Corrected Altitude word.

Data: - IAS label commonly 0x174 (octal 174, decimal 124). We will use label 0174 (octal) → binary 0x74. - Altitude (feet) can be encoded as BNR with LSB = 1 ft.

// app/main.c (ARINC service demo)
#include "..." // same includes as generic demo + a429_service.h, arinc429.h

static uint8_t a429_rx_store[512]; static ring8_t a429_rx_ring;
static uart_hw_t uart_a429_hw; static serial_port_t sp_a429;
static frame_link_t a429_link; static a429_service_t a429;

int main(void){
    BOARD_InitBootPins(); BOARD_InitBootClocks();
    uint32_t clk = BOARD_DebugConsoleSrcFreq();

    // Debug UART for logs
    UART_HW_Init(&uart_debug_hw, LPUART1, clk, 115200); UART_HW_AttachRing(&uart_debug_hw, rx_storage, RING_CAP); SerialPort_fromUart(&sp_debug, &uart_debug_hw); log_bind(&sp_debug);

    // Second UART connected to ADK‑8582
    UART_HW_Init(&uart_a429_hw, LPUART3 /*example*/, clk, 115200); UART_HW_AttachRing(&uart_a429_hw, a429_rx_store, sizeof a429_rx_store); SerialPort_fromUart(&sp_a429, &uart_a429_hw);

    rb_init(&a429_rx_ring, a429_rx_store, sizeof a429_rx_store); frame_init(&a429_link, &sp_a429, &a429_rx_ring); a429_service_init(&a429, &a429_link);

    timebase_init(SystemCoreClock);
    log_printf("ARINC 429 gateway starting");

    // Periodically transmit baro altitude (example: 12345 ft)
    a429_fields_t tx; tx.label = 0xA1; /* example label for altitude */ tx.sdi=0; tx.ssm=0;
    tx.data = a429_bnr_from_float(12345.0f, 1.0f, 19);

    uint32_t tTx = time_ms();
    while(1){
        if(time_ms()-tTx >= 100){ a429_tx(&a429, tx, /*repeat*/1, /*gap_ms*/0); tTx = time_ms(); }
        a429_fields_t rx;
        if(a429_poll(&a429, &rx)){
            if(rx.label == 0174 /* IAS */){
                float ias = a429_bnr_to_float(rx.data, /*LSB*/0.5f, /*width*/19); // example scale
                log_printf("IAS=%.1f kt (SDI=%u, SSM=%u)", ias, rx.sdi, rx.ssm);
            }
        }
    }
}

What this demonstrates: - The service layer offers a business‑level API (a429_tx, a429_poll). No driver types or registers leak upward. - Swapping UART instance or baudrate is a BSP concern; the service and middleware remain unchanged.

Integration note: Adapt labels/SSM/BNR scaling to your aircraft ICD (Interface Control Document). The ADK‑8582’s exact UART protocol can be fit behind the a429_service.c payload rules without changing application code.

## 14) Hands‑on lab sequence (with solutions)

### Lab 1 — Bring‑up and serial abstraction

- Start from the SDK’s lpuart/interrupt example. Create a new project and copy drivers/uart_hw.* and middleware/serial_port.*.

- In main.c, initialize LPUART1 with BOARD_DebugConsoleSrcFreq() and bind a serial_port_t.

- Verify you can sp_debug.v->write() a banner and see it on a terminal at 115200‑8‑N‑1.

Solution highlight: LPUART_GetDefaultConfig, LPUART_Init, LPUART_TransferCreateHandle, LPUART_TransferStartRingBuffer are used exactly as in the SDK examples.

### Lab 2 — CRC16 and framed link

- Add crc16_ccitt.* and frame_link.*. Use the generic demo.

- With a PC tool, inject intentionally corrupted frames; verify they are discarded (no crash, no echo).

Solution highlight: Frame verification compares computed CRC to the received trailer before accepting payload.

### Lab 3 — ARINC 429 utilities

- Add arinc429.*; unit‑test a429_make_word and a429_parse_word on the host (compile with -DUNIT_TEST replacing main).

- Verify parity errors are detected. Confirm BNR scaling round‑trips for a few values.

Solution highlight: Parity is odd over bits 0..30 in the C representation.

### Lab 4 — A429 service over UART

- Add a429_service.* and connect the EVKB UART to the ADK‑8582.

- Subscribe to a label (extend a429_service.c with a CMD_RX_SUB).

- Log received words; format the label in octal to match avionics ICDs.

Solution highlight: Only a429_service.c knows the UART payload schema; app logic stays invariant.

### Lab 5 — Flight‑realistic scenario

Scenario: The FCC (Flight Control Computer) requires IAS at 50 Hz and altitude at 10 Hz. Implement two periodic jobs in the main loop driven by time_ms(). On RX of IAS, compute trend (ΔIAS/Δt) and log an amber caution if trend > 10 kt/s while flaps are extended (simulate flap input via GPIO).

Solution approach: Two timers, a simple FIR filter for the trend, GPIO read via fsl_gpio. No dynamic memory.

### Lab 6 — Robustness and recovery

- Force UART overflow (momentarily stop servicing RX). Confirm the frame parser resynchronizes.

- Toggle the ADK‑8582 power: detect link loss (no frames for 200 ms), raise an SSM = Failure for published parameters.

Solution highlight: Add a watchdog timer and a link‑timeout finite‑state machine in the service.

## 15) Best practices (avionics‑aimed)

- Layer contracts in headers: Every *.h carries a brief contract and constraints (blocking behavior, maximum sizes, timing guarantees). This feeds DO‑178C low‑level requirements.

- Immutable configuration: Keep baud, pin maps, and label tables in const structs placed in a dedicated linker section; version them in the build.

- No hidden allocation: All buffers are static; sizes are ICD‑driven and justified. Provide watermarks and asserted capacities.

- Time‑bounded operations: Any loop that waits on hardware has a timeout or is driven by interrupts. Document worst‑case latencies.

- MISRA‑C compliance: Stay within MISRA‑C:2012; document deviations (e.g., use of uint8_t unions) with rationale.

- Unit tests on middleware/services: Build the same code for host (-DHOST) with fakes for serial_port_t to exercise framing and A429 packing.

- Error taxonomy: Convert driver status_t to project‑wide error codes; never leak vendor types across layers.

- Traceability: Map each safety requirement to a function; embed requirement IDs as comments that tools can scrape.

- Diagnostics: Provide a structured health page over the framed link: firmware version, uptime, RX/TX counters, last CRC error timestamp.

- Watchdog and brownout: Enable IWDT or software watchdog; on brownout events, fail safe and log SSM = Failure.

- Interrupt hygiene: Keep ISRs short; defer parsing to the main loop; guard shared state with atomic flags or disable/enable IRQ only around single writes.

- Deterministic logging: Rate‑limit logs; avoid printf floating formats in tight ISRs; prefer preformatted integers.

## 16) Traceability matrix snapshot (example)

| Requirement | Layer | Artifact | Verification |
| --- | --- | --- | --- |
| R‑UART‑001: System shall transmit framed packets with CRC | Middleware | frame_link.c | Unit test: inject bit errors; observe discard |
| R‑A429‑002: System shall detect parity errors | Service | arinc429.c | Unit test with flipped parity bit |
| R‑LINK‑003: System shall resync within 200 ms after noise burst | Middleware | frame_rx_poll | HIL test: inject noise via PC tool |

## 17) What to adapt for your aircraft program

- Labels & scaling: Replace example labels (e.g., IAS 0174) and BNR LSBs per your ICD. Provide a const lookup to scale and SSM semantics.

- Pin muxing: Select LPUART instance for ADK‑8582 in pins.c only; keep services middleware‑pure.

- Rates: If you must guarantee, e.g., 100 Hz TX with 100 µs jitter, move frame_tx onto EDMA‑driven UART TX and gate by a hardware timer ISR; the service API stays unchanged.

## 18) Checklist before flight‑test

- Builds are reproducible (compiler version, linker script, -fno‑common, -fstack‑protector as applicable).

- Stack usage measured worst‑case (map file + test harness); buffer sizes justified.

- Every ISR execution time measured and logged under worst case.

- Fault injection performed: CRC errors, UART overflow, parity errors, cable disconnects.

- All acronyms expanded at first use in the documentation; ICD references embedded in code comments.

## 19) Appendix — Minimal BSP snippets

// bsp/board_init.c
#include "board.h"
void BOARD_InitHardware(void){ BOARD_InitBootPins(); BOARD_InitBootClocks(); BOARD_InitDebugConsole(); }

// bsp/pins.c (sketch)
#include "fsl_iomuxc.h"
void BOARD_InitPins(void){
    // Configure pins for LPUART3 RX/TX as needed for ADK‑8582 connection
    // IOMUXC_SetPinMux(...);
    // IOMUXC_SetPinConfig(...);
}

Keep all IOMUX and clock specifics in BSP. Higher layers remain untouched if the UART instance or pins change.

## 20) Summary

We now have a layered, cert‑friendly firmware template: drivers → middleware → services → application, demonstrated with a robust framed UART link and a realistic ARINC 429 gateway scenario using the EVKB‑IMXRT1050 SDK APIs. Extend the service layer to add label filtering, SSM rules, and maintenance commands without touching drivers or application logic.