# Task Registration & Scheduling — Application Task Lifecycle (i.MX RT1050‑EVKB)

## 1) Why this topic matters (avionics context)

- In avionics, especially for DAL (Design Assurance Level) A/B functions under DO‑178C (Software Considerations in Airborne Systems and Equipment Certification), every application task must have a predictable lifecycle.

- From power‑up to shutdown, we need to know when a task starts, how often it runs, how long it takes (its WCET — Worst‑Case Execution Time), and what happens if it overruns.

- On the i.MX RT1050‑EVKB (NXP i.MX RT1052, Arm® Cortex‑M7), we can build a robust, traceable task lifecycle with bare‑metal code using the MCUXpresso SDK and a thin cooperative scheduler.

- This module walks you from fundamentals to advanced concepts with realistic avionics use cases, including ARINC 429 (Aeronautical Radio, Incorporated 429) receive/transmit flows via ADK‑8582 interfaced over UART (Universal Asynchronous Receiver/Transmitter).

## 2) Essential definitions (expanded on first use)

- Task: A unit of work with a defined purpose, period, priority, inputs, outputs, and timing budget. In bare metal, a task is typically a C function plus metadata (period, state, etc.).

- ISR (Interrupt Service Routine): A short, bounded‑time routine that handles asynchronous events (e.g., timer tick, UART RX). ISRs should defer non‑urgent work to tasks.

- Scheduler: Code that decides which task runs when. We will implement a TTC (Time‑Triggered Cooperative) scheduler; you will also learn how to extend it toward fixed‑priority preemption later.

- Period/Frequency: How often a periodic task is released (e.g., 10 ms period = 100 Hz).

- Release: The instant a task becomes eligible to run. In TTC, tasks run when released and the CPU is free.

- Deadline: Latest acceptable completion time for a task instance.

- WCET (Worst‑Case Execution Time): Upper bound on the task’s execution time.

- Jitter: Variation in task start/finish time around the nominal schedule.

- Sporadic task: Runs on demand, with a minimum inter‑arrival time (e.g., fault reporter).

- Aperiodic task: Runs when an event occurs without a guaranteed spacing (e.g., maintenance command).

- Overrun: When a task has not finished before the next release time.

## 3) Platform specifics you will use

- CPU: Arm Cortex‑M7 @ up to 600 MHz (i.MX RT1050).

- Tick source: SysTick timer (1 ms tick) from Arm core.

- SDK: MCUXpresso SDK (file provided: SDK_25_06_00_EVKB-IMXRT1050.zip). We will reuse board init, clock init, pin mux, and the debug console.

- I/O for labs:

  - Debug UART via BOARD_InitDebugConsole() (LPUART1) — for logs (PRINTF).

  - User LED BOARD_USER_LED_GPIO/BOARD_USER_LED_GPIO_PIN — for timing visibility.

  - ADK‑8582 on a separate LPUART instance (e.g., LPUART3) — for ARINC 429 bridging exercises. You will map pins in pin_mux.c with MCUXpresso ConfigTools.

Tip: All pin/clock/console macros are already present in the SDK’s board.h within each example; we will follow the SDK patterns to stay aligned with your codebase.

## 4) Application Task Lifecycle (concept & state model)

A robust lifecycle enforces determinism and traceability:

States

1. UNREGISTERED → not known to the scheduler.

2. REGISTERED → in scheduler table with metadata.

3. INITIALIZED → init() ran once; resources allocated.

4. READY → released by time or event, waiting to run.

5. RUNNING → run() executing to completion.

6. SUSPENDED → temporarily disabled (e.g., degraded mode, missing dependency).

7. ERROR → faulted; requires recovery policy.

Transitions - Power‑up: UNREGISTERED → REGISTERED → INITIALIZED (during system bring‑up). - Scheduler tick release: INITIALIZED/READY → READY when now >= next_release. - Dispatcher: READY → RUNNING → READY (on success) or → ERROR (on failure/overrun policy). - Mode control: READY/RUNNING → SUSPENDED (crew command/degraded mode) and back.

Artefacts per task - Name, init(void*), run(void*), context pointer, period (ms), priority (for tie‑breakers), next release time, counters (overruns/executions), and state.

## 5) Registration patterns on bare metal

### 5.1 Static table (DAL‑friendly)

Define tasks at compile time in a constant table. Benefits: traceability, no heap use, deterministic footprint, easy certification evidence.

### 5.2 Dynamic registration (careful)

Allow (de)registration at runtime from a bounded static pool. You must prove boundedness: fixed pool size, strict error handling if pool is full, time‑bounded list operations, no heap.

In this course, we implement both. The production default is static table; dynamic is used only for mode switching or optional features.

## 6) Scheduling models (what we implement & why)

- Super‑loop: simplest; no explicit timing — not sufficient for avionics periodicity.

- TTC (Time‑Triggered Cooperative): periodic scheduler driven by SysTick; tasks run to completion and must be short. Very deterministic, certification‑friendly. We implement this.

- Fixed‑priority preemptive (e.g., Rate Monotonic — RM): tighter latency for high‑rate tasks but added complexity (context switch, stack per task). Optional extension is outlined for later.

- Time partitioning (ARINC 653 style): generally done with an RTOS/hypervisor; out of scope for this bare‑metal module, but concepts (windows, budgets) are mirrored in our TTC parameters.

## 7) Avionics use case (realistic scenario)

Scenario: A Data Concentrator Unit (DCU) ingests ARINC 429 high‑speed (100 kbps) words from multiple sensors via an ADK‑8582 bridge on UART, performs integrity checks, and republishes selected parameters. Typical labels include Airspeed (label 203), Altitude (label 204), Angle of Attack (label 102). The DCU must:

- Receive ARINC 429 via ADK‑8582 UART stream, parse to structured words, time‑stamp, and write to a shared database.

- Run periodic monitors (e.g., BIT — Built‑In Test; HM — Health Monitoring) every 100 ms.

- Republish computed data at 20 Hz (every 50 ms) to a secondary link or log.

- Degrade gracefully: if the UART link is quiet for >500 ms, suspend the ARINC tasks and raise a caution.

Mapped tasks (periods chosen to match typical update rates): - arinc429_rx_task — 1 ms period (budget 50 µs): pull bytes from UART ring buffer, frame words, enqueue. - arinc429_tx_task — 50 ms period: package selected labels for transmission (or echo). - bit_100ms_task — 100 ms: BIT/HM metrics. - telemetry_task — 200 ms: log counters via debug console. - led_hb_task — 1000 ms: heartbeat LED.

## 8) Hands‑on — build a TTC scheduler on i.MX RT1050

All code below follows the MCUXpresso SDK patterns. Start from the SDK hello_world or led_blinky example to inherit board.c, board.h, pin_mux.c, clock_config.c, and BOARD_InitDebugConsole().

### 8.1 File: scheduler.h

#ifndef SCHEDULER_H
#define SCHEDULER_H

#include <stdint.h>
#include <stdbool.h>

typedef enum {
    TASK_UNREGISTERED = 0,
    TASK_REGISTERED,
    TASK_INITIALIZED,
    TASK_READY,
    TASK_RUNNING,
    TASK_SUSPENDED,
    TASK_ERROR
} task_state_t;

typedef struct task_tag {
    const char *name;
    void (*init)(void *ctx);
    void (*run)(void *ctx);
    void *ctx;
    uint32_t period_ms;      // 0 for aperiodic; use event to release
    uint32_t next_release_ms;
    uint8_t priority;        // 0 = highest
    volatile task_state_t state;
    volatile uint32_t executions;
    volatile uint32_t overruns;
} task_t;

void scheduler_init(uint32_t tick_hz);
bool task_register(task_t *t);
void task_suspend(task_t *t);
void task_resume(task_t *t, uint32_t delay_ms);
void scheduler_run(void);
void scheduler_tick_isr(void); // call from SysTick_Handler
uint32_t scheduler_now_ms(void);

#endif // SCHEDULER_H

### 8.2 File: scheduler.c

#include "scheduler.h"
#include "fsl_common.h"
#include "fsl_debug_console.h"
#include <string.h>

#ifndef MAX_TASKS
#define MAX_TASKS 16
#endif

static volatile uint32_t g_ms_ticks = 0;
static task_t *g_tasks[MAX_TASKS];
static uint32_t g_task_count = 0;

static inline int cmp_prio(const task_t *a, const task_t *b) {
    return (a->priority < b->priority) ? -1 : (a->priority > b->priority);
}

void scheduler_init(uint32_t tick_hz)
{
    SystemCoreClockUpdate();
    (void)tick_hz; // we use fixed 1 ms below for simplicity
    if (SysTick_Config(SystemCoreClock / 1000U)) {
        while (1) { /* error */ }
    }

    // Enable DWT cycle counter for timing (optional but useful)
    CoreDebug->DEMCR |= CoreDebug_DEMCR_TRCENA_Msk;
    DWT->CYCCNT = 0;
    DWT->CTRL |= DWT_CTRL_CYCCNTENA_Msk;
}

bool task_register(task_t *t)
{
    if ((t == NULL) || (t->run == NULL)) return false;
    if (g_task_count >= MAX_TASKS) return false;

    t->state = TASK_REGISTERED;
    t->executions = 0;
    t->overruns = 0;
    t->next_release_ms = g_ms_ticks + t->period_ms; // first release after one period

    if (t->init) {
        t->init(t->ctx);
    }
    t->state = TASK_INITIALIZED;

    g_tasks[g_task_count++] = t;
    return true;
}

void task_suspend(task_t *t)
{
    if (t) t->state = TASK_SUSPENDED;
}

void task_resume(task_t *t, uint32_t delay_ms)
{
    if (!t) return;
    t->next_release_ms = g_ms_ticks + delay_ms;
    t->state = TASK_INITIALIZED; // will move to READY when released
}

void scheduler_tick_isr(void)
{
    g_ms_ticks++;
}

uint32_t scheduler_now_ms(void)
{
    return g_ms_ticks;
}

void scheduler_run(void)
{
    for (;;) {
        uint32_t now = g_ms_ticks;
        task_t *best = NULL;

        // Pick the highest priority READY task whose release time has arrived
        for (uint32_t i = 0; i < g_task_count; i++) {
            task_t *t = g_tasks[i];
            if (!t) continue;
            if (t->state == TASK_SUSPENDED || t->state == TASK_ERROR) continue;

            if (t->period_ms == 0U) {
                // aperiodic: only runs when someone sets next_release_ms == now
            }

            if (now >= t->next_release_ms) {
                // Make it READY implicitly for cooperative run
                if (best == NULL) {
                    best = t;
                } else {
                    // choose by priority, then by earliest next_release
                    if (cmp_prio(t, best) < 0 ||
                        (t->priority == best->priority && t->next_release_ms < best->next_release_ms)) {
                        best = t;
                    }
                }
            }
        }

        if (best) {
            // Dispatch
            uint32_t start = DWT->CYCCNT;
            best->state = TASK_RUNNING;
            best->run(best->ctx);
            best->executions++;
            best->state = TASK_READY;

            // Schedule next release
            uint32_t expected = best->next_release_ms + best->period_ms;
            best->next_release_ms = expected;

            // Overrun accounting: if we are already past next release, fix up
            if (g_ms_ticks > expected) {
                best->overruns++;
                // catch-up: push to the next boundary after now
                uint32_t late = g_ms_ticks - expected;
                uint32_t periods_missed = 1U + late / (best->period_ms ? best->period_ms : 1U);
                best->next_release_ms = expected + periods_missed * best->period_ms;
            }

            (void)start; // available for timing prints if needed
        } else {
            __WFI(); // save power; wake on next tick/interrupt
        }
    }
}

### 8.3 File: app_tasks.c — example tasks (LED, telemetry, button)

#include "board.h"
#include "fsl_debug_console.h"
#include "fsl_gpio.h"
#include "scheduler.h"

// --- LED heartbeat (1 Hz) ---
static void led_init(void *ctx)
{
    (void)ctx;
    USER_LED_INIT(LOGIC_LED_OFF);
}

static void led_run(void *ctx)
{
    (void)ctx;
    USER_LED_TOGGLE();
}

// --- Telemetry: print counters every 200 ms ---
extern task_t t_led, t_telemetry; // declared in main.c or app_tasks.h

static void telemetry_init(void *ctx) { (void)ctx; }

static void telemetry_run(void *ctx)
{
    (void)ctx;
    static uint32_t hb = 0;
    PRINTF("[telemetry] tick=%lu led.exec=%lu led.ovr=%lu\r\n",
           (unsigned long)scheduler_now_ms(),
           (unsigned long)t_led.executions,
           (unsigned long)t_led.overruns);
    if (++hb % 5U == 0U) {
        PRINTF("[telemetry] alive\r\n");
    }
}

// --- Button sampler (10 ms) ---
#ifndef BOARD_USER_BUTTON_GPIO
#define BOARD_USER_BUTTON_GPIO GPIO5
#define BOARD_USER_BUTTON_GPIO_PIN 0U
#endif

static void button_init(void *ctx)
{
    (void)ctx;
    // Input by default via pin mux; ensure clock to GPIO enabled in board init
}

static void button_run(void *ctx)
{
    (void)ctx;
    static uint8_t stable = 1U;
    static uint8_t last = 1U;
    uint8_t now = (uint8_t)GPIO_PinRead(BOARD_USER_BUTTON_GPIO, BOARD_USER_BUTTON_GPIO_PIN);
    if (now == last) {
        if (stable != now) {
            stable = now;
            PRINTF("[button] state=%u at %lu ms\r\n", (unsigned)stable, (unsigned long)scheduler_now_ms());
        }
    }
    last = now;
}

// Public task descriptors
#include <stddef.h>

task_t t_led       = { .name = "led",       .init = led_init,       .run = led_run,       .ctx = NULL, .period_ms = 1000U, .priority = 3 };
task_t t_telemetry = { .name = "telemetry", .init = telemetry_init, .run = telemetry_run, .ctx = NULL, .period_ms = 200U,  .priority = 4 };
task_t t_button    = { .name = "button",    .init = button_init,    .run = button_run,    .ctx = NULL, .period_ms = 10U,   .priority = 2 };

### 8.4 File: main.c — bring‑up & run scheduler

#include "board.h"
#include "pin_mux.h"
#include "clock_config.h"
#include "fsl_debug_console.h"
#include "scheduler.h"

// Declarations from app_tasks.c
extern task_t t_led, t_telemetry, t_button;

void SysTick_Handler(void)
{
    scheduler_tick_isr();
}

int main(void)
{
    BOARD_InitBootPins();
    BOARD_InitBootClocks();
    BOARD_InitDebugConsole(); // LPUART1

    PRINTF("\r\nIMXRT1050 TTC scheduler demo\r\n");

    scheduler_init(1000U); // 1 kHz tick

    (void)task_register(&t_button);
    (void)task_register(&t_led);
    (void)task_register(&t_telemetry);

    scheduler_run(); // never returns
}

What you should see - LED toggles once per second. - UART (115200 8N1 on LPUART1) prints telemetry every 200 ms and button events when pressed.

All macros such as USER_LED_*, BOARD_InitBoot*, and debug console are provided by the SDK’s board.h in the EVKB examples you started from.

## 9) ARINC 429 integration via ADK‑8582 (UART) — tasks & skeleton

The ADK‑8582 is attached to a separate UART (e.g., LPUART3) so we do not interfere with the debug console on LPUART1. Configure TX/RX pins for LPUART3 in pin_mux.c using MCUXpresso ConfigTools. Keep DMA optional; interrupt‑driven ring buffer is sufficient at 115200 bps.

### 9.1 UART driver glue (ring buffer) — adk_uart.c

#include "board.h"
#include "fsl_lpuart.h"

#ifndef ADK_LPUART
#define ADK_LPUART LPUART3
#endif

#ifndef ADK_LPUART_CLK_FREQ
#define ADK_LPUART_CLK_FREQ CLOCK_GetFreq(kCLOCK_UartClk) // or BOARD_DebugConsoleSrcFreq() if routed similarly
#endif

#ifndef ADK_BAUD
#define ADK_BAUD 115200U
#endif

#define RX_RING_BUFFER_SIZE 256U

static lpuart_handle_t s_handle;
static uint8_t s_rx_ring[RX_RING_BUFFER_SIZE];

void ADK_UART_Init(void)
{
    lpuart_config_t cfg;
    LPUART_GetDefaultConfig(&cfg);
    cfg.baudRate_Bps = ADK_BAUD;
    cfg.enableRx = true;
    cfg.enableTx = true;
    LPUART_Init(ADK_LPUART, &cfg, ADK_LPUART_CLK_FREQ);
    LPUART_TransferCreateHandle(ADK_LPUART, &s_handle, NULL, NULL);
    LPUART_TransferStartRingBuffer(ADK_LPUART, &s_handle, s_rx_ring, RX_RING_BUFFER_SIZE);
}

size_t ADK_UART_Read(uint8_t *dst, size_t maxLen)
{
    size_t n = 0;
    (void)LPUART_TransferReceiveNonBlocking(ADK_LPUART, &s_handle, dst, maxLen, &n);
    return n; // bytes actually copied from ring buffer
}

void ADK_UART_Write(const uint8_t *src, size_t len)
{
    lpuart_transfer_t xfer = { .data = (uint8_t *)src, .dataSize = len };
    (void)LPUART_TransferSendNonBlocking(ADK_LPUART, &s_handle, &xfer);
}

### 9.2 ARINC 429 tasks — arinc_tasks.c

The ADK typically emits bytes that can be framed into ARINC 429 words (parity, label, SDI, data, SSM). Your exact protocol may vary; here we sketch a minimal parser shell.

#include "scheduler.h"
#include "fsl_debug_console.h"
#include <string.h>

#define QUIET_TIMEOUT_MS 500U

typedef struct {
    uint32_t words_rx;
    uint32_t words_tx;
    uint32_t last_rx_ms;
    uint8_t  link_ok;
} arinc_ctx_t;

static arinc_ctx_t g_arinc;

// --- RX (1 ms) ---
static void arinc_rx_init(void *ctx)
{
    (void)ctx; g_arinc.link_ok = 0U; g_arinc.last_rx_ms = scheduler_now_ms();
}

static void arinc_rx_run(void *ctx)
{
    (void)ctx;
    uint8_t buf[64];
    size_t n = ADK_UART_Read(buf, sizeof(buf));
    if (n > 0U) {
        g_arinc.last_rx_ms = scheduler_now_ms();
        g_arinc.link_ok = 1U;
        // TODO: parse bytes into ARINC 429 word(s); demo increments counter
        g_arinc.words_rx += (uint32_t)n; // placeholder for demo
    } else {
        if ((scheduler_now_ms() - g_arinc.last_rx_ms) > QUIET_TIMEOUT_MS) {
            g_arinc.link_ok = 0U; // quiet — declare link down
        }
    }
}

// --- TX (50 ms) ---
static void arinc_tx_init(void *ctx) { (void)ctx; }

static void arinc_tx_run(void *ctx)
{
    (void)ctx;
    if (g_arinc.link_ok) {
        const char msg[] = "TX: label203 mock\r\n"; // replace with real framing for ADK‑8582
        ADK_UART_Write((const uint8_t*)msg, sizeof(msg)-1U);
        g_arinc.words_tx++;
    }
}

// --- BIT/HM (100 ms) ---
static void bit_init(void *ctx) { (void)ctx; }

static void bit_run(void *ctx)
{
    (void)ctx;
    PRINTF("[BIT] rx=%lu tx=%lu link=%u\r\n",
           (unsigned long)g_arinc.words_rx,
           (unsigned long)g_arinc.words_tx,
           (unsigned)g_arinc.link_ok);
}

// Expose task descriptors
#include <stddef.h>

task_t t_arinc_rx = { .name = "arinc_rx", .init = arinc_rx_init, .run = arinc_rx_run, .ctx = NULL, .period_ms = 1U,   .priority = 0 };
task_t t_arinc_tx = { .name = "arinc_tx", .init = arinc_tx_init, .run = arinc_tx_run, .ctx = NULL, .period_ms = 50U,  .priority = 1 };
task_t t_bit      = { .name = "bit_100ms",.init = bit_init,      .run = bit_run,      .ctx = NULL, .period_ms = 100U, .priority = 1 };

### 9.3 Add ARINC tasks to main.c

extern task_t t_arinc_rx, t_arinc_tx, t_bit;
void ADK_UART_Init(void);

int main(void)
{
    BOARD_InitBootPins();
    BOARD_InitBootClocks();
    BOARD_InitDebugConsole();

    ADK_UART_Init(); // LPUART3; configure pins in pin_mux.c

    scheduler_init(1000U);

    (void)task_register(&t_arinc_rx);
    (void)task_register(&t_arinc_tx);
    (void)task_register(&t_bit);
    (void)task_register(&t_led);
    (void)task_register(&t_button);
    (void)task_register(&t_telemetry);

    scheduler_run();
}

Validation steps 1. Connect ADK‑8582 UART to EVKB LPUART3 (3.3 V levels). Configure pins with ConfigTools. 2. Power up; observe [BIT] lines every 100 ms and link=0 until data arrives. 3. Drive ADK to emit ARINC words; rx counter increases; link flips to 1; TX messages appear every 50 ms. 4. Disconnect cable; within ~500 ms link returns to 0; tasks continue safely.

For certification‑style testing, record timestamps to compute jitter and compare to budgets.

## 10) From cooperative to preemptive (optional extension)

- Assign BASEPRI mask for critical sections; keep ISRs short.

- Use SysTick for periodic tick and PendSV for context switch if you later implement fixed‑priority preemption. Each task would then need its own stack. This adds complexity (context save/restore) and increases verification scope under DO‑178C; use only if latency demands justify it. For many DCU‑style functions, TTC suffices with proper WCET control.

## 11) Best practices (Airbus‑grade)

- Deterministic design: Prefer static tables and bounded operations. Avoid heap; allocate all buffers at compile time.

- Tight ISRs: Do the minimum in interrupts (e.g., push byte into ring buffer), schedule work to tasks.

- Time budgets: Define WCET per task (measured with DWT cycle counter) and margin; document in a timing sheet.

- Jitter control: Use release‑time computation next += period (not now + period) to avoid drift; handle catch‑up on overrun.

- Critical sections: Use __disable_irq()/__enable_irq() sparingly or __set_BASEPRI() for fine‑grained masking; keep sections very short.

- Memory placement: Put hot code/data in ITCM/DTCM when necessary; be aware of caches on Cortex‑M7 (enable/flush/invalidate correctly around DMA).

- Interfaces: Separate drivers (adk_uart.c) from tasks (arinc_tasks.c) to keep clear abstraction boundaries for reviews.

- Diagnostics: Count executions, overruns, last error. Telemetry at a low rate (e.g., 5 Hz) helps flight test without flooding links.

- MISRA‑C:2012: Follow rule set (no dynamic allocation, bounded loops, explicit casts, no hidden side effects). Keep a deviation log.

- Safety nets: Implement TASK_ERROR policy — e.g., suspend and raise a maintenance message; never spin forever in an ISR.

## 12) Generic example vs. avionics use case summary

- Generic: LED, button, and telemetry tasks mapped to TTC with 1 ms tick. Demonstrates registration, release, dispatch, and overrun handling.

- Avionics: ARINC 429 via ADK‑8582 over UART; RX (1 ms), TX (50 ms), BIT/HM (100 ms), heartbeat (1 s), telemetry (200 ms), with quiet‑link detection and suspend/resume policy.

## 13) Exercise checklist

- Build the TTC kernel (scheduler.[ch]), integrate with SysTick, and verify LED heartbeat.

- Add telemetry and button; confirm periods and jitter visually.

- Instrument WCET using DWT cycle counter; log max execution time per task for 60 s.

- Wire ADK‑8582 to LPUART3; enable ring buffer; verify RX/TX counters and quiet‑link behavior.

- Demonstrate task_suspend/task_resume by simulating a mode change (e.g., maintenance mode suspends TX).

Keep your project based on the SDK example tree so all board/clock/console code remains identical to NXP’s reference, easing integration and certification evidence collection.