The HI-8582’s ARINC 429 transmitter rate (high vs low speed) and the mandatory word-gap timing are hardware-generated, so “effective bandwidth use” mostly becomes a label/period scheduling problem: how many 32-bit words per second you can (and should) push into the TX FIFO, and how you spread them over time.

## 1) First: what “bandwidth” really is on the HI-8582 (words/sec, not bits/sec)

The transmitter’s timing is defined in clocks; at high speed the data bit time is 10 clocks and the word gap is 40 clocks; at low speed the data bit time is 80 clocks and the word gap is 320 clocks.
Also, the part’s ARINC timing is based on a 1 MHz clock (so 1 clock ≈ 1 µs when operated in the ARINC-compliant mode).

So an ideal minimum-time ARINC word on the HI-8582 is:

- High speed (≈100 kbps):
Word time ≈ 32 bits × 10 µs + 40 µs gap = 360 µs/word
⇒ ~2778 words/sec max

- Low speed (≈12.5 kbps):
Word time ≈ 32 bits × 80 µs + 320 µs gap = 2880 µs/word
⇒ ~347 words/sec max

And the HI-8582 selects those rates via CR13: 0 → 100 kbit/s, 1 → 12.5 kbit/s (and it also changes output slope).

## 2) A practical scheduling rule: budget in words/sec, then pick a frame size

For periodic labels, the required bus load is roughly:

Words/sec required = i1Ti×Ri

- Ti: period of label (i) (seconds)

- Ri:redundancy factor (1 if you send once per period; 2 if you deliberately repeat, etc.)

Then compare that to capacity:

- High speed capacity ≈ 2778 wps

- Low speed capacity ≈ 347 wps

In real avionics you usually don’t run near the max. A common engineering target is ≤50% bus load so jitter, retries, and growth don’t break you.

## 3) Example A — Same label set: works great at high speed, fails at low speed

### The label set

Assume you want to transmit these periodic labels :

- 8 labels @ 50 Hz (T = 20 ms) → 8 × 50 = 400 wps

- 12 labels @ 10 Hz (T = 100 ms) → 12 × 10 = 120 wps

- 20 labels @ 1 Hz (T = 1 s) → 20 × 1 = 20 wps

Total required = 540 words/sec

### High speed result

540 / 2778 ≈ 19% load → plenty of headroom.

### Low speed result

540 / 347 ≈ 156% load → impossible.

Takeaway: if you need lots of fast-changing periodic labels, you almost always must use high speed, or split across multiple ARINC buses, or reduce update rates / number of labels on the low-speed bus.

## 4) Example B — A concrete high-speed periodic schedule using a 10 ms “minor frame”

At high speed, in 10 ms you can send about:
2778 wps × 0.010 s ≈ 27 words per 10 ms

So pick:

- Minor frame = 10 ms

- Major frame = 100 ms (10 minor frames)

A simple deterministic schedule:

- Every 20 ms (50 Hz group): send the 8 “fast” labels in minor frames 0, 2, 4, 6, 8

- Every 100 ms (10 Hz group): send the 12 “medium” labels in minor frame 1

- Every 1 s (1 Hz group): send 20 “slow” labels only once every 10 major frames (e.g., major frame #0)

This works well because it spreads the bus usage (no huge bursts), and it matches the HI-8582 TX FIFO behavior: it can hold up to 32 words, and ignores extra loads if full.

### FIFO-service implication (important on HI-8582)

At high speed, 32 words drain in:
32 × 360 µs ≈ 11.5 ms

So if you want a near-continuous stream, your firmware should “service” the FIFO about every ≤10 ms, or top it up in blocks when a threshold triggers.

The part gives you a hint pin/flag for this: when the TX FIFO has <16 words, the half-full flag indicates you can start a 16-word block write sequence.

## 5) Example C — A low-speed schedule that stays efficient and realistic

At low speed, capacity is only ~347 wps, and in 10 ms you only “earn”:
347 × 0.010 ≈ 3.5 words

So don’t use 10 ms ticks unless you like constant underflow/overhead. Instead pick:

- Minor frame = 100 ms

- Major frame = 1 s (10 minor frames)

Now each 100 ms minor frame can carry ~34 words comfortably.

### A feasible low-speed label set

- 4 labels @ 10 Hz → 40 wps

- 8 labels @ 1 Hz → 8 wps

- 10 labels @ 0.2 Hz (every 5 s) → 2 wps
Total = 50 wps → ~14% load (very safe).

### FIFO-service implication at low speed

At low speed, 32 words drain in:
32 × 2.88 ms ≈ 92 ms

So a 100 ms minor frame aligns nicely: you can refill once per minor frame and stay stable.

## 6) “Effective use” trick when you’re bandwidth-limited: filter aggressively on receive

If your scenario includes reading received labels and forwarding them elsewhere (e.g., over UART to an i.MX RT board), use the HI-8582’s label recognition so unwanted labels never enter the receive FIFO. The datasheet notes that when label recognition is enabled, the chip compares incoming labels; if there’s no match, no indicators of receiving ARINC data are presented (i.e., it effectively ignores them).

That’s often the difference between “UART link melts” and “UART is fine”.

## 7) Quick note if you’re bridging over UART (ADK8582 ↔ i.MX RT): the bus may be faster than your pipe

High-speed ARINC can hit ~2778 wps. If you naïvely forward every TX/RX word as 4 raw bytes, that’s ~11.1 kB/s before framing/checksums—very close to what 115200 UART can sustain in practice. So even on high-speed ARINC, you typically:

- forward only the labels you care about (label recognition),

- reduce update rates for forwarded labels,

- or compress/pack at the application layer.