# ARINC 429 Protocol Fundamentals: Word Structure and Timing Requirements

## 0) Big picture: what ARINC 429 is and why it matters

- ARINC 429, formally Mark 33 Digital Information Transfer System (DITS), is a simplex, self‑clocking avionics bus used extensively in transport aircraft.

- A single transmitter fans out over a shielded, twisted pair to multiple receivers (point‑to‑multipoint).

- There are two standardized bit rates: 12.5 kbps (“low‑speed”) and 100 kbps (“high‑speed”).

- Signaling is Bipolar Return‑to‑Zero (BPRZ), so the line carries both data and timing; no separate clock wire exists.

Why focus on word structure and timing?

- If you place the wrong bits into the wrong fields, downstream Line‑Replaceable Units (LRUs) will either drop your data or misinterpret it.

- If you violate bit timing, rise/fall limits, or the mandatory NULL gap between words, receivers cannot reliably frame or decode your traffic.

## 1) The 32‑bit ARINC word — fields, numbering, and transmission order

Every ARINC message is a fixed‑length 32‑bit word subdivided into five fields by bit number (Bit 1 … Bit 32). The canonical layout is:

Bit#:   1        8 9  10       11                                29 30 31 32
        | Label  |SDI|             DATA (19 bits)                 | SSM | P |
        +--------+---+--------------------------------------------+-----+---+

Fields: -

- Label (Bits 1–8): An 8‑bit identifier, traditionally written in octal (e.g., label 203₈). The label tells any LRU what the word is (e.g., baro altitude), independent of who sent it. –

- SDI — Source/Destination Identifier (Bits 9–10): Often used to differentiate sources (e.g., Air Data Computer 1 vs 2) or to select a destination on legacy receivers. When unused, it may be repurposed as data per the program’s Interface Control Document (ICD). –

- DATA (Bits 11–29): A 19‑bit payload.

Common encodings are

- BNR — Binary Number Representation (two’s‑complement fixed‑point),

- BCD — Binary‑Coded Decimal (packed digits), or discrete flags. –

- SSM — Sign/Status Matrix (Bits 30–31): Conveys sign and/or validity, typically including codes like Normal Operation (NO), Functional Test (FT), Failure or No Computed Data (NCD). Meaning depends on the data type and label family. –

- Parity (Bit 32): Odd parity over Bits 1–31. Set Bit 32 so the total number of ‘1’ bits in the entire 32‑bit word is odd.

### 1.1 Bit numbering vs. bit transmission order — the classic gotcha

- ARINC’s bit numbering (1..32) is a logical view.

- On the wire, the Label field (Bits 1–8) is transmitted first and inside that byte the most‑significant bit (MSB) is sent first.

- The remainder of the word (Bits 9–32) is then transmitted least‑significant‑bit first.

- Many host libraries or analyzers hide this by flipping the label byte for you; others expect you to do it.

- Always check your adapter’s API.

Practical rule:

- In your software, build words in the bit‑numbered layout above.

- Before handing a word to a device that expects an on‑the‑wire bitstream, ensure the Label byte is bit‑reversed if required by that device.

### 1.2 Computing odd parity (Bit 32)

- Assemble Bits 1–31 (Label, SDI, DATA, SSM).

- Count the number of ‘1’ bits.

- If the count is even, set Bit 32 to 1. If it’s odd, set Bit 32 to 0. After this, the total count of ’1’s across 32 bits is odd.

Worked parity example:

- Suppose Bits 1–31 currently contain 17 one‑bits (odd).

- Then Bit 32 must be 0 (leave odd alone). If Bits 1–31 contain 18 one‑bits (even), set Bit 32 to 1 to make 19 (odd).

### 1.3 Encodings inside DATA (Bits 11–29)

- BNR (Binary Number Representation): two’s‑complement fixed‑point. Choose LSB (least significant bit) scale to fit the range (e.g., LSB = 1/128 °/s). Negative values are represented by two’s‑complement. Sign is conveyed by the numeric value; status lives in SSM.

- BCD (Binary‑Coded Decimal): decimal digits packed into nibbles; often used for times, headings, altitudes, or settings where decimal readability is important. Sign may live in SSM or an agreed bit.

- Discrete: individual bits are on/off flags (e.g., gear down, spoilers deployed). SSM still carries validity/state.

SDI and SSM policy are ICD‑defined.

Decide and document, for example, “SDI 00 = ADC #1, 01 = ADC #2” and “SSM 00 = Normal Operation, 01 = Functional Test, 10 = Failure, 11 = No Computed Data,” or whatever your program specifies.

## 2) Timing requirements — bit times, waveform, and gaps

ARINC 429 transmitters must meet three major timing domains:

- Bit rate accuracy and stability.

  - High‑speed: 100 kbps (bit time 10 µs).

  - Low‑speed: 12.5 kbps nominal (bit time 80 µs). Bit‑time and half‑bit windows include tight tolerances for accuracy and jitter.

- Waveform shape — Bipolar Return‑to‑Zero (BPRZ).

  - Logical ‘1’: +10 V differential for the first half of the bit time, then return to 0 V (NULL) for the second half.

  - Logical ‘0’: −10 V differential for the first half of the bit time, then NULL.

  - Rise/fall times (10–90 %) are bounded so receivers can reconstruct timing without excessive ISI (inter‑symbol interference). High‑speed rise/fall is microseconds; low‑speed is an order of magnitude slower.

- Inter‑word gap (NULL spacing).

  - Between two words there must be ≥ 4 bit‑times of continuous NULL (0 V differential).

  - Therefore, the absolute minimum “time budget” per word is 36 bit‑times (32 bits + 4 bits gap). At 100 kbps that’s 360 µs per word; at 12.5 kbps it’s 2.88 ms per word.

Throughput rule of thumb: Max words/s ≈ bit_rate / 36. At 100 kbps ⇒ ≈ 2777 words/s.

Real systems add margin for bus loading, flight phase transients, and adapter FIFOs. Keeping steady‑state channel load ≤ 50 % is common practice.

Scheduling implication:

- Because the bus is self‑clocking, receivers rely on clean NULL gaps to frame words.

- Bursting at the absolute minimum gap repeatedly can stress older LRUs.

- Prefer 6–8 bit‑time gaps when possible; your adapter’s hardware will enforce the on‑line gap once you set it.

## 3) Worked examples (clear and realistic)

### 3.1 Generic example — BNR packing

We must transmit body‑X angular rate +3.25 °/s as BNR with LSB = 1/128 °/s.

- Scaled integer = 3.25 × 128 = 416 (0x01A0).

- Place into DATA (Bits 11–29). SDI = 0. SSM = Normal Operation.

- Compute odd parity over Bits 1–31; set Bit 32 accordingly.

- If the adapter expects the on‑the‑wire bitstream, bit‑reverse the Label byte before transmit.

### 3.2 Real avionics scenario — ADIRU to PFD/FMS stream

An ADIRU — Air Data/Inertial Reference Unit streams on a 100 kbps channel to both the PFD — Primary Flight Display and FMS — Flight Management System:

- Airspeed (label A) at 50 Hz

- Baro altitude (label B) at 50 Hz

- Body rates p, q, r (labels C, D, E) at 100 Hz each

Words/s = 2×50 + 3×100 = 400 words/s. Capacity headroom at 100 kbps (max ≈ 2777 words/s) is ample. Target an average gap of ≥ 6 bit‑times to be courteous to legacy receivers. Document SDI mapping (e.g., SDI 00 = ADIRU #1, 01 = ADIRU #2) and SSM policy in the ICD.

### 3.3 Real avionics scenario — Flight Controls timing budget

- A Flight Control Computer (FCC) multicast channel carries:

  - Elevator servo command (label F) at 200 Hz (control loop)

  - Surface position feedback (label G) at 200 Hz

  - Monitoring discretes (label H) at 25 Hz

  - Mode/status (label J) at 10 Hz

- Total words/s = 200 + 200 + 25 + 10 = 435 words/s. At a conservative 8 bit‑time average gap, each word consumes 32 + 8 = 40 bit‑times ⇒ 100 kbps / 40 = 2500 words/s capacity.

- Bus load ≈ 435 / 2500 = 17.4 % — healthy.

- Ensure rising‑edge slew and stub lengths meet installation limits; test worst‑case cold‑soak (slow edges) and hot‑soak (timing drift).

## 4) Hands‑on labs — i.MX RT1050‑EVKB driving Holt ADK‑8582 over UART

Hardware - EVKB‑IMXRT1050 board (MIMXRT1052 MCU — Microcontroller Unit).

ADK‑8582 evaluation kit for the HI‑8582/HI‑8583 ARINC‑429 interface ICs.

The kit’s on‑board MCU presents a UART — Universal Asynchronous Receiver/Transmitter control/telemetry console; the ARINC line driver/receiver on the kit enforces BPRZ waveform and line levels on the twisted pair.

Connectivity –

- Choose a free EVKB LPUART — Low‑Power UART instance (e.g., LPUART3).

- Route its TX/RX to a header using the Pins tool. Keep LPUART1 for the debug console.

- Wire EVKB TX → ADK RX, EVKB RX → ADK TX, and GND ↔ GND. Level is 3.3 V UART logic.

- The ADK’s ARINC output goes to your analyzer or loopback as desired.

Software prerequisites –

- MCUXpresso SDK for EVKB‑IMXRT1050 (provided).

- Driver headers you’ll use: fsl_lpuart.h, fsl_pit.h, plus board init (board.h, clock_config.h, pin_mux.h). –

- Start from an SDK lpuart driver example (boards/evkbimxrt1050/driver_examples/lpuart/*) and convert it to an application task.

### Lab 1 — Build an ARINC word packer (BNR) with odd parity

Goal: Create a reusable packer that assembles Label, SDI, DATA, SSM, and computes Bit 32 odd parity. Keep the API agnostic to whether the adapter expects you to bit‑reverse the Label byte.

#include <stdint.h>
#include <stdbool.h>

/* Reverse bit order within the 8‑bit Label byte: b7..b0 → b0..b7 */
static inline uint8_t arinc429_reverse_label_bits(uint8_t label)
{
    uint8_t x = label;
    x = (uint8_t)(((x * 0x0802U & 0x22110U) | (x * 0x8020U & 0x88440U)) * 0x10101U >> 16);
    return x;
}

/* 32‑bit population count */
static inline uint8_t popcount32(uint32_t v)
{
    v = v - ((v >> 1) & 0x55555555U);
    v = (v & 0x33333333U) + ((v >> 2) & 0x33333333U);
    return (uint8_t)((((v + (v >> 4)) & 0x0F0F0F0FU) * 0x01010101U) >> 24);
}

/* Pack an ARINC429 word in bit‑number order (Bit 1 at LSB). */
uint32_t arinc429_pack(uint8_t label /*octal*/, uint8_t sdi /*0..3*/, uint32_t data19 /*19 bits*/,
                       uint8_t ssm /*0..3*/, bool reverseLabelBitsForTx)
{
    uint32_t w = 0;
    uint8_t lab = reverseLabelBitsForTx ? arinc429_reverse_label_bits(label) : label;

    w |= (uint32_t)(lab & 0xFFu);             /* Bits 1..8  */
    w |= (uint32_t)(sdi & 0x3u) << 8;         /* Bits 9..10 */
    w |= (uint32_t)(data19 & 0x1FFFFu) << 10; /* Bits 11..29 */
    w |= (uint32_t)(ssm & 0x3u) << 29;        /* Bits 30..31 */

    /* Odd parity over bits 1..31 ⇒ set Bit 32 so total ones (1..32) is odd */
    uint8_t ones = popcount32(w);
    if ((ones & 1u) == 0u) { w |= 0x80000000u; } /* set MSB (bit 32) */
    return w;
}

/* Helper: BNR two’s‑complement fixed‑point for 19‑bit field */
uint32_t arinc429_make_bnr_data19(float value, float lsb)
{
    const float scale = 1.0f / lsb;
    int32_t i = (int32_t)(value * scale + (value >= 0 ? 0.5f : -0.5f));
    if (i >  0x3FFFF) i =  0x3FFFF;           /*  19‑bit max  */
    if (i < -0x40000) i = -0x40000;           /* −19‑bit min  */
    return (uint32_t)i & 0x1FFFFu;
}

Unit‑test idea:

- Pack label 203₈, SDI = 0, SSM = Normal, with +3.25 °/s and LSB = 1/128. Confirm the total ‘1’ bits across 32 bits is odd.

### Lab 2 — Transmit a word to ADK‑8582 over LPUART

Goal:

- Bring up an EVKB LPUART (not the debug console) and send ASCII/HEX commands that the ADK’s demo firmware understands to transmit an ARINC word on the line.

Key SDK pieces:

- fsl_lpuart.h, board.h, clock_config.h, pin_mux.h. Use the MCUXpresso Pins tool to route your chosen LPUART instance (e.g., LPUART3) to a header.

#include "board.h"
#include "pin_mux.h"
#include "clock_config.h"
#include "fsl_lpuart.h"

#define ADK_UART              LPUART3
#define ADK_UART_CLK_FREQ     CLOCK_GetFreq(kCLOCK_UartClk)
#define ADK_UART_BAUD         115200U   /* Confirm with ADK guide */
#define ADK_CMD_BUF_MAX       64

static void ADK_UART_Init(void)
{
    lpuart_config_t cfg;
    LPUART_GetDefaultConfig(&cfg);
    cfg.baudRate_Bps  = ADK_UART_BAUD;
    cfg.parityMode    = kLPUART_ParityDisabled;
    cfg.dataBitsCount = kLPUART_EightDataBits;
    cfg.isMsb         = false;
    LPUART_Init(ADK_UART, &cfg, ADK_UART_CLK_FREQ);
}

static void ADK_SendString(const char *s)
{
    LPUART_WriteBlocking(ADK_UART, (const uint8_t*)s, (size_t)strlen(s));
}

static void ADK_SendWord(uint32_t word)
{
    /* Many ADK firmwares accept: "TX <8 hex chars>\r\n". Adjust to your firmware. */
    char buf[ADK_CMD_BUF_MAX];
    (void)snprintf(buf, sizeof(buf), "TX %08lX\r\n", (unsigned long)word);
    ADK_SendString(buf);
}

int main(void)
{
    BOARD_InitBootPins();
    BOARD_BootClockRUN();
    BOARD_InitDebugConsole(); /* LPUART1 stays for printf */

    ADK_UART_Init();

    const uint8_t  label203 = 0203; /* octal literal */
    const uint8_t  sdi = 0, ssmNO = 0b00;
    const bool reverseLabel = true;  /* Set per ADK expectations */

    while (1) {
        float value = 3.25f; /* example value */
        uint32_t data19 = arinc429_make_bnr_data19(value, 1.0f/128.0f);
        uint32_t w = arinc429_pack(label203, sdi, data19, ssmNO, reverseLabel);
        ADK_SendWord(w);
        SDK_DelayAtLeastUs(20000, SystemCoreClock); /* 50 Hz pacing */
    }
}

Pin‑mux note:

- The EVKB schematics and SDK pin_mux.c templates show which pins can serve LPUART3/5/6 TX/RX.

- Avoid conflicting with the debug console.

### Lab 3 — Enforce update rates and inter‑word gaps with PIT

Goal:

- Use the EVKB PIT — Periodic Interrupt Timer to schedule label‑specific update rates (e.g., 50 Hz, 100 Hz) and avoid adapter FIFO overflows.

Steps:

1) Initialize PIT with a known source clock (e.g., Bus clock).

2) Configure a channel to interrupt at your desired period.

3) In the ISR, build words and call ADK_SendWord() for that label.

- Use separate PIT channels or a simple software scheduler for multiple rates.

Skeleton:

#include "fsl_pit.h"

static void PIT_Init_50Hz(void)
{
    pit_config_t pitCfg;
    PIT_GetDefaultConfig(&pitCfg);
    PIT_Init(PIT, &pitCfg);
    uint32_t srcClk = CLOCK_GetFreq(kCLOCK_BusClk);
    PIT_SetTimerPeriod(PIT, kPIT_Chnl_0, USEC_TO_COUNT(20000U, srcClk)); /* 20 ms */
    PIT_EnableInterrupts(PIT, kPIT_Chnl_0, kPIT_TimerInterruptEnable);
    EnableIRQ(PIT_IRQn);
    PIT_StartTimer(PIT, kPIT_Chnl_0);
}

void PIT_IRQHandler(void)
{
    if (PIT_GetStatusFlags(PIT, kPIT_Chnl_0) & kPIT_TimerFlag) {
        PIT_ClearStatusFlags(PIT, kPIT_Chnl_0, kPIT_TimerFlag);
        /* Build and send one or more words at 50 Hz */
        /* ... call ADK_SendWord(w) ... */
    }
}

Inter‑word gap:

- Many ARINC adapters allow configuring the gap they emit on the ARINC side.

- If your ADK exposes that setting via UART, choose ≥ 6–8 bit‑times.

- If not, pace host commands so the adapter’s FIFO never underflows into minimum‑gap bursts.

### Lab 4 — Receive, validate, and decode returned words

Goal:

- Enable the ADK’s loopback or connect an ARINC source, then read back received words over UART and verify parity, Label, SDI, DATA, and SSM.

Simple UART line reader:

static int ADK_ReadLine(char *dst, size_t maxLen, uint32_t timeoutMs)
{
    size_t n = 0; uint8_t ch; uint32_t t0 = SDK_Millis();
    while (n + 1 < maxLen) {
        if (LPUART_ReadByte(ADK_UART, &ch) == kStatus_Success) {
            if (ch == '\n') break;
            if (ch != '\r') dst[n++] = (char)ch;
        }
        if ((SDK_Millis() - t0) > timeoutMs) break;
    }
    dst[n] = '\0';
    return (int)n;
}

Parsing:

- If the ADK emits lines like RX 12345678, parse the 8 hex digits to a uint32_t, recompute parity with popcount32(), and then extract fields with masks/shifts.

- Apply Label bit reversal if the adapter presents raw on‑the‑wire ordering.

### Lab 5 (optional) — Mix BNR and BCD, exercise SSM states

- Transmit a BCD‑encoded altitude and deliberately toggle SSM among Normal, Functional Test, and No Computed Data. Confirm that downstream tools show the expected validity/state.

- Inject an odd‑parity error (flip Bit 32) and observe that receivers reject the word.

### Build & run checklist

- Clocks: BOARD_BootClockRUN() must set UART/PIT source clocks to the expected frequencies.

- Pins: Verify LPUART instance routing in pin_mux.c and conflict‑free with debug UART.

- Baud: Match the ADK console baud (likely 115200‑8‑N‑1).

- ARINC rate: Select 100 kbps or 12.5 kbps in the ADK command set before transmitting.

- Verification: With a scope or ARINC analyzer, confirm:

  - Bit rate within tolerance (10 µs per bit at 100 kbps; 80 µs per bit at 12.5 kbps).

  - BPRZ pulse width ≈ half‑bit time, proper polarity for ‘1’ and ‘0’.

  - Inter‑word NULL gap ≥ 4 bit‑times; target 6–8.

  - Parity is odd across full 32 bits.

## 5) Best practices

- Treat Labels as octal everywhere: code, comments, and ICDs. This avoids mis‑matches with standard tables.

- Centralize packing/unpacking and scaling so parity, Label bit reversal, and BNR/BCD handling are uniform.

- Document SDI/SSM policy in the ICD and enforce it in code reviews. Never leave SSM ambiguous.

- Budget channel load and avoid minimum gaps in steady state; leave margin for maintenance traffic and phase‑of‑flight bursts.

- Verify on real hardware: rise/fall times, pulse widths, gaps, and parity. Vary temperature and supply.

- Grounding and cabling: use 78 Ω shielded twisted pair; respect stub length limits and shield termination practice.

- Error handling: on RX, check parity and SSM; discard or flag words with errors or invalid status. On TX, fail safe (e.g., transmit NCD) when data becomes stale.

## 6) Common pitfalls (and how to avoid them)

- Label bit order misunderstood → PFD shows nonsense. Fix: always confirm whether your adapter expects the label byte bit‑reversed.

- Parity computed over wrong bits → receivers drop words. Fix: compute over Bits 1–31 only; set Bit 32 afterwards.

- Over‑aggressive bursting → violates effective gaps. Fix: use PIT‑based pacing and set adapter gap to ≥ 6–8 bit‑times.

- Assuming SSM semantics → downstream misinterpretation. Fix: follow your ICD for each label family.

- Ignoring installation effects → ringing, slow edges. Fix: scope at the line under worst‑case load and temperature.

## 7) Summary: what to remember

- ARINC 429 uses fixed 32‑bit words with odd parity; Label is Bits 1–8 and is transmitted first (MSB‑first) while Bits 9–32 go LSB‑first.

- Two rates: 100 kbps (10 µs/bit) and 12.5 kbps (80 µs/bit). Signaling is BPRZ with a mandatory ≥ 4 bit‑time inter‑word NULL gap.

- Prefer scheduled transmissions with PIT and keep average gaps ≥ 6–8 bit‑times to be friendly to legacy receivers.

- Encode DATA using BNR, BCD, or discretes as your ICD specifies; use SSM to convey validity/state.

## Appendix A — Notation quick‑ref

- ADIRU: Air Data/Inertial Reference Unit

- ADC: Air Data Computer

- BNR: Binary Number Representation (two’s‑complement fixed‑point)

- BCD: Binary‑Coded Decimal

- BPRZ: Bipolar Return‑to‑Zero

- DITS: Digital Information Transfer System (ARINC 429 formal name)

- FCC: Flight Control Computer

- FMS: Flight Management System

- ICD: Interface Control Document

- ISR: Interrupt Service Routine

- LRU: Line‑Replaceable Unit

- LSB: Least Significant Bit (also least‑significant bit weight in fixed‑point)

- LPUART: Low‑Power Universal Asynchronous Receiver/Transmitter

- NCD: No Computed Data

- NO: Normal Operation

- PFD: Primary Flight Display

- PIT: Periodic Interrupt Timer

- RZ: Return‑to‑Zero signaling

- SDI: Source/Destination Identifier

- SSM: Sign/Status Matrix

## Appendix B — Alternate (portable) parity helper

static inline uint32_t arinc429_apply_odd_parity(uint32_t w31)
{
    /* w31: Bits 1..31 populated; Bit 32 clear */
    uint32_t v = w31;
    v ^= v >> 16; v ^= v >> 8; v ^= v >> 4; v &= 0xFu;
    /* parity LUT for 0..15 nibble popcount parity */
    static const uint8_t lut[16] = {0,1,1,0,1,0,0,1,1,0,0,1,0,1,1,0};
    if (lut[v]) { /* odd already */ return w31; }
    else { return w31 | 0x80000000u; }
}

## Appendix C — Field extraction masks (for RX)

#define A429_LABEL_MASK   0x000000FFu
#define A429_SDI_MASK     0x00000300u
#define A429_DATA_MASK    0x0FFFFC00u
#define A429_SSM_MASK     0x60000000u
#define A429_PARITY_MASK  0x80000000u

#define A429_LABEL(x)     ((uint8_t)((x) & A429_LABEL_MASK))
#define A429_SDI(x)       ((uint8_t)(((x) & A429_SDI_MASK) >> 8))
#define A429_DATA19(x)    ((uint32_t)(((x) & A429_DATA_MASK) >> 10))
#define A429_SSM(x)       ((uint8_t)(((x) & A429_SSM_MASK) >> 29))
#define A429_PARITY(x)    (((x) & A429_PARITY_MASK) ? 1u : 0u)