# ARINC 429 Protocol Fundamentals — Parity and Error Detection

## 1) What “parity” means on ARINC 429

- ARINC (Aeronautical Radio, Incorporated) 429—also called the Mark 33 Digital Information Transfer System (DITS)—moves information as fixed 32‑bit “words” over a unidirectional, two‑wire, differential bus using bipolar return‑to‑zero (BPRZ) signalling.

- The 32nd bit of every word is the parity bit. ARINC 429 uses odd parity:

  - Count the number of logic ‘1’ bits in bits 1–31. Set bit 32 so that the total number of ’1’s in all 32 bits is odd.

- If the receiver counts an even number of ’1’s across bits 1–32, it flags a parity error and—per the system Interface Control Document (ICD)—must discard or de‑weight the word and usually set a status (for example, “No Computed Data”).

### 1.1 ARINC 429 word layout (conventional bit numbering)

- Bits 1–8 — Label (8 bits): Identifies what the word means (e.g., airspeed, altitude). On the wire, the label’s LSB is transmitted first.

- Bits 9–10 — SDI (Source/Destination Identifier, 2 bits): Multiplexing cue (which source or which destination the word is intended for).

- Bits 11–29 — DATA (19 bits): The payload, most commonly Binary Number Representation (BNR) or Binary‑Coded Decimal (BCD).

- Bits 30–31 — SSM (Sign/Status Matrix, 2 bits): Sign and validity/status coding (e.g., Normal, Functional Test, No Computed Data, Failure).

- Bit 32 — Parity (1 bit): Odd parity across bits 1–31.

Implementation tip: It is common to store an ARINC word in a uint32_t where bit 0 ↔ ARINC bit 1 and bit 31 ↔ ARINC bit 32. This makes masking/parity math simple even though the label’s on‑the‑wire bit order is reversed.

## 2) Why parity matters (and what it cannot do)

- Parity is simple, fast, and cheap to check in hardware or software, and it guarantees detection of any odd number of bit flips in a 32‑bit word (single‑bit errors are the dominant random error on a clean bus).

- It cannot detect errors that flip an even number of bits within the same word.

Therefore ARINC 429 robustness relies on more than parity:

- SSM checks (e.g., word marked Failure or No Computed Data).

- Reasonableness/Range checks (value within expected physical limits).

- Rate‑of‑change monitors (e.g., airspeed doesn’t jump 100 kt in 10 ms).

- Redundancy/Voting across multiple sources (e.g., dual Air Data Computers).

- Sequence/Refresh timing monitors (label arrives at required rate).

For certification, combine parity with those higher‑level monitors to meet RTCA DO‑178C (Software Considerations in Airborne Systems and Equipment Certification) and RTCA DO‑254 (Design Assurance Guidance for Airborne Electronic Hardware).

## 3) Worked example — compute the parity bit (generic)

- Assume we want to transmit an ARINC word with:

  - Label = 0xAF (binary 1010 1111)

  - SDI = 01b

  - DATA (BNR) = 0x12345 masked into 19 bits

  - SSM = 00b (Normal)

- Pack bits 1–31 first (leave bit 32 = 0 while calculating).

- Count the number of ’1’s in bits 1–31.

- If that count is even, set bit 32 = 1. If it’s odd, set bit 32 = 0.

- The total 1‑bit count across 32 bits will then be odd.

A compact software way is to XOR‑reduce (population count & 1) and invert as needed. See arinc429_parity_odd_bit() in the lab code below.

## 4) Avionics use‑case scenario — detecting a corrupted airspeed word

- A Primary Flight Display (PFD) receives Indicated Airspeed from an Air Data Computer (ADC) on a high‑speed (100 kbps) ARINC 429 bus.

- The aircraft is in cruise at 280 kt. Due to a transient disturbance on the wiring harness, bit 17 of the airspeed word flips during transmission.

- Because only a single bit flipped, the parity at the receiver becomes even, the PFD’s ARINC interface flags a parity error, discards that word, and holds the last valid value.

- If subsequent words also fail parity or the SSM transitions to No Computed Data, the PFD annunciates “AIRSPEED DATA” caution and transitions to reversionary/standby indications, per the system safety analysis.

- If, instead, two bits flipped in the same word, the parity check alone might not catch it; this is where range and rate monitors (e.g., “airspeed change must be ≤ 10 kt per 40 ms”) and multi‑source cross‑checks prevent hazardous effects.

## 5) Lab platform: IMXRT1050‑EVKB + ADK‑8582 (UART bridge)

- For training, we treat ADK‑8582 as a UART‑controlled ARINC 429 bridge/transceiver connected to the NXP i.MX RT1050 EVKB.

- The EVKB side uses LPUART1 (pins GPIO_AD_B0_12 TX and GPIO_AD_B0_13 RX).

- We compute parity in software on the EVKB, then deliver packed 32‑bit words to the ADK‑8582 over UART for transmission on the ARINC bus.

- On receive, we read 32‑bit words from the ADK‑8582 and verify parity before passing data up to application logic.

Note:

- Different ARINC‑UART bridges package words differently (raw 4‑byte binary, big‑endian, little‑endian, ASCII frames, checksums).

- The UART framing helpers below isolate that detail—adjust just those few lines to match your specific ADK‑8582 command set/manual while keeping the parity logic and SDK driver usage unchanged.

### 5.1 Minimal, SDK‑based UART bring‑up (EVKB)

The snippet follows the EVKB SDK style (same APIs as the EVKB lpuart_polling example). It shows:

- LPUART1 init on EVKB.

- ARINC word packing with odd parity.

- Send/receive helpers that you can adapt to your ADK‑8582 protocol.

// File: arinc429_parity_demo.c
#include "board.h"
#include "app.h"                 // DEMO_LPUART, DEMO_LPUART_CLK_FREQ
#include "fsl_lpuart.h"
#include "fsl_common.h"
#include <stdint.h>
#include <stdbool.h>
#include <string.h>
#include <assert.h>

// -------------------- ARINC 429 bitfield helpers --------------------
#define ARINC_LABEL_MASK   (0xFFu)       // bits 1..8
#define ARINC_SDI_MASK     (0x3u)        // bits 9..10
#define ARINC_DATA_MASK    (0x7FFFFu)    // bits 11..29 (19 bits)
#define ARINC_SSM_MASK     (0x3u)        // bits 30..31

// Bit positions in a uint32_t where bit0 == ARINC bit1, bit31 == ARINC bit32
#define ARINC_LABEL_POS    (0u)
#define ARINC_SDI_POS      (8u)
#define ARINC_DATA_POS     (10u)
#define ARINC_SSM_POS      (29u)
#define ARINC_PARITY_POS   (31u)

// Compute odd parity for bits 0..30 and return 0/1 to place into bit31.
// This produces bit31 such that the total number of 1s in all 32 bits is odd.
static inline uint32_t arinc429_parity_odd_bit(uint32_t w31)
{
    // XOR-reduce over bits 0..30.
    // If popcount is even => xor = 0 => parity bit must be 1 (to make total odd).
    // If popcount is odd  => xor = 1 => parity bit must be 0.
    // So: parity_bit = (~xor) & 1
    uint32_t x = w31 ^ (w31 >> 16);
    x ^= (x >> 8);
    x ^= (x >> 4);
    x ^= (x >> 2);
    x ^= (x >> 1);
    uint32_t xor1 = x & 1u;
    return (xor1 ^ 1u); // invert to get odd parity
}

// Pack bits 1..31, compute and insert bit32 (odd parity)
static inline uint32_t arinc429_pack(uint8_t label, uint8_t sdi, uint32_t data19, uint8_t ssm)
{
    uint32_t w = 0;
    w |= ((uint32_t)(label & ARINC_LABEL_MASK) << ARINC_LABEL_POS);
    w |= ((uint32_t)(sdi   & ARINC_SDI_MASK)   << ARINC_SDI_POS);
    w |= ((uint32_t)(data19 & ARINC_DATA_MASK) << ARINC_DATA_POS);
    w |= ((uint32_t)(ssm   & ARINC_SSM_MASK)   << ARINC_SSM_POS);
    // compute parity on bits 0..30
    uint32_t p = arinc429_parity_odd_bit(w);
    w |= (p << ARINC_PARITY_POS);
    return w;
}

// Verify odd parity: return true if word passes parity
static inline bool arinc429_check_parity(uint32_t word)
{
    // Total 1s across all 32 bits must be odd => XOR-reduce must be 1
    uint32_t x = word ^ (word >> 16);
    x ^= (x >> 8);
    x ^= (x >> 4);
    x ^= (x >> 2);
    x ^= (x >> 1);
    return (x & 1u) == 1u;
}

// -------------------- ADK-8582 UART framing (adjust to your board) --------------------
// Option A: raw 4-byte big-endian word
static status_t adk_send_word_raw_be(uint32_t w)
{
    uint8_t frame[4] = { (uint8_t)(w >> 24), (uint8_t)(w >> 16), (uint8_t)(w >> 8), (uint8_t)(w) };
    LPUART_WriteBlocking(DEMO_LPUART, frame, sizeof(frame));
    return kStatus_Success;
}

static status_t adk_read_word_raw_be(uint32_t *out)
{
    uint8_t buf[4];
    LPUART_ReadBlocking(DEMO_LPUART, buf, 4);
    *out = ((uint32_t)buf[0] << 24) | ((uint32_t)buf[1] << 16) | ((uint32_t)buf[2] << 8) | buf[3];
    return kStatus_Success;
}

// -------------------- EVKB LPUART setup --------------------
static void uart_init_115200_8N1(void)
{
    lpuart_config_t config;
    LPUART_GetDefaultConfig(&config);
    config.baudRate_Bps = 115200;
    config.enableTx = true;
    config.enableRx = true;
    LPUART_Init(DEMO_LPUART, &config, DEMO_LPUART_CLK_FREQ);
}

// Example: transmit one word and then read one word back and check parity
void arinc_parity_demo(void)
{
    uint8_t  label = 0xAF;       // example label (ICD will define the real one)
    uint8_t  sdi   = 0x1;
    uint32_t data  = 0x12345;    // 19-bit application data
    uint8_t  ssm   = 0x0;        // Normal
    uint32_t wtx   = arinc429_pack(label, sdi, data, ssm);

    adk_send_word_raw_be(wtx);

    // Receive a word and validate parity
    uint32_t wrx = 0;
    adk_read_word_raw_be(&wrx);
    bool ok = arinc429_check_parity(wrx);

    // In a real app, do not printf on the flight CPU path; use queues/state.
    const char *msg = ok ? "PARITY OK\r\n" : "PARITY ERROR\r\n";
    LPUART_WriteBlocking(DEMO_LPUART, (const uint8_t*)msg, (uint32_t)strlen(msg));
}

int main(void)
{
    BOARD_InitHardware();   // From SDK template; includes pins/clock.
    uart_init_115200_8N1();
    while (1) {
        arinc_parity_demo();
        // Simple pacing; replace with proper scheduler/RTOS tick in real code
        for (volatile uint32_t i = 0; i < 12000000; ++i) { __NOP(); }
    }
}

Why this matches the EVKB SDK: It uses the standard driver layer (fsl_lpuart.h), the LPUART_GetDefaultConfig / LPUART_Init / *_WriteBlocking / *_ReadBlocking calls, and assumes the EVKB LPUART instance/pins used by the official examples. Replace the two adk_* functions with the exact frame your ADK‑8582 requires (binary vs ASCII, endianness, checksums).

### 5.2 Bench wiring and configuration checklist

- Levels: Ensure EVKB UART is 3.3 V TTL and matches the ADK‑8582 UART levels.

- Ground: Common ground between EVKB and ADK‑8582.

- UART settings: Start with 115200 8‑N‑1; adjust to the ADK‑8582’s documented default.

- ARINC port rate: Configure ADK‑8582 channel as Low‑speed (12.5 kbps) or High‑speed (100 kbps) per test.

- Byte order: The example sends big‑endian (network order). Change if your bridge expects little‑endian.

- Label bit order: Do not reverse label bits in memory—only the physical line order is reversed; parity math remains unchanged.

## 6) Hands‑on exercises (with solutions)

### Exercise A — Parity generator/validator (software)

Goal: Write two functions on the EVKB: arinc429_pack() and arinc429_check_parity().

Pass criteria: For 1000 random words, packing + checking always passes; flipping any single bit in bits 1–32 always fails parity.

Solution scaffold: Functions provided above. To test, add a loop that flips each bit b:

for (uint32_t b = 0; b < 32; ++b) {
    uint32_t w = arinc429_pack(0x22, 0x0, 0x12345, 0x1);
    uint32_t wbad = w ^ (1u << b);
    assert(arinc429_check_parity(w) == true);
    assert(arinc429_check_parity(wbad) == false);
}

### Exercise B — Inject and detect an error end‑to‑end

Goal: Demonstrate a single‑bit flip causes a parity failure at the receiver.

- On the EVKB, compute a good word.

- Before sending to the ADK‑8582, deliberately flip one randomly chosen bit among bits 1–31 (don’t flip the parity bit; we want an odd error count).

- Transmit.

- On the return path (EVKB receiving from ADK‑8582 or from a loopback), call arinc429_check_parity() and increment a counter on failure.

Snippet:

uint32_t inject_single_bit_error(uint32_t w)
{
    uint32_t b = (SysTick->VAL % 31u);   // toy pseudo‑random bit index 0..30
    return w ^ (1u << b);
}

Expected: Every injected frame fails parity.

### Exercise C — SSM behavior on parity error (avionics use case)

Goal: Emulate a Primary Flight Display (PFD) input monitor.

- If parity OK and data in range, forward the value with SSM = Normal.

- If parity error, discard the word, hold last good, and set a downstream status to “NCD (No Computed Data)” after N consecutive parity failures.

- If your ADK‑8582 lets you force SSM, send the same label with SSM = Failure and verify the receiver logic treats it as invalid even if parity is OK.

This shows parity is necessary but not sufficient; SSM and reasonableness logic complete the safety case.

## 7) Practical details that often trip people up

- Label bit order vs. storage: On the wire, the label’s LSB goes first; in memory, keep the label as an 8‑bit value and don’t reverse bits unless your bridge specifically requires it. Parity calculation is unaffected by wire order—always compute across bits 1–31 as stored.

- Endianness over UART: EVKB is little‑endian; many bridges expect big‑endian words. The example explicitly builds a big‑endian frame—change if needed.

- Parity over what? Only bits 1–31. Never include bit 32 in the parity calculation.

- Don’t “fix up” bad words: If a received word fails parity, discard it; do not try to guess the flipped bit.

- Refresh/rates: Even perfect‑parity words that stop arriving at the expected refresh rate must be treated as invalid by higher‑level logic.

- Determinism: In RTOS builds, do parity checks in the driver ISR or DMA completion path so applications only see validated words.

## 8) Best practices (Airbus‑style rigor)

- Compute parity where the word is formed and verify parity where the word is consumed. Do not depend on “someone upstream checked it.”

- Instrument error counters (per label and per source). Trend them and expose via built‑in test (BIT) to catch intermittent wiring faults.

- Validate SSM semantics end‑to‑end during integration. Parity‑OK + SSM = Failure must be treated as invalid data.

- Rate/range/sanity checks: Implement label‑specific monitors (e.g., airspeed ∈ [0, 450] kt, Δv/Δt bounded).

- Keep parity logic constant‑time and side‑effect free. The XOR‑reduction compiles to a handful of shifts/XORs with no branches.

- Lock down a UART‑to‑ARINC ICD early (byte order, escapes, checksums). Only the adk_* helpers should change when you update the ICD; parity and SDK usage stay fixed.

- Handle noise bursts: Use glitch counters and timeout‑based de‑weighting rather than instantaneous failover to prevent nuisance reconfigurations.

- Certification evidence: Tie your unit tests (including the “flip any single bit” test) to DO‑178C verification objectives and store logs as artifacts.

- Clocking and pins: Verify LPUART1 clock source and IO mux match EVKB defaults; mismatches lead to subtle framing errors you may misinterpret as parity faults.

- Telemetry for integration: Provide a debug counter of parity_errors_per_label_per_minute to aid flight‑line troubleshooting.

## 9) Quick reference: pack/unpack masks

- Pack:
word = (label<<0) | (sdi<<8) | (data19<<10) | (ssm<<29); then set bit 31 using arinc429_parity_odd_bit(word).

- Check:
bool ok = arinc429_check_parity(word);

## 10) Resources

- Source file: arinc429_parity_demo.c (drop into an EVKB SDK application skeleton; it uses the same app.h/BOARD_InitHardware() pattern as the SDK examples).

- Pinout: EVKB LPUART1 on GPIO_AD_B0_12 (TX) / GPIO_AD_B0_13 (RX) to ADK‑8582 UART.

- One‑page ICD: Define the UART frame for an ARINC word (binary or ASCII, byte order, and any checksum). Only the two adk_* helpers change when you update the ICD; the parity logic and SDK usage stay fixed.