# ARINC 429 Protocol – All Aspects in Great Detail

## 1. Overview of ARINC 429

- Standard name: ARINC Specification 429 – “Mark 33 Digital Information Transfer System (DITS)”.

- Purpose: Defines a unidirectional, point-to-multipoint digital data bus used extensively in commercial transport aircraft.

- Typical use: Communication between avionics LRUs (Line Replaceable Units) such as:

  - Air Data Computers (ADC)

  - Inertial Reference Systems (IRS)

  - Flight Management Computers (FMC)

  - Flight Control Computers

  - Radios, Navigation receivers, etc.

- Communication model:

  - Each physical link is simplex (one-way only).

  - One transmitter (talker) per pair of wires.

  - Up to 20 receivers (listeners) can be connected to one transmitter on that pair.

  - If bidirectional exchange is required, two links are used: A→B and B→A.

- Design goals:

  - Deterministic behavior (no bus arbitration like CAN or Ethernet).

  - Very high reliability in a harsh EMI environment.

  - Simple, fixed word structure to ease certification and analysis.

## 2. Physical Layer

### 2.1 Cabling and Impedance

- Medium: Shielded twisted pair cable.

- Characteristic impedance: Nominal 78 Ω.

- Transmitter output impedance: Approximately 75 Ω ± 5 Ω, balanced.

- Receiver input impedance: ≥ 8 kΩ, allowing many receivers on one pair.

- Shielding: Cable shield is grounded appropriately to reduce electromagnetic interference (EMI).

Effects: - Matched impedance plus differential signaling → reduced reflections and noise. - High receiver impedance → one transmitter can fan out to many receivers without overloading.

### 2.2 Differential Signaling – BPRZ

ARINC 429 uses Bipolar Return-to-Zero (BPRZ) signaling on a differential twisted pair labeled typically as A and B:

- Three effective differential levels (measured A–B):

  - Logic 1 (HIGH): about +10 V at transmitter (receiver sees roughly +6.5 to +13 V).

  - NULL: about 0 V differential (receiver roughly within –2.5 V to +2.5 V).

  - Logic 0 (LOW): about –10 V at transmitter (receiver sees roughly –6.5 to –13 V).

- Return-to-zero behavior:

  - Each bit time is split into two halves.

    - First half: line driven to HIGH or LOW for a ‘1’ or ‘0’.

    - Second half: line returns to NULL.

  - This ensures frequent transitions → easier clock recovery and reduced DC offset.

Advantages: - Differential + twisted pair → high common-mode noise rejection. - RZ scheme → easier to maintain signal integrity and timing.

## 3. Bit Rates and Timing

Two standard data rates are supported on a given bus:

- Low speed: Nominal 12.5 kbps (approximately 12–14.5 kbit/s allowed).

- High speed: Nominal 100 kbps (±1%).

A single physical bus operates at one of these speeds; you don’t mix them on the same pair.

### 3.1 Timing at 100 kbps

- Bit time: ~10 µs.

- Bit structure at 100 kbps:

  - First half-bit (~5 µs): line is driven HIGH or LOW.

  - Second half-bit (~5 µs): line returns to NULL.

- Rise and fall times are constrained (order of 1–2 µs) to control EMI and ensure proper sampling.

### 3.2 Timing at 12.5 kbps

- Bit time: ~80 µs.

- Same BPRZ principle, just stretched in time.

- Rise/fall times are correspondingly longer (order of ~10 µs).

### 3.3 Inter-word Gap

- Between successive ARINC 429 words, the transmitter must insert a minimum of 4 bit times of NULL (no data).

- This gap helps receivers separate words and maintain synchronization.

### 3.4 Throughput Intuition

- At 100 kbps:

  - 32 bits per word → 320 µs.

  - Add ≥ 4 bit times of gap → +40 µs.

  - So one word takes ≈ 360 µs → around 2,700–2,800 words/s maximum, in theory.

- At 12.5 kbps everything is 8× slower.

System designers choose word repetition rates (e.g., 10 Hz, 20 Hz, 40 Hz, 100 Hz) so that all required data comfortably fits within capacity.

## 4. Bus Architecture and Topology

### 4.1 Simplex, Single Talker / Multi Listener

- Each ARINC 429 channel is simplex:

  - Exactly one transmitter on that pair.

  - Multiple receivers (up to ~20) are listeners.

- Receivers never drive the bus → no collision / arbitration at this level.

### 4.2 Typical Topologies

- Star: One transmitter feeding separate short cable runs to each receiver.

- Trunk with stubs: Main cable with short branch stubs to LRUs.

- Systems often have many independent 429 channels connecting LRUs.

### 4.3 Redundancy

- Aircraft typically implement dual or multiple redundant buses:

  - E.g., ADC1 output on Bus A, ADC2 on Bus B.

  - Or cross-strapped buses where critical data is available on more than one channel.

- Redundancy is handled at the system architecture level, not by the 429 protocol itself.

## 5. ARINC 429 Word Format (32 Bits)

Every ARINC 429 word is exactly 32 bits long. The logical layout (bit numbering) is:

Bit numbers (logical view, MSB on left):

32          31 30           29 … 11            10  9      8 … 1
+-----------+------+--------+------------------+------+-----------+
| Parity    | SSM  |  Data  | (19 bits)        | SDI  | Label     |
+-----------+------+--------+------------------+------+-----------+

- Bits 1–8 : Label (8 bits)

- Bits 9–10 : Source/Destination Identifier (SDI) (2 bits)

- Bits 11–29 : Data field (19 bits)

- Bits 30–31 : Sign/Status Matrix (SSM) (2 bits)

- Bit 32 : Parity (1 bit, odd parity)

Important conventions: - The spec often draws bit 32 on the left (MSB) and bit 1 on the right (LSB) in documentation. - However, transmission order on the wire is special and must be understood separately.

## 6. Bit Transmission Order and Label Peculiarities

### 6.1 Transmission Order

- The word bits are logically numbered from 1 to 32.

- On the wire, bits are transmitted in the order:

  - Bit 1 first, then 2, 3, …, 32.

### 6.2 Label Bit Order

- Label (bits 1–8) is special:

  - Within the label field, bit 8 is transmitted first, then 7, 6, …, down to bit 1.

  - So the label appears MSB first.

- For the rest of the word (SDI, DATA, SSM, Parity), many implementations treat each byte as transmitted LSB first within the byte, but the exact handling can be implementation-specific.

Practical consequence: - Many ARINC 429 interface chips expect you to load the label byte in a bit-reversed order compared to how you write it in octal on paper. - For example, the label 203₈ might need to be written as a particular hex value with reversed bits to be transmitted correctly.

This is one of the most common “gotchas” when first programming ARINC 429 hardware.

## 7. Detailed Field Definitions

### 7.1 Label (Bits 1–8)

- Purpose: Identifies the type of data contained in the word.

- 8-bit value, traditionally expressed in octal.

- ARINC documentation defines standardized label sets per equipment type:

  - Air Data Computer labels (altitude, airspeed, temperature, etc.).

  - Inertial Reference System labels (pitch, roll, heading, acceleration, etc.).

  - Flight Management labels (waypoints, performance data, etc.).

Example: - A specific label (e.g., 203₈) may represent barometric altitude from an ADC.

Important: - Labels are contextual: their meaning depends on the equipment type and sometimes on SDI. - To interpret a word, you need to know both the label and which LRU / channel it came from.

### 7.2 SDI – Source/Destination Identifier (Bits 9–10)

- 2-bit field used for:

  - Identifying the source channel of the data (e.g., ADC #1 vs ADC #2).

  - Or specifying the intended destination in older or specialized schemes.

Common usage today: - SDI represents which instance of a system the data belongs to (e.g., left vs right system). - Combined with bus/channel and label, SDI helps uniquely identify a data stream.

### 7.3 Data Field (Bits 11–29, 19 bits)

The core payload of the word. ARINC 429 supports several data coding schemes:

- BNR (Binary Number Representation) – signed binary fraction.

- BCD (Binary Coded Decimal) – decimal digits coded in 4-bit nibbles.

- Discrete – individual bits representing boolean flags.

- Miscellaneous or mixed formats for special applications.

For each label, the ARINC spec (and aircraft documentation) define: - Encoding type (BNR/BCD/discrete). - Bit significance and resolution. - Units, ranges, and scaling factors.

#### 7.3.1 BNR (Binary Number Representation)

- Used for continuous numeric values: angles, velocities, temperatures, etc.

- Representation:

  - Bit 29: Sign bit (1 = negative, 0 = positive) for BNR.

  - Bits 28–11: Magnitude bits in fractional two’s-complement.

- Each bit has a defined weight:

  - Bit 28 is the most significant magnitude bit.

  - Weighting is typically defined relative to full-scale or a scaling factor (e.g., 2⁻¹, 2⁻², etc.).

- Resolution and range are determined by the label definition.

Example concept: - A BNR field might represent an angle from –180 to +180 degrees with some resolution. - Scaling might be such that one LSB = 0.01°, for instance (exact values are given per label in the spec).

#### 7.3.2 BCD (Binary Coded Decimal)

- Used for numeric values where decimal representation is convenient (e.g., altitude, flight number).

- Data bits are grouped into 4-bit nibbles, each representing a decimal digit 0–9.

- There may be up to 5 digits; sometimes the most significant group uses fewer bits.

- Often the SSM bits encode sign or direction (e.g., plus/minus, north/south).

Example: - A BCD altitude could encode digits of an altitude in feet. - SSM may indicate plus or minus or other directional info.

#### 7.3.3 Discrete Data

- Each data bit is a separate boolean flag.

- Used for on/off or status indicators such as:

  - Gear down

  - Autopilot engaged

  - Flight director on

  - etc.

- The meaning of each bit position is defined for that label.

### 7.4 SSM – Sign/Status Matrix (Bits 30–31)

These two bits summarize sign and/or status, with detailed meaning depending on data type (BNR vs BCD vs discrete).

Typical encodings (simplified view):

- For BNR values:

  - One combination indicates Normal Operation.

  - Others indicate No Computed Data (NCD), Functional Test (FT), and Failure Warning (FW).

- For BCD values:

  - Combinations can encode sign/direction (plus/minus, N/S, E/W) and/or status.

- For discretes:

  - Used mostly for status: normal, test, NCD, failure, etc.

SSM enables downstream systems to differentiate between: - Valid numeric data. - Data under test mode. - Missing or invalid data. - Known failure conditions.

### 7.5 Parity Bit (Bit 32)

- ARINC 429 uses odd parity for each 32-bit word.

- The total number of ‘1’ bits in bits 1–32 must be odd.

- The transmitter sets the parity bit accordingly.

- This provides a single-bit error detection mechanism.

- If a receiver detects parity error, it normally discards the word or flags it as invalid.

Note: There is no built-in retransmission at the ARINC 429 level; higher-level logic must decide what to do.

## 8. System-Level Behavior and Data Rates

### 8.1 Word Repetition and Scheduling

- Each data source (e.g., ADC, IRS) repeats its important labels at fixed rates:

  - Example: attitude data at 50 Hz or 100 Hz.

  - Slower parameters (e.g., configuration or maintenance data) may be at 1 Hz or lower.

- Designers ensure:

  - All required data can fit in the available bandwidth.

  - Critical data has sufficiently high update rate for control tasks.

### 8.2 Determinism

- Because each bus has only one talker and no arbitration, transmission timing is determined purely by the transmitter’s schedule.

- This simplifies worst-case latency analysis, which is important for certification.

## 9. EMI Robustness and Environmental Considerations

ARINC 429 is designed to operate reliably in the avionics environment:

- Differential signaling and twisted pair reduce susceptibility to common-mode noise.

- Shielding and controlled rise/fall times reduce emitted radiation and ringing.

- RZ coding ensures frequent transitions and avoids long runs of constant voltage, helping with clock recovery and EMI.

- Equipment is qualified per DO-160 (environmental conditions and test procedures for airborne equipment).

## 10. Addressing and Identification in ARINC 429

Unlike CAN or Ethernet, ARINC 429 does not have an explicit node address in the word itself.

Identification comes from a combination of:

- Bus/channel (physical wiring / routing).

- Transmitting LRU and port.

- Label and SDI.

Additionally: - LRUs have equipment IDs and may use standardized label sets. - Higher-level protocols (file transfer, data loading, maintenance) may introduce additional addressing or identifiers.

## 11. Special and Extended Features

### 11.1 25-bit Word Format

Some implementations support a shortened 25-bit word format:

- Bits 1–8: Label

- Bits 10–24: Data (16 bits)

- Bit 25: Parity

This is a special-case format used in certain applications; the same general principles (label + data + parity) apply.

### 11.2 File Transfer and Data Loading Protocols

ARINC 429 provides the physical and word-level protocol. On top of it, higher-level protocols exist, such as:

- ARINC 429 Part 3 / Williamsburg/Buckhorn protocol for structured message exchange.

- ARINC 615 for software/data loading over 429 (e.g., uploading navigation databases or avionics software).

These define framing, sequencing, acknowledgments, and other functions for large data transfers.

### 11.3 Evolution Beyond ARINC 429

- ARINC 629: A later standard with multi-transmitter capability and TDMA access control; used on some aircraft but not as ubiquitous as 429.

- ARINC 664 Part 7 (AFDX): An Ethernet-based, deterministic network providing virtual links that emulate dedicated data paths (in many ways conceptually similar to multiple 429 links over IP/Ethernet).

- Modern aircraft often use a mix of ARINC 429 and AFDX, with gateways translating between them.

## 12. Comparisons with Other Avionics Buses

### 12.1 ARINC 429 vs CAN (Controller Area Network)

- ARINC 429:

  - Simplex, single talker per bus.

  - Fixed 32-bit words, fixed structure.

  - No arbitration, no ACK; deterministic by scheduling.

  - Widely used in commercial transport avionics.

- CAN:

  - Multi-master bus with bitwise arbitration.

  - Messages identified by IDs; payload is separate (0–8 bytes in classical CAN).

  - Has CRC, ACKs, error confinement mechanisms.

  - Common in automotive and some avionics subsystems, especially where flexibility and multi-master behavior is needed.

### 12.2 ARINC 429 vs MIL-STD-1553

- ARINC 429:

  - One-way traffic per channel (simplex).

  - No bus controller; each transmitter just sends on its schedule.

  - Simpler, lighter-weight protocol.

- MIL-STD-1553:

  - Dual-redundant, multi-drop bus.

  - Uses a Bus Controller (BC) that centrally schedules all communication.

  - Remote Terminals (RTs) and Monitor Terminals.

  - More flexible but more complex.

Both are widely used in aerospace, often in different roles.

## 13. Practical Implementation Notes

When implementing or interfacing with ARINC 429 in hardware or software, keep in mind:

- Label bit order:

  - Understand how your interface hardware expects the label bytes.

  - Often, you must bit-reverse the label from the octal representation.

- Scaling and units:

  - For each label, know the specific BNR/BCD encoding, resolution, and units.

  - Misinterpreting scale or sign can lead to incorrect engineering values.

- SSM handling:

  - Always check SSM bits to distinguish:

    - Normal operation

    - No Computed Data (NCD)

    - Functional test

    - Failure conditions

- Parity checking:

  - Implement parity checks and discard/flag words with parity errors.

- Repetition rates:

  - Know the expected update rate of each label; large gaps may indicate a fault.

## 14. Summary

- ARINC 429 is a simplex, deterministic, word-oriented avionics bus.

- It uses differential BPRZ signaling over 78 Ω twisted pair at fixed bit rates (12.5 or 100 kbps).

- Each 32-bit word is structured as Label, SDI, Data, SSM, and Parity.

- Data fields support BNR, BCD, and discrete encodings, with detailed per-label definitions.

- SSM and parity provide basic integrity and status information.

- System-level design (multiple channels, redundancy, scheduling) delivers the overall robustness and safety required in transport-category aircraft.

These fundamentals form the basis for deeper topics like ARINC label mapping for specific LRUs, gateway design, and integration with more modern networks like AFDX.