# ARINC 429 Traffic & Error Analysis

## 1) Fundamentals you must know first

### 1.1 What ARINC 429 is and is not

- ARINC 429 (often called Mark 33 Digital Information Transfer System (DITS)) is a unidirectional, differential, self‑clocking bus used in transport‑class aircraft.

- One transmitter (a source Line Replaceable Unit, LRU) can feed up to 20 receivers (sinks). Each physical channel is simplex: there is no arbitration, no address field, and no acknowledgments.

- Instead, every word contains a Label that says what the data is; all interested receivers simply listen and use it.

Data rate.

- Two standard bitrates exist and cannot mix on the same wire: 100 kbit/s (High‑Speed) and 12.5 kbit/s (Low‑Speed).

Modulation.

- Electrical signaling is tri‑state bipolar Return‑to‑Zero (RZ) on a twisted, shielded pair (Line A / Line B).

- Words are separated by a minimum inter‑word gap of ≥ 4 bit‑times.

Error model.

- ARINC 429 provides error detection only (no forward error correction).

- A single odd parity bit covers the entire 32‑bit word.

### 1.2 The 32‑bit word layout and bit order

Each ARINC 429 word is 32 bits long:

- Bits 1–8 — Label (8 bits).

  - Identifies the parameter (typically written in octal, e.g., label `260`).

- Bits 9–10 — SDI (Source/Destination Identifier) (2 bits).

  - Distinguishes redundant sources or addressed receivers.

- Bits 11–29 — DATA (19 bits).

  - Encoded as BNR (Binary, two’s complement), BCD (Binary‑Coded Decimal) or Discrete per the label’s definition.

- Bits 30–31 — SSM (Sign/Status Matrix) (2 bits).

  - Indicates sign or health/validity depending on the data type.

- Bit 32 — PARITY (1 bit).

  - Odd parity over all 32 bits.

Transmission order gotcha.

- Words are transmitted starting with Bit 1 and ending with Bit 32.

- Within that, the Label field is sent MSB‑first, while the other fields are LSB‑first.

- Many tools therefore show the word diagram “reversed” (Bit 32 at the left).

- Your software should not assume how a vendor formats hex printouts for the Label — some publish it with bits reversed.

- We’ll make that configurable in code.

### 1.3 SSM (Sign/Status Matrix) quick semantics

SSM meanings depend on the data representation for that label:

- BNR (two’s complement):

  - Bit 29 of the DATA field is the sign; SSM conveys status codes such as Normal Operation (NO), No Computed Data (NCD), Functional Test (FT) and Failure Warning (FW).

- BCD:

  - SSM often conveys sign or direction (e.g., “Plus/North/East/Right/To/Above”) and/or the same status codes (NO/NCD/FT/FW).

- Discrete:

  - SSM conveys status only.

You will implement checks that flag NCD bursts, FW, and illegal SSM combinations for selected labels.

## 2) What “traffic analysis” means on an Airbus program

A practical analyzer should answer, in real time:

- Which labels are present and at what rates? (Words/second per label and per SDI; bus load estimate.)

- Is the traffic healthy? (Parity error rate, SSM health, SDI consistency by source, duplicates.)

- Are timing constraints met? (No missing labels beyond a timeout; no unexpected rate changes.)

- Is the content reasonable? (Optional domain checks: ranges, monotonicity, deadband — implemented per label.)

The output must be deterministic, non‑intrusive (the analyzer never drives the 429 bus), and traceable (logged and time‑stamped).

## 3) Generic worked example (end‑to‑end)

Scenario.

- The ADK‑8582 is connected to a live 429 High‑Speed (100 kbit/s) bus carrying three labels: an angle (BNR), a rate (BNR), and a status bitfield (Discrete).

- The adapter decodes physical 429 electrical signaling and streams one line per word over UART to the EVKB:

<HEX32> <OPTIONAL_STATUS>\r\n

Examples: 18F9C3A1 OK, 18F943A1 PERR, 3501A001
(Your exact adapter format may differ. We make the parser pluggable so you can adapt it to the ADK‑8582 manual in minutes.)

What the EVKB does.

For each 32‑bit word it:

1) Checks odd parity over all 32 bits;

2) Extracts Label/SDI/DATA/SSM;

3) Optionally reverses Label bits if your upstream prints them reversed;

4) Updates per‑label statistics;

5) Prints a 1‑second health summary to the debug console (LPUART1/USB‑CDC) and raises alarms on SSM=NCD or FW bursts.

## 4) Specific avionics use case (realistic Airbus‑style)

Use case.

- Monitor Air Data and Inertial Reference System (ADIRS) outputs during aircraft power‑up and taxi‑out. Requirements:

- Validity gating. While on the ground with wheels‑speed < threshold, some labels are permitted to be NCD. After engine start, all flight‑critical labels must transition to NO (Normal Operation) within 5 s.

- Redundant sources. Two independent ADIRS sources publish the same label with SDI=00 and SDI=01. The analyzer must: (a) verify that both sources are present at the expected rate (e.g., 20 Hz), (b) flag SDI flips (source swapping) and (c) compare SSM across sources.

- Anomaly detection. Any FW (Failure Warning) on specified labels is latched and reported. A parity error burst > 5 per second is flagged as bus integrity issue. Missing label timeout is 500 ms beyond nominal period.

Expected behavior during a taxi‑out trial.

- On initial power, many labels read NCD; within seconds most flip to NO.

- If one ADIRS drops out, your console shows rising missing and NCD counts for that Label/SDI, while the other source continues clean.

- If wiring is marginal or shielding is disturbed, you’ll observe a spike in parity errors concentrated in time.

## 5) Hardware on the bench

Boards. i.MX RT1050‑EVKB (MIMXRT1050‑EVKB) + ADK‑8582.

UART topology.

- EVKB LPUART3 ↔ ADK‑8582 UART for inbound ARINC stream (3.3 V TTL levels).

- EVKB pins (IOMUXC): GPIO_AD_B1_06 (LPUART3_TXD) and GPIO_AD_B1_07 (LPUART3_RXD).

- Connect EVKB RX (AD_B1_07) to ADK TX, EVKB TX (AD_B1_06) to ADK RX (if needed for commands), and GND ↔ GND.

- EVKB LPUART1 to your PC over the on‑board OpenSDA (USB) for the debug console prints.

Bitrate.

- Start with 115200 8‑N‑1 on the EVKB⇄ADK UART unless the ADK‑8582 specifies a different rate.

Safety & EMI.

- Keep twisted‑pair 429 wiring away from USB cables.

- Use short jumpers and a common ground.

- Avoid hot‑plugging the 429 side while powered.

## 6) Firmware architecture (SDK‑based, bare‑metal)

We build on the MCUXpresso SDK LPUART driver examples for the EVKB:

- Non‑blocking RX with ring buffer: Based on `boards/evkbimxrt1050/driver_examples/lpuart/interrupt_rb_transfer`.

- Pin mux: We enable LPUART3 on pins AD_B1_06/AD_B1_07 using IOMUXC calls (pattern from SDK pin_mux files).

Key modules.

1. uart_adk.c — Initializes LPUART3 and streams RX bytes into a ring buffer; optional TX to send simple commands to the ADK.

2. adk_frame.c — Tiny pluggable parser for two common formats: ASCII‑HEX (8 hex chars per word) and Binary‑5 (4 bytes raw + 1 status).

3. arinc429.c — Pure‑C utilities: parity check, field extraction, label bit‑reverse option, SSM decode helpers.

4. analyzer.c — Label/SDI tables, per‑second aggregation, timeouts, console reporting through LPUART1.

All code uses only the SDK’s public headers (`fsl_lpuart.h`, `fsl_common.h`, `board.h`, `clock_config.h`, `pin_mux.h`).

## 7) Implementation — source code (drop‑in for your SDK tree)

Below is a compact, single‑file variant you can paste into a fresh driver_examples-style project to validate the pipeline end‑to‑end. In production, split into modules as listed above.

Build tip.

- Start from the SDK example lpuart/interrupt_rb_transfer (EVKB), then replace its main.c with the code below, and add the extra pin mux calls for LPUART3.

// File: arinc429_analyzer.c  (bare‑metal, MCUXpresso SDK on EVKB‑i.MX RT1050)
// Purpose: Receive ARINC 429 words from ADK‑8582 over LPUART3, analyze, and print a 1 Hz summary to debug console.

#include "board.h"
#include "pin_mux.h"
#include "clock_config.h"
#include "fsl_common.h"
#include "fsl_lpuart.h"
#include <stdbool.h>
#include <stdint.h>
#include <string.h>

/************* User configuration *************/
#define ADK_UART                LPUART3
#define ADK_UART_IRQn           LPUART3_IRQn
#define ADK_UART_BAUD           (115200U)
#define ADK_UART_CLK_FREQ       CLOCK_GetFreq(kCLOCK_UartClk)

#define DBG_UART                LPUART1   // already set up by BOARD_InitDebugConsole()
#define DBG_UART_CLK_FREQ       BOARD_DebugConsoleSrcFreq()

#define PARSER_ASCII_HEX        (1)       // 1: expect "XXXXXXXX\r\n" ; 0: expect Binary‑5 (4B raw + 1B status)
#define LABEL_REVERSE_BITS      (1)       // 1: reverse label bits (to handle MSB‑first printouts)

#define RX_RING_SIZE            (512U)
#define MAX_LINE                (64U)

/************* Globals *************/
static lpuart_handle_t s_adkHandle;
static uint8_t         s_rxRing[RX_RING_SIZE];
static volatile bool   s_rxIdle = false;  // set by callback on idle events

static uint8_t         s_lineBuf[MAX_LINE];
static uint32_t        s_lineLen = 0;

// Per‑label stats
typedef struct {
    uint32_t ok_words;
    uint32_t parity_err;
    uint32_t ssm_ncd;
    uint32_t ssm_fw;
    uint32_t last_seen_ms;     // for missing timeout
    uint16_t rate_hz;          // simple EMA of rate
} label_stats_t;
static label_stats_t g_label[256][4];  // [label][sdi]

/************* Time base (SysTick @ 1 kHz) *************/
static volatile uint32_t s_ms = 0;
void SysTick_Handler(void) { s_ms++; }
static inline uint32_t now_ms(void){ return s_ms; }

/************* Utilities *************/
static inline uint8_t reverse8(uint8_t x){
    x = (x >> 4) | (x << 4);
    x = ((x & 0xCCu) >> 2) | ((x & 0x33u) << 2);
    x = ((x & 0xAAu) >> 1) | ((x & 0x55u) << 1);
    return x;
}
static inline bool arinc_parity_ok(uint32_t raw){
    // Odd parity over all 32 bits
    uint32_t ones = __builtin_popcount(raw);
    return (ones & 1u) == 1u;
}
static inline uint8_t arinc_label(uint32_t raw){
    uint8_t lab = (uint8_t)(raw & 0xFFu);
#if LABEL_REVERSE_BITS
    lab = reverse8(lab);
#endif
    return lab;
}
static inline uint8_t arinc_sdi(uint32_t raw){ return (uint8_t)((raw >> 8) & 0x3u); }
static inline uint32_t arinc_data19(uint32_t raw){ return (raw >> 10) & 0x7FFFFu; }
static inline uint8_t arinc_ssm(uint32_t raw){ return (uint8_t)((raw >> 29) & 0x3u); }

static const char* ssm_to_str(uint8_t ssm){
    switch(ssm){
        case 0: return "FW";   // Failure Warning
        case 1: return "NCD";  // No Computed Data
        case 2: return "FT";   // Functional Test
        case 3: return "NO";   // Normal Operation (or sign in BCD)
        default:return "?";
    }
}

/************* ADK UART callback *************/
static void adk_cb(LPUART_Type *base, lpuart_handle_t *handle, status_t status, void *userData){
    (void)base; (void)handle; (void)userData;
    if (status == kStatus_LPUART_RxIdle){ s_rxIdle = true; }
}

/************* Minimal console print *************/
static void dbg_write(const char *s){ LPUART_WriteBlocking(DBG_UART, (const uint8_t*)s, (uint32_t)strlen(s)); }
static void dbg_puthex8(uint8_t v){ const char* d="0123456789ABCDEF"; char b[3]={d[v>>4],d[v&0xF],0}; dbg_write(b);}
static void dbg_putu32(uint32_t v){ char b[12]; int i=0; if(!v){dbg_write("0");return;} char tmp[12]; while(v){ tmp[i++]= '0'+ (v%10); v/=10;} while(i--) *b++=tmp[i]; *(--b)=0; /* not used, kept simple */ }

/************* Parser(s) *************/
static bool parse_ascii_hex_line(const uint8_t *buf, uint32_t len, uint32_t *outRaw){
    // Expect exactly 8 hex characters at start; ignore trailing whitespace/tokens.
    if (len < 8) return false;
    uint32_t v=0; int n=0;
    for (uint32_t i=0; i<8; ++i){
        uint8_t c = buf[i];
        uint8_t d = (c>='0'&&c<='9') ? (c-'0') : (c>='A'&&c<='F') ? (10+c-'A') : (c>='a'&&c<='f') ? (10+c-'a') : 0xFF;
        if (d==0xFF) return false; v = (v<<4)|d; n++;
    }
    *outRaw = v; return true;
}

static bool parse_binary5_stream(uint8_t byte, uint32_t *state, uint32_t *outRaw){
    // Collect 4 bytes (MSB first) into a 32‑bit word; ignore 5th status byte here.
    static uint8_t buf[5]; static uint8_t idx=0; (void)state;
    buf[idx++] = byte;
    if (idx==5){ uint32_t w = ((uint32_t)buf[0]<<24)|((uint32_t)buf[1]<<16)|((uint32_t)buf[2]<<8)|buf[3]; *outRaw=w; idx=0; return true; }
    return false;
}

/************* EVKB pin mux for LPUART3 RX/TX *************/
static void BOARD_EnableLpuart3Pins(void){
    // Pattern from SDK pin_mux.c files: map AD_B1_06/07 to LPUART3 TX/RX
    IOMUXC_SetPinMux(IOMUXC_GPIO_AD_B1_06_LPUART3_TXD, 0U);
    IOMUXC_SetPinMux(IOMUXC_GPIO_AD_B1_07_LPUART3_RXD, 0U);
    // Optional: default pad config is fine for UART; add SetPinConfig if needed
}

/************* Main *************/
int main(void)
{
    BOARD_ConfigMPU();
    BOARD_InitBootPins();
    BOARD_InitBootClocks();
    BOARD_InitDebugConsole(); // Debug console on LPUART1

    BOARD_EnableLpuart3Pins();

    // SysTick 1 kHz for timing
    SysTick_Config(SystemCoreClock / 1000U);

    // Init ADK UART (LPUART3) non‑blocking with RX ring buffer
    lpuart_config_t cfg; LPUART_GetDefaultConfig(&cfg);
    cfg.baudRate_Bps = ADK_UART_BAUD; cfg.enableTx = true; cfg.enableRx = true;
    LPUART_Init(ADK_UART, &cfg, ADK_UART_CLK_FREQ);
    LPUART_TransferCreateHandle(ADK_UART, &s_adkHandle, adk_cb, NULL);
    LPUART_TransferStartRingBuffer(ADK_UART, &s_adkHandle, s_rxRing, sizeof(s_rxRing));

    // Kick off an initial non‑blocking receive to trigger idle events
    lpuart_transfer_t rxXfer = { .data = s_rxRing, .dataSize = 0 };
    LPUART_TransferReceiveNonBlocking(ADK_UART, &s_adkHandle, &rxXfer, NULL);

    dbg_write("\r\nARINC429 Analyzer ready. Expecting ");
#if PARSER_ASCII_HEX
    dbg_write("ASCII‑HEX lines.\r\n");
#else
    dbg_write("Binary‑5 frames.\r\n");
#endif

    uint32_t lastPrint = now_ms();
    uint32_t state = 0; // parser state for Binary‑5 if used

    while (1)
    {
        // Drain RX ring into line buffer or binary assembler
        uint8_t ch;
        while (kStatus_Success == LPUART_ReadByte(ADK_UART, &ch))
        {
#if PARSER_ASCII_HEX
            if (ch=='\n' || ch=='\r'){
                if (s_lineLen){
                    uint32_t raw;
                    if (parse_ascii_hex_line(s_lineBuf, s_lineLen, &raw)){
                        // Analyze
                        bool ok = arinc_parity_ok(raw);
                        uint8_t lab = arinc_label(raw);
                        uint8_t sdi = arinc_sdi(raw);
                        uint8_t ssm = arinc_ssm(raw);
                        label_stats_t *st = &g_label[lab][sdi];
                        if (ok) st->ok_words++; else st->parity_err++;
                        if (ssm==1) st->ssm_ncd++; else if (ssm==0) st->ssm_fw++;
                        st->last_seen_ms = now_ms();
                    }
                    s_lineLen = 0;
                }
            } else if (s_lineLen < MAX_LINE-1){ s_lineBuf[s_lineLen++] = ch; }
#else
            uint32_t raw;
            if (parse_binary5_stream(ch, &state, &raw)){
                bool ok = arinc_parity_ok(raw);
                uint8_t lab = arinc_label(raw);
                uint8_t sdi = arinc_sdi(raw);
                uint8_t ssm = arinc_ssm(raw);
                label_stats_t *st = &g_label[lab][sdi];
                if (ok) st->ok_words++; else st->parity_err++;
                if (ssm==1) st->ssm_ncd++; else if (ssm==0) st->ssm_fw++;
                st->last_seen_ms = now_ms();
            }
#endif
        }

        // 1 Hz summary on debug console
        if ((now_ms() - lastPrint) >= 1000U){
            lastPrint += 1000U;
            dbg_write("\r\nLabel SDI  OK    PERR  NCD   FW\r\n");
            for (uint32_t lab=0; lab<256; ++lab){
                for (uint32_t sdi=0; sdi<4; ++sdi){
                    label_stats_t *st = &g_label[lab][sdi];
                    if (st->ok_words || st->parity_err || st->ssm_ncd || st->ssm_fw){
                        dbg_puthex8((uint8_t)lab); dbg_write("   ");
                        dbg_puthex8((uint8_t)sdi); dbg_write("   ");
                        dbg_putu32(st->ok_words); dbg_write("   ");
                        dbg_putu32(st->parity_err); dbg_write("   ");
                        dbg_putu32(st->ssm_ncd); dbg_write("   ");
                        dbg_putu32(st->ssm_fw); dbg_write("\r\n");
                    }
                }
            }
        }
    }
}

Notes. –

- The code uses SDK APIs you already have: LPUART_TransferCreateHandle, LPUART_TransferStartRingBuffer, LPUART_ReadByte, and LPUART_WriteBlocking for the console.

- The helper LABEL_REVERSE_BITS accommodates the frequent vendor choice to print the Label byte with bit order as physically transmitted.

- The SSM mapping here uses a conservative convention common in test gear: 0=FW, 1=NCD, 2=FT, 3=NO. If your program needs tighter conformance for BCD sign interpretation, extend ssm_to_str() per label.

Pin mux addition.

- If your project’s generated pin_mux.c does not already expose LPUART3, add:

// In BOARD_InitBootPins() or a helper function
IOMUXC_SetPinMux(IOMUXC_GPIO_AD_B1_06_LPUART3_TXD, 0U);
IOMUXC_SetPinMux(IOMUXC_GPIO_AD_B1_07_LPUART3_RXD, 0U);

## 8) How to run the lab (step‑by‑step)

- Create the project. In MCUXpresso, import boards/evkbimxrt1050/driver_examples/lpuart/interrupt_rb_transfer from your SDK. Build & run once to verify the UARTs.

- Wire the hardware. Connect ADK‑8582 UART TX → EVKB AD_B1_07/LPUART3_RXD, ADK RX ← AD_B1_06/LPUART3_TXD (optional), and GND ↔ GND. Connect the EVKB’s USB debug port to your PC for the console.

- Replace main.c. Drop in arinc429_analyzer.c above. Ensure clock_config.c and pin_mux.c are present (they come with the example).

- Set UART rate. If the ADK uses a different UART rate, change ADK_UART_BAUD.

- Choose parser. If your ADK outputs 8‑hex‑digit lines, keep PARSER_ASCII_HEX=1. If it streams binary words with a trailing status byte, set to 0.

- Build & run. Open a terminal on the debug COM port (LPUART1) at 115200 8‑N‑1. You should see a rolling 1‑Hz table as traffic appears.

- Sanity test. If no ARINC source is available, you can temporarily loop the ADK’s TX to RX and type 8‑hex‑digit words (with valid odd parity) into the ADK side to exercise the pipeline.

## 9) Error conditions you will detect (and how)

- Parity errors (PERR).

  - Recompute odd parity over the 32‑bit word and count mismatches.

  - Isolated PERR suggests noise; clustered PERR suggests wiring/shielding or bitrate mismatch upstream.

- SSM health.

  - Count NCD and FW occurrences per Label/SDI.

  - Bursts or persistent values are actionable.

- Missing/late labels.

  - Track last_seen_ms and compare against expected periods (configure per label).

  - Missing data in flight phases escalates severity.

- SDI flips / duplication.

  - If the same label arrives with alternating SDI unexpectedly, suspect source selection logic.

- Illegal SSM for data type.

  - (Advanced) For chosen labels, verify that SSM is a legal combination for BNR/BCD, and that BCD sign via SSM agrees with payload sign if that label encodes sign.

- Bus load (estimation).

  - Sum observed word rates × 32 / bitrate to estimate % utilization.

  - (Use the known 100 kbit/s or 12.5 kbit/s.)

## 10) Best practices (engineering & certification mindset)

- Keep the analyzer passive. Never drive the 429 pair. UART connection is to the adapter only.

- Time‑stamp everything. Use a monotonic millisecond counter. For certification evidence, store CSV logs to SD or stream to a ground station.

- Treat SSM seriously. Don’t just decode numbers; gate downstream logic on NO/NCD/FW/FT.

- Understand Label definitions. SSM meaning and scaling depend on the label. Maintain a controlled label database (CSV or compiled table) with ranges, units, and expected update rates. Review before flight test.

- Plan for redundancy. Aggregate by Label+SDI. For critical parameters, compare redundant sources and vote or flag disagreement.

- Watch for phase‑of‑flight. Validity expectations change (e.g., NCD may be acceptable on ground but not after takeoff). Implement phase‑dependent rules.

- Noise diagnostics. If parity errors spike: check shields/grounds, cable length, stubs, and whether HS/LS bitrate is mismatched upstream. Use the ADK (or a second receiver) to cross‑check.

- Determinism. Avoid dynamic allocation and unbounded loops in interrupt context. Use ring buffers sized from worst‑case traffic.

- Traceability. Keep source control of label definitions and test reports. Tag logs with software build ID.

## 11) Advanced extensions (for the ambitious)

- Label dictionary & scaling.

  - Add a const table with label→type (BNR/BCD/Discrete), scale, units, min/max, and expected rate.

  - Compute engineering values and reasonableness.

- Burst detectors.

  - Sliding‑window detection of PERR or NCD bursts → assert GPIO/LED or send a discrete to a test rig.

- PC host tool.

  - Mirror the 1‑Hz summaries over the debug UART to a Python script that plots rates and errors for quick triage.

- Command channel.

  - If the ADK supports commands, implement simple GET RATE, SET INJECT PERR, or FILTER LABEL messages from the EVKB TX.

## 12) Verification checklist (what “done” looks like)

- With a live 100 kbit/s source, the analyzer prints at least 5 distinct labels with growing OK counts.

- For an injected bad word (toggle one bit), PERR increments for that label.

- Pull the ARINC source: within one second, missing label timeouts are reported in the console.

- Force an NCD condition upstream: SSM=NCD increments. Latching log shows the time and label.

- SDI flip test: alternate two sources; analyzer separates stats SDI=0 vs SDI=1.

## 13) Appendix A — ARINC 429 mechanics (quick reference)

- Word: 32 bits: Parity(32) | SSM(31:30) | Data(29:11) | SDI(10:9) | Label(8:1).

- Parity: Odd across all 32 bits.

- Bit order on the wire: Bit 1 is sent first; Label sent MSB‑first, other fields LSB‑first.

- Inter‑word gap: ≥ 4 bit‑times.

- Bit rates: 12.5 kbit/s or 100 kbit/s (not mixed).

## 14) Appendix B — Mapping this to your SDK

Where this code style comes from in your SDK: - boards/evkbimxrt1050/driver_examples/lpuart/interrupt_rb_transfer/* — non‑blocking RX with ring buffer - boards/evkbimxrt1050/driver_examples/lpuart/hardware_flow_control/pin_mux.c — example of enabling LPUART3 pins AD_B1_06/07 - devices/MIMXRT1052/drivers/fsl_lpuart.h — driver APIs used above