## 1) What the ARINC 429 specification says that matters for the HI-8582

From the ARINC 429 protocol specification:

- ARINC 429 is BPRZ (bipolar return-to-zero) differential signaling: ONE ≈ +10 V diff, ZERO ≈ −10 V diff, NULL ≈ 0 V diff.

- Two standard rates: 100 kbps (high) and 12.5 kbps (low), and a minimum NULL word gap between words.

- The word is 32 bits: Label (1–8), SDI (9–10), Data (11–29), SSM (30–31), Parity (32), and the note explicitly warns about transmission order / label bit order being a common “gotcha”.

The HI-8582 is designed to implement exactly these physical/timing requirements and to present a parallel-bus view that can either be “legacy scrambled” or “clean unscrumbled”.

## 2) How the HI-8582 implements ARINC 429 in hardware

### 2.1 Receiver physical layer + timing enforcement (the “real ARINC” part)

The HI-8582 has analog differential receivers and comparators that classify the incoming differential pair into ONE / NULL / ZERO using ARINC-style thresholds, and then a digital timing checker driven by an internal sampling clock (derived from CLK).

It enforces ARINC timing by:

- requiring runs of consecutive samples to accept a ONE/ZERO/NULL,

- checking that each bit arrives within an expected sample window (rate discrimination),

- enforcing a NULL word gap before the next word is accepted.

There is also a pin NFD (“No Frequency Discrimination”) that disables strict bit-rate discrimination (it will accept any 32 bits, but still requires a valid word gap). This is useful in “non-standard” timing situations.

### 2.2 Receiver parity handling (important subtlety!)

On receive, the HI-8582 does not simply give you the raw parity bit as “bit 32”. Instead, it stores a parity flag in the “32nd bit position” of the FIFO word:

- 0 → good odd parity received

- 1 → parity error detected

So when you read a received 32-bit word, bit 32 is a status flag, not necessarily the original parity bit value.

That matters when you’re comparing your “protocol doc’s bit 32 = parity” view to what software sees.

### 2.3 Transmitter physical layer + timing generation

On transmit, the HI-8582 contains:

- a FIFO,

- a 32-bit shift register,

- a sequencer that emits DATA then NULL for each bit (BPRZ),

- automatic word gap insertion,

- a line driver that generates the differential output.

It supports high/low speed and controls the output slope via the control register (TX rate + rise/fall shaping).

## 3) Bit ordering “on the wire” vs “on the bus” for the HI-8582

### 3.1 Transmission order (bit 1 … bit 32)

The HI-8582 transmitter shifts the word out as ARINC bit 1 first, then bit 2, …, then bit 32, and it labels its own timing diagrams as “DATA BIT 1 … DATA BIT 32”.

So: you do not do a full 32-bit reversal for the HI-8582. You load “bit positions”, and the chip serializes them in ARINC’s bit-number order.

### 3.2 The “reversal” you may still need: the label octal gotcha

The HI-8582 does not provide a dedicated “reverse label bits for me” control. In fact, it explicitly says label significance is not changed by the scramble/unscramble setting: label compare is always BD00–BD07 ↔ ARINC bits 1–8.

Practical implication:

- If your software works with labels in the usual ARINC “octal label” convention, you typically need a bit-reverse of the 8-bit label when mapping that label into ARINC bits 1–8 (and reverse again when interpreting received labels), because the documentation convention for label presentation often doesn’t match “bit1..bit8 as you’d store them in a byte”. Your ARINC note explicitly calls this out as a common hardware-programming pitfall.

A very common sanity check:

- If someone expects label 001(octal) and you see 0x80 (binary 1000 0000) patterns involved, you’re in the “bit-reversed label” world.

### 3.3 Scramble vs Unscramble (CR15) — this is the HI-8582’s main “ordering handle”

The HI-8582 gives you a data format mapping control:

- CR15 = 0 → “Scramble ARINC data”

- CR15 = 1 → “Unscramble ARINC data”

This does not change what is transmitted on the wire; it changes how the 32 ARINC bit positions are mapped onto the 16-bit bus during reads/writes.

Why this exists: legacy HI-8282-style software often expects a “scrambled” layout where label and some status/parity bits sit in convenient bus positions. Unscramble gives a clean “bit-position contiguous” view.

Best practice if you want minimal confusion:

- Use CR15 = 1 (unscramble) so your parallel writes/reads correspond to straightforward ARINC bit positions across two 16-bit transfers (lower half then upper half).

Then your only “reversal” concern is usually label-octal ↔ label-byte mapping, not the whole 32-bit word.

## 4) Do we need reversal logic?

### You do not need “reverse the entire word” logic

Because the chip transmits bit 1 → bit 32 itself.

### You may need label-bit reversal in software

Because labels are commonly handled in octal / MSB-first conventions in documentation, while the chip’s programming model is “ARINC bits 1–8 sit in BD00–BD07”. The protocol note explicitly warns this is a common gotcha with ARINC interface chips.

### CR15 is not label reversal

CR15 changes the bus mapping, but label compare remains BD00–BD07 ↔ ARINC bits 1–8 regardless.

## 5) What programming handles you have (what you can “control”)

### 5.1 Core configuration handle: Control Register (CR0–CR15)

You load the 16-bit control register via BD00–BD15 when CWSTR is pulsed. Key bits you’ll use most for “protocol compliance”:

- CR0 / CR14: Receiver 1 / Receiver 2 data rate select (CLK/10 vs CLK/80)

- CR13: Transmitter data rate + output slope (high speed vs low speed shaping)

- CR15: Scramble vs Unscramble mapping

- CR4: Transmit bit 32 is parity or data

- CR12: Odd vs even parity selection when parity enabled

- CR1: Label memory read/write sequencing

- CR2/CR3: Enable label recognition on RX1/RX2

- CR6–CR11: Optional decoder match on ARINC bits 9/10 (SDI-like filtering)

### 5.2 Data movement handles (parallel bus “transactions”)

Transmit (to FIFO):

- Put 16-bit “Byte 1” on BD[15:0], pulse PL1

- Put 16-bit “Byte 2” on BD[15:0], pulse PL2

- The word is queued in the TX FIFO; TX/R, HFT, FFT indicate readiness/levels.

Receive (from FIFO):

- Use EN1 / EN2 to enable receiver output onto the bus

- Use SEL to choose “Byte 1 vs Byte 2”

- D/R1 / D/R2, HF1/2, FF1/2 expose FIFO status.

### 5.3 Label programming handle (16 labels per receiver)

- Set CR1=1, then perform 16 label writes (label byte on BD00–BD07, pulse PL1 for RX1 labels / PL2 for RX2 labels)

- Label reads are likewise sequenced with EN1/EN2.

### 5.4 “Strict ARINC timing vs accept-any-32-bits”

- NFD pin gives you the “turn off strict rate discrimination” option (still enforces word gap).

## 6) A clean mental model to implement correctly (recommended)

If you want the cleanest alignment between your protocol description and what you program:

- Set CR15 = 1 (unscramble) so bus mapping matches ARINC bit positions in a contiguous way.

- Build the word by assigning Label→bits 1–8, SDI→9–10, Data→11–29, SSM→30–31.

- If your labels are coming from ARINC tables in octal, apply a bit-reverse of the label byte when placing it into bits 1–8 (and reverse back when reading).

- Decide how you want bit 32:

  - Normal ARINC: CR4=1 (bit 32 is parity), and choose odd/even via CR12.

  - Special/nonstandard payload: CR4=0 (bit 32 becomes data).

- On receive, remember: “bit 32” you read is parity status, not the original parity bit.

The HI-8582 Receiver

## 0) The key idea in one sentence

The HI-8582 receiver does oversampling + pattern validation:

- It samples the line at a fixed tick (derived from an accurate 1 MHz reference),

- it only believes a value (ONE / ZERO / NULL) if it sees it stable for 3 consecutive samples,

- it verifies each bit looks like RZ (data level then returns to NULL inside the bit time),

- it verifies the bit period is correct (next bit arrives 8–12 “ticks” later),

- it verifies a word gap (enough continuous NULL) before accepting the next 32-bit word,

- and NFD can disable the bit-rate check.

## 1) What is the “sample tick” in high speed vs low speed?

Your excerpt says the timing checker relies on an accurate 1 MHz clock.

The receiver then uses two effective sampling modes:

### High speed (≈100 kbps)

- Sample tick = 1 µs (1 MHz)

- A 10-bit sampling shift register spans 10 µs

- Nominal bit time at 100 kbps is 10 µs

- So: 10 samples ≈ one ARINC bit cell

### Low speed (≈12.5 kbps)

The receiver effectively stretches the tick by 8× (that’s consistent with the “80 clocks” wording in the word-gap logic and the low-speed bit rate range they compute).

- Effective sample tick ≈ 8 µs

- 10-bit shift register spans 80 µs

- Nominal bit time at 12.5 kbps is 80 µs

- So again: 10 samples ≈ one ARINC bit cell

This is why the logic can reuse the same “10-bit window rules” for both rates.

## 2) Step (2): “10-bit sampling shift registers” + “3 consecutive” rule

### 2.1 What is being sampled?

At each sample tick, the analog front end classifies the line into one of three states:

- ONE (positive pulse)

- ZERO (negative pulse)

- NULL (near 0)

Think of each sample as producing a symbol: O, Z, or N.

### 2.2 Why “3 consecutive”?

To reject noise and slow edges, the chip requires stability:

- Three consecutive O → a valid ONE level exists

- Three consecutive Z → a valid ZERO level exists

- Three consecutive N → a valid NULL exists

So a single noisy spike won’t be treated as real data.

### Example A (High speed): Valid ONE pulse

A correct ARINC RZ “ONE” at 100 kbps is roughly:

- +level for ~5 µs, then

- NULL for ~5 µs

With 1 µs sampling, a typical 10-sample window inside that bit might look like:

O O O O O N N N N N

- Upper part has ≥3 consecutive O

- Lower part has ≥3 consecutive N
✅ Accepted as a valid data bit (“ONE with return-to-zero”)

### Example B (High speed): Too narrow pulse (rejected)

Suppose the positive part is only 2 µs (too narrow):

O O N N N N N N N N

Upper part has only 2 O (not 3)
❌ Not accepted as a valid ONE

This is how the “minimum pulse width” is indirectly enforced.

### Example C (Low speed): Valid ZERO pulse

Low speed bit cell ~80 µs, pulse width ~35–42 µs.
With ~8 µs sampling tick, ~40 µs corresponds to about 5 ticks:

Z Z Z Z Z N N N N N

Again:

- ≥3 consecutive Z

- followed by ≥3 consecutive N
✅ Accepted as valid ZERO

## 3) The specific RZ check: “ONE/ZERO in upper bits must be followed by NULL in lower bits”

This is the important nuance: it’s not enough to see a stable ONE or ZERO.

For a data bit, the receiver also demands that within the bit time:

- It sees ONE or ZERO in the earlier samples (“upper bits” of the shift register),

- and then sees NULL in the later samples (“lower bits”).

That’s exactly ARINC return-to-zero behaviour.

### Example D: A stuck-at level (rejected)

If the line stays positive the whole time (no return-to-zero):

O O O O O O O O O O

You have ≥3 O, but you do not see NULL in the lower part.
❌ Rejected (not a valid ARINC RZ bit)

This is how it rejects “NRZ-like” streams or a shorted driver.

### Example E: Word gap NULL requirement (stronger NULL check)

For “NULL in the word gap”, it wants:

- Three consecutive NULLs in both upper and lower parts of the sampling window

Meaning: it wants the line to be NULL throughout the observation window, not just returning to NULL briefly.

That prevents a new word from starting too soon, and prevents glitching from being treated as a gap.

## 4) Step (3): The bit-rate check: “each data bit must follow its predecessor by 8 to 12 samples”

Once it has validated one bit, it expects the next bit to occur in a tight timing window.

### High speed

- Tick = 1 µs

- Next bit must arrive 8–12 ticks later

- So bit time must be between 8 µs and 12 µs

- That corresponds to 125 kbps down to 83 kbps, exactly as listed.

#### Example F: Too fast (rejected)

If bits arrive every 6 µs:

Bit boundary spacing = 6 ticks
6 < 8 → ❌ rejected as out-of-rate

#### Example G: Too slow (rejected)

If bits arrive every 15 µs:

15 > 12 → ❌ rejected as out-of-rate

### Low speed

Effective tick ≈ 8 µs
So 8–12 ticks means:

- 8 ticks × 8 µs = 64 µs (max rate ≈ 15.6 kbps)

- 12 ticks × 8 µs = 96 µs (min rate ≈ 10.4 kbps)

Again matching the table in the excerpt.

## 5) Step (4): Word gap enforcement (why the chip won’t “back-to-back accept” words)

After the 32nd bit of a valid word, the receiver won’t even start looking for the next word until a minimum gap of continuous NULL is confirmed.

Mechanism described:

- After the last valid bit,

- the Word Gap timer checks the NULL shift register periodically:

  - every 10 input clocks in high speed

  - every 80 input clocks in low speed

- Each time it checks:

  - if NULL is present → increment a counter

- When the counter reaches 3, the receiver arms itself to accept the next word.

### What this means in real time

With a 1 MHz base clock:

#### High speed

- checks every 10 µs

- requires 3 successive “NULL present” checks
→ needs about 30 µs of confirmed NULL before next reception can start

#### Low speed

- checks every 80 µs

- requires 3 checks
→ needs about 240 µs of confirmed NULL

### Example H: Too little gap (rejected)

If a transmitter tries to start the next word after only ~10 µs of NULL at high speed:

- Word gap counter might reach only 1

- It never reaches 3
❌ next word start is ignored until sufficient gap is seen

This is exactly how the chip enforces the ARINC “word gap” concept in hardware.

## 6) What changes when NFD is held low?

- NFD high → strict timing enforcement (accept only within tolerances)

- NFD low → frequency discrimination disabled:

  - any stream totalling 32 bits is accepted,

  - even with gaps between bits,

  - but word gap is still required.

### Example I: “Bit-banged ARINC” with irregular bit spacing

Imagine a microcontroller emulating ARINC pulses but with jitter:

- bit 1 occurs,

- next bit occurs 20 µs later,

- next bit occurs 5 µs later,

- etc.

With NFD high, the “8–12 sample” rule fails → ❌ rejected.
With NFD low, the receiver does not apply the “8–12” timing gate → ✅ it can accept the word as long as it still sees 32 valid bit shapes and then sees the required word gap.

This mode is basically:
“Don’t enforce rate; still require a proper word boundary.”

## 7) A very concrete “walkthrough” of receiving one high-speed word

Let’s say the receiver sees three bits: 1, 0, 1 (high speed).

### Bit 1 (“1”)

- samples: O O O O O N N N N N

- 3 O in upper part + 3 N in lower part → valid RZ data bit

- bit timestamp recorded

### Bit 2 (“0”)

- must occur 8–12 µs later

- samples: Z Z Z Z Z N N N N N

- valid RZ data bit

### Bit 3 (“1”)

- again 8–12 µs later

- samples: O O O O O N N N N N

After bit 32, the receiver looks for word gap:

- it checks for NULL at 10 µs intervals

- needs 3 successive “still NULL”
Only then it arms for the next 32-bit word.

NFD on the HI-8582/HI-8583 is the “No Frequency Discrimination” input pin.

### What it normally does (NFD = HIGH)

The receiver performs frequency (bit-rate) discrimination:

- It checks that each data bit arrives at the right time relative to the previous one: 8 to 12 sample ticks later.

  - With a 1 MHz reference, that corresponds to accepted rates of roughly:

    - High speed: 83–125 kbps (centered around 100 kbps)

    - Low speed: 10.4–15.6 kbps (centered around 12.5 kbps)

If the timing is outside that window, the receiver rejects the stream.

### What happens when you assert NFD (NFD = LOW)

Frequency discrimination is disabled:

- The receiver does not enforce the “8–12 samples between bits” rule.

- It will accept any data stream that totals 32 valid bits, even if there are irregular gaps/jitter between bits.

But: it still enforces the word gap rule (it still needs a proper NULL gap between words before it will accept the next word).

### Why you would use it

- To receive ARINC-like data from a source that is not tightly within 100 kbps / 12.5 kbps tolerances (jittery generator, bit-banged source, unusual bit rate).

- For debugging when you suspect your transmitter’s bit timing is slightly off.

So, in plain terms:

NFD = “don’t reject my word just because the bit timing isn’t exactly ARINC-rate.”
It relaxes the bit-rate check, not the word-gap requirement.

The HI-8582 Transmitter

## 1) What the transmitter block is doing (mental model)

Inside the chip, the transmitter is basically:

FIFO (up to 32 words) → 32-bit shift register → sequencer → line driver

and alongside it:

- a parity generator (optional)

- a DATA/NULL timer (to create ARINC “return-to-zero” inside each bit)

- a word-gap timer (to force the ARINC inter-word NULL gap)

That exactly matches the timing diagram on p.8: every ARINC bit is DATA then NULL, then after bit 32 you get WORD GAP, then next word starts again at bit 1.

## 2) The physical output waveform: what comes out on TXAOUT/TXBOUT

The line driver creates the ARINC-style differential signaling:

- ONE: about +10 V differential

- ZERO: about −10 V differential

- NULL: about 0 V differential

(Your V+ and V− supplies feed this. The datasheet also notes series output resistance: HI-8582 has 37.5 Ω, HI-8583 has 10 Ω.)

### Key point: each bit is Return-to-Zero (RZ) in hardware

For every bit cell, the chip outputs:

- DATA portion (ONE or ZERO) for a fixed time

- then NULL portion for a fixed time

So you never have “stuck high” or “stuck low” within a bit—hardware forces it back to NULL every bit.

## 3) Timing generation: high-speed vs low-speed (CR13)

The transmitter timing is derived from the chip clocking, and the datasheet gives the bit-cell breakdown (p.7):

### High speed (CR13 = 0)

- ARINC bit time = 10 clocks

- DATA time = 5 clocks

- NULL time = 5 clocks

- WORD GAP = 40 clocks

### Low speed (CR13 = 1)

- ARINC bit time = 80 clocks

- DATA time = 40 clocks

- NULL time = 40 clocks

- WORD GAP = 320 clocks

If you use the common CLK = 1 MHz (as recommended for accurate timing), that becomes:

| Mode | Bit time | DATA | NULL | Word gap |
| --- | --- | --- | --- | --- |
| High speed | 10 µs | 5 µs | 5 µs | 40 µs |
| Low speed | 80 µs | 40 µs | 40 µs | 320 µs |

### Rise/fall slope is also tied to CR13

Datasheet p.7 states:

- CR13=0 → ~1.5 µs slope (high speed)

- CR13=1 → ~10 µs slope (low speed)

So CR13 is your single “big lever” for rate + edge shaping.

## 4) How you load a word to transmit (PL1 / PL2 + CR15 mapping)

### 4.1 The chip expects a 32-bit ARINC word loaded as two 16-bit bus writes

From the timing diagram (p.8):

- Put BYTE 1 on BD[15:0] → pulse PL1

- Put BYTE 2 on BD[15:0] → pulse PL2
→ the word is now complete in the TX FIFO and can be transmitted.

(PL2 is conceptually the “second half” latch.)

### 4.2 CR15 decides which ARINC bit goes to which BD pin

Datasheet p.4 gives the exact mapping.

#### If you choose CR15 = 1 (Unscramble) (recommended for sanity)

This is the clean mapping:

- BYTE 1 = ARINC bits 1..16

  - BD00 = bit 1, BD01 = bit 2, … BD15 = bit 16

- BYTE 2 = ARINC bits 17..32

  - BD00 = bit 17, … BD14 = bit 31, BD15 = bit 32 (parity/data)

✅ This makes software packing straightforward.

#### If you choose CR15 = 0 (Scramble)

BYTE 1 is “mixed” (label + SDI + parity/SSM bits appear in convenient places), and BYTE 2 holds the mid data bits. You typically use this only for legacy compatibility.

## 5) When does transmission actually start? (ENTX + FIFO → shift register)

Datasheet (p.7) describes it clearly:

- When ENTX goes high, transmission is enabled.

- The transmitter pulls the next word from the FIFO into its shift register.

- Within ~2.5 “data clocks” the first transmitted bit appears on TXAOUT/TXBOUT.

- Bits are then output sequentially until the word is complete.

So ENTX is effectively your “gate”:

- ENTX low → outputs are not actively sending a word

- ENTX high → FIFO feeds the shift register and the sequencer starts outputting ARINC bits

## 6) Bit-by-bit behavior with a clear example (matching the datasheet waveform)

Let’s mimic the datasheet timing diagram (p.8) which shows bits 30–32, then word gap, then bit 1 of next word.

### Example A: High-speed, last 3 bits = 1, 0, 1

Assume CLK=1 MHz and CR13=0.

Each bit cell is 10 µs:

- First 5 µs: DATA (ONE or ZERO)

- Next 5 µs: NULL

Now for the three bits:

#### Bit 30 = 1

- TXAOUT goes “positive side”, TXBOUT goes “negative side” for 5 µs

- then both go to NULL for 5 µs

#### Bit 31 = 0

- polarity flips for 5 µs

- then NULL for 5 µs

#### Bit 32 = 1 (parity or data depending on CR4)

- again polarity for 5 µs

- then NULL for 5 µs

#### Word gap

- driver holds NULL for 40 µs

#### Next word starts at bit 1

- bit 1 DATA/NULL resumes

That is exactly the “DATA–NULL, DATA–NULL, … WORD GAP … NEXT WORD” picture on datasheet p.8.

## 7) Parity behavior (CR4 and CR12) with examples

### Controls (from control register table)

- CR4 = 1 → bit 32 is parity (generated by chip)

- CR4 = 0 → bit 32 is data (you provide it)

- CR12 = 0 → generate odd parity

- CR12 = 1 → generate even parity

Datasheet p.7: parity generator “counts the Ones in the 31-bit word” and chooses bit 32 accordingly.

### Example B: Odd parity generation (typical ARINC)

Assume CR4=1 and CR12=0 (odd).

- Suppose bits 1..31 contain 17 ones (already odd).

  - To keep total ones odd, parity bit (bit 32) must be 0.

- Suppose bits 1..31 contain 18 ones (even).

  - To make total ones odd, parity bit must be 1.

So you don’t manually compute parity if CR4=1—chip does it.

### Example C: Using bit 32 as data (non-standard payload)

If you set CR4=0, then bit 32 is just another data bit.

- Parity generator is bypassed.

- Useful if you’re treating ARINC framing as a generic UART-like framed protocol for a lab exercise (but it won’t be “ARINC-compliant parity” anymore).

## 8) FIFO + status pins: what TX/R, HFT, FFT mean in practice

From datasheet p.7 and pin/status descriptions:

- TX FIFO depth: 32 words

- If FIFO is full, additional loads are ignored

- TX/R goes high when transmission completes and FIFO is empty (p.7)

- HFT / FFT give FIFO level indications:

  - HFT ~ “half full” threshold

  - FFT ~ “full”

### Example D: Two queued words (continuous transmission)

- You load word #1 (PL1 then PL2)

- You load word #2 (PL1 then PL2)

- ENTX is high

What happens:

- shift register takes word #1 and transmits it

- word gap is automatically inserted

- then word #2 transmits immediately after the gap

- TX/R only returns high after word #2 finishes and FIFO becomes empty

So the transmitter produces a clean ARINC stream without software timing.

## 9) Repeater mode (important “gotcha” example)

Datasheet p.7 explains repeater operation:

- A received word can be “read out” and simultaneously loaded into the transmitter FIFO.

- But during repeater mode, the lower byte must be read first, because the transmitter FIFO is always loaded lower byte first.

### Example E: Why “lower byte first” matters

If you accidentally read “upper byte first” in repeater mode:

- the TX FIFO would get the halves swapped

- transmitted ARINC bits would be wrong (bit positions scrambled)

- label/SDI/data fields won’t line up

So repeater mode is efficient, but it imposes that ordering rule.

# Practical “programming handles” summary (transmitter side)

If you want a checklist of the knobs you actually use:

- CR13: speed + slope (HS/LS and edge rate)

- CR4 / CR12: parity generation on/off, odd/even

- CR15: bus mapping (scramble/unscramble)

- ENTX pin: enable/disable transmission sequencing

- PL1 / PL2: push 16-bit halves into TX FIFO

- TX/R, HFT, FFT (+ status register SR bits): monitor FIFO empty/full, ready/busy

- MR / TEST / CR5 (if you use self-test modes): force null / loopback behaviors (optional but useful in lab bring-up)

ARINC 429 label handling and filtering implemented in the HI-8582/HI-8583

.

## 1) What the chip treats as a “Label” (and why it matters)

In ARINC 429, every 32-bit word has:

- Bits 1–8 = Label

- Bits 9–10 = SDI

- Bits 11–29 = Data

- Bits 30–31 = SSM

- Bit 32 = Parity (odd, per the ARINC convention)

The label is the primary identifier for “what this word means.” Filtering on label is therefore the most common hardware acceptance filter.

Important transmission nuance (ARINC-level): label bits are transmitted with the label MSB first on the wire (bit 8 first), which is why many software stacks talk about “bit reversal.”

HI-8582 key point: the chip reconstructs the received ARINC word into its internal bit numbering (bits 1..32) and then performs label compare using ARINC bits 1..8. So label filtering is based on the logical label field, not “wire order.”

## 2) The label filter resources inside HI-8582

Each receiver has its own label filter block:

- Two independent receivers (Receiver 1 and Receiver 2)

- Each receiver has an on-chip 16 × 8 “Label Memory” (so: up to 16 labels per receiver)

- There is a Label/Decode/Compare block in the receiver pipeline (see receiver block diagram) that decides whether a received word is loaded into the receiver FIFO or ignored.

## 3) Where filtering happens in the receive pipeline

After the receiver has detected a valid 32-bit ARINC word (timing + 32 bits), it generates EOS (End of Sequence). Only then does it do the filtering checks:

- Parity check happens

  - The receiver uses the “32nd bit position” in the FIFO word as a Parity Flag, not as the received parity bit.

  - If odd parity is correct: it stores 0 in bit 32; if parity error: it stores 1 in bit 32.

  - Either way, the word can still be written to FIFO if it passes the label/SDI filtering.

- Optional SDI (bits 9–10) decode filter (per receiver)

- Optional Label recognition filter (per receiver)

- If the enabled checks pass → load into 32×32 receiver FIFO; else → ignore word (do not load FIFO)

## 4) The exact “filter knobs” you can program (Control Register bits)

From the control register definitions:

### A) Enable/disable label recognition

- CR2 = Enable Label Recognition (Receiver 1)

- CR3 = Enable Label Recognition (Receiver 2)

If enabled, incoming label must match one of the 16 stored labels, otherwise the word is ignored (not put into FIFO).

### B) Enable/disable SDI (bits 9–10) “decoder” filter

Receiver 1:

- CR6 enables the decoder for Receiver 1

- CR7 sets required value for ARINC bit 9

- CR8 sets required value for ARINC bit 10

Receiver 2:

- CR9 enables the decoder for Receiver 2

- CR10 sets required value for ARINC bit 9

- CR11 sets required value for ARINC bit 10

If decoder is enabled, then ARINC bits 9 and 10 must match those programmed bits, or the word is ignored.

### C) Important: CR15 does not affect label compare meaning

The datasheet explicitly states that label bit significance is not changed by CR15, and that label bits BD00–BD07 are always compared to received ARINC bits 1–8 respectively.

So:

- You do not do any “reversal logic” for label filtering based on CR15.

- CR15 mainly affects how bits are mapped onto the 16-bit parallel bus during read/write of full words.

## 5) The chip’s filtering truth table (what gets loaded vs ignored)

The datasheet provides the combined behaviour in a table. Conceptually it is:

- If label recognition disabled, label match is “don’t care”

- If decoder disabled, SDI match is “don’t care”

- Only when an enabled feature says “must match” and it doesn’t → the word is ignored

In plain English:

### Case 1: No filters enabled

- Label recognition OFF, decoder OFF → every valid 32-bit word loads into FIFO

### Case 2: Label filter only

- Label recognition ON, decoder OFF → load only if label is in the 16-label list

### Case 3: SDI filter only

- Label recognition OFF, decoder ON → load only if (bit9,bit10) match your programmed SDI

### Case 4: Label + SDI filter together

- Label recognition ON, decoder ON → load only if BOTH match

## 6) Programming the 16-label list (load + readback) — step-by-step

### 6.1 Loading labels into label memory (CR1 “label memory access mode”)

To load the 16 labels:

- Write Control Register with CR1 = 1 (this puts the chip into label-memory load mode).

- Perform 16 label writes (Label #1 … Label #16):

  - Put the 8-bit label value on BD00–BD07

  - Pulse:

    - PL1 to load into Receiver 1 label memory

    - PL2 to load into Receiver 2 label memory

- Write Control Register with CR1 = 0 to return to normal operation.

Operational constraints (important):

- ARINC word reception is suspended during the label-memory write sequence (so do this during init, or at a safe time).

- Label recognition must be disabled (CR2/CR3 = 0) during label read sequence (and it’s a good practice during writes too).

- Access all 16 label locations before leaving the mode; then set CR1 back to 0.

- Label 00h is treated like any other label (not a wildcard).

### 6.2 Reading labels back (verification)

To read the 16 stored labels back:

- Write Control Register with CR1 = 1

- Do 16 reads of the selected receiver using:

  - EN1 reads Receiver 1 labels

  - EN2 reads Receiver 2 labels
Label data appears on BD00–BD07.

- Write Control Register with CR1 = 0 when done.

## 7) “Do I need label bit reversal?” — practical answer for HI-8582

### For receive-side filtering (label recognition):

No. You program labels exactly as the logical label byte (ARINC bits 1–8).

Reason: the chip compares BD00..BD07 ↔ ARINC bits 1..8 regardless of CR15.

### Where “reversal confusion” usually comes from

ARINC label values are often written in octal and described in terms of transmission order (bit 8 first). Some systems therefore tell you “reverse bits.”

With HI-8582, the receiver already reconstructs the word into the proper bit positions and then compares bits 1–8, so for label filtering you just load the correct 8-bit label value.

Tip to stay consistent: keep labels in your software as an 8-bit value representing bits 1..8, and convert from octal once at the boundary (documentation → code).

## 8) Worked examples

### Example A — Accept only a small set of labels (label filter only)

Goal: Receiver 1 should accept only labels {0x83, 0x1A, 0x55}.

- Set CR2=0 (disable recognition while loading)

- Set CR1=1

- Write 16 label slots via PL1, putting the 3 labels you care about in the first 3 slots; fill unused slots with values you don’t care about (but still must write them).

- Set CR1=0

- Set CR2=1 (enable label recognition)

Result: only words with those labels get loaded into Receiver 1 FIFO.

### Example B — Accept one label but only from SDI = 01 (label + SDI decode)

Goal: accept label 0x83 only when ARINC bits 9–10 = 01.

Receiver 1:

- Load label list with 0x83 present

- Enable SDI decoder:

  - CR6=1

  - CR7=1 (bit 9 must be 1)

  - CR8=0 (bit 10 must be 0)

- Enable label recognition:

  - CR2=1

Result: the receiver loads FIFO only if (label matches) AND (bits9–10 match).

### Example C — Accept everything but tag parity errors in software

Goal: don’t filter by label/SDI, but detect parity issues.

- Disable filters:

  - CR2=0 and CR6=0 (Receiver 1)

  - CR3=0 and CR9=0 (Receiver 2)

- Read words from FIFO normally.

- Check the stored “bit 32 parity flag”:

  - 0 = good odd parity

  - 1 = parity error

Result: you get all traffic, and you can decide in software what to do with parity-error words.

## 9) What you observe externally when a label doesn’t match

The datasheet states: if label recognition is enabled and no match is found, then “no indicators of receiving ARINC data are presented.”

In practice that means:

- The word is not written into FIFO

- So D/R (data-ready) won’t assert for that word

- Status register “data ready” bit won’t set for it

- From your MCU point of view, it’s as if the word never arrived

This is exactly what you want for hardware filtering: it reduces interrupts/polling load and FIFO churn.