# MCUXpresso IDE Debug Features for Bare‑Metal on EVKB‑i.MXRT1050

## 1) Fundamentals: What “debug” means on a Cortex‑M7, and what MCUXpresso provides

### 1.1 Definitions

- Bare‑metal: Software running directly on the microcontroller with no operating system. You control reset, clocks, pin mux, interrupts, memory, and application logic.

- CoreSight: ARM® debug/trace architecture present in Cortex‑M7. It exposes hardware resources (breakpoints, watchpoints, trace) over JTAG/SWD.

- EVKB‑i.MXRT1050: NXP evaluation kit for the i.MX RT1050 crossover MCU (Cortex‑M7 @ up to 600 MHz).

- CMSIS‑DAP (Cortex Microcontroller Software Interface Standard – Debug Access Port): The on‑board debug probe interface on EVKB used by MCUXpresso’s LinkServer.

- ITM/SWO (Instrumentation Trace Macrocell / Single‑Wire Output): Low‑intrusion, hardware‑assisted message/printf and event streaming over a single pin.

- ETM (Embedded Trace Macrocell): Instruction trace (not always routed/available on all boards without external hardware).

- DWT (Data Watchpoint and Trace): Hardware comparators/counters for watchpoints and cycle counting.

- FPB (Flash Patch and Breakpoint): Hardware instruction breakpoints in flash.

- LPUART (Low‑Power Universal Asynchronous Receiver/Transmitter): i.MX RT1050 UART peripheral used in our ARINC 429 lab.

- DO‑178C (Software Considerations in Airborne Systems and Equipment Certification): Primary avionics software standard; requires determinism, evidence, and traceability.

- ARINC 429 (Aeronautical Radio, Incorporated 429): Unidirectional 2‑wire data bus used in avionics. We will ingest it via a UART bridge (ADK‑8582) for labs.

### 1.2 CoreSight resources we will use

- PC breakpoints: Halt when Program Counter reaches an address/line.

- Watchpoints (DWT): Halt on read/write of a specific memory location; ideal to catch unexpected variable changes without instrumentation.

- Single‑step controls: Step‑into/over/out; run‑to‑line.

- Core & Peripheral register access: Inspect CPU (xPSR, PRIMASK, BASEPRI) and SoC peripheral registers live.

- ITM/SWO prints: Real‑time logging with minimal timing impact.

- DWT cycle counter: Cycle‑accurate timing for latency/jitter bounds.

Why it matters for avionics:

- Use hardware‑assisted observability (watchpoints, SWO, cycle counter) to gather objective evidence with minimal perturbation—preferable to intrusive printf over UART.

## 2) MCUXpresso IDE: Your debugging workbench

Key views/panels:

- Quickstart → Debug: Launch session via LinkServer (CMSIS‑DAP).

- Breakpoints: Add/enable/disable line breakpoints; create Watchpoints (DWT data breakpoints).

- Expressions: Watch C variables and raw addresses; enable continuous refresh for live variables.

- Peripherals+ (NXP): Decoded peripheral registers (GPIO, IOMUXC, LPUART, etc.).

- Memory: Hex/ASCII view, selectable widths.

- Console(s): MCUXpresso console for semihosting;

- SWO Trace Console for ITM/SWO.

- Disassembly: Interleaved C + instructions; useful for instruction‑level tracing.

- Registers: CPU core regs, fault status (CFSR, HFSR), fault addresses (BFAR/MMFAR).

Reset/connect modes: -

- Halt on reset: Recommended for low‑level bring‑up.

- Attach to running: Non‑intrusive attach to an already running target.

- System vs Core reset: System reset reinitializes peripherals;

- Core reset is faster but preserves peripheral state.

Semihosting vs SWO: -

- Semihosting uses BKPT traps → intrusive, can stall; OK for early bring‑up only. –

- SWO provides low‑intrusion, high‑bandwidth telemetry; preferred for timing‑sensitive work.

## 3) Hands‑On Labs (SDK‑based, compile‑ready)

Each lab includes a generic example and an avionics use case. Paths are relative to the NXP SDK for EVKB‑i.MXRT1050.

### Lab 1 — Breakpoints, stepping, Peripherals+ (LED Blinky)

Project:

- boards/evkbimxrt1050/demo_apps/led_blinky/

Steps:

- Import into MCUXpresso (File → Import → Existing Projects into Workspace).

- Build and Debug with LinkServer (CMSIS‑DAP).

- Set a line breakpoint inside led_blinky.c where GPIO_PinWrite() toggles the LED.

- Step‑over and observe logic.

- Open Peripherals+ → GPIO for the LED port; watch the DR (data) register bit change as you step.

- Add the LED variable or register address to Expressions and enable live refresh.

- Compare System Reset vs Core Reset and note peripheral state differences.

Generic example:

- Verify a basic state machine that toggles the LED only when a condition is true.

Avionics use case:

- Treat the LED as a discrete output line driving a Weight‑On‑Wheels (WOW) signal.

- Place a breakpoint at the WOW decision gate and verify that debounced state transitions alone modify the discrete output.

- This mirrors verifying a safety‑critical discrete output path before enabling downstream functions.

Evidence to save:

- Peripherals+ screenshot of GPIO bit toggling and a note on which reset mode you used and why.

### Lab 2 — Low‑intrusion logging with ITM/SWO

Project:

- boards/evkbimxrt1050/demo_apps/hello_world_swo/

Demo behavior:

- Board init enables trace clocks; SWO console prints messages from PRINTF() configured for SWO (see hardware_init.c and app.h).

Steps:

- Import, build, and debug the project.

- Open SWO Trace Console; configure to the demo’s SWO baud (default 4 MHz in app.h).

- Trigger button events and observe SWO messages; observe periodic SysTick prints.

- Set a breakpoint in the button’s GPIO interrupt handler;

- step and inspect Registers (PRIMASK/BASEPRI) to confirm interrupt masking state.

Generic example:

- Replace button events with a small state machine and log transitions via SWO.

Avionics use case:

- Use SWO as maintenance/logging telemetry in an Air Data Computer prototype—for example, log “pitot fault injected” events without disturbing UART bandwidth budgets needed elsewhere (e.g., an ARINC 429 bridge).

Why SWO:

- Lower intrusion than semihosting or UART prints; suitable for timing‑sensitive paths.

### Lab 3 — Watchpoints + UART RX with ADK‑8582 (ARINC 429 over UART)

Goal:

- Use a DWT watchpoint to halt exactly when a specific ARINC 429 label arrives from an external converter (ADK‑8582 → UART → EVKB).

- Step ISR and inspect a ring buffer.

Project:

- boards/evkbimxrt1050/driver_examples/lpuart/interrupt_transfer/

Physical setup:

- Configure ADK‑8582 to output ARINC 429 32‑bit words as ASCII hex lines: XXXXXXXX\r\n, 115200‑8‑N‑1 into EVKB LPUART. Wire GND↔GND, ADK TX → EVKB RX.

Parsing fields (SDK‑style C):

/* ARINC 429 field extraction (labels commonly documented in OCTAL).
   For this lab treat 'label' as the low 8 bits of the 32‑bit word. */
typedef struct {
    uint8_t  label;     // bits 1..8 (LSB)
    uint8_t  sdi;       // bits 9..10
    uint32_t data;      // bits 11..29
    uint8_t  ssm;       // bits 30..31
    bool     parity;    // bit 32 (MSB)
} arinc429_t;

static inline arinc429_t parse_arinc429(uint32_t w)
{
    arinc429_t f;
    f.label  = (uint8_t)(w & 0xFFu);
    f.sdi    = (uint8_t)((w >> 8) & 0x3u);
    f.data   = (uint32_t)((w >> 10) & 0x1FFFFFu);
    f.ssm    = (uint8_t)((w >> 29) & 0x3u);
    f.parity = ((w >> 31) & 0x1u) ? true : false;
    return f;
}

Probe variables for watchpoints:

volatile arinc429_t g_lastRx;
volatile uint8_t    g_lastLabel; // convenience copy for watchpoints

SDK‑pattern LPUART init (based on fsl_lpuart.h example):

lpuart_config_t config;
LPUART_GetDefaultConfig(&config);
config.baudRate_Bps = 115200U;
config.enableRx = true;
config.enableTx = true;
LPUART_Init(DEMO_LPUART, &config, CLOCK_GetFreq(kCLOCK_UartClk));

static lpuart_handle_t g_lpuartHandle;
LPUART_TransferCreateHandle(DEMO_LPUART, &g_lpuartHandle, LPUART_UserCallback, NULL);
// Start first receive (non‑blocking)
static uint8_t ch;
lpuart_transfer_t rx = { .data = &ch, .dataSize = 1 };
LPUART_TransferReceiveNonBlocking(DEMO_LPUART, &g_lpuartHandle, &rx, NULL);

In the RX path, assemble a line → parse → update probe vars:

#define RX_LINE_MAX  16
static uint8_t rxLine[RX_LINE_MAX];
static size_t  rxLen = 0;

void LPUART_UserCallback(LPUART_Type *base, lpuart_handle_t *handle,
                         status_t status, void *userData)
{
    if (kStatus_LPUART_RxIdle == status) {
        // When IDLE, check if we have a full line like "XXXXXXXX\n"
        if (rxLen >= 8) {
            rxLine[8] = '\0';
            uint32_t raw = (uint32_t)strtoul((const char*)rxLine, NULL, 16);
            g_lastRx = parse_arinc429(raw);
            g_lastLabel = g_lastRx.label;
        }
        rxLen = 0; // prepare for next word
    }
}

Create a DWT Watchpoint in MCUXpresso:

- Debug the app.

- Breakpoints view → New → Watchpoint.

- Address: symbol g_lastLabel. Size: 1 byte. Condition: Break on write.

- Resume. The core halts exactly when a new label is parsed.

Avionics use case:

- Track baro‑corrected altitude using label 203 (octal) = decimal 131 = 0x83.

- Stop when g_lastLabel == 0x83 (or add a conditional BKPT).

- This proves your Input Data Concentrator correctly detects the required label without UART printf jitter. Use Peripherals+ → LPUART to confirm FIFO/status bits.

Evidence to save:

- Screenshot of watchpoint hit (g_lastLabel == 0x83) plus LPUART status view.

### Lab 4 — Timing analysis with DWT Cycle Counter + SWO

Goal:

- Measure ISR latency and code section execution time at cycle precision.

Helpers:

#include "fsl_device_registers.h"  // brings in DWT/DEMCR definitions

static inline void DWT_InitCycleCounter(void)
{
    CoreDebug->DEMCR |= CoreDebug_DEMCR_TRCENA_Msk; // enable DWT/ITM
    DWT->CYCCNT = 0;
    DWT->CTRL  |= DWT_CTRL_CYCCNTENA_Msk;
}
static inline uint32_t DWT_GetCycles(void) { return DWT->CYCCNT; }

Use in code:

DWT_InitCycleCounter();
uint32_t t0 = DWT_GetCycles();
// section under test
uint32_t dt = DWT_GetCycles() - t0;
PRINTF("section cycles=%u us=%u\r\n", dt,
        (unsigned)((uint64_t)dt * 1000000ULL / SystemCoreClock));

Avionics use case:

- Bound latency from UART word arrival (RX complete) to integrity check completion (parity/SSM verification).

- Record min/mean/max over ≥10,000 events via SWO.

- Convert cycles to microseconds using SystemCoreClock and attach results as WCET evidence for a DAL‑B/C prototype.

## 4) Deep‑Dive Techniques

### 4.1 Breakpoints (software vs hardware)

- Software breakpoints patch BKPT into flash (managed by IDE); hardware breakpoints use FPB comparators. If you run out, remove some or use Run to Line.

### 4.2 Watchpoints (DWT)

- Watchpoints trigger on read/write/modify of aligned addresses (1/2/4 bytes). Mark variables volatile to prevent optimization from eliding them.

### 4.3 Fault analysis (SCB registers)

- Inspect CFSR, HFSR, BFAR, MMFAR when stopping in HardFault to identify the faulting access and instruction. Combine with Disassembly view.

### 4.4 SWO Trace Console tips

- Match SWO baud to project (app.h in sample uses 4 MHz). Ensure trace clock enabled in board init. Prefer SWO over UART inside ISRs.

### 4.5 Reset strategy

- Use System Reset + Halt when changing clock trees/pin mux for a clean start. Use Attach to running for endurance tests that must not be reset.

### 4.6 Linker Map & memory insight

- Open the .map file to verify code/data placement. Track BSS/stack headroom during stress tests.

## 5) Strong Avionics Scenarios

- IOMUXC mis‑mux / drive strength:

  - With Peripherals+ open, verify pad mux & drive for an ARINC 429 TX‑enable pin before enabling the line driver.

  - Watchpoint on the GPIO direction register guarantees output mode is set exactly once.

- UART framing errors in noise:

  - In Peripherals+ → LPUART, monitor FE/NE/PE flags while ADK‑8582 streams data.

  - Watchpoint a shadowed status variable to halt on FE and dump a short diagnostic.

- Timing regression after refactor:

  - Wrap the ARINC label parser with DWT cycle timing.

  - If cycles exceed a threshold, issue a SWO alert and log a timestamp.

- Induced HardFault boundary test:

  - Intentionally overrun a test buffer; on HardFault, capture PC/LR/CFSR/BFAR in a minimal fault handler.

  - This becomes part of your laboratory safety case.

## 6) Practical SDK‑aligned Snippets

### 6.1 SWO init/prints (mirrors hello_world_swo)

#include "fsl_debug_console.h"
#include "board.h"
#include "app.h"

int main(void)
{
    BOARD_InitHardware();
    BOARD_InitDebugConsoleSWO(DEMO_DEBUG_CONSOLE_SWO_PORT, DEMO_DEBUG_CONSOLE_SWO_BAUDRATE);
    while (1) {
        PRINTF("SWO alive\r\n");
        for (volatile uint32_t i=0; i<1000000; ++i) {}
    }
}

### 6.2 LPUART non‑blocking receive (SDK driver pattern)

#include "fsl_lpuart.h"
#include "board.h"
#include "app.h"

#define RX_LINE_MAX  16
static uint8_t rxLine[RX_LINE_MAX];
static size_t  rxLen = 0;

volatile arinc429_t g_lastRx;
volatile uint8_t    g_lastLabel;

static lpuart_handle_t g_lpuartHandle;

void LPUART_UserCallback(LPUART_Type *base, lpuart_handle_t *handle,
                         status_t status, void *userData)
{
    if (kStatus_LPUART_RxIdle == status) {
        if (rxLen >= 8) {
            rxLine[8] = '\0';
            uint32_t raw = (uint32_t)strtoul((const char*)rxLine, NULL, 16);
            g_lastRx = parse_arinc429(raw);
            g_lastLabel = g_lastRx.label;
        }
        rxLen = 0;
    }
}

static void uart_init_and_start(void)
{
    lpuart_config_t config;
    LPUART_GetDefaultConfig(&config);
    config.baudRate_Bps = 115200U;
    config.enableRx = true;
    config.enableTx = true;
    LPUART_Init(DEMO_LPUART, &config, CLOCK_GetFreq(kCLOCK_UartClk));

    LPUART_TransferCreateHandle(DEMO_LPUART, &g_lpuartHandle, LPUART_UserCallback, NULL);

    static uint8_t ch;
    lpuart_transfer_t rx = { .data = &ch, .dataSize = 1 };
    LPUART_TransferReceiveNonBlocking(DEMO_LPUART, &g_lpuartHandle, &rx, NULL);
}

### 6.3 DWT cycle counter helper

static inline void DWT_InitCycleCounter(void) {
    CoreDebug->DEMCR |= CoreDebug_DEMCR_TRCENA_Msk;
    DWT->CYCCNT = 0;
    DWT->CTRL  |= DWT_CTRL_CYCCNTENA_Msk;
}

## 7) Best Practices (Airbus‑grade)

- Prefer non‑intrusive observability. Use watchpoints and SWO prints; avoid semihosting and UART prints in critical paths.

- Freeze build settings per test. Optimization/inlining impact code placement and evidence reproducibility. Record -O level, LTO, and linker options.

- Make probe state explicit. Mark watched variables volatile; keep them in RAM and visible to the debugger.

- Clocks & pins as single source of truth. Step through clock_config.c and pin_mux.c once per board/BSP change while watching Peripherals+.

- Reset discipline. Use System Reset when changing clocks/mux; document reset method in your evidence set.

- Bound worst‑case, not average. Use DWT cycles for WCET claims; SWO timestamps are supportive but not primary.

- Faults as first‑class artifacts. Include a tiny HardFault report (PC, LR, CFSR, BFAR) for lab builds; disable for flight.

- ARINC labels: be explicit with bases. Document label numbers in octal and hex (e.g., 203(oct) = 0x83) to prevent confusion.

- Probe bandwidth awareness. Tune SWO baud (demo uses 4 MHz). If you outgrow SWO, move to external trace rather than more UART prints.

- Configuration control. Archive launch configs (.launch), map files, and SWO logs with test reports to support DO‑178C reviews.

## 8) Session Checklist

- EVKB power and CMSIS‑DAP (LinkServer) enumerated.

- Launch configuration: Halt on reset, correct SWO baud, semihosting disabled (unless used intentionally).

- Optimization level noted (-O0 for bring‑up; -O2 for timing runs) and unchanged during evidence capture.

- Variables for watchpoints marked volatile and in scope.

- ARINC lab: ADK‑8582 at 115200‑8‑N‑1, ASCII hex framing; GND common; TX→RX wired.

## 9) Artifacts

- Lab 1: Peripherals+ screenshot of GPIO bit toggling; note on reset mode and reasoning.

- Lab 2: SWO console capture of timed events; screenshot stepping inside GPIO ISR showing PRIMASK/BASEPRI state.

- Lab 3: Watchpoint hit on g_lastLabel == 0x83 with LPUART status; brief throughput/overflow note.

- Lab 4: Min/mean/max cycles for ISR path, SystemCoreClock value, computed microseconds, and test conditions.

### Appendix A — Troubleshooting SWO on EVKB‑i.MXRT1050

- Ensure board init enables TRACE clock (the SWO demo’s hardware_init.c does this).

- In SWO Trace Console, match the baud rate from app.h (default 4 MHz). If output is garbled, try lower rates (e.g., 2 MHz).

- Disable semihosting in the launch configuration if the target stalls unexpectedly.

### Appendix B — Minimal HardFault Report (for lab builds)

void HardFault_Handler(void)
{
    volatile uint32_t hfsr = SCB->HFSR;
    volatile uint32_t cfsr = SCB->CFSR;
    volatile uint32_t bfar = SCB->BFAR;
    (void)hfsr; (void)cfsr; (void)bfar; // inspect in debugger or print via SWO if safe
    __BKPT(0);
    while (1) { }
}