# Minimizing Interrupt Latency and Jitter (i.MX RT1050 EVKB, ARM Cortex‑M7)

## 1) Why latency and jitter matter (with precise definitions)

- Interrupt latency is the elapsed time from when a hardware event asserts an interrupt line until the first instruction of the corresponding Interrupt Service Routine (ISR) executes.

- Interrupt jitter is the variation of that latency from one occurrence to the next (i.e., the spread of the distribution, not its mean).

- In avionics, we need predictability as much as speed.

- Accurate sampling, time‑aligned data fusion, and bus timestamping must meet worst‑case budgets for certification under RTCA DO‑178C (Software Considerations in Airborne Systems and Equipment Certification).

- Jitter is often more damaging than average latency because it creates rare but dangerous outliers that break assumptions in control loops and data correlation.

## 2) Cortex‑M7 latency path (i.MX RT1050)

From event to first ISR instruction, the path is:

- Peripheral → NVIC (Nested Vectored Interrupt Controller): peripheral asserts an interrupt; request traverses on‑chip interconnect.

- Masking and priority checks: PRIMASK (global mask), BASEPRI (mask at/under threshold), and FAULTMASK (masks even faults) can delay service. If not masked and of higher priority than the current context, preemption proceeds.

- Automatic stacking: core saves R0–R3, R12, LR, PC, xPSR; optionally Floating‑Point Unit (FPU) state (depending on lazy stacking policy).

- Vector fetch & instruction fetch: handler address is read from the vector table (SCB->VTOR), then the first instruction fetch occurs.

Micro‑architectural helpers when priorities are set well: -

- Tail‑chaining: switches directly between back‑to‑back ISRs, cutting return/entry overhead.

- Late arrival: a higher‑priority interrupt can preempt during the entry of a lower‑priority ISR, protecting the latency of the more important event.

One cannot delete these steps, but one can remove variability from them.

## 3) Primary sources of latency jitter on i.MX RT1050

- Execute‑In‑Place (XIP) from external flash via FlexSPI and the AXI (Advanced eXtensible Interface) fabric: I‑cache misses and bus contention make vector fetch and ISR fetch time variable.

- Data cache and DMA (Direct Memory Access) interactions: without explicit clean/invalidate discipline, buffer sharing between CPU and EDMA (Enhanced DMA) introduces unpredictable maintenance points and stale reads/writes.

- Lazy FPU stacking: the first floating‑point instruction in an ISR triggers FP state save/restore, creating sporadic long entries.

- Masking strategy: long __disable_irq() (global mask) windows inflate jitter across the system; BASEPRI misuse can also block a lifeline ISR if priorities are mis‑assigned.

- Long or branchy ISRs: medium‑priority ISRs that run long block higher‑priority ISRs; heavy branching increases path‑length variance on Cortex‑M7.

- Clock and power management: late enable of clocks or bus gating adds wake‑up variability.

## 4) Design rules that consistently reduce latency and jitter

### 4.1 Use priorities and masking deliberately

- Assign the truly timing‑critical ISR a numerically small priority (e.g., 1; 0 is the absolute highest). Document a priority map.

- Avoid global masking (PRIMASK); guard short critical sections with BASEPRI so the lifeline ISR can still preempt instantly.

### 4.2 Make memory deterministic

- Place the vector table and timing‑critical ISRs in ITCM (Instruction Tightly Coupled Memory); place their hot data in DTCM (Data TCM). This removes XIP/cache/AXI variability.

- If executing from flash is unavoidable, keep ISRs tiny and straight‑line so they stay I‑cache hot (still not as deterministic as ITCM).

### 4.3 Keep ISRs tiny, constant‑time, and branch‑light

- Acknowledge the device, capture timestamp/counter, enqueue a compact record, and exit. Push all parsing/logging to a bottom half.

### 4.4 Floating‑point policy

- Prefer no float in ISRs. If float is required, disable lazy stacking to pay a constant entry cost every time. Do not allow rare first‑use penalties to become outliers.

### 4.5 Cache discipline with DMA

- Easiest deterministic choice: place DMA rings in non‑cacheable memory.

- If buffers must be cacheable, enforce a documented clean/invalidate + barrier protocol at handoff points (producer cleans before DMA‑out; consumer invalidates after DMA‑in).

### 4.6 Strictly periodic scheduling

- Program next_deadline = previous_deadline + period, not now + period, to prevent drift and convert occasional delays into non‑accumulating phase errors.

### 4.7 Stabilize clocks

- Keep the GPT (General Purpose Timer) / PIT (Periodic Interrupt Timer) and relevant interconnect clocks enabled at known divisors during critical operation.

## 5) Measuring what matters (before optimizing)

Use two timebases: -

- DWT (Data Watchpoint and Trace) CYCCNT for core‑cycle timing. –

- GPT/PIT as a peripheral domain clock to timestamp ISR entry relative to scheduled compare events.

- Also toggle a GPIO (e.g., EVKB user LED on GPIO1_IO09) at ISR entry/exit and view on a logic analyzer.

- Collect min/mean/max and p99.9 under idle and worst‑case contention (UART floods, DMA bursts).

- If you cannot measure the worst case, you do not own the worst case.

## 6) Generic worked example: a rock‑steady 10 kHz timestamp ISR

Goal: ≤ 2 µs worst‑case entry, ≤ 0.5 µs jitter.

Concept:

- GPT1 free‑runs;

- schedule strictly periodic compares.

- The ISR (in ITCM, high priority) sets a probe GPIO high, reads GPT, computes latency = now − expected, schedules expected + period, clears the flag, sets GPIO low, exits.

- No float, no logging, no branches beyond the essential.

Why it works:

- vector and ISR fetch are single‑cycle (ITCM), data hits DTCM, preemption is guaranteed by priority and BASEPRI discipline, and the code path is constant‑time.

SDK‑style snippet (compiles on EVKB‑IMXRT1050)

#include "fsl_gpt.h"
#include "fsl_gpio.h"
#include "fsl_clock.h"
#include "fsl_common.h"
#include "board.h"

static inline void DWT_EnableCycleCounter(void) {
    CoreDebug->DEMCR |= CoreDebug_DEMCR_TRCENA_Msk;
    DWT->CYCCNT = 0;
    DWT->CTRL  |= DWT_CTRL_CYCCNTENA_Msk;
}

volatile uint32_t g_last_target;
volatile uint32_t g_latency_ticks;
volatile uint32_t g_isr_count;

#define GPTx            GPT1
#define GPTx_IRQn       GPT1_IRQn
#define PERIOD_US       (100U)
static uint32_t gpt_ticks_per_us;

#ifndef BOARD_USER_LED_GPIO
#define BOARD_USER_LED_GPIO     GPIO1
#define BOARD_USER_LED_GPIO_PIN 9U
#endif

__attribute__((section(".itcm_func")))
void GPT1_IRQHandler(void)
{
    GPIO_PinWrite(BOARD_USER_LED_GPIO, BOARD_USER_LED_GPIO_PIN, 1);
    uint32_t now    = GPT_GetCurrentTimerCount(GPTx);
    uint32_t expect = g_last_target;
    g_latency_ticks = now - expect; /* natural wrap */
    g_isr_count++;

    g_last_target = expect + (PERIOD_US * gpt_ticks_per_us);
    GPT_SetOutputCompareValue(GPTx, kGPT_OutputCompare_Channel1, g_last_target);

    GPT_ClearStatusFlags(GPTx, kGPT_OutputCompare1Flag);
    GPIO_PinWrite(BOARD_USER_LED_GPIO, BOARD_USER_LED_GPIO_PIN, 0);
    __DSB(); __ISB();
}

static void LatencyDemo_Init(void)
{
    BOARD_BootClockRUN();
    gpio_pin_config_t out = { kGPIO_DigitalOutput, 0, kGPIO_NoIntmode };
    GPIO_PinInit(BOARD_USER_LED_GPIO, BOARD_USER_LED_GPIO_PIN, &out);

    gpt_config_t cfg; GPT_GetDefaultConfig(&cfg); cfg.enableFreeRun = true;
    GPT_Init(GPTx, &cfg);
    GPT_SetClockDivider(GPTx, 1);
    uint32_t gpt_freq = CLOCK_GetFreq(kCLOCK_PerClk);
    gpt_ticks_per_us  = gpt_freq / 1000000U;

    GPT_StartTimer(GPTx);
    uint32_t now = GPT_GetCurrentTimerCount(GPTx);
    g_last_target = now + 10 * gpt_ticks_per_us;
    GPT_SetOutputCompareValue(GPTx, kGPT_OutputCompare_Channel1, g_last_target);
    GPT_EnableInterrupts(GPTx, kGPT_OutputCompare1InterruptEnable);

    NVIC_SetPriority(GPTx_IRQn, 1U);
    NVIC_EnableIRQ(GPTx_IRQn);

    DWT_EnableCycleCounter();
}

Linker/startup note:

- Add a section .itcm_func mapped to ITCM;

- Relocate the vector table (SCB->VTOR → ITCM) with __DSB(); __ISB();

- After the write.

Place hot ISR data in DTCM similarly.

## 7) Avionics worked example: ADK‑8582 (ARINC 429) → UART with tight timestamps

- ARINC 429 (Aeronautical Radio, Incorporated 429) transmits 32‑bit words at 100 kbps (10 µs/bit) or 12.5 kbps (80 µs/bit).

- The ADK‑8582 board decodes ARINC 429 and streams decoded words over LPUART (Low‑Power Universal Asynchronous Receiver/Transmitter).

- We must avoid RX overruns during bursts and timestamp labels within, say, ≤ 5 µs at 100 kbps.

Deterministic architecture: -

- Timestamp is sovereign: GPT ISR in ITCM at priority 1; it only captures time and marks boundaries.

- Bulk RX via EDMA: LPUART uses EDMA into non‑cacheable rings; the EDMA completion ISR (priority 3–5) advances pointers and stores a concise “buffer‑ready @ time T” record.

- Parsing in thread context: label decoding and parity checks happen outside ISRs; short BASEPRI regions protect pointer swaps without globally masking.

SDK‑style skeleton

#include "fsl_lpuart.h"
#include "fsl_dmamux.h"
#include "fsl_edma.h"
#include "fsl_cache.h"
#include "board.h"

AT_NONCACHEABLE_SECTION_ALIGN(static uint8_t s_rxRing[2048], 16);
static edma_handle_t           s_edmaHandle;
static lpuart_edma_handle_t    s_lpuartEdmaHandle;

static void UART_EDMA_UserCallback(LPUART_Type *base,
                                   lpuart_edma_handle_t *handle,
                                   status_t status,
                                   void *userData)
{
    (void)base; (void)handle; (void)userData;
    if (status == kStatus_LPUART_RxIdle || status == kStatus_LPUART_IdleLineDetected)
    {
        uint32_t tstamp = GPT_GetCurrentTimerCount(GPT1); /* minimal timestamp */
        /* record {buffer_index, tstamp}; do not parse here */
    }
}

static void UART_Rx_EDMA_Init(void)
{
    CLOCK_EnableClock(kCLOCK_Dma0);
    CLOCK_EnableClock(kCLOCK_Dmamux0);
    DMAMUX_Init(DMAMUX0);
    EDMA_Init(DMA0);

    lpuart_config_t cfg; LPUART_GetDefaultConfig(&cfg);
    cfg.baudRate_Bps = 2000000U; cfg.enableTx = false; cfg.enableRx = true;
    LPUART_Init(LPUART1, &cfg, CLOCK_GetFreq(kCLOCK_OscClk));

    EDMA_CreateHandle(&s_edmaHandle, DMA0, 0);
    LPUART_TransferCreateHandleEDMA(LPUART1, &s_lpuartEdmaHandle,
                                    UART_EDMA_UserCallback, NULL,
                                    &s_edmaHandle, NULL);

    lpuart_transfer_t xfer = { .data = s_rxRing, .dataSize = sizeof(s_rxRing) };
    LPUART_TransferReceiveEDMA(LPUART1, &s_lpuartEdmaHandle, &xfer);

    NVIC_SetPriority(DMA0_0_IRQn, 3U); /* below GPT, above UART */
    NVIC_SetPriority(LPUART1_IRQn, 5U);
}

Why this minimizes jitter: the timestamp ISR always preempts because the system uses BASEPRI, not global masking; its code/data run from TCM; UART traffic does not generate per‑byte interrupts; DMA buffers are non‑cacheable, avoiding unpredictable cache maintenance in ISRs.

## 8) Hands‑on labs (with MCUXpresso SDK paths)

A. Baseline latency & jitter with GPT
Start from boards/evkbimxrt1050/driver_examples/gpt/timer/. Add the 10 kHz ISR above, place it and vectors in ITCM, and toggle GPIO1_IO09 on entry/exit. Measure min/mean/max and p99.9 under idle.

B. Stress with UART floods (interrupt mode)
Start from boards/evkbimxrt1050/driver_examples/lpuart/interrupt/. Flood RX at 1–2 Mbps. Compare jitter when background code uses BASEPRI vs. global masking (__disable_irq()): the latter inflates jitter.

C. Switch UART RX to EDMA
Start from boards/evkbimxrt1050/cmsis_driver_examples/lpuart/edma_transfer/. Use AT_NONCACHEABLE_SECTION_ALIGN for RX rings; keep the EDMA callback minimal and timestamp it with GPT. Jitter improves due to lower interrupt pressure and short ISRs.

D. ARINC 429 end‑to‑end with ADK‑8582
Connect ADK‑8582 to LPUART. Keep GPT ISR at priority 1 (ITCM), EDMA at 3–5, UART at 5–7. Demonstrate zero RX overrun during a scripted 100 kbps burst and timestamp error ≤ 5 µs.

## 9) Best practices (with reasons)

- Tiny, constant‑time ISRs: acknowledge, timestamp, queue, exit. Parsing/logging in bottom halves only.

- Prefer BASEPRI to PRIMASK: allow lifeline ISR preemption; jitter collapses when you stop pausing the world.

- ITCM/DTCM for critical paths: removes XIP/cache/AXI variability.

- Non‑cacheable DMA rings: the simplest deterministic coherency model.

- No printf or allocation in ISRs: unbounded latency and cache churn.

- Written priority map: avoid accidental inversions; treat priorities as architecture, not a tweak.

- Program periodic timers from previous deadline: prevents phase drift and jitter accumulation.

- Instrument and keep distributions: worst‑case and p99.9 are design‑closing numbers; means are not.

## 10) Acceptance criteria and verification

- Latency: GPT compare ISR worst‑case entry ≤ 2 µs (example budget) at 600 MHz core; measured over ≥ 10^6 events with UART/DMA load present.

- Jitter: p99.9 of entry latency ≤ 0.5 µs.

- No data loss: 0 LPUART RX overruns during N minutes with worst‑case ARINC 429 bursts.

- Regression guard: testbench fails build if worst‑case widens by >10% from baseline.

## 11) Troubleshooting guide

- Rare long outlier on first ISR run → FPU lazy stacking. Fix: ban floats in ISRs or disable lazy stacking to pay a constant cost.

- Heavy tail only with XIP → I‑cache/AXI variance. Fix: move vectors/ISR to ITCM; keep hot data in DTCM.

- Timing drift over minutes → scheduled from now + period. Fix: schedule previous + period.

- UART overruns under load → per‑byte RX interrupts or cacheable DMA. Fix: EDMA + non‑cacheable rings.

- Stale or duplicated DMA data → missing cache maintenance. Fix: non‑cacheable buffers or explicit clean/invalidate + barriers.

## 12) Reusable snippets (SDK‑compatible)

Enable DWT CYCCNT

static inline void DWT_Init(void) {
    CoreDebug->DEMCR |= CoreDebug_DEMCR_TRCENA_Msk;
    (void)DWT->CYCCNT; /* probe presence */
    DWT->CYCCNT = 0;
    DWT->CTRL  |= DWT_CTRL_CYCCNTENA_Msk;
}

BASEPRI helpers

static inline uint32_t irq_lock_basepri(uint32_t prio) {
    uint32_t old = __get_BASEPRI();
    __set_BASEPRI(prio << (8 - __NVIC_PRIO_BITS));
    __DSB(); __ISB();
    return old;
}
static inline void irq_unlock_basepri(uint32_t old) {
    __set_BASEPRI(old);
    __DSB(); __ISB();
}

Non‑cacheable DMA buffers

AT_NONCACHEABLE_SECTION_ALIGN(static uint8_t s_dmaRing[2048], 16);

Relocate vector table to ITCM

extern uint32_t __isr_vector_start__;
SCB->VTOR = ((uint32_t)&__isr_vector_start__) & SCB_VTOR_TBLOFF_Msk;
__DSB(); __ISB();

## 13) Closing thought

Four choices usually get you to avionics‑friendly determinism on i.MX RT1050:

- place vectors and critical ISRs in ITCM with data in DTCM,

- use BASEPRI instead of global masking,

- keep ISRs tiny and defer work, and

- handle heavy I/O with EDMA into non‑cacheable rings.

The labs above, built on your EVKB SDK, let you measure and prove both latency and jitter budgets for DO‑178C evidence.